3GPP TSG-RAN WG2 Meeting #129bis	R2-2502810

Wuhan, China, 7th -11th April, 2025

Agenda Item:	7.0.2.13

Source: 	ASUSTeK

Title:  	Discussion on supporting 8Tx

Document for:	Discussion and Decision

- Introduction

In RAN2#128 online MIMO session, modeling methods for supporting 8Tx in MAC specification is discussed and the following modeling assumption was made:

- ?? For 8Tx 2TB transmission, we assume the following from MAC point of view: two uplink grants for two HARQ processes, one HARQ process ID, 2 HARQ buffer. Detailed changes to MAC will be discussed in offline.

An LS R2-2411220 [1] was sent to check RAN1’s view on the above model assumption and 2-TB generation for 8Tx:

1. To support UL 8Tx in the MAC specification, RAN2 made the following assumption regarding how to model UL 8Tx operation from MAC point of view to minimize specification impacts. For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.. 2. For UL 8Tx 2TB transmission scheduled by one DCI, if MAC entity only generates a single TB due to the uplink skipping, would there be any issue for PHY layer?

In the previous meeting, reply from RAN1 in LS R1-2501636 [2] was as follows:

RAN1 thanks RAN2 for the LS on UL 8Tx, RAN 1 made the following agreements related to the Questions from RAN 2: Agreement - Proposal 1: Regarding Item 1 of the incoming LS R1-2500012, RAN1 is fine with RAN2’s assumption, with the understanding that it is RAN2 internal assumption to implement RAN2 specifications on UL 8Tx. There is no RAN1 specification change needed based on the RAN2 assumption. - Proposal 2: Regarding Item 2 of the incoming LS R1-2500012, for UL 8Tx 2TB transmission scheduled by one DCI, for the case where MAC entity only generates a single TB due to the uplink skipping, this case was not considered in previous RAN1 meetings and the current RAN1 specifications do not define UE behaviours to support this case..

In this contribution, we discuss MAC specification impacts to support 8Tx based on RAN1’s response and past meeting discussions.

- Discussion

2.1 Confirmation on modelling 2 assumption

According to the reply LS, RAN1 is fine with RAN2’s model 2 assumption (i.e., two uplink grants scheduled by one DCI for two HARQ processes, one HARQ process ID, and two HARQ buffers). Therefore, we first propose to confirm the assumption of using the model 2 for supporting 8Tx in MAC.

Proposal 1:	Confirm the assumption that for UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

## 2.2 MAC changes for modelling method #2

In summary of discussion [AT128][202], R2-2410952 [3], impacts and proposed TPs on MAC spec were discussed. In this discussion paper, we list and discuss each of the proposed changes below:

    - 2.2.1 UL grant reception and number of HARQ processes for 2 TBs

In order to support modeling method in 8Tx, corresponding description should be captured in UL grant reception and the previous change on one HARQ process supporting two TBs should be reverted.

### 5.4.1 UL Grant reception Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. For uplink spatial multiplexing, the MAC layer can receive up to two uplink grants (one per HARQ process) on the PDCCH that schedules two TBs as specified in TS 38.212 [9]. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant. … #### 5.4.2.1 HARQ Entity … Each HARQ process supports one TB.

Proposal 2:	In 5.4.1, add in description that MAC layer can receive up to two uplink grants for two TBs, and in 5.4.2.1, revise the description so that each HARQ process supports one TB.

    - 2.2.2 HARQ process ID for the HARQ processes

According to the model, only one HARQ process id will be used for 8Tx 2 TB transmission. Corresponding changes of the description is captured as below:

#### 5.4.2.1 HARQ Entity … For uplink spatial multiplexing, two associated HARQ processes are associated with one HARQ process identifier. Otherwise, each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response or for UL transmission for MSGA payload, HARQ process identifier 0 is used.

Proposal 3:	In 5.4.2.1, add a description so that the two HARQ processes for 8Tx transmission shares a same HARQ process id.

    - 2.2.3 Intra-UE multiplexing

As proposed in R2-2411076 [4] by ZTE during RAN2#128, extra handling for the two UL grants would be needed for intra-UE multiplexing in order to avoid unnecessary prioritization and de-prioritization between the two TBs (e.g., the two TBs should be prioritized or deprioritized together and are not considered as conflict with each other). After some offline discussion, a Note is added as below:

### 5.4.1 UL Grant reception … NOTE x: For uplink spatial multiplexing, for the MAC entity configured with lch-basedPrioritization, the MAC entity considers the two uplink grants received on the PDCCH that schedules two TBs are prioritized or deprioritized together and the priority is determined by the highest priority among priorities of the logical channels that are multiplexed or have data available that can be multiplexed in the two MAC PDUs, according to the mapping restrictions as described in clause 5.4.3.1.2.

Proposal 4:	In 5.4.1, add a NOTE regarding handling of the two TBs for intra-UE multiplexing.

    - 2.2.4 Ensuring 2-TB generation

As indicated in reply LS from RAN1 [2], RAN1 does not support the case where only a single TB is generated for a UL 8Tx 2TB transmission due to uplink skipping:

Proposal 2: Regarding Item 2 of the incoming LS R1-2500012, for UL 8Tx 2TB transmission scheduled by one DCI, for the case where MAC entity only generates a single TB due to the uplink skipping, this case was not considered in previous RAN1 meetings and the current RAN1 specifications do not define UE behaviours to support this case.

That is, the MAC entity should provide the PHY layer with either both or no TBs when UL 8Tx 2 TB transmission is scheduled. To ensure the MAC entity provides two TBs to PHY layer (i.e., 2 new MAC PDUs, or a new MAC PDU and a MAC PDU for retransmission), a proposed change in 5.4.3.1.3 is as below:

##### 5.4.3.1.3 Allocation of resources … NOTE 1a: For uplink spatial multiplexing as specified in clause 5.4.1, if at least one MAC PDU is to be generated or to be retransmitted for a PDCCH that schedules two TBs, the MAC entity generates MAC PDU(s) corresponding to all UL grants indicated by the PDCCH.

Proposal 5:	In 5.4.3.1.3, add a NOTE regarding ensuring 2-TB generation for 8Tx UL grant.

A CR [5] is provided with the above proposed changes and can be used as a baseline for supporting UL 8Tx transmission in MAC specification.

Proposal 6:	Agree the changes in CR R2-2502835 for supporting 8Tx in MAC specification.

## 2.2 Stage-2 change for modelling method #2

In current TS 38.300 [6], it is indicated that for downlink/uplink spatial multiplexing, a single HARQ process supports one or multiple TBs. Based on the current model and RAN1 input, we propose to change the stage-2 description above so that two HARQ processes support the multiple TBs for UL spatial multiplexing:

### 6.2.4 HARQ The HARQ functionality ensures delivery between peer entities at Layer 1. A single HARQ process supports one TB when the physical layer is not configured for downlink/uplink spatial multiplexing. When the physical layer is configured for downlink spatial multiplexing, a single DL HARQ process supports one or multiple TBs, and when the physical layer is configured for uplink spatial multiplexing, a single UL HARQ process supports one TB.

Proposal 7:	Agree the stage-2 changes in CR R2-2502812 to support 2 HARQ processes for 2TB UL transmissions.

- Conclusion

We have the following proposals for 8Tx in MAC specification and stage 2 specification:

Proposal 1:	Confirm the assumption that for UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

Proposal 2:	In 5.4.1, add in description that MAC layer can receive up to two uplink grants for two TBs, and in 5.4.2.1, revise the description so that each HARQ process supports one TB.

Proposal 3:	In 5.4.2.1, add a description so that the two HARQ processes for 8Tx transmission shares a same HARQ process id.

Proposal 4:	In 5.4.1, add a NOTE regarding handling of the two TBs for intra-UE multiplexing.

Proposal 5:	In 5.4.3.1.3, add a NOTE regarding ensuring 2-TB generation for 8Tx UL grant.

Proposal 6:	Agree the changes in CR R2-2502835 for supporting 8Tx in MAC specification.

Proposal 7:	Agree the stage-2 changes in CR R2-2502812 to support 2 HARQ processes for 2TB UL transmissions.

- Reference

[1] R2-2411220, LS on UL 8Tx, Samsung

[2] R1-2501636, Reply LS on UL 8Tx, Samsung

[3] R2-2410952, Report of [AT127bis][202][MIMOevo] Discuss on the modelling and review the MAC CR for 8Tx 2TB if the modelling is agreeable (ASUSTeK, ZTE)

[4] R2-2411076, Harmonization of 8Tx in MAC specification, ZTE Corporation

[5] R2-2502835, Correction on supporting 8Tx in MAC specification, ASUSTeK

[6] 3GPP TS 38.300 v 18.4.0