3GPP TSG-RAN WG2 Meeting #129bis	R2-2502812

Wuhan, China, 7th -11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0987 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage-2 Correction on UL 8Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The 8Tx UL has been introduced by RAN1 in release 18 MIMO, and RAN2 made the modeling assumption (method #2) in RAN2 #128 meeting: - For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers. Based on LS (R1-2501636), RAN1 is fine with above assumption. However, in the current 38.300 specification, it is indicated that a single HARQ process supports one or multiple TBs for UL spatial multiplexing, which is not aligned with the model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the description in 6.2.4 so that when the physical layer is configured for uplink spatial multiplexing, a single UL HARQ process supports one TB. Impact analysis Impacted functionality: UL HARQ operation. Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no inter-operability issue. - If the network is implemented according to this CR but the UE is not, the UE may not perform correct new transmsision or retransmission for multiple TB transmissions in uplink spatial multiplexing. Impacted 5G architecture options: NR SA, NE-DC and NR-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot perform uplink spatial multiplexing transmissions properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### Start of change

6.2.4	HARQ

The HARQ functionality ensures delivery between peer entities at Layer 1. A single HARQ process supports one TB when the physical layer is not configured for downlink/uplink spatial multiplexing. When the physical layer is configured for downlink spatial multiplexing, a single DL HARQ process supports one or multiple TBs, and when the physical layer is configured for uplink spatial multiplexing, a single UL HARQ process supports one TB.

##### End of change