3GPP TSG-RAN WG2 Meeting #129bis		       R2-2502816

Wuhan, China: 7th April – 11th April 2025

Agenda Item:	8.1.2.2

Source:	ASUSTeK

Title:	Discussion on LCM for UE-sided models

Document for:	Discussion and Decision

- Introduction

The WI NR_AIML_air agreed to work on the signalling mechanism of applicable functionalities /models. In the previous meeting, RAN2 had lots of discussion on the options A and B provided by RAN1. Generally, RAN2 agreed option A is based on inference configuration, whereas the design of inference configuration is still FFS. For option B, RAN2 still has some aspects to clarify. We would like to address some of the issues in this document.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models

- Discussion

In the LS to RAN2 (R1-2410898), RAN1 provided some information regarding the terminologies, what may be the content provided in each step of the applicability reporting procedure, and provided 2 potential directions A and B.

From RAN1 LS response:

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

Agreements (RAN2 #129) 1. Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). 2. The full inference configuration is sent in CSI-ReportConfig. 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. 4. FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

Agreements applicability reporting and management (RAN2 #129)  Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause  If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).  The provided periodic CSI configuration should be consistent with reported UE capabilities  FFS option B  Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: Semi-persistent reporting, activated by MAC CE/DCI Aperiodic CSI reporting, activated by DCI

During RAN2 #129, RAN2 agreed the general procedure of option A, i.e., inference configuration(s) are provided in step 3, UE reports the applicability/inapplicability based on the received inference configuration(s) and starts performing inference based on the configuration(s). However, a remaining issue is that, what happens to the inference configuration(s) that are determined inapplicable by the UE?

- Option 1: The UE discards the inapplicable configuration(s) (e.g., automatically release).

- Option 2: The UE keeps the inapplicable configuration(s). The UE may train or obtain an AI/ML model afterwards to be applicable for the configuration(s). The NW may also release the inapplicable configuration(s) based on NW implementation.

Proposal 1: For option A, RAN2 discuss whether the inapplicable configurations provided in step 3 are discarded or kept as deactivated (e.g., for future applicability change or training).

For option B, it is unclear of what is option B. We think a first step is to align the companies understanding on what content is included in a configuration for option B. In our opinion, a configuration for option B may be one of the options:

- Option 1: option B is subset of the full inference configuration. FFS what is included in the inference configuration.

- Option 2: option B is assistance information for the NW to determine the applicability of an inference configuration (i.e., option B may include additional information not included in the full inference configuration, for example, multiple candidate sets of RS resources).

Proposal 2: For option B, RAN2 discuss what is the content of option B.

Proposal 3: If option B is subset of the full inference configuration, whether a configuration for option B is associated with the full inference configuration (e.g., reporting applicable for a configuration for option B equals reporting applicable for the associated full inference configuration).

For the report of the applicability/inapplicability in step 4, there may be different possibilities based on the design of option A and B. In our opinion, to achieve a unified structure for potential AI/ML functionalities, the applicability/inapplicability report can be decoupled from the inference configuration (e.g., option A), as different functionalities may utilize different framework (e.g., CSI framework, L3 measurement framework) for inference. We suggest to report the applicability/inapplicability based on otherConfig (instead of inference configuration/framework). For example, an ID may be used to indicate a subset of the full inference configuration related to AI/ML (e.g., configuration for option B). The ID may be included in the full inference configuration. The ID is used to report the applicability/inapplicability of configurations.

Proposal 4: The reporting of applicability/inapplicability is based on otherConfig instead of inference configuration/framework.

To further simplify the complexity of supporting both option A and B, we think the two options can be used in different messages and not together, reducing the ambiguity of the content of applicability/inapplicability report (e.g., which configurations to report).

Proposal 5: An RRCReconfiguration message is either for option A or option B.

- Conclusion

Proposal 1: For option A, RAN2 discuss whether the inapplicable configurations provided in step 3 are discarded or kept as deactivated (e.g., for future applicability change or training).

Proposal 2: For option B, RAN2 discuss what is the content of option B.

Proposal 3: If option B is subset of the full inference configuration, whether a configuration for option B is associated with the full inference configuration (e.g., reporting applicable for a configuration for option B equals reporting applicable for the associated full inference configuration).

Proposal 4: The reporting of applicability/inapplicability is based on otherConfig instead of inference configuration/framework.

Proposal 5: An RRCReconfiguration message is either for option A or option B.

References

[1] RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

[3] Chair Notes for RAN2 #129

Annex