3GPP TSG-RAN WG2 Meeting #129bis                                                      R2-2502832

Wuhan, China, April 7th – 11th, 2025

Source:	TCL

Title:	Discussion on NW side data collection for positioning

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

Data collection is a very important function in the AI/ML-based LCM, which is used to provide the input data to the different LCM aspects (i.e., model inference, monitoring, and training). In the recent RAN2#129 meeting, there were full discussions on logging events, availability indication, content of collected data, sending of large data and handling of configuration/data during mobility/state transition. Some conclusions were drawn as follows [1].

Agreements - Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log - Low power bit indication is supported - Data availability indication is supported. FFS when this would be triggered - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. - UE retains logged data during handover (HO). FFS if there are scenarios where the UE needs to release the data and how does the UE know and if control from network is needed - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - FFS how to handle idle/inactive and RLF cases and whether we have a unified.

In this contribution, we provide our views on NW side data collection for the positioning use case, focusing on the first priority Case 3a and Case 3b.

- Discussion

2.1 NW side data collection for Case 3a

The training data can be autonomously collected by gNB or OAM for Case 3a. As for gNB-centric data collection, SRS is configured reusing the legacy RRC message for measurement data collection. The label data (intermediate positioning measurement) can be calculated based on the UE location information, which can be contained in the RRM measurement report associated with the corresponding SRS configuration for data collection to match the measurement data and label data. The SRS configuration can be contained in the RRM measurement report configuration or associated to the RRM measurement report configuration via a measurement ID.

Proposal 1: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

As for OAM-centric data collection, the Immediate MDT with RAN measurement configuration can be extended. The channel measurement (e.g., DP, PDP, CIR) by RAN should be additionally supported for Immediate MDT so that OAM can collect the measurement data. The content of channel measurement waits for RAN1 discussion. The measurement collection can be triggered by event or at the end of measurement collection period, and the trigger events can be further discussed. Besides, the UE location information provision is supported in the legacy Immediate MDT so there should be no extension for the label data collection.

Proposal 2: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a. The content of channel measurement waits for RAN1 discussion.

2.2 NW side data collection for Case 3b

The label data can be generated by PRU, non-PRU UE and LMF for Case 3b, which is captured as follows [2].

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

If the label data is generated by PRU and LMF, LMF can easily match the measurement data and label data. However, the non-PRU UE may generate the label data via PRS. In this case, the UE may move within the time gap between the PRS reception and SRS transmission, which leads to the mismatch between the measurement data and label data. Hence, the data association should be considered to guarantee the reliability of model training. Intuitively, the side information can be transmitted along with the time stamp of label data. One way is to transmit the record time of the time stamp of label data and also transmit the validity duration of label data as side information. For example, the time stamp of label data is recorded when the UE location changes. If the label data is the old location, the validity duration indicates the duration in which the old location maintains before the location changes. Otherwise, the validity duration indicates the duration in which the new location maintains after the location changes. The information for data association waits for RAN1 discussion, which is captured as follows[3].

Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

Proposal 3: The data association procedure should be considered for Case 3b and the information for data association waits for RAN1 discussion.

- Conclusion

We have the following proposals:

Proposal 1: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

Proposal 2: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a. The content of channel measurement waits for RAN1 discussion.

Proposal 3: The data association procedure should be considered for Case 3b and the information for data association waits for RAN1 discussion.

- References

- RAN2 #129 Chair Notes

- RAN1 #117 Chair Notes

- RAN1 #120 Chair Notes