3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502833

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	 8.12.3

Source:	 Huawei, HiSilicon

Title:	 Enhancements for UE-initiated/event-driven beam management

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, according to the WID as shown below and recent working assumption made by RAN2, objectives relevant to UE-initiated/event-driven beam management (UEIBM) are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure, which are outlined in the WID.

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting.

In this contribution, we present detailed analysis and provide our considerations on how RAN2 can start working on UEIBM, based on RAN1 agreements so far.

2	UE Initiated/Event-driven Beam Management

In order to reduce the periodic CSI reporting overhead and latency, event-based CSI reporting was agreed by RAN1 for Rel-19 MIMO WI i.e., the UE reports CSI only when the event is triggered. This includes specification of the event triggering determination procedure and of the reporting procedure after the event is triggered. In the following, we provide our considerations on the trigger event for event-driven beam management, beam reporting and UL transmission.

According to the recent RAN2 meeting, RAN2 has made the working assumption that the event evaluation for UE-initiated beam reporting is handled at the MAC layer. We then proceed to consider how this can be effectively modelled within the MAC specifications.

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision. No need to send a LS to R1 on this.

2.1	Event evaluation

2.1.1	Application of Time window length and M

The following agreement was made for event triggering in RAN1:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

According to the RAN1 agreements, an event-2 instance occurs when the L1-RSRP of the new beam is a threshold value greater than the current beam. This threshold is configured by the network. Then, there are two cases:

- Case-1: upon occurrence of an event-2 instance, the UE initiates the reporting procedure.

- Case-2 (subject to a UE capability): when M event-2 instances for the same beam occur within a time window, the UE initiates the reporting procedure.

Based on the RAN1 agreements of Case-2, RRC configures the threshold, time window length and M. According to the RAN1 agreements, Event-1 and Event-7 are both supported for the UE-initiated/event-driven beam reporting. It is likely that this event evaluation procedure can also be applied to Event-1 and Event-7.

Observation 1: RRC should configure a threshold, time window length and M for the event-triggering determination procedure of the UEIBM.

Agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

UE should maintain a timer and counter to operate during the event triggering determination procedure. According to the recent RAN1 agreement, the event instance(s) counting(counter) is per new beam for Event-2 and Event-7. Therefore, the timer should also be maintained for each new beam. In contrast, for Event-1, there is only a single timer and counter operating within the event-triggering determination procedure.

Proposal 1: For the Event-1 triggering determination procedure, the UE maintains a counter and a timer for current beam. For the Event-2 and Event-7 triggering determination procedure, the UE maintains a counter and a timer for each new beam.

Currently, BF and RLF are declared upon receiving a certain number of instances of an event from lower layers, which is similar to the RAN1 agreement for Case-2 above. However, these procedures have some differences:

- In the BF detection procedure, the timer is (re)started upon the reception of an instance, and the counter is incremented. Upon timer expiry, the UE resets the counter to 0. When the counter reaches the configured number of event instances, the UE considers beam failure to be detected.

- In the RLF procedure, the N310 “out-of-sync” instances need to be consecutive, then a timer is started, which can by stopped if N311 consecutive “in-sync” instances are received. At the point of timer expiration, RLF is declared.

The RAN1 agreement does not consider consecutive instances, but a number of instances in a time window. A time window of e.g., 300ms with 3 instances, requires recording the time of up to 2 instances, then when a third one occurs, if the first one occurred less than 300ms ago, the event is triggered, otherwise the first instance is discarded. If the number of instances to trigger an event is more like 30, the UE implementation impact is significantly higher.

Even though it does not strictly match with the RAN1 agreement, an alternative method could be like the method used for BF i.e., the UE uses a single timer for an event and at every instance, the timer is (re)started and the counter is reset if the timer expires. If the timer is 10ms and instances are occurring more frequently than every 10ms, the effect is similar to a 300ms window with 30 instances.

As the number of event instances is not decided yet, RAN2 can send a LS to RAN1, asking for the expected range for the count of such instances. Then, whether the BFD approach can be determined.

Proposal 2: For each event, at each even instance, the timer is (re)started and the counter is incremented. If the timer expires, the counter is reset to 0 (like for BF detection). Besides, RAN2 can ask RAN1 the expected range for the count of the event instances in a LS.

Additionally, the text proposal is also shown in Annex.

2.2	Specification aspects

2.2.1	MAC Layer

It is assumed that the event evaluation procedure for UE-initiated beam reporting is performed at the MAC layer. Subsequently, we need to discuss how this procedure can be modelled. Two potential approaches for modelling event evaluation can be considered:

-	Approach 1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

-	Approach 2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

For approach 1, if the comparison occurs in the PHY layer, the PHY layer sends the instance to the higher layers. Alternatively, for approach 2, if conducted in the higher layers, the PHY layer sends the beam quality metrics, such as L1-RSRP, to the higher layers, allowing them to perform the comparison based on beam quality. Both methods are feasible. In the Rel-19 event-triggered LTM, approach 1 has been selected. RAN2 may consider applying a similar mechanism in the event evaluation procedure for UERIM. The final decision could up to RAN1.

Proposal 3: It’s up to RAN1 to discuss which approach is used. Following this, a LS should be sent to inform RAN1.

- Approach 1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Approach 2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

2.2.1	RRC configurations

The parameters described above are part of the recent RAN1 Liaison Statement to RAN2 (R1-2410758), which focuses on the RRC impacts for Rel-19 MIMO UEIBM. Below is a specific description of the new parameters:

- The parameter newBeamResourceSetEvent2-r19 defines the new beam resource set for Event-2.

- The parameter eventType-r19 indicates the event type used for UEI beam reporting.

- The parameter valueOfQ-r19 is used to specify the value of Q for Event-7.

- The parameter configuredResourceForSecondChannelOfModeB-r19 informs Type-1 CG PUSCH resource for second channel in mode-B.

- The parameter eventThresholdEvent1-r19 sets a threshold value for trigger event detection regarding Event-1.

- The parameter firstPUCCHResourceConfig-UEIBR-r19 configures the periodic PUCCH resource for first PUCCH for both mode-A and mode-B.

Which options is preferred – add currently defined UEIBM parameters in CSI-ReportConfig or in CSI-MeasConfig? In case of CSI-ReportConfig, the legacy reportConfigType can be clarified in the field description to be ignored by a UE receiving configuration for UEIBM. It should also be noted that the running CR for now is based on the RAN1 input and thus includes the parameters in CSI-ReportConfig, but it can be updated depending on the conclusion of the discussion.

For Rel-19 MIMO, event-triggered reporting is introduced, along with corresponding configurations. The parameters above are proposed to be under the IE CSI-ReportConfig. It is up to RAN2 to decide how to capture these parameters in RRC. We prefer to follow the legacy structure to add UEIBM in CSI-ReportConfig and have further clarifications in FD.

Proposal 4: UEIBM parameters are configured in CSI-ReportConfig.

2.3	Beam reporting and UL transmission

For MIMO, the UE sends the L1 report to the network using UCI. Two modes (Mode-A and Mode-B) have been agreed by RAN1 for the beam reporting procedure.

•	Mode-A: beam report is transmitted in a UCI in dynamically allocated UL resource. One advantage of Mode-A is that the scheduling flexibility, which can help to avoid collision issue on the second channels. Although the minor drawback is that the dynamic resource allocation of step-2 in Mode-A could introduce some latency, the allocated resources are really used. Therefore, the Mode-A was agreed by RAN1 as a fundamental feature.

•	Mode-B: beam report is transmitted in a UCI in pre-configured UL resources. This mode significantly reduces the complexity for the network as the network does not need to monitor whether UEI beam report is carried in the pre-configured resource at all times. Additionally, Mode-B offers better resource efficiency, as the pre-configured resources can be shared among UEs.

2.3.1	First UL channel Transmission

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type. - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

RAN1 has decided that the first PUCCH channel is a new UCI type for both Mode-A and Mode-B, excluding the option of using the first PUCCH as a dedicated SR. This channel is associated with a periodic PUCCH resource configuration but is not linked to the SchedulingRequestId. The first PUCCH channel transmission can be notified from PHY, which is similar to PRACH, as we described in section 2.2.1, and the details can be further discussed based on further RAN1 discussion.

Observation 2: The first PUCCH channel configuration is a new UCI type with RRC parameter firstPUCCHResourceConfig-UEIBR for both mode-A and mode-B. This configuration does not serve as a Scheduling Request.

2.3.2	Second UL channel Transmission

2.3.2.1	DRX Impact on UEIBM

The UE may be configured with DRX (Discontinuous Reception). DRX operation determines whether the MAC entity monitors the PDCCH or not. When DRX is configured, the UE does not need to continuously monitor the PDCCH, it only needs to wake up during the DRX active time to monitor the PDCCH.

In mode-A beam reporting procedure of UEIBM, the UE transmits the first PUCCH to request resources for PUSCH. Subsequently, the network sends the response via the PDCCH, which indicates the allocated PUSCH resources for beam reporting.

TS 38.321 clause 5.7 When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: …… - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

Similar to the SR procedure, if DRX is configured, the UE must remain awake after transmitting the first PUCCH, until the DL scheduling information for resource allocation is received.

In mode-B beam reporting procedure of UEIBM, the second UL channel transmission is CG PUSCH, which is not subject to PDCCH monitoring, and thus we think there is no impact.

Proposal 5: In mode-A of UEIBM, the UE should remain in DRX active time to receive DCI for the second UL transmission. In mode-B of UEIBM, the second UL transmission is not subject to DRX active time.

2.3.2.2	Deactivation of SCell(s) Impact on UEIBM

Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group. - (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability

Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification. - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability - Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signalling.

According to the RAN1 agreement, UE-initiated or event-driven beam reporting can also be extended to the cross-CC beam reporting scenario. In this context, a CSI reporting configuration for CC#A is associated with a measurement resource configuration for CC#B. This cross-CC RS measurement framework has been introduced to enable the reporting of measurement results for cells operating in FR2 via cells operating in lower frequency bands. Since the current NR standards already support intra-CC RS measurement, this framework can be reused for R19 UEIBM.

Observation 3: UEIBM supports cross-CC beam reporting scenarios, enabling a CSI reporting configuration in CC#A to be linked with a measurement resource configuration in CC#B.

In a Carrier Aggregation scenario, when the UE is configured with one or more SCells, the network has the flexibility to activate or deactivate these SCells as needed. When an SCell is deactivated, the UE is no longer required to receive PDCCH or PDSCH on the SCell and cannot transmit on the associated uplink.

Observation 4: For legacy active, periodic, semi-persistent, and aperiodic CSI reporting, there is no need to transmit a measurement report for a deactivated SCell.

Consequently, in the UEIBM procedure, there is also unnecessary to transmit a beam report for a deactivated SCell. In the context of cross-CC RS measurement, if CC#B (configured with a measurement resource) is deactivated while CC#A (configured with a CSI reporting configuration) remains active, periodic, semi-persistent, and aperiodic CSI reporting are not performed. Further discussion is required on how the UEI beam reporting procedure should be applied in such scenarios.

TS 38.321-i40 clause 5.9 1> else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or 1> if the sCellDeactivationTimer associated with the activated SCell expires; or 1> if the SCG associated with the activated SCell is deactivated: 2> deactivate the SCell according to the timing defined in TS 38.213 [6]; 2> stop the sCellDeactivationTimer associated with the SCell; 2> stop the bwp-InactivityTimer associated with the SCell; 2> deactivate any active BWP associated with the SCell; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively; 2> clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell; 2> suspend any configured uplink grant Type 1 associated with the SCell; 2> flush all HARQ buffers associated with the SCell; 2> cancel, if any, triggered consistent LBT failure for the SCell.

In mode-B of UEI beam reporting, a configured CG PUSCH resource is used for the second uplink transmission. When the reported cell is deactivated in a same-CC beam reporting scenario, the corresponding CG PUSCH resource is cleared. In the cross-CC RS reporting scenario, if CC#B is deactivated, the corresponding CG PUSCH resource in CC#A allocated for UEI beam reporting should also be cleared to maintain consistency. The change to the specification, as depicted in the text proposal, is also outlined in the Annex.

Proposal 6: For mode-B of UEIBM, in the cross-CC beam reporting transmission procedure, if CC#B is deactivated, the UE should release any PUSCH resources allocated for UEIBM of CC#B in CC#A. Additionally, the text proposal is provided in the Annex.

2.3.2.2	Dormant BWP Impact on UEIBM

Additionally, to facilitate fast SCell activation in CA configurations, a dormant BWP can be defined for an SCell. If the BWP of an activated SCell is a dormant BWP, the UE stops monitoring the PDCCH and stops transmitting SRS, PUSCH, and PUCCH on the SCell, but continues performing CSI measurements. The legacy Rel-15 CSI reporting procedure is limited to periodic, semi-persistent, and aperiodic beam measurements. Therefore, the impact of a dormant BWP on the UEIBM procedure also requires further discussion.

TS 38.300-i40 clause 10.6 10.6 Activation/Deactivation Mechanism To enable fast SCell activation when CA is configured, one dormant BWP can be configured for an SCell. If the active BWP of the activated SCell is a dormant BWP, the UE stops monitoring PDCCH and transmitting SRS/PUSCH/PUCCH on the SCell but continues performing CSI measurements, AGC and beam management, if configured. A DCI is used to control entering/leaving the dormant BWP for one or more SCell(s) or one or more SCell group(s). The dormant BWP is one of the UE's dedicated BWPs configured by network via dedicated RRC signalling. The SpCell and PUCCH SCell cannot be configured with a dormant BWP.

Due to the definition of the dormant BWP, we suggest that for Rel-19, CSI measurements for UE-initiated/event-triggered event reporting continues to be performed in the dormant BWP, in alignment with the previous specification.

Proposal 7: The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP.

TS 38.321-i40 clause 5.15.1 1> if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP: 2> stop the bwp-InactivityTimer of this Serving Cell, if running. 2> not monitor the PDCCH on the BWP; 2> not monitor the PDCCH for the BWP; 2> not receive DL-SCH on the BWP; 2> not report CSI on the BWP, report CSI except aperiodic CSI for the BWP; 2> not transmit SRS on the BWP; 2> not transmit on UL-SCH on the BWP; 2> not transmit on RACH on the BWP; 2> not transmit PUCCH on the BWP; 2> clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively; 2> suspend any configured uplink grant Type 1 associated with the SCell; 2> if configured, perform beam failure detection and beam failure recovery for the SCell if beam failure is detected; 2> if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured: 3> not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

To reduce power consumption in dormant mode, the UE will not monitor the PDCCH on the dormant BWP. Consequently, there will be no dynamic grant for CSI reporting on this BWP. In this case, the UE will only perform periodic or semi-persistent reporting for the dormant BWP and will not report aperiodic CSI. If aperiodic CSI reporting is supported, the UE has to wait the triggering of aperiodic CSI at any time and be ready for measuring the CSI RS. This prevent the UE from going to deep sleep. The mode-A beam reporting in UEIBM is similar to aperiodic CSI reporting, so the same rule should apply. In contrast, mode-B beam reporting is similar to periodic reporting, so it can continue as usual. The change to the specification is also outlined in the Annex.

Proposal 8: If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

# 3	Conclusion

In this paper, we have addressed the objective regarding initial analysis on RAN2 impact for MIMO. There are the following observations and proposals, and the draft LS to RAN1 and text proposal is attached in Annex.

Event evaluation

Observation 1: RRC should configure a threshold, time window length and M for the event-triggering determination procedure of the UEIBM.

Proposal 1: For the Event-1 triggering determination procedure, the UE maintains a counter and a timer for current beam. For the Event-2 and Event-7 triggering determination procedure, the UE maintains a counter and a timer for each new beam.

Proposal 2: For each event, at each even instance, the timer is (re)started and the counter is incremented. If the timer expires, the counter is reset to 0 (like for BF detection). Besides, RAN2 can ask RAN1 the expected range for the count of the event instances in a LS.

Specification aspects

Proposal 3: It’s up to RAN1 to discuss which approach is used. Following this, a LS should be sent to inform RAN1.

- Approach 1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Approach 2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to upper layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

Proposal 4: UEIBM parameters are configured in CSI-ReportConfig.

Beam reporting and UL transmission

Observation 2: The first PUCCH channel configuration is a new UCI type with RRC parameter firstPUCCHResourceConfig-UEIBR for both mode-A and mode-B. This configuration does not serve as a Scheduling Request.

Proposal 5: In mode-A of UEIBM, the UE should remain in DRX active time to receive DCI for the second UL transmission. In mode-B of UEIBM, the second UL transmission is not subject to DRX active time.

Observation3: UEIBM supports cross-CC beam reporting scenarios, enabling a CSI reporting configuration in CC#A to be linked with a measurement resource configuration in CC#B.

Observation4: For legacy active, periodic, semi-persistent, and aperiodic CSI reporting, there is no need to transmit a measurement report for a deactivated SCell.

Proposal 6: For mode-B of UEIBM, in the cross-CC beam reporting transmission procedure, if CC#B is deactivated, the UE should release any PUSCH resources allocated for UEIBM of CC#B in CC#A. Additionally, the text proposal is provided in the Annex.

Proposal 7: The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP.

Proposal 8: If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

# 4	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- RAN1 #119 chairman notes;

- TS 3GPP 38.300 V18.4.0;

- TS 3GPP 38.321 V18.4.0.

# 5	Annex – LS

3GPP TSG-RAN WG2 Meeting #129-bis	R2-25xxxxx

Wuhan, China, 7th – 11th April, 2025

Title:	(draft) LS on Rel-19 UEIBM

Response to:	-

Release:	Rel-19

Work Items:	NR_MIMO_Ph5-Core

Source:	RAN WG2, Huawei, HiSilicon

To:	RAN WG1

CC:	-

Contact Person:

Name:					TBD

E-mail Address:		TBD

Attachment:	               -

1. Overall Description:

RAN2 would like to ask the following question:

1. RAN2 discussed two options on how to model the specification on UEIBM and would like to ask RAN1.

Question 1: which option is used for event triggering determination.

- Option-1: L1 provides the RSRP to MAC layer, and MAC layer check whether condition is met based on the threshold, timer and counter.

- Option-2: The L1 checks whether RSRP condition for an event instance is met based on the threshold and sends an event instance to MAC layer if condition is met. MAC checks whether event is met based on counter and timer for event instance.

2. Actions:

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to consider above information for Rel-19 MIMO and to provide answers to the above questions.

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG2 Meeting #130				May 19 to 23, 2025				Malta, MT

TSG-RAN WG2 Meeting #131 				Aug 25 to 29, 2025				India (TBC), IN

# 6	Annex – TP

## 5.X	 	UE Initiated/Event-driven Beam Management

The MAC entity may be configured by RRC per CSI-ResourceConfig with a UE Initiated/Event-driven Beam Management procedure, to indicate to the network whether a beam (SSB/CSI-RS) satisfies the corresponding events.

RRC configures the following parameters in the CSI-ReportConfig:

-	EventInstanceMaxCount for the UEIBM;

-	EventDetectionTimer for the UEIBM;

The following UE variables are used:

-	UEIBM_COUNTER (per beam in the associated CSI-ResourceConfig): counter for event instance indication which is initially set to 0.

The MAC entity shall, for each beam configured for UE Initiated/Event-driven Beam Management:

1>	if the CSI-ReportConfig is set to eventType and eventType is set to Event1, or Event2, or Event7:

2>	if an event instance indication for a beam in the associated CSI-ResourceConfig has been received from lower layers:

3>	start or restart the EventDetectionTimer of the new beam;

3>	increment UEIBM_COUNTER of the beam by 1;

3>	if UEIBM_COUNTER of the beam >= EventInstanceMaxCount:

4>	inform the lower layer to trigger a UEI beam reporting procedure for this CSI-ReportConfig;

1>	if the EventDetectionTimer expires; or

1>	if EventDetectionTimer, EventInstanceMaxCount, or any of the reference signals used for UEIBM is reconfigured by upper layers:

2>	set UEIBM_COUNTER to 0 for each associated beam.

1>	if lower layers indicate that the UEI beam report is successfully sent:

2>	set UEIBM_COUNTER to 0 for each associated beam;

2>	stop the EventDetectionTimer, if configured;