3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502834

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	 8.12.2

Source:	 Huawei, HiSilicon

Title:	 Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, objectives relevant to asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts. Progress on the MAC CE definition approach for this topic has been made in RAN1 and RAN2 meetings. In the last meeting, RAN2 agreements were as follows:

Agreement - One PL offset value is indicated for each TCI state included in the new MAC CE. - The new MAC CE contains one serving cell ID and one BWP ID - TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) - The new MAC CE can include flexible number of PL offset values.

In this contribution, we present detailed analysis on RAN2 work on the remaining issues on asymmetric DL sTRP/UL mTRP based on RAN1 and RAN2 agreements so far, and provide our considerations.

2	Discussion

2.1	MAC CE Format

2.1.1	New MAC CE for PL offset update

In pre-Rel-19 mTRP scenarios, each TRP can transmit DL RS and the UE can measure the path loss directly based on each DL RS from the TRP. In Rel-19, a new scenario is considered, in which one TRP does not transmit any DL RS. For the TRP with DL RS transmission, the UE calculates the pathloss (PL) based on the DL RS. However, for the UL-only TRP without DL RS transmission, a new mechanism is needed e.g., the pathloss is calculated using the PL to the TRP with DL transmission and a PL offset is applied. RAN1 agreed that different PL offset values can be configured for different TCI states of a TRP, and RAN2 agreed to use a new MAC CE for PL offset update.

According to the RAN2 agreements, the new MAC CE can include flexible number of PL offset values. However, it would cause extra complexity for UE to decode if the number of updated PL offsets is implicitly indicated by the L field of MAC subheader. Therefore, it is preferable that the new MAC CE carries indication bits to make it explicit for UE to decode all the updated PL offsets and the corresponding TCI State ID. Two options for the new MAC CE format design are provided as follows:

Proposal 1: Select one from the following two options for the new MAC CE format.

Option1: Use 1-bit reserved field in the new MAC CE to indicate whether the next TCI State/PL offset exists, as shown in Figure 3.

Option2: Use a new field in the new MAC CE to indicate the exact number of PL offsets, as shown in Figure 4.

In Figure 3 (Option1), the new 1-bit reserved field is named as F, if F=1, it means there will be a next TCI State/PL offset after the current TCI State/PL offset (indicated by F) for UE to decode. If F=0, it means there will be no next TCI State/PL offset after the current TCI State/PL offset, which means the current TCI State/PL offset is the last one in this MAC CE. This new field uses the reserved bit, which causes no extra overhead in the new MAC CE.

In Figure 4 (Option2), the new field (<=1 byte) is named as PL offsets number to indicate the exact number of PL offsets included in the current MAC CE. UE will decode all the updated TCI State/PL offsets according to this number. This option requires one additional octet overhead in the new MAC CE but keep the “R” bit for better future proof.

Figure 3. PLO update MAC CE format of Option1 		Figure 4. PLO update MAC CE format of Option2

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Based on the RAN1 agreement, both joint TCI mode and separate TCI mode are supported for PL offset update. In our view, one single MAC CE would be sufficient for both modes (joint and separate). The TCI mode (joint or separate) of each MAC CE can be distinguished by the unifiedTCI-StateType of the Serving Cell indicated by the Serving Cell ID field. In separate TCI mode, only the UL TCI state ID is associated with a PL offset, thus the PL offset update MAC CE only contains UL TCI State ID without DL TCI state ID. In joint TCI mode, the joint TCI state ID is included. Therefore, joint/separate TCI mode can share one common MAC CE format for updating PL offset.

Proposal 2: One common MAC CE format for PL offset update can be used for joint and separate TCI mode.

### 2.1.2	MAC CE for TCI State activation/deactivation

For the asymmetric DL sTRP/UL mTRP deployment scenario, RAN1 agreed to reuse the Rel-18 unified TCI framework:

Agreement - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

The R18 legacy Enhanced TCI state activation/deactivation MAC CE format is shown in Figure 4.

Figure 5. Rel-18 Enhanced TCI state activation/deactivation MAC CE format

According to TS 38321[1], the Rel-18 activation/deactivation MAC CE can support the {two DL TCI states + two UL TCI states} configuration, which covers the combination of {one DL TCI state + up to two UL TCI states} in RAN1 agreement. Therefore, the legacy MAC CE format can be reused in the asymmetric DL sTRP/UL mTRP scenario. Obviously, there will be no impact on the joint TCI mode. However, for the separate TCI mode, since there is no DL TCI State for the UL-only TRP, the corresponding Fi,j field should set to be 0, and the UE should ignore this Fi,j field.

Proposal 3: Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

Proposal 4: When the TCI activation/deactivation MAC CE for separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j field for the TCI state of the UL-only TRP is set to 0 and the UE ignores this Fi,j field.

2.2	PL offset Update

Based on the agreement, RAN2 should introduce in RRC a PL offset for each joint and UL TCI state and a new MAC CE with eLCID for updating the PL offset associated with TCI states after RRC configuration. The calculation of PL for multi-TRPs is shown in Figure 6.

Figure 6. PL calculation in asymmeteric mTRP scenario

On the existence between RRC and MAC CE approaches, in the last RAN2 meeting, the following WA is achieved

Working assumption UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found.

RRC supports delta signalling, in which the UE is required to keep fields that are absent. For example, if a number of TCI-UL-State were previously configured in BWP-UplinkDedicated for some UL BWP, and the network performs a reconfiguration of some DL parameters, or of some other UL BWP, the UE keeps the existing TCI-UL-State configured in this UL BWP.

Under handover scenario, as shown in Figure 6, when the PL offset is included in TCI-UL-State, after the reconfiguration, it would be problematic if UE uses the path loss offset modified by MAC CE because the target gNB may keep the original RRC configured PL offset, and this will lead to different PL offsets kept by UE and target gNB, which will further impact the PRACH power control and power headroom report.

Figure 6. Different PL offsets between UE and Target gNB under handover scenario

Observation: During handover procedure, in case delta signalling is used, the UE may store the latest PL offset value received in MAC CE, while the target gNB is only aware of RRC configured value by the source gNB.

Proposal 5: Discuss how to handle the problem that UE and target gNB may use different PL offsets caused by the conflict between MAC CE update and RRC delta signaling under handover scenario.

2.3	RACH Procedure

For PRACH transmission, RAN1 agreements are listed as follows:

Agreement - Support applying PL offset on PDCCH-order PRACH towards a UL-only TRP in FR1. - Support to apply PL offset on PDCCH-order PRACH in FR2. The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19. - The PL offset associated with one of the indicated joint/UL TCI state for UL-only TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. - A one-bit DCI field indicates whether the PL offset of the indicated joint/UL TCI state is used for PRACH transmission.

According to the agreements, PL offset is applied to PDCCH-order PRACH and the transmission power of PRACH should be calculated by PL offset. However, it is still not clear whether it refers to both PDCCH-order CBRA and PDCCH-order CFRA or not. In our view, PDCCH-order CBRA is typically triggered when UL asyn occurs while DL transmission is required, and there is no dedicated preamble for UL synchronization. Since UL-only TRP has no DL transmisson, there is no use case for PDCCH-order CBRA.

PDCCH-order PRACH with UL-only TRP is aimed to obtain TA, while for the UE-initiated CBRA, which is mainly triggered for initial RRC setup or BFR/SR, PL offset is not necessary because such functions can be achieved by legacy RA power control with DL-UL TRP. Therefore, the target SSB and carrier is indicated in the PDCCH order and we don't see any further impact to MAC with respect to pathloss offset.

Proposal 6: For PRACH transmission, PL offset is applied only to PDCCH-order CFRA, and no further MAC impact is needed.

# 3	Conclusion

From the analysis on RAN2 impacts for MIMO, we made the following proposals:

Proposal 1: Select one from the following two options for the new MAC CE format.

- Option1: Use 1-bit reserved field in the new MAC CE to indicate whether the next TCI State/PL offset exists, as shown in Figure 3.

- Option2: Use a new field in the new MAC CE to indicate the exact number of PL offsets, as shown in Figure 4.Proposal 2: The new MAC CE for PL offset update includes the impacted TCI State ID(s).

Proposal 2: One common MAC CE format for PL offset update can be used for joint and separate TCI mode.

Proposal 3: Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

Observation: During handover procedure, in case delta signalling is used, the UE may store the latest PL offset value received in MAC CE, while the target gNB is only aware of RRC configured value by the source gNB.

Proposal 4: When the TCI activation/deactivation MAC CE for separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j field for the TCI state of the UL-only TRP is set to 0 and the UE ignores this Fi,j field.

Proposal 5: Discuss how to handle the problem that UE and target gNB may use different PL offsets caused by the conflict between MAC CE update and RRC delta signaling under handover scenario.

Proposal 6: For PRACH transmission, PL offset is applied only to PDCCH-order CFRA, and no further MAC impact is needed.

# 4	Reference

- TS 3GPP 38.321 V18.2.0