3GPP TSG-RAN WG2 Meeting #129bis	R2-2502842

Wuhan, China, 7 - 11 Apr, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on satellite switch with resync

Agenda Item:	7.0.2.17

Document for:	Discussion and decision

- Introduction

In RAN2#128, the SMTC issue in soft satellite switch was discussed in RAN2 [1], and the following agreement was made:

- Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response

The LS to RAN1/RAN4 was sent in [2]. The replies from RAN1 and RAN4 have been received in [3] and [4] respectively. In this contribution, we continue to discuss the issue based on RAN1 and RAN4 inputs.

- Discussion

  - Replies to RAN1 questions

In RAN1 LS, the following questions were raised:

RAN1 has discussed the question in LS R2-2410978, and RAN1 would like to ask RAN2 for further clarification on the question and assumptions below: - Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed? - RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset). - RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

On Q1, RAN2 has never discussed RAN1 specification changes, the motivation for the original LS is to ask whether the pre-acquired DL sync can be maintained. The reason why RAN1 raised the question is, the wording “shift” in previous RAN2 LS [2] gives the impression that RAN2 want to change the fixed SSB pattern (within half frame) as defined in clause 4.1 of TS 38.213:

For a half frame with SS/PBCH blocks, the first symbol indexes for candidate SS/PBCH blocks are determined according to the SCS of SS/PBCH blocks as follows, where index 0 corresponds to the first symbol of the first slot in a half-frame. - Case A - 15 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes of 2,8+14⋅n. - For operation without shared spectrum channel access: - For carrier frequencies smaller than or equal to 3 GHz, n=0,1. - For carrier frequencies within FR1 larger than 3 GHz, n=0,1,2,3. - For operation with shared spectrum channel access, as described in [15, TS 37.213], n=0, 1, 2, 3, 4. …

The similar concern was expressed by RAN4 (about whether the ssb-TimeOffset is in the granularity of 5ms). The existing ssb-TimeOffset configuration provides a granularity of 1ms, however, to avoid affecting the SSB pattern specified in half-frame granularity as in clause 4.1 of TS 38.213, we agree with RAN4 that ssb-TimeOffset should be configured as 5 or multiples of 5. This should be clarified in the RAN2 specification, for instance:

SatSwitchWithReSync field descriptions

ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes and the value can only be 5 or multiples of 5.

Proposal 1: Add the following in the field description of ssb-TimeOffset: “It is given in number of subframes and the value can only be 5 or multiples of 5”.

With the above clarification, the SSB pattern within half frame will not be affected, the SSB configuration is only “implicitly reconfigured” and RAN2 does not intend to change R1 specification.

Proposal 2: On Q1 from RAN1 LS, RAN2 answer is that: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. The ssb-TimeOffset will be configured in 5ms or multiples of 5ms, RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213, the SSB configuration is only “implicitly reconfigured” from RAN2 perspective.

On Q2, one condition for soft satellite switching without PCI change is “non-colliding SSB with same PCI at UE side” and the network needs to manipulate the SSB timing difference between source and target satellite, and this difference is created by SSB time offset. Therefore, we think the SSB time offset is the only essential change needed.

Proposal 3: On Q2 from RAN1 LS, RAN2 answer is that: RAN2 confirms the only change is SSB time offset (determined by ssb-TimeOffset).

On Q3, starting from R17, it is up to UE implementation to maintain a valid SIB19 (containing the satellite assistance information), as specified in clause 5.2.2.4.21 of TS 38.331:

Upon receiving SIB19 in an NTN cell, the UE in RRC_CONNECTED shall: 1> start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell. NOTE: UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

Proposal 4: On Q3 from RAN1 LS, RAN2 answer is that: Starting from R17, it is up to UE implementation to reacquire SIB 19 before the end of serving cell validity duration, and RAN2 assumes in the soft satellite switch case it is also up to UE implementation to maintain a valid ephemeris information of target satellite and ensures it is not outdated at satellite switch.

2.2 Replies to RAN4 questions

In RAN4 LS, the following conclusions and questions were provided:

Under following assumptions, - Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point; - The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service]; RAN4 reached the conclusion that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. RAN4 would like to note that the temporarily SSB pattern shift (i.e., subject to ssb-TimeOffset) during [t-ServiceStart, t-Service] and shifting back after t-Service was not considered during RAN4 requirement definition and it may require additional UE processing for shifting. Also, it may have impact to UE performing initial access during [t-ServiceStart, t-Service]. It is also noted that the RAN4 requirements are defined assuming the parameter ssb-TimeOffset is the relative time offset between SSB of the source satellite and the SSB of the target satellite with same SSB index. In addition, RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset, - Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point - Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2: 1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5. 2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

On the assumptions mentioned by RAN4:

-	Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point;

-	The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service];

The first assumption means the timing of source satellite is the same as that of the target satellite. This is in line with RAN2 understanding, as RAN2 has ruled out Option 1 (i.e. shifted cell timing) based on previous RAN2 agreements:

Option 1: Shifted cell timing

Option 2: Shifted SSB pattern (2a: via reconfiguration; 2b: auto SMTC adjustment)

Option 3: Temporarily shifted SSB pattern

R2-2410971 Report of [AT128][301][NR NTN] SMTC issues Huawei discussion Rel-18 NR_NTN_enh-Core Proposal 1: RAN2 to further discuss whether to specify the following for soft satellite switching: Option 2b: After t-Service, UE applies ssb-TimeOffet to the SMTC configured by source satellite. Option 3: Temporarily shifted SSB pattern (determined by ssb-TimeOffset) is used during [t-Start, t-Service] for early synchronization with target satellite, FFS whether RAN4 input is needed. - vivo is fine to consider option 3 and send a LS to RAN4/RAN1 asking for feedback - QC thinks it’s late to introduce a change and prefers not to send the LS - Google/Sequans are fine to send the LS. Sequans thinks option 2b is not complete Send a LS to RAN1/RAN4 on the feasibility of option 3. RAN2 will work on solution 2b or solution 3 depending on the LS response

The second assumption is related to the interpretation of SSB time offset. RAN4 understanding is as follows:

It is also noted that the RAN4 requirements are defined assuming the parameter ssb-TimeOffset is the relative time offset between SSB of the source satellite and the SSB of the target satellite with same SSB index.

We think this understanding is aligned with RAN2. In RAN2#125bis, RAN2 had some discussion on this. No clarification is made but it seems companies were on the same page: using the start subframe of the SSB to indicate the offset is the same as using the SSB of same index to indicate the offset.

Proposal 3: Clarify that the ssb-TimeOffset refers to the time offset between the start subframe of the SSB burst from source and target satellite. - HW agrees with the issue and thinks we could refer to the specific SSB index - CATT thinks this proposal is to avoid referring to SSB index. - ZTE is not sure anything else needs to be clarified: the ssb-TimeOffset is the same between different SSB indices We don’t introduce additional clarifications

RAN2 further clarified ssb-TimeOffset in RAN2#127 but didn’t mention “of the same SSB index”, companies might consider it as common understanding so didn’t emphasize this aspect in the field description

R2-2407612 Report of [AT127][301][R18 NR NTN] corrections for satellite switch with re-sync Qualcomm Incorporated discussion Rel-18 NR_NTN_enh-Core Proposal 1 The field description of ssb-TimeOffset is updated as “Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. It is given in number of subframes”. Agreed

Therefore, it is clear that Option 1 from RAN4 LS is the correct understand.

Proposal 5: Regarding the two options of ssb-TimeOffset from RAN4 LS, RAN2 answer is that: Option 1 is aligned with RAN2 understanding.

RAN4 further asked whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5 and how the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service]. The issue of ssb-TimeOffset granularity has been discussed and clarified in P1.

The half frame indication is indicated in PBCH payload, as specified in clause 7.1.1 of TS 38.212:

Generate the following additional timing related PBCH payload bits , where: - are the 4th, 3rd, 2nd, and 1st LSB of SFN, respectively; - is the half frame bit ; - if Lmax=10 as defined in Clause 4.1 of [5, TS38.213], …

The SFN indication is provided in MIB contents:

MIB ::=                             SEQUENCE {

systemFrameNumber                   BIT STRING (SIZE (6)),

subCarrierSpacingCommon             ENUMERATED {scs15or60, scs30or120},

ssb-SubcarrierOffset                INTEGER (0..15),

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

pdcch-ConfigSIB1                    PDCCH-ConfigSIB1,

cellBarred                          ENUMERATED {barred, notBarred},

intraFreqReselection                ENUMERATED {allowed, notAllowed},

spare                               BIT STRING (SIZE (1))

}

In legacy mechanism, the SFN indication in MIB and half frame bit in PBCH payload will be corresponding to the actual time domain location where the SSB is transmitted. In soft satellite switch case, we think it is the same, the SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Proposal 6: Regarding RAN4 question on ssb-TimeOffset granularity and half-frame indication, RAN2 answer is that: ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

2.3 Way forward

According RAN4 LS, the DL synchronization can be maintained:

Under following assumptions, - Starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point; - The parameter ssb-TimeOffset is the relative offset between the SSB of the source satellite and the SSB of the target satellite during [t-ServiceStart, t-Service]; RAN4 reached the conclusion that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. RAN4 would like to note that the temporarily SSB pattern shift (i.e., subject to ssb-TimeOffset) during [t-ServiceStart, t-Service] and shifting back after t-Service was not considered during RAN4 requirement definition and it may require additional UE processing for shifting. Also, it may have impact to UE performing initial access during [t-ServiceStart, t-Service]. It is also noted that the RAN4 requirements are defined assuming the parameter ssb-TimeOffset is the relative time offset between SSB of the source satellite and the SSB of the target satellite with same SSB index.

On the initial access issue, in our understanding the satellite switch with resync mechanism is designed for Connected mode UEs. Also considering the time duration of [t-ServiceStart, t-Service] would be quite short, the UEs that could be affected during initial access would be very few, that’s also why RAN4 said “may have impact”.

Based on the RAN4 conclusion, we think RAN2 can adopt Option 3, unless RAN1 raised further concern.

Proposal 7: Option 3 is adopted, unless RAN1 raised further concern. The target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

Option 2b should not be adopted, because it is never studied by RAN1 and RAN4.

If RAN2 cannot decide on Option 3, we think the only way out is to keep the existing spec and rely on network implementation to update SMTC. In this case, the benefits of satellite switch with resync come from the following: 1) HO procedure is not needed and MAC is not reset; 2) Avoid signalling burst of RRCReconfiguration messages, as the network can send RRCReconfiguration message later after satellite switch; 3) Reduce RRCReconfiguration messages to some extent, considering it is not urgent to perform measurements right after accessing a new cell, and the network can wait a bit longer to update SMTC (the SMTC in NTN scenario is anyway changing continuously and the network needs to reconfigure it every once in a while.)

The reply LS in the annex is drafted assuming Option 3 is adopted.

Proposal 8: Approve the reply LS in the Annex.

- Conclusion

This contribution discussed the SMTC issue in soft satellite switch with resync. Proposals are summarized as follows:

Proposal 1: Add the following in the field description of ssb-TimeOffset: “It is given in number of subframes and the value can only be 5 or multiples of 5”.

Proposal 2: On Q1 from RAN1 LS, RAN2 answer is that: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. The ssb-TimeOffset will be configured in 5ms or multiples of 5ms, RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213, the SSB configuration is only “implicitly reconfigured” from RAN2 perspective.

Proposal 3: On Q2 from RAN1 LS, RAN2 answer is that: RAN2 confirms the only change is SSB time offset (determined by ssb-TimeOffset).

Proposal 4: On Q3 from RAN1 LS, RAN2 answer is that: Starting from R17, it is up to UE implementation to reacquire SIB 19 before the end of serving cell validity duration, and RAN2 assumes in the soft satellite switch case it is also up to UE implementation to maintain a valid ephemeris information of target satellite and ensures it is not outdated at satellite switch.

Proposal 5: Regarding the two options of ssb-TimeOffset from RAN4 LS, RAN2 answer is that: Option 1 is aligned with RAN2 understanding.

Proposal 6: Regarding RAN4 question on ssb-TimeOffset granularity and half-frame indication, RAN2 answer is that: ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Proposal 7: Option 3 is adopted, unless RAN1 raised further concern. The target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

Proposal 8: Approve the reply LS in the Annex.

- References

- R2-2410971, Report of [AT128][301][NR NTN] SMTC issues, Huawei

- R2-2410978, LS on soft satellite switch with resync, RAN2 (contact company: Huawei)

- R1-2501606, Reply LS on soft satellite switch with re-sync, RAN1 (contact company: Huawei)

- R4-2502701, reply LS on soft satellite switch with re-sync, RAN4 (contact company: Huawei)

- Annex – draft LS to RAN1/RAN4

# 1. Overall Description:

RAN2 thanks RAN1 and RAN4 for the replies, and provide the following inputs after further RAN2 discussion:

RAN1 Question 1: Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed?

RAN2 answer: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. The ssb-TimeOffset will be configured in 5ms or multiples of 5ms, RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213, the SSB configuration is only “implicitly reconfigured” from RAN2 perspective.

RAN1 Question 2: RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

RAN2 answer: RAN2 confirms the only change is SSB time offset (determined by ssb-TimeOffset).

RAN1 Question 3: RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

RAN2 answer: Starting from R17, it is up to UE implementation to reacquire SIB 19 before the end of serving cell validity duration, and RAN2 assumes in the soft satellite switch case it is also up to UE implementation to maintain a valid ephemeris information of target satellite and ensures it is not outdated at satellite switch.

RAN4 Question 1: RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset,

-	Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point

-	Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite

RAN2 answer: Option 1 is aligned with RAN2 understanding.

RAN4 Question 2: Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2:

1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5.

2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

RAN2 answer: ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Furthermore, RAN2 has agreed that the target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service based on RAN4 inputs, unless further concern is raised by RAN1/RAN4.

# 2. Actions:

To RAN1

ACTION: 	RAN2 kindly asks RAN1 and RAN4 to consider the above question and provide feedback if any.

# 3. Date of Next RAN2 Meetings:

3GPP TSG RAN WG2#130	19 - 23, May, 2025		Malta, MT

3GPP TSG RAN WG2#131	25 - 29, Aug, 2025		India, IN