3GPP TSG-RAN WG2#129-bis		      					R2-2502846

Wuhan, China, April 7 – 11, 2025

Agenda item:	8.6.4

Source: 	Kyocera

Title: 	Considerations on CLTM TAT handling

Document for:	Discussion

Introduction

RAN2#129 agreed to the following statements with regard to the MAC issues [1]:

Agreements on MAC issues: 1. CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell. 2. If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid. 3. If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM. 4. The CLTM TAT is per candidate configuration. 5. FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution. 6. Absolute TA value is included in the MAC CE for CLTM early TA acquisition. 7. The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

In this contribution, the handling of new timer, i.e., CLTM TAT, is discussed.

Discussion

CLTM TAT and legacy TAT handlings upon CLTM execution

For Rel-19 Conditional LTM (CLTM), the TA value acquired in the early sync phase is indicated via a new MAC CE which is different from Rel-18 Cell Switch Command MAC CE. So, there is a time difference between the reception of the new MAC CE with the TA value at the UE and the occurrence of the CLTM execution. Therefore, after the early sync phase, the UE needs to check whether the stored TA value is valid before the RACH-less CLTM execution. For this purpose, it was agreed that “Candidate cell TA is maintained by a new timer” in the RAN2#128 meeting [2]. In this paper, this new timer is referred as CLTM TAT.

Before looking at the CLTM TAT for LTM candidate cell’s TA, the existing Time Alignment Timer (TAT) for the serving cell’s TA management is summarized. According to TS38.321 [4], the UE uses TAT to manage the validity of the TA value for UL transmissions to the serving cell. Upon TAT expiry, the UE MAC proceeds to flush HARQ buffer, to indicate RRC to remove the PUCCH/SRS and to cancel already received downlink assignment/uplink grants. TAT is configured per Timing Advance Group (TAG) associated with multiple serving cells.  For clarification, TAT is referred as legacy TAT in the following discussions.

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG;

On the other hand, according to the current MAC running CR [5], the UE uses the CLTM TAT to manage the validity of the stored TA value for the CLTM candidate cell, whereby the stored TA value was obtained in the last early sync phase. Each CLTM TAT is associated with one candidate configuration for CLTM.

- ltm-Candidate-TimeAlignmentTimer which controls how long the MAC entity considers the CLTM candidate cells associated with this timer to be uplink time aligned. Each ltm-Candidate-TimeAlignmentTimer is associated with one candidate configuration for CLTM.

It should be noted that the CLTM TAT works for the TA value stored for the CLTM candidate cell, which is not for the TA value applied for the serving cell handled by the legacy TAT.

Besides, it could be considered what the UE behaviour upon the CLTM TAT expiry. In the legacy TAT, the UE stops UL/DL transfer upon timer expiry. On the other hand, upon CLTM TAT expiry, the UE does not need to stop UL/DL transfer towards the serving cell (i.e., the source cell) while UE considers the TA for the candidate cell is invalid as RAN2 agreed.

According to the current MAC running CR [5], after a CLTM execution is initiated, the TA value corresponding to the CLTM candidate cell (i.e., the target cell) on which the LTM execution was performed is applied to the PTAG, and the legacy TAT associated with the PTAG is started or restarted with timeAlignmentTimer provided within the applied LTM candidate configuration.

1> when a conditional LTM cell switch procedure is triggered for a LTM candidate cell as in clause 5.y.3 or indicated by upper layer, and the ltm-Candidate-TimeAlignmentTimer associated with the LTM candidate cell is running: 2> apply the stored TA value associated with the LTM target cell for the PTAG as specified in clause 6.1.3.4x; 2> start or restart the timeAlignmentTimer associated with the PTAG.

The legacy TAT is started/restarted with the TAT value provided in the LTM candidate configuration, when the CLTM cell switch procedure is triggered.

However, it’s questionable whether it is appropriate to apply timeAlignmentTimer within the LTM candidate configuration simply as in the current running CR above, since the validity of the TA value may not be guaranteed in some cases. The problematic scenario where the validity of TA value is not guaranteed is shown with 3 steps as below and depicted in Figure 1:

Step 1 (Just before CLTM execution, in the source cell): The remaining time of CLTM TAT is 300ms when the CLTM execution is triggered, which means the stored TA value is valid for the next 300ms. As the stored TA value is valid, the UE initiates the RACH-less CLTM cell switch.

Step 2 (Upon CLTM execution to the candidate cell): The legacy TAT value (i.e., timeAlignmentTimer) is 500ms in the corresponding CLTM candidate configuration, so the UE starts the legacy TAT with 500ms when the stored TA value is applied for the target cell. Note that this step is based on the current MAC running CR above.

Step 3 (after CLTM execution, in the target cell): The applied TA value is actually invalid for 200ms after 300ms lapses since the legacy TAT was started, but the UE still considers the current TA value as valid, which is the issue.

Figure 1 Scenario where a validity of TA value is not guaranteed

The issue in Observation 2 under the scenario above may be prevented by gNB implementation, i.e., the target cell always sends the Timing Advance Command MAC CE to reset the legacy TAT, whenever the target cell detects the RRC Reconfiguration Complete from the UE for RACH-less CLTM. However, it mandates the gNB behaviour upon the RACH-less CLTM execution and causes the unnecessary signalling overhead

On the other hand, another way to prevent the issue is to modify Step 2, i.e., upon applying the stored TA value for the CLTM candidate cell (i.e., the target cell), the UE sets the remaining time of the CLTM TAT to the legacy TAT for this target cell. In this way, the legacy TAT in the target cell starts with 300ms in Step 2, so the inconsistency between CLTM TAT and the legacy TAT for the same TA value before/after the CLTM execution can be avoided, i.e., the time period of 200ms in Step 3 does not happen.

RAN2 should discuss if the UE sets the remaining time of the CLTM TAT associated with the CLTM candidate cell (i.e., the target cell) to the timer value of the legacy TAT associated with the PTAG, when the UE applies the stored TA value of the CLTM candidate cell upon the CLTM execution.

CLTM TAT handling for other candidates at CLTM execution

Furthermore, it’s still unclear how to handle the CLTM TATs associated with other stored TA values for other CLTM candidate cells, upon the CLTM execution.

In Rel-16 Conditional Handover, the TA value is always acquired upon the CHO execution (i.e., only RACH-based CHO is supported), so no TA value is stored for CHO candidate cells in advance. In Rel-18, RACH-less CHO is supported for the NTN and mobile IAB use cases, but the TA value provided in Conditional Reconfiguration is static and always applicable (i.e., zero or same as source), so any new validity timer (like CLTM TAT) is not needed.  It’s also noted that CHO does not support subsequent CHO execution.

In Rel-18 LTM, the subsequent LTM execution is introduced, whereby the multiple LTM executions are performed without RRC Reconfiguration and by keeping the LTM candidate configuration upon LTM execution. According to TS38.300 [3], the steps of the early synchronization, LTM cell switch execution, and LTM cell switch completion can be performed multiple times for the subsequent LTM, using the LTM candidate configuration(s) provided in LTM preparation phase. It should be noted that the early sync phase is not mandated to be performed for the subsequent LTM and the TA values for LTM candidate cells are managed by the network. So, for more efficient subsequent LTM, it’s possible as a gNB implementation that the TA value acquired before an LTM execution is indicated to the UE for the next LTM execution (i.e., the TA value is not acquired in the current cell but in the previous serving cell; so, it can be seen as an “early sync-less RACH-less subsequent LTM”.)

In Rel-18 LTM, the TA values for candidate cells are managed by the network. So, it’s possible as a network implementation to reuse the already acquired TA values set in CSC MAC CE for subsequent LTM executions (i.e., without additional early sync), as long as the network considers the TA value as valid.

With regards to the subsequent CLTM, it’s natural that the same principle as Rel-18 is applied, i.e., the subsequent CLTM is performed without the LTM preparation phase, i.e., the UE maintains CLTM candidate configurations, so RRC Reconfiguration is not needed for the next CLTM execution. In addition, as similar to the smart network implementation in Rel-18 LTM as above, the early sync phase should not be mandated for the RACH-less subsequent CLTM.

RAN2 should agree that the same principle as Rel-18 subsequent LTM is applied to the subsequent CLTM, i.e., the LTM preparation (RRC Reconfiguration) and the early sync are not mandated for subsequent CLTM execution, regardless of whether RACH-less access or not.

If Proposal 2 is agreeable, it should be considered how to avoid the early sync for the next RACH-less subsequent CLTM execution.  In general, the stored TA value depends on the physical distance between the UE and the CLTM candidate cell. In practice, the same stored TA value should be valid as long as the UE is located within a geographical range, e.g., tens of meters, and the validity of stored TA values are not related to the occurrences of the CLTM cell switch but just subject to the CLTM TAT. In this sense, it’s possible to reuse the stored TA value for the RACH-less subsequent CLTM execution, so it is better to maintain the stored TA values for other CLTM candidate cells as long as they are still valid, even after a CLTM to a candidate cell is executed. In order to manage the validities of the stored TA values after the CLTM execution, the corresponding CLTM TATs should not be stopped after the CLTM execution. With this mechanism, the UE can perform RACH-less subsequent CLTM without requiring early sync phase after the CLTM executions.

On the other hand, it’s considered simple if all the TA values are discarded every time after the CLTM execution, but early sync will be needed again for RACH-less subsequent CLTM as discussed above. If the execution condition is fulfilled for the subsequent CLTM before the UE obtains the TA value in the early sync phase, RACH-based CLTM execution will be needed, which is inefficient in terms of the signalling overhead, the increased latency due to the random access procedure and the risk of RACH failure.

Given the considerations above, the UE should keep the stored TA values upon a CLTM execution(s) as long as these’re considered valid, and the UE should also keep the corresponding CLTM TATs running continuously.

RAN2 should agree that the UE can keep the CLTM TATs running and continue storing the TA values even after CLTM execution, in order to support more efficient Subsequent CLTM.

In case of the subsequent CLTM, the source cell may be configured as a CLTM candidate cell when the UE is still served by the source cell. After the UE executes CLTM to the target cell (which is the new source cell), the UE may perform the CLTM from the new source cell (i.e., the original target cell) back to the original source cell, if the UE returns to the original source cell. In this case, the early sync is needed again to execute RACH-less CLTM to the source original cell.

Figure 2 Case where the source cell is configured as a LTM candidate cell

The source cell may be also configured as a CLTM candidate cell due to the subsequent CLTM.

However, the TA value applied in the source cell just before the CLTM execution is still valid even after the CLTM execution as long as the　legacy TAT is running at the time of the CLTM execution to the target cell, but it is discarded at the time of the CLTM execution because the UE applies the stored TA value of the CLTM candidate cell to the PTAG. Therefore, when RACH-less CLTM is executed again to the source cell, early sync needs to be performed again.

If the legacy TAT is running upon the CLTM execution, the UE can support more efficient subsequent CLTM by storing the TA value, which is currently applied for the source cell, into the CLTM candidate configuration corresponding to the source cell, before applying the stored TA value to the PTAG of the target cell. With this operation, the valid TA value for the source cell can be kept, which enables RACH-less CLTM without requiring early sync procedure.

RAN2 should discuss whether the UE is allowed to store the TA value applied in the source cell after the CLTM execution to the target cell, if there is a CLTM candidate configuration corresponding to the source cell and the legacy TAT was still running upon the CLTM execution.

Conclusion

In this contribution, the issues for supporting conditional intra-CU LTM are discussed. RAN2 is kindly asked to take into account the observations and the proposals below:

Observation 1	It should be noted that the CLTM TAT works for the TA value stored for the CLTM candidate cell, which is not for the TA value applied for the serving cell handled by the legacy TAT.

Observation 2	The legacy TAT is started/restarted with the TAT value provided in the LTM candidate configuration, when the CLTM cell switch procedure is triggered.

Proposal 1	RAN2 should discuss if the UE sets the remaining time of the CLTM TAT associated with the CLTM candidate cell (i.e., the target cell) to the timer value of the legacy TAT associated with the PTAG, when the UE applies the stored TA value of the CLTM candidate cell upon the CLTM execution.

Observation 3	In Rel-18 LTM, the TA values for candidate cells are managed by the network. So, it’s possible as a network implementation to reuse the already acquired TA values set in CSC MAC CE for subsequent LTM executions (i.e., without additional early sync), as long as the network considers the TA value as valid.

Proposal 2	RAN2 should agree that the same principle as Rel-18 subsequent LTM is applied to the subsequent CLTM, i.e., the LTM preparation (RRC Reconfiguration) and the early sync are not mandated for subsequent CLTM execution, regardless of whether RACH-less access or not.

Proposal 3	RAN2 should agree that the UE can keep the CLTM TATs running and continue storing the TA values even after CLTM execution, in order to support more efficient Subsequent CLTM.

Observation 4	The source cell may be also configured as a CLTM candidate cell due to the subsequent CLTM.

Proposal 4	RAN2 should discuss whether the UE is allowed to store the TA value applied in the source cell after the CLTM execution to the target cell, if there is a CLTM candidate configuration corresponding to the source cell and the legacy TAT was still running upon the CLTM execution.

References

- R2-2501331 “Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#129

- R2-2410911 “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#128

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS38.321, “Medium Access Control (MAC) protocol specification”, 3GPP

- R2-2502158 “Running MAC CR for Mob Ph4”, vivo (Rapporteur), RAN2#129bis