3GPP TSG-RAN WG2 Meeting #129bis	R2-250xxxx

Wuhan, China, 7th April – 11th April, 2025

Agenda Item		:	8.5.2

Source		:	LG Electronics Inc.

Title		:	Discussion on on-demand SSB SCell operation

Document for		:	Discussion and Decision

1. Introduction

In this contribution, we provide our views on remaining issues for OD-SSB.

2. Discussion

2-1. Details on MAC CE format

In RAN1#120 meeting, RAN1 agreed on introducing the parameters for OD-SSB configurations (i.e., od-ssb-config). The OD-SSB configurations are configured via RRC message. New MAC CE indicates one of the OD-SSB configuration to apply the pattern for activated OD-SSB transmission. To indicate the OD-SSB configuration, a field for indicating OD-SSB configuration should be introduced.

Proposal 1	OD-SSB configuration, i.e. od-ssb-config is configured by the RRC message.

Proposal 2	Indicates one of the OD-SSB configurations via a new MAC CE.

Proposal 3	Introduce a field for indicating which OD-SSB configuration is applied for activated OD-SSB transmission in a new MAC CE.

Based on the existing SCell activation/deactivation MAC CE format, we can consider two types of MAC CE for activation/deactivation of OD-SSB transmission as illustrated in Figure 1. Ci is 1 to indicate activation OD-SSB transmission on SCell#i. Ci is 0 for deactivation The fields for OD-SSB configuration are included in ascending order based on the index i where Ci=1:

Figure 1. Example of a new MAC CE format indicating activation/deactivation of OD-SSB transmission. 
(Left: short-form; Right: long-form)

In RAN1#119 meeting, RAN1 agreed on introducing two options for OD-SSB transmission.

- Option-1: Explicitly indicate activation/deactivation of OD-SSB transmission

- Option-2: Transmit number N of OD-SSB bursts after activation indication

For Option-2, RAN1 agreed to introduce the parameter: Number N of OD-SSB bursts to be transmitted after OD-SSB transmission is indicated (i.e., od-ssb-nrofTx). Option-2 can be signalled via new MAC CE indicating OD-SSB configuration containing od-ssb-nrofTx.

Let us consider the scenario that network indicates the MAC CE consecutively.

- First MAC CE indicating OD-SSB activation on SCell#X with OD-SSB configuration#1

  - OD-SSB configuration#1 contains od-ssb-nrofTx

- Second MAC CE indicating OD-SSB activation on SCell#Y

When the second MAC CE is indicated during SCell#X OD-SSB transmission, the network indicates OD-SSB configuration #1 in the second MAC CE. In that case, the network intends to indicate two different UE behavior. First (Intention-1), the network intends the UE to keep the OD-SSB configuration applied by the first MAC CE. Second (Intention-2), the network intends the UE to re-apply the number of OD-SSB bursts to be transmitted according to the OD-SSB configuration#1. However, the UE cannot distinguish between two network intentions only with the field for OD-SSB configuration.

When the UE behavior is not matched with the network intention, power dissipation problem occurs. For Intention-1, the UE performs measurement despite the OD-SSB transmission is terminated. For Intention-2, the network transmits OD-SSB bursts despite the UE does not perform measurement. Therefore, we propose discussing how to indicate the network intention on such case.

Observation 1	Network cannot indicate its intention whether maintaining OD-SSB configuration or re-applying OD-SSB configuration when SCell is transmitting number N of OD-SSB bursts.

Proposal 4	RAN2 to discuss how to indicate the network maintains the OD-SSB configuration or re-applies the OD-SSB configuration when SCell is transmitting number N of OD-SSB bursts.

2-2. OD-SSB measurement

According to TS38.331, whenever the UE has a measurement configuration, the UE shall perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured, and include the measurement results of the serving cell in every measurement report message.

In the previous meeting, RAN2 agreed for Case #1 (no always-on SSB on the cell) that UE does not expect to measure SSB when on-demand SSB transmission is deactivated. If servingCellMO is configured for the OD-SSB SCell, the RAN2 agreement conflicts with the current UE behaviour. To resolve the contradiction between the current UE behaviour and the RAN2 agreement, two solutions can be considered as follows:

- Option 1. Do not configure servingCellMO for OD-SSB SCell

- Option 2. Change the procedural text to meet the RAN2’s agreement

If servingCellMO is not configured for OD-SSB SCell, the measurement results of the OD-SSB SCell will not be reported even when the OD-SSB transmission is activated, unless the measurement reporting condition associated with OD-SSB SCell is met. Since the SCell requires tight management though the measurement results frequently reported by UE, option 1 does not seem acceptable. Therefore, it seems more desirable to modify the procedural text related to the servingCellMO.

UE is not required to include the SSB based measurement results of the SCell configured with servingCellMO in the measurement report, if the SSB based measurement results of the SCell are not available. Therefore, even if the SSB transmission is deactivated, it will not impact the L3 measurement reporting.

Proposal 5	The servingCellMO can be configured for the OD-SSB SCell.

Proposal 6	Even if the servingCellMO is configured for the OD-SSB SCell, UE does not derive the SSB based measurement results of the OD-SSB SCell when the on-demand SSB transmission is deactivated.

According to RAN1’s agreement, RAN2 should do the signaling support for the on-demand SSB and the always-on SSB based L3 measurements. To analyze the impact of them on the existing measurement configuration, we should first consider whether the existing measurement configuration can be re-used for OD-SSB/always-on SSB based measurements.

The characteristic of the always-on SSB is it is transmitted with a long periodicity. Although RAN1 agreed that the always-on SSB is SSB supported in Rel-18 specifications, considering that the primary measurement timing configuration, i.e. smtc1, is used also for measurements on neighbor cells, it does not seem suitable to configure the always-on SSB using the existing smtc1.

The measurement object configuration, i.e. MeasObjectNR, contains also smtc2, smtc3list, and smtc4list, to support the secondary measurement timing which is different from the primary measurement timing for the listed cells. For always-on SSB based SCell measurements, similar signaling can be used, e.g. smtc5. The new measurement timing configuration for always-on SSB is applicable only to an SCell which is associated with the measurement object, so the cell list is not needed.

Proposal 7	Introduce smtc5 in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of always-on SSB.

RAN1 also agreed that multiple periodicities should be support for on-demand SSB. It is already possible in the measurement object configuration to configure multiple measurement timing for particular cell using smtc3list or smtc4list. The usage of them is limited to IAB-MT and NTN cells, respectively. Therefore, it seems reasonable to introduce smtc6list to configure multiple measurement timing configuration for on-demand SSB.

Proposal 8	Introduce SSB-MTC6List in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of on-demand SSB.

3. Conclusion

In this contribution, following statements are proposed:

2-1. Details on MAC CE format

Proposal 1	OD-SSB configuration, i.e. od-ssb-config is configured by the RRC message.

Proposal 2	Indicates one of the OD-SSB configurations via a new MAC CE.

Proposal 3	Introduce a field for indicating which OD-SSB configuration is applied for activated OD-SSB transmission in a new MAC CE.

Observation 1	Network cannot indicate its intention whether maintaining OD-SSB configuration or re-applying OD-SSB configuration when SCell is transmitting number N of OD-SSB bursts.

Proposal 4	RAN2 to discuss how to indicate the network maintains the OD-SSB configuration or re-applies the OD-SSB configuration when SCell is transmitting number N of OD-SSB bursts.

2-2. OD-SSB measurement

Proposal 5	The servingCellMO can be configured for the OD-SSB SCell.

Proposal 6	Even if the servingCellMO is configured for the OD-SSB SCell, UE does not derive the SSB based measurement results of the OD-SSB SCell when the on-demand SSB transmission is deactivated.

Proposal 7	Introduce smtc5 in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of always-on SSB.

Proposal 8	Introduce SSB-MTC6List in MeasObjectNR associated with the OD-SSB SCell to configure the measurement timing of on-demand SSB.

4. Appendix

RAN2#127 Agreements: 1. Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). 2. For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. RAN2#127bis Agreement: 1. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 2. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

RAN1#118bis Agreement Agreement - For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options RAN1#119 Agreement Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer RAN1#120 Agreement Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list