3GPP TSG-RAN2 Meeting #129bis	R2- 2502848

# Wuhan, China, Apr 7th – 11st, 2025

Agenda item:		8.7.4.1 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Remaining issues on LCP enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to define additional Logical channel priority handling based on the delay/deadline information.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In RAN2#128 meeting [2], it is agreed to use additional LCH priority with LCH priority-adjusted data for both the first round and the second round of the LCP procedure as a baseline, but fallback to default priority at second round is supported with optional capability.

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In addition, in RAN2#129 meeting [3], some details using additional LCP are agreed, including whether the additional priority can be applied for intra-UE prioritization.

There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination No additional PBR is needed for priority adjusted data FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

This document discusses remaining issues on LCP enhancement for XR.

2.	Discussion

Details on intra-UE prioritization with additional priority

In RAN2#129 meeting [3], it is agreed that additional LCP should be applied for intra-UE prioritization between the UL grant, if the additional LCP is used during the LCP procedure for initial transmission. However, several details are remained as FFS, including following aspects:

- Issue 1) Whether the priority for UL grant should be further updated for retransmission;

- Issue 2) Whether any clarification is needed in the current specification; and

- Issue 3) Which priority should be applied for SR transmission

For Issue 1, during the MAC CR discussion, some companies wanted to clarify whether the priority for UL grant should be updated for retransmission. Specifically, after the initial transmission, if some of data multiplexed in the MAC PDU becomes an LCH-priority adjusted data at the time of retransmission, it should be clarified whether the priority of the uplink for this retransmission should be updated to the additional priority.

However, it would be too complicated if the UE always checks whether there is any LCH-priority adjusted data for generated MAC PDU for retransmission case, since UE needs to check remaining time of the generated MAC PDU whenever the UE receives an UL grant, in order to update the priority of the uplink grant for each transmission. In our understanding, if legacy priority is used for an LCH at the time of the MAC PDU generation, network would have enough time to allocate the retransmission UL grant within the delay budget, so there is no critical reason to update the priority of the retransmission UL grant based on real-time delay status of the generated MAC PDU. Therefore, for simplicity, it should be enough to consider the applied priority at the time of the MAC PDU generation, i.e., when the LCP procedure is performed.

Proposal 1. The priority of uplink grant is determined based on applied logical channel priority at the time of LCP procedure. No further update on retransmission case.

For Issue 2, based on MAC CR discussion, it seems that there are different understandings on the current wording, regarding whether the additional priority is applied for intra-UE prioritization based on the current text. In addition, there are also different understanding on whether the priority can be further updated on the retransmission, based on the delay status on data included in generated MAC PDU. Therefore, in order to avoid any ambiguity on desired UE behaviour, it would be better to clearly explicitly specify the intended behaviour, based on RAN2 agreements.

Therefore, if proposal 1 is agreed, it should be clearly specified that priority of an uplink grant is determined based on the additional priority instead of legacy priority, if there is any LCH priority-adjusted data at the time of the LCP procedure (i.e., based on applied priority at the time of MAC PDU generation).

Proposal 2. For intra-UE prioritization, it should be explicitly specified that priority of an uplink grant is determined based on additional priority instead of the legacy priority, if LCH priority-adjusted data is included at LCP procedure.

An example TP for proposal 2 is as follows:

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this determination, the priority of a data logical channel shall be its additionalPriority if it satisfies the conditions of logical channel priority adjustment at the LCP procedure (see clause 5.4.3.1.3).

For Issue 3, according to the current MAC specification [4] , the priority of SR is determined by the priority of the logical channel that “triggered” the SR:

(…omitted) 2> if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

On the other hand, in RAN2#129 meeting [3], the additional priority is not applied for triggering SR.

There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority.

In addition, the LCP procedure is applied for a new transmission, and not for the SR transmission. Therefore, it would be straightforward to apply existing priority for the SR priority determination as well, regardless of whether LCH priority adjusted data is available or not.

Proposal 3. For priority determination of SR, always apply the legacy priority, i.e., no need to apply the additional priority.

LCH priority adjusted data in the first round when Bj < 0

In RAN2#129 meeting [3], it is agreed to not introduce additional PBR for LCH-priority adjusted data. However, it was remained as FFS on whether the LCH-priority data can be included at the first round of LCP procedure, even though the Bj for the corresponding logical channel is less than or equal to zero (i.e., Bj<0).

In our understanding, the intention of introducing additional priority for LCH-priority adjusted data is to prioritize the transmission of the urgent data with short remaining time. Therefore, if the LCH with LCH-priority data is not considered at the first round of the LCP when the Bj of LCH with LCH priority-adjusted data is negative, data included in other LCHs would be considered prior to the LCH-priority adjusted data. Then the transmission of LCH-priority adjusted data would be delayed, even though the priority of the corresponding LCH is adjusted to the higher priority.

It is true that it is not common case that Bj is negative when the LCH-priority data exists in the corresponding LCH. However, if there is the case, LCH-priority data should considered in the first round of LCP as well, in order to ensure the transmission of the urgent data. Therefore, in order to ensure the prioritized transmission of LCH-priority data (i.e., urgent data), if the additional priority is applied, the corresponding LCH should be considered in the first round of LCP procedure, regardless of its Bj value.

Proposal 4. When the additional LCH priority is applied for an LCH, the LCH is considered in 1st round of LCP regardless of its Bj value.

Capability for fallback in the second round of LCP procedure

In RAN2#128 meeting [2], it is agreed that UE may support the fallback from the additional priority to default priority at the second round of LCP procedure, if all of LCH-priority adjusted data is included during the first round of LCP procedure.

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

The intention of introducing the optional capability is to have a common understanding between the network and the UE on whether the UE fallbacks to the legacy priority on the second round of LCP procedure. In this sense, it would be straightforward to define the UE behaviour on Rel-19 LCP based on the network configuration, including whether the UE updates the priority from additional priority to legacy priority in case of there is no further LCH-priority adjusted data for the corresponding LCH. Here, the network may configure whether to fallback from additional priority to legacy priority in the second round of LCP procedure, after the network receives the UE capability on whether the UE supports the corresponding functionality.

Proposal 5. In the configuration of additional priority for LCH-priority adjusted data, the network may configure whether to fallback from additional priority to legacy priority in case of all the LCH-priority data included in the first round of LCP procedure, based on UE capability signalling.

3.	Conclusion

In this contribution, it is discussed for the LCP enhancement for XR. This document includes following proposals.

Proposal 1. The priority of uplink grant is determined based on applied logical channel priority at the time of LCP procedure. No further update on retransmission case.

Proposal 2. For intra-UE prioritization, it should be explicitly specified that priority of an uplink grant is determined based on additional priority instead of the legacy priority, if LCH priority-adjusted data is included at LCP procedure.

Proposal 3. For priority determination of SR, always apply the legacy priority, i.e., no need to apply the additional priority.

Proposal 4. When the additional LCH priority is applied for an LCH, the LCH is considered in 1st round of LCP regardless of its Bj value.

Proposal 5. In the configuration of additional priority for LCH-priority adjusted data, the network may configure whether to fallback from additional priority to legacy priority in case of all the LCH-priority data included in the first round of LCP procedure, based on UE capability signalling.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024

[3] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17-21 February, 2025

[4] TS 38.321 v18.5.0