3GPP TSG-RAN2 Meeting #129bis	R2- 2502851

# Wuhan, China, Apr 7th – 11st, 2025

Agenda item:		8.11.3 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Other aspects on SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD [1], it is agreed to support the Random Access procedure using the SBFD symbols.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request

This document further discusses RAN2 aspects on other aspects for SBFD.

2.	Discussion

Time/frequency configuration of SBFD symbol

In RAN3 LS [2], RAN3 indicated that SBFD time/frequency information is exchanged between the gNB using the Served Cell Information NR IE, and asked RAN2 to work on IE design on SBFD time and frequency configuration.

RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, and also between gNB-DU and gNB-CU in case of split architecture. RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration information, which should be based on parameters provided by RAN1. With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN2 to work on the IE design of SBFD time and frequency configuration information similar to the TDD-UL-DL-ConfigurationCommon IE which is signalled over Xn and F1.

Based on the above RAN3 LS, during the post e-mail discussion, it was suggested to include the NZP CSI-RS resource information in the inter-node message MeasurementTimingConfiguration IE.

Issue description: It is understood that RAN3 (R3-250888) concluded that the IE design on SBFD time and frequency configuration information for the SBFD information exchange among gNBs is up to RAN2. According to [25] P7, the resource configuration can be signalled as below. Solutions: For SBFD gNB-to-gNB CLI measurement, to include the set of one or more periodic NZP CSI-RS resources configuration into inter-node message MeasurementTimingConfiguration.

However, it should be noted that the information exchange among gNB on “Semi-static cell-specific SBFD time and frequency location configuration” and the “periodic NZP CSI-RS resources configuration (i.e., measurement resource configuration)” are discussed separately with different purpose in RAN1/RAN3, as noted in WID. Therefore, it is not suitable to define NZP CSI-RS resources configuration into inter-node message MeasurementTimingConfiguration IE as a solution, since it is not aligned with the RAN3 request mentioned in the LS.

Therefore, based on the request of RAN3 LS, it should be first discussed about how to configure SBFD time and frequency location, not NZP CSI-RS resources configuration.

For SBFD time location configuration, it seems that the detailed configuration is agreed in RAN1 parameter list [3]. Specifically, following IEs should be included in TDD-UL-DL-Pattern IE:

- SBFD-StartingSlotIndex

- SBFD-StartingSymbolIndex

- SBFD-EndingSlotIndex

- SBFD-EndingSymbolIndex

Therefore, it is straightforward to define the agreed parameters in RAN2 as well, in order to let RAN3 define additional parameters in Served Cell Information NR IE based on RRC configuration on SBFD time frequency location defined in RAN2.

Proposal 1. Based on the RAN1 parameter list, define SBFD time location configuration in TDD-UL-DL-Pattern IE.

For SBFD frequency location configuration, in RAN1#119 [4], it is agreed that one RIV is indicated for SBFD UL subband and one or two RIVs are indicated for SBFD DL subband, for each SCS configuration in SCS-SpecificCarrierList.

Agreement Confirm the following working assumption made in RAN1#118. Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

However, it seems that RAN1 has not agreed on the detailed signaling structure, so RAN2 can wait for RAN1 until the detailed signaling is provided via additional RAN1 parameter list.

Proposal 2. For SBFD frequency location configuration, wait for RAN1 on additional parameter list.

NZP CSI-RS resource exchange

In the post e-mail discussion, it was suggested to add an NZP CSI-RS configuration in the inter-node message. On the other hand, for NZP CSI-RS exchange among gNB, RAN3 is already discussing between these two options [5], and has not concluded yet:

- Option 1: to put a new IE in Served Cell Information NR IE with reference to RAN2 definition

- Option 2: ask RAN2 to include in inter-node message (i.e., in MeasurementTimingConfiguration IE).

Therefore, there is no need to duplicate the discussion in RAN2 and RAN3, i.e., RAN2 can just wait for RAN3 conclusion. In our view, considering that both options are feasible, it is preferred to define a new IE in Served Cell Information NR IE, in order to avoid additional LS exchange between RAN2 and RAN3.

Proposal 3. Wait for RAN3 on exchange of NZP CSI-RS configuration among gNBs.

Support of DC/CA

During the post e-mail discussion, several companies raised issues on whether SBFD operation can be supported for CA and DC.

| Company | Comments |
| --- | --- |
| Ericsson | RAN2 should discuss if SBFD operation can be supported for CA and DC. |
| ZTE | Agree with Ericsson. RAN1 has already support multiple carrier scenario with SBFD. RAN2 should discuss the spec impact of the CA and DC work together with SBFD. |

On the other hand, regarding SBFD support in multi-carrier scenario, following agreement is made in RAN1#120 [5]:

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

In other words, in CA scenario, SBFD operation both in PCell and SCell is not supported.

In our understanding, the above agreement was made mainly due to CLI issues in intra-band CA scenario. However, it is not explicitly specified in the RAN1 agreement whether the above agreement is only for intra-band CA or not, so it is not clear whether the CA is not supported in inter-band CA as well.

Therefore, based on the above agreement, RAN2 may assume that for CA case, SBFD operation can only be supported in one carrier as well, unless further agreements are made in RAN1.

Proposal 4. RAN2 assumes that SBFD operation is supported only in one carrier in CA scenario, unless RAN1 has made further agreement to update this assumption.

Similarly, if the SBFD operation is supported both in MN and SN in DC scenario, RAN1 impact may be occurred, e.g., to identify whether there is any issue, e.g., due to CLI based on DC configuration. Therefore, before supporting SBFD operation both MN and SN in NR-DC scenario, it should be checked with RAN1 whether there are any issues to support this. However, given that RAN1 has only two meetings left, it may not be possible to send an LS to check with RAN1 and resolve any issues to support DC before finishing the WI.

Therefore, it would be safer to not support the DC in SBFD, i.e., in NR-DC, only SN or MN can be configured with the SBFD symbol.

Proposal 5. For DC scenario, only one of SN or MN can support SBFD operation.

3.	Conclusion

In this contribution, it is discussed about other aspects on SBFD. This document includes following proposals.

Proposal 1. Based on the RAN1 parameter list, define SBFD time location configuration in TDD-UL-DL-Pattern IE.

Proposal 2. For SBFD frequency location configuration, wait for RAN1 on additional parameter list.

Proposal 3. Wait for RAN3 on exchange of NZP CSI-RS configuration among gNBs.

Proposal 4. RAN2 assumes that SBFD operation is supported only in one carrier in CA scenario, unless RAN1 has made further agreement to update this assumption.

Proposal 5. For DC scenario, only one of SN or MN can support SBFD operation.

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] R2-2501731 SBFD information exchange among gNBs for CLI mitigation

[3] R2-2501724 LS on Rel-19 higher layers parameters list Post RAN1#120

[4] Report of 3GPP TSG RAN WG1 #119

[5] R3-250803 Summary of SBFD offline discussion

[6] Report of 3GPP TSG RAN WG1 #120