3GPP TSG-RAN WG2 Meeting #129bis	R2-2502856

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UL Capacity and Throughput Enhancement

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#106 the work item on NR NTN (Non-Terrestrial Networks) enhancements was updated [1]. Among the objectives of the work item, there is uplink capacity/throughput enhancement for FR1-NTN, using orthogonal cover codes (OCC). The objective description is as follows:

Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we provide our views on the possible RAN2 impacts.

# 2	Discussion

In RAN1 #119, an agreement was reached for the OCC scheme to support for Rel-19:

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

Based on the RAN1 agreement, both 2 and 4 UEs multiplexing are supported by using inter-slot OCC.  As the OCC is an optional feature for NTN UEs, RAN1 agreed there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE feature requirement to support the inter-slot OCC is still in discussion in RAN1.

Proposal 1: Introduce separate UE capabilities for OCC with length 2 and OCC with length 4. The UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE capability description can be finalized based on the RAN1 LS on feature list.

To enable orthogonal transmission (and reception) of multiple UEs over the same time/frequency resources using OCC, methods for indication of OCC operation to corresponding UEs need to be introduced. It is indeed fundamental that UEs scheduled in the same time frequency resources use different and orthogonal codes for their transmissions, and this can be guaranteed via gNB indication and management of UEs OCC code selection. Therefore, the code characteristics including OCC length, actual code index and application timing should be clearly known at the UE.

Semi-static RRC signalling and dynamic DCI signalling may be used together to have a trade-off between efficiency and overhead. In particular, the OCC set of sequences may be configured by RRC and the exact OCC characteristics for each user is user specific and indicated to the UE dynamically via DCI. The exact field on the indications in DCI, e.g., either via a new DCI field or via repurposing an existing DCI field, is still in discussion in RAN1. Furthermore, for configured grant Type1 where the CG resource periodicity and PRBs/MCS are configured via RRC without DCI activation/deactivation, it seems the OCC sequence length and OCC sequence index can be indicated by RRC only.

Proposal 2: The OCC sequence length and OCC sequence index should be indicated to UE via RRC and/or DCI. The exact solution can wait for further progress from RAN1.

According to the WID [1], the OCC enhancement is not applied for initial access. Furthermore, RAN1-120 meeting also agreed Msg3 PUSCH with OCC is not in the scope of current WI. This means the UL capacity enhancement would only be for the “regular PUSCH” scheduled by DCI or configured grant. Whether and how the UE use the OCC resource can be controlled by network in a UE-specific way via DCI or RRC. In this sense, on top of existing NTN barring bit, there is no need to introduce dedicated access control mechanism for OCC-capable UEs.

Proposal 3: There is no need to introduce dedicated access control mechanism for OCC capable UEs.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Introduce separate UE capabilities for OCC with length 2 and OCC with length 4. The UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE capability description can be finalized based on the RAN1 LS on feature list.

Proposal 2: The OCC sequence length and OCC sequence index should be indicated to UE via RRC and/or DCI. The exact solution can wait for further progress from RAN1.

Proposal 3: There is no need to introduce dedicated access control mechanism for OCC capable UEs.

# References

- RP-243300, ”Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”