3GPP TSG-RAN WG2 Meeting #129bis	                                 R2-2502868

Wuhan, China, Apr. 7th – 11th , 2025

Source:        ZTE Corporation

Title:           Clarification on the MUSIM Gap and Measurement Gap Collision

Agenda item:   7.0.2.12

Document for:  Discussion and Decision

- Introduction

In the R18, for the UE supporting musim-GapPriorityPreference, the network can configure MUSIM gap together with concurrent measurement gap, and how to solve the collision between the MUSIM Gap and the measurement Gap was captured in RAN4 spec. However, according to the latest RAN4 spec [1], it’s still unclear on how to solve the collision between the MUSIM Gap without priority and the measurement Gap with priority. In this contribution, we share our views on this issue.

- Discussion

In the latest RAN4 spec, processing on the collision between the MUSIM Gap and the measurement Gap was described as below:

#### 9.1.10.5 Collisions between MUSIM gaps and measurement gaps ... Collisions between MUSIM gaps and measurement gaps configured via GapConfig-r17 with assigned priority but without preConfigInd-r17 or ncsgInd-r17 are resolved based on their assigned priorities: - If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. For each collision, the occasion of the MUSIM gap or measurement gap with higher priority shall be kept and the occasion of the MUSIM gap or measurement gap with lower priority shall be dropped. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4. - Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their priorities. Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps: - If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply to these gaps. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4. - Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP following the above mentioned collision rule.

Based on the above spec, we can see that only the first 3 cases as in the Table 1 were included, for the firth case, it’s unclear.

Table 1: Collision between the MUSIM Gap and the Measurement Gap

|  | Meas Gap Priority | MUSIM Gap Priority | Collision Processing |
| --- | --- | --- | --- |
| 1 | N | Yes | Based on the MGRP |
| 2 | N | N | Based on the MGRP |
| 3 | Y | Y | Based on the Priority |
| 4 | Y | N | Not specified yet |

For the case 4, it can be happened at least for the below 3 scenarios:

- Scenario 1: The UE supports musim-GapPriorityPreference but doesn’t indicate any preference on the MUSIM Gap priority, and the network doesn’t configure MUSIM Gap priority even it conflicts with the measurement Gap.

- Scenario 2: The UE only indicates the keep indication to the network as clarified in [2].

- Scenario 3: The UE indicates the preference on the MUSIM Gap priority, but the network configures neither priority nor the keep indication

Observation 1: For the UE supporting musim-GapPriorityPreference, the network may configure MUSIM gap without priority together with concurrent measurement gap with priority, and it’s unclear how to solve the collision between these 2 gap types.

If we look back at the MUSIM discussion history, we can find that this case was previously addressed in Proposal 7 of [3], but no conclusion was reached.

R2-2304398 Report of [AT121bis-e][231][MUSIM] RAN4 aspects of MUSIM (Samsung) Samsung discussion Rel-18 NR_DualTxRx_MUSIM-Core Proposal 7: RAN2 to discuss whether the following question needs to be checked with RAN4 - Does network always configure priority levels to all configured periodic MUSIM gaps if UE is allowed to indicate periodic MUSIM gap priority preference? If not, is there any need to specify a default UE behavior?

Then in the next meeting, RAN2 only made an agreement for the below case as in [4].

Agreements RAN2#123: If network doesn’t retain the relative priorities among MUSIM gaps, UE behaviour is not specified

Even with this agreement, it’s still unclear how to solve the collision between the MUSIM Gap without priority and the measurement Gap with priority. To solve this issue, there are 3 options:

- Option 1: Take the lowest priority for the MUSIM Gap.

- Option 2: For the UE supporting musim-GapPriorityPreference, if the network decides to configure MUSIM gap together with concurrent measurement gap (with priority), the network should always provide the MUSIM Gap with priority when there is a collision between these two gap types.

- Option 3: Send a LS to ask RAN4 on how to solve this issue.

The solution 1 is a quite easy way, the solution 2 would introduce some unnecessary complexity, e.g. as clarified in the last meeting, the UE may only indicate keep indication without any priority indication and RAN4 has agreed that it’s up network to determine whether to configure the priority at the very beginning [5]. The solution 3 is also acceptable if no consensus can be achieved from RAN2.

Proposal 1: RAN2 to discuss which option is preferred for resolving the collision between a MUSIM gap without priority and a concurrent measurement gap with priority

- Option 1: Take the lowest priority for the MUSIM Gap.

- Option 2: Send a LS to ask RAN4 on how to solve this issue.

The TP for the option 1 is also provided as in the Annex.

Proposal 1a: Agree with the TP in the Annex if the option 1 is selected.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: For the UE supporting musim-GapPriorityPreference, the network may configure MUSIM gap without priority together with concurrent measurement gap with priority, and it’s unclear how to solve the collision between these 2 gap types.

Proposal 1: RAN2 to discuss which option is preferred for resolving the collision between a MUSIM gap without priority and a concurrent measurement gap with priority

- Option 1: Take the lowest priority for the MUSIM Gap.

- Option 2: Send a LS to ask RAN4 on how to solve this issue.

Proposal 1a: Agree with the TP in the Annex if the option 1 is selected.

- Reference

- 38.133-i80  Requirements for support of radio resource management  3GPP

- R2-2501511	Corrections on MUSIM gap	Samsung	CR	Rel-18	38.331	18.4.0	5271

- R2-2304398	Report of [AT121bis-e][231][MUSIM] RAN4 aspects of MUSIM (Samsung) Samsung

- R2-2309401 Report of 3GPP TSG RAN WG2 meeting #123, Toulouse, France

- R2-2302430	LS on priority for MUSIM gaps (R4-2303249; contact: vivo)	RAN4

- Annex

–	MUSIM-GapConfig

The IE MUSIM-GapConfig specifies the MUSIM gap configuration and controls setup/release of MUSIM gaps.

MUSIM-GapConfig information element

-- ASN1START

-- TAG-MUSIM-GAPCONFIG-START

MUSIM-GapConfig-r17 ::=                  SEQUENCE {

musim-GapToReleaseList-r17               SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17               OPTIONAL, -- Need N

musim-GapToAddModList-r17                SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17                 OPTIONAL, -- Need N

musim-AperiodicGap-r17                   MUSIM-GapInfo-r17                                       OPTIONAL, -- Need N

...,

[[

musim-GapToAddModListExt-v1820           SEQUENCE (SIZE (1..3)) OF MUSIM-GapExt-v1820            OPTIONAL, -- Need N

musim-GapKeep-r18                        ENUMERATED {true}                                       OPTIONAL  -- Need N

]]

}

MUSIM-Gap-r17 ::=                        SEQUENCE {

musim-GapId-r17                          MUSIM-GapId-r17,

musim-GapInfo-r17                        MUSIM-GapInfo-r17

}

MUSIM-GapExt-v1820 ::=                   SEQUENCE {

gapPriority-r18                          GapPriority-r17

}

-- TAG-MUSIM-GAPCONFIG-STOP

-- ASN1STOP

MUSIM-GapConfig field descriptions

musim-AperiodicGap Indicates the MUSIM aperiodic gap as specified in TS 38.133 [14] clause 9.1.10. If UE indicates the musim-Starting-SFN-AndSubframe when requesting aperiodic gap the network can only configure the aperiodic gap with the same start point or no aperiodic gap. If the field musim-Starting-SFN-AndSubframe is absent for aperiodic gap, network can configure any timing as the starting point for aperiodic gap or configure no aperiodic gap.

musim-GapInfo Indicates the values for musim-GapLength and musim-GapRepetitionAndOffset. When network provides periodic gap, network always signals the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred MUSIM gap configuration.

musim-GapKeep Indicates the UE is allowed to use "keep solution" for colliding MUSIM periodic/aperiodic gaps. If "keep solution" is not granted, collisions between MUSIM periodic gaps are resolved based on the assigned MUSIM gap priorities as specified in TS 38.133[14].

musim-GapToAddModListExt Indicates the priority of MUSIM periodic gap(s). For the UE supporting musim-GapPriorityPreference, if the network configures MUSIM gap without priority together with measurement gap with priority, the UE shall take the MUSIM Gap as the lower priority if it collides with the measurement gap with priority. If the network includes musim-GapToAddModListExt-v1820, it includes the same number of entries, and listed in the same order, as in musim-GapToAddModList-r17. For the priority of MUSIM aperiodic gap, the MUSIM aperiodic gap is always kept (not dropped) from UE perspective in case of collisions with other gaps (i.e. all gaps including MUSIM gaps, etc).

musim-GapToAddModList List of MUSIM periodic gap patterns to add or modify.

musim-GapToReleaseList List of MUSIM periodic gap patterns to release.