3GPP TSG RAN WG2 #129bis		R2-2502875

Wuhan, China, April 07th – 11th , 2025

Source:	CEWiT

Title:	Discussion on random access for Ambient IoT

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #129 [1] meeting, the following agreements have been made on random access aspects related to A-IoT.

- Agreements - For Rel-19, only 3-step CBRA is supported for A-IOT - We will specify both CBRA and CFRA. - Re-use the subsequent paging message to trigger re-access. There is no need to differentiate Msg1 resource for initial access vs re-access - NACK based mechanism is supported for D2R messages to determine re-access for at least Msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. - RAN2 assumes that device randomly selects among FDMA occasions as the baseline. - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we discuss our views on random access procedure related aspects for A-IoT.

- Random access

2.1 Determination of resources for random access

2.2.1 For MSG1

RAN2 has agreed that slotted-ALOHA is the baseline for A-IoT random access procedure. In this process, each A-IoT device randomly selects one of the time-frequency resources to transmit its A-IoT MSG1. In the RAN2 #129 [1] meeting, it is agreed that paging message can include the total number of resources for MSG1. For A-IoT random access, to further select the resources for the MSG1 transmission a separate R2D trigger message can be used after the paging message. This ensures that the A-IoT device is aware of the resources available for transmitting its initial message. As shown in fig. 1, a predefined time window, referred to as the RACH time window can be used for A-IoT device to select the time resources randomly from this window. This RACH window allows multiple devices to backscatter their responses. The RACH window represents the duration allocated for Msg1 transmissions, during which multiple devices can transmit at different time instances. In the RACH window different occasions for MSG1 transmission are available. The R2D trigger message can be used to indicate the start of the RACH window. The time offset between the reception of R2D trigger message and the start of RACH window is predefined and called as TR2D_min. Similar to RFID, the A-IoT device can use similar approach i.e. random number based on Q value approach to further chose one time resource form the RACH window for transmitting the MSG1. In this case, the A-IoT device generates a random number based on Q value, loads it to a timer, start decrementing the timer and transmits the MSG1 at the expiry of the timer. The Q value can be configured through the paging message.

Fig.1 TDMA of Msg1 Transmission

Proposal 1: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 2: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

- The time duration between the reception of R2D trigger and the start of RACH time window is indicated by the TR2D_min.

Proposal 3: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

When multiple devices backscatter their MSG1, the reader responds by transmitting MSG2 to these devices. In RAN1#120 [2] meeting, RAN1 has agreed that MSG2 supports both options which are given below:

- Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

To ensure proper reception, the monitoring of Msg2 transmission is scheduled to take place after the RACH window. The specific time window during which devices are expected to listen for Msg2 is referred to as the RAR time window, as illustrated in Fig. 2. The RAR time window can start after a predefined time gap ( TD2R_min) from the end of RACH time window. The RAR time window is divided into multiple RAR occasions similar to RACH occasions within RACH window.

Fig.2 Random access procedure

Proposal 4. Monitoring for Msg2 transmission using a RAR time window after the end of the RACH time window.

- The time duration between the end of RACH time window and start of RAR time window is indicated by the TD2R_min.

For Msg3’s transmission resource configuration, the resources can be communicated in two possible ways i.e. the reader can indicate the resources in MSG2 based on the received MSG1 which provides a more efficient resource allocation process or the A-IoT device can reuse the same resources that were previously used for transmitting Msg1

Proposal 5: Support the following options to determine the transmission resources for MSG3.

- MSG2 indicates the resources for MSG3 transmission.

- Reuse the same resources of MSG1 transmission

2.2 NACK as feedback

According to the RAN2#129 [1] meeting agreement, if the reader fails to receive Msg3 from the device, it sends a NACK to indicate that the device should attempt retransmission. However, if Msg3 is successfully received, the reader does not send an explicit ACK to minimize unnecessary acknowledgments, as most D2R transmissions are successful. This strategy helps reduce ACK signaling overhead.

After transmitting Msg3, the device remains uncertain about whether it should continue monitoring for an R2D message. If no further information is sent by the reader, the device will continue monitoring transmissions, leading to potential inefficiencies. To address this, a timer based approach should be adopted. In the timer-based approach, the device will start a timer after transmitting Msg3. During this timer, the reader can send a follow-up message if needed. If no message is received before the timer expires, the device will assume that no further communication is required. Additionally, if the device does not receive a NACK within the expected timeframe, it will assume Msg3 was successfully received.

Proposal 6: Support the timer based approach for monitoring the NACK feedback by the A-IoT device in case of MSG3 failure.

- Conclusion

In this contribution, we provide our views on random access for A-IoT. The proposals are summarized as follows.

Proposal 1: For TDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access:

- Based on the slotted aloha in a predefined RACH time window.

Proposal 2: R2D trigger used to determine the start of the RACH time window for transmission of MSG1 is supported.

- The time duration between the reception of R2D trigger and the start of RACH time window is indicated by the TR2D_min.

Proposal 3: Support to generate the random number based on Q value configured in paging message by the A-IoT device for transmission of MSG1 in the predefine RACH time window.

Proposal 4. Monitoring for Msg2 transmission using a RAR time window after the end of the RACH time window.

- The time duration between the end of RACH time window and start of RAR time window is indicated by the TD2R_min.

Proposal 5: Support the following options to determine the transmission resources for MSG3.

- MSG2 indicates the resources for MSG3 transmission.

- Reuse the same resources of MSG1 transmission

Proposal 6: Support the timer based approach for monitoring the NACK feedback by the A-IoT device in case of MSG3 failure.

# References

[1]   RAN2#129 Chair Notes.

[2]  RAN1#120 Chair Notes.