3GPP TSG-RAN WG2 Meeting #129bis	R2- 2502883

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.7.6

Source:	Nokia, Nokia Shanghai Bell

Title:	XR rate control

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). - Companies to check with their SA4 colleagues whether there are any issues with this

# 2	Discussion

## 2.1	New table for XR rate control

In RAN2#129, RAN2 received a reply LS from SA4 in R2-2500056 indicating that:

-	the uplink bit rate range may vary from 100kbps to 40Mbps.

-	typical uplink data rates differ by application, e.g., 200kbps for cloud gaming and 20Mbps for AR conversational.

-	a finer granularity of 1% is preferred, though coarser granularity of 10% could also be acceptable.

Based on this feedback, RAN2 has decided to specify a new table since the current table may not adequately serve the traffic characteristics mentioned by SA4.

The codepoints included in the table vary based on the target granularity and the range, which would also influence the MAC CE design. Therefore, it is necessary to first establish the target granularity and range. However, as SA4 pointed out, the bit rate range varies by application, and higher granularity is generally preferred. Defining a single table with fixed range and granularity, e.g., 10%, in the specification could compromise the flexibility needed to accommodate diverse application requirement. While one might argue that a multiplier could address this issue, it affects both granularity and range simultaneously, making it suboptimal to, e.g., AR conversational traffic which may require finer control. Thus, considering both flexibility and performance, it seems beneficial to allow the use of different codepoints depending on the traffic characteristic and congestion level. One approach could be defining multiple tables with distinct ranges and granularities, and allowing the network to select the appropriate table – which can be signalled via MAC CE for rate control using e.g., multiplier field. Alternatively, a more dynamic method would be to form the table dynamically as needed based on configurable parameters such as min/max bit rates and desirable granularity, without explicitly defining the tables in the specification.

Proposal 1: Multiple tables with distinct ranges and granularities are specified for XR rate control.

## 2.2	Query for rate control

In RAN2#129, it was assumed that rate query MAC CE is supported with the option to disable it via a prohibit timer, similar to legacy. The remaining question is whether the prohibit timer should be per QoS flow or per DRB. The prohibit timer serves two functions:

-	it prevents the UE from transmitting queries too frequently to the gNB by enforcing a configured prohibit interval.

-	it disables the query function when the prohibit timer is not configured since the UE cannot generate a MAC CE in that case.

If the goal is to prevent the UE from sending queries too frequently, then configuring the prohibit timer at the DRB level seems to be sufficient. In the previous RAN2 discussion, one primary purpose of the query was to allow the UE for XR – different from voice service – to request an increased bit rate back when congestion is alleviated. Once the gNB receives a query for a DRB, the gNB can adjust the uplink bit rate in accordance with the current congestion situation, similar to the initial adjustment.

Even when the purpose is to disable or enable the query function, there seems to be no clear advantage to disable/enable the query function at the QoS flow level. Therefore, it is deemed sufficient to configure the prohibit timer at the DRB level.

Proposal 2: For XR rate control, the prohibit timer for query function is defined/configured at the DRB level (as legacy).

Currently, the gNB implementation determines when to send updated bit rate information, even though the UE’s query can request an adjustment. This might lead to unnecessary transmission of uplink bit rate or transmission of outdated bit rate values because, applications – such as HTTP Adaptive Streaming – may only adjust their bit rates once per segment (typically every 2-10 seconds. Therefore, it would be advantageous if the UE could receive the updated bit rate information that aligns with the application’s adaptation timing, for which the gNB needs to know the applications adaptation timing information.

Proposal 3: RAN2 discuss if any assistance information can help determine the timing for the network to transmit the rate control MAC CE.

## 2.3	Per QoS flow vs. per DRB MAC CE

For control of uplink bit rate, there are still two options on the table:

-	Option 1: QoS flow information for rate control is included in the rate control MAC CE.

-	Option 2: The QoS flow information for rate control is provided via RRC, while the DRB information is included in the rate control MAC CE.

Both options control the bit rate at QoS flow level, but they may affect F1 interface differently since, in option 1, DU gets the QoS flow information from CU. Since RAN3 will further evaluate the implication for the F1 interface, the final decision should be postponed until their conclusion is available.

Proposal 4: RAN2 wait for further progress in SA4 and RAN3 to decide the design of MAC CE for rate control.

However, from RAN2 perspective, it would be important to clarify and further look into the option 2, which was spontaneously introduced during online discussions.

For Option 2, the general assumption seems to be that RRC indicates a single QoS flow, for which the uplink bit rate can be signalled via MAC CE (with DRB ID). This approach requires RRC reconfiguration to modify the QoS flow for which the uplink bit rate can be signalled, making it less flexible especially when there are multiple QoS flows which are capable of rate adjustment within a single DRB. An alternative assumption is configuring multiple QoS flows and allowing the UE to adjust the uplink bit rates of those flows by implementation. However, this alternative was not favoured by UE vendors due to lack of controllability.

If we aim to perform the rate control for multiple QoS flows while still using DRB indication in the MAC CE, one approach could be to apply the MAC CE sequentially to the QoS flows in a certain order. From a network perspective, when congestion occurs, it is preferable to adapt the uplink bit rate starting with the QoS flow which is not only capable of rate control but also most effectively reduces congestions. Therefore, if such order can be provided to the UE in advance, the UE can simply apply the MAC CE according to the order of QoS flows. Since Option 2 already assumes that the QoS flows subject to rate control are configured by RRC, adding the order of QoS flows does not add any significant signalling complexity. In addition, specifying that the UE determines the order of applying the MAC CE to the respective QoS flows would not be particularly challenging from a specification perspective. For instance, this can be implemented using a window or timer.

Proposal 5: RAN2 discuss how the MAC CE with DRB indication can be applied to multiple QoS flows.

## 2.4	Scenarios with dual connectivity

In the scenarios with dual connectivity (DC) and split bearer, the data from a single DRB or QoS flow is sent over multiple cells e.g. MCG and SCG. When congestion occurs either in MCG or SCG or both, the UE may receive the recommended bit rate MAC CE with updated bit rate from at least one of the connected cells in case no coordination between MCG and SCG. In this case, due to different congestion situations in the connected cells, UE may or may not need to change the bit rate at application layer. As another possibility, it is also feasible that only MCG sends the updated recommended bit rate MAC CE to cover the latest recommended bit rate from both cells. From UE point of view, it might be easier to determine whether to change the application bit rate or not based on a single MAC CE. While as the first step, RAN2 needs to discuss whether to consider rate control for DC scenarios in Rel-19 XRM work.

Proposal 6: RAN2 to specify how to support XR rate control in case with DC.

# 3	Conclusion

Proposal 1: Multiple tables with distinct ranges and granularities are specified for XR rate control.

Proposal 2: For XR rate control, the prohibit timer for query function is defined/configured at the DRB level (as

legacy).

Proposal 3: RAN2 discuss if any assistance information can help determine the timing for the network to transmit the rate control MAC CE.

Proposal 4: RAN2 wait for further progress in SA4 and RAN3 to decide the design of MAC CE for rate control.

Proposal 5: RAN2 discuss how the MAC CE with DRB indication can be applied to multiple QoS flows.

Proposal 6: RAN2 to specify how to support XR rate control in case with DC.