3GPP TSG-RAN WG2 Meeting #129bis		                               R2-2502890

Wuhan, China, April. 7th – 11th, 2024

Agenda Item:		8.7.6

Source:			MediaTek Inc.

Title:				Discussion on XR rate control

Document:			Discussion and decision.

### 1	Introduction

This document discusses the objective: uplink congestion control in R19 XR WID [1].

-	Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

### 2	Discussion

In RAN plenary #105 meeting [2], companies bring a joint contribution to elaborate that RAN-Aware XR Rate Control is important to XR user experience. Compare to packet drop due to congestion problem which leads to broken image, reducing the data rate (lower resolution but not broken image) has lower negative impact to user experience. Therefore, to know whether there is congestion along the path to destination or not is important for XR services.

In RAN2 #128 meeting [3], we already agreed that ”Rate indication from gNB to the UE on a per QoS flow level is supported.” The FFS part is the exact signalling mechanism we adopted, two mechanisms are listed during RAN2 #128 meeting:

1) flows are indicated by MAC CE or

2) by RRC while MAC CE is per DRB.

In MAC spec, we already have Recommended bit rate MAC CE to indicate the data rate. However, the granularity of current Recommended bit rate MAC CE is per logical channel (per DRB), which is not finer enough to support XR services, which might have multiple QoS mapped into the same DRB/logical. Another reason is that if we still adopt the second option, there is no difference between legacy Recommended bit rate MAC CE and the new rate indication MAC CE. Therefore, it is better to use QoS flow ID rather than logical channel ID, and according to TS 38.331, the maxQFI is 63, 6 bits is enough to cover all QoS Flow ID.

Proposal 1: RAN2 agree to define a new MAC CE, the new MAC CE include 6 bits to indicate QoS Flow ID

### 3	Conclusion

This document promulgated the following proposals:

Proposal 1: RAN2 agree to define a new MAC CE, the new MAC CE include 6 bits to indicate QoS Flow ID

### 4	Reference

[1]	RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).

[2]       RP-242224, On RAN Awareness and UL Rate Control for XR, Meta, CATT, CMCC, Google, vivo, ZTE Corporation, Sanechips, Huawei, HiSilicon, Spreadtrum, Samsung, Apple, MediaTek Inc., Futurewei

[3]       R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei).