3GPP TSG-RAN WG2 #129bis 	R2-2502905

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.4

Source:	KT Corp.

Title:	Discussion on random access aspects of Ambient IoT

Document for:	Discussion/Decision

WID/SID:	Ambient_IoT_solutions

1	Introduction

In the previous RAN2#128 and RAN2#129 meeting, RAN2 agreed that re-transmission of Msg2 and NACK based mechanism for at least Msg3 as shown in below. We would like to discuss how and when a A-IoT reader (re-)transmits MSG2 and NACK to specific device(s).

Agreements (RAN2#128) - RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID - If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported).

Agreements (RAN2#129) - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. - RAN2 assumes that device randomly selects among FDMA occasions as the baseline

2	Discussion

Here is the 3-step CBRA procedure, which is based on the agreements so far.

Figure 1. 3-step CBRA based on current agreements

As shown in figure 1, according to the agreement from the previous RAN2#129 meeting, it was agreed that the reader would send a Msg4, which includes a NACK (failure) message for at least Msg3 to determine re-access. In other words, if the reader does not receive Msg3, it can send a NACK message to specific devices. However, in the RAN2#128 meeting, it was agreed that reader can re-transmit Msg2 to devices from which Msg3 was not received. In other to support and integrate both functions, it is essential to consider the perspectives of both the reader and the device.

Figure 2. Problem due to duplicate retransmission of Msg2 or Msg3 feedback by reader

Reader perspective:

As shown in above figure 2, the reader can expect to receive Msg3 after sending Msg2. However, the reader does not know whether the device has successfully received Msg2 or not. Therefore, if the reader fails to receive Msg3, it may re-transmit (Re-)Msg2 to those specific devices or transmit a NACK message.

Reader does not know whether the device has successfully received Msg2 or not.

Device perspective:

The device can expect to receive Msg2 after sending Msg1. If the device successfully receives Msg2, it can transmit Msg3. Subsequently, the device can expect to receive Msg4(NACK) during the next monitoring period. Until now, the device does not know whether the reader has successfully received the transmitted Msg3 itself. If the transmission of Msg3 fails and the device receives a NACK from the reader, it should retransmit MSG3. There is no issue. However, if the transmission of MSG3 fails and the device receives (Re-)MSG2 instead of NACK from the reader, problems can occur.

Even after the device sends Msg3, it does not know whether the reader has successfully received the transmitted Msg3.

If the reader decides to re-transmit Msg2 to the specific devices that failed to transmit Msg3 (regardless of failure of Msg2 reception device perspective), the device may need a separate monitoring window to receive (re-)Msg2. Therefore, we would like to discuss the monitoring windows for receiving the (re-)Msg2 such as Random Access Response (RAR).

RAN2 to discuss the monitoring windows for receiving the (re-)Msg2 such as Random Access Response (RAR).

If the reader decides to send a NACK to the specific devices that failed to transmit Msg3, the device may need a monitoring window to receive a 1-bit or small size of NACK after sending Msg3. Therefore, we would like to discuss monitoring windows that match the NACK size.

RAN2 to discuss monitoring windows for receiving the MSG3 feedback (NACK) that match the NACK size.

3	Conclusion

In the previous sections we made the following observations:

Observation 1	Reader does not know whether the device has successfully received Msg2 or not.

Observation 2	Even after the device sends Msg3, it does not know whether the reader has successfully received the transmitted Msg3.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the monitoring windows for receiving the (re-)Msg2 such as Random Access Response (RAR).

Proposal 2	RAN2 to discuss monitoring windows for receiving the MSG3 feedback (NACK) that match the NACK size.

4	References

RAN2#128 chairman’s notes.

TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR” V19.0.0 (2024-12).