3GPP TSG RAN WG2 Meeting #129bis

Wuhan, China, 7th – 11th April, 2025                                                       R2-2502906

Agenda Item:	8.1.4

Source:	Rakuten Mobile

Title:	Discussion on UE Side Data collection

Document for:	Discussion and Decision

# Introduction

Following agreements were made in RAN2#129 on UE Side Data Collection, This contribution discusses additional aspects of UE side data collection especially focusing on how to control the UE Side data collection from NW Side.

RAN2#129 discussed UE-side data collection for the UE-side use cases (including the beam management use case), focusing on data collection configuration, and made the following agreements: - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

# Discussion

## UE-Initiated Collection Requests and NG-RAN Handling

The agreements at RAN2#129 allow the UE to initiate data collection via RRC signaling, but the structure and semantics of the request remain undefined. In practice, AI/ML data collection sessions may involve distinct measurement resource groups (e.g., CSI-RS subsets), associated model IDs, and timing preferences. Without standardized fields in the request, NG-RAN lacks sufficient context to make informed decisions about whether to authorize or reject the request.

Moreover, the NG-RAN’s policy in handling such requests is not yet fully specified. To support consistent behavior across vendors, the decision to accept, defer, or reject collection must be explicitly supported at the protocol level.

Proposal 1: Define a new RRC message AIDataCollectionRequest containing optional IEs:

- associatedID: referencing a previously configured measurement set

- collectionIntent: indicating training or monitoring

- preferredWindow: a suggested start time or duration for collection

Proposal 1-1: Extend the response to include rejectionCause and optional retryWindow. If the NG-RAN declines the request (e.g., due to cell load), the UE should not retry immediately but follow the configured backoff.

## Network-Controlled Constraints for UE-Side Data Collection

Although full scheduling of UE collection by NG-RAN is not desirable due to signaling overhead, operators need a means to define basic constraints, such as:

- How frequently UEs can collect data

- When collection is allowed

These constraints enable resource planning and reduce the risk of uplink congestion caused by overlapping data collection from many UEs.

Observation 1:
The absence of network-defined constraints for collection frequency and timing may lead to uncontrolled uplink behavior and complicate scheduling.

Proposal 2:
Introduce a new IE AIMLDataCollectionConstraint in AIMLDataCollectionConfiguration message. This IE includes:

- maxSamplesPermeaurementtime: limiting per-UE contribution

- collectionTimeWindow: defining allowed collection periods

## Differentiation and Control of AI/ML Data on the User Plane

It seem that AI/ML data collected by the UE will be transported via existing DRBs or QFI-based flows. However, these data flows are indistinguishable from other best-effort traffic. This limits the NG-RAN’s ability to apply differentiated scheduling or resource treatment. Furthermore, in cells with high uplink load, uncontrolled AI/ML uploads may interfere with latency-sensitive services.

Given that AI/ML data is usually non-time-critical, the NG-RAN should be able to delay, throttle, or deprioritize such traffic. Doing so requires some degree of identification — preferably at the SDAP layer.

Observation 2:
AI/ML data flows are opaque to NG-RAN, and cannot be deprioritized or shaped unless they are carried in dedicated DRBs, which are not always justified.

Proposal 3:
Introduce a new SDAP-level marking for AI/ML traffic using existing SDAP header extension fields (TS 38.321). Define a trafficPurpose field with values such as:

- training

- inferenceFeedback

- diagnostic

This allows the NG-RAN to apply scheduling rules that distinguish between user traffic and internal UE-generated AI data.

## Scheduling Flexibility and Congestion Control for AI/ML Uploads

UEs may collect and store large amounts of measurement data, uploading them asynchronously during idle or low-load periods, the network doesn’t have a mechanism to suggest optimal upload timing or to delay uploads dynamically during congestion. AI/ML data uploads would be able to tolerate moderate latency, making them suitable for scheduled or rate-controlled transmission — if such control mechanisms are defined..

Observation 3:
There is no RRC-level mechanism for the NG-RAN to advise the UE when to upload AI/ML data, nor any congestion signaling specific to such traffic.

Proposal 4-1: Define a new optional IE AIMLUploadWindow in AIMLDataCollectionConfiguration, specifying preferred time windows or conditions for data upload. If omitted, the UE is free to upload at its discretion.

Proposal 4-2: Introduce a new RRC/MAC-level indication AIMLUploadThrottlingIndicator, allowing the NG-RAN to dynamically reduce AI/ML upload priority during congestion periods. This can be mapped to uplink scheduling behavior or DRB suspension.

RRC Level ASN.1 AI-UploadControl ::= SEQUENCE { throttleMode ENUMERATED { noRestriction, -- Full-rate upload allowed pauseImmediately, -- UE shall stop AI/ML upload temporarily reduceRate -- UE shall throttle upload to 'maxRateKbps' }, maxRateKbps INTEGER (1..100000) OPTIONAL, -- Applicable when 'reduceRate' is selected durationSec INTEGER (1..3600) OPTIONAL, -- Duration (in seconds) for which the command applies ... }

## Maintaining Data Collection Context Across Mobility and RRC state Transitions

UEs collecting data for AI/ML training typically should not over extended time windows and may remain in the same collection session across multiple cell changes. In the current specification, there is no guarantee that the associated configuration — including measurement resources, collection intent, or associatedID — is valid in the target cell post-handover. If not maintained, the result may be an unintended collection gap, wasted radio resources, or model inconsistency.

While handover mechanisms already support transferring measurement configurations and DRB context, there is no equivalent transfer for data collection session metadata. Without such support, the UE must treat mobility as the end of one session and re-request a new one, which adds signaling and causes collection delay.

This is particularly problematic in deployments involving distributed model training, where continuity of collected data under stable measurement assumptions is essential to maintain model quality.

Observation 4:
Data collection context is not preserved across intra- mobility events, risking session fragmentation and inconsistent training inputs.

Proposal 5:
Extend the handover preparation signaling to optionally include the active AIMLDataCollectionConfiguration context, including associatedID, measurement resources, and session timing. Upon successful handover, the target cell shall either continue the session or trigger a reconfiguration.

Proposal 5-1: If context transfer is not possible define a mandatory behavior for the UE to indicate its ongoing collection state via UAI or AIMLDataCollectionStatus immediately post-handover.

## Runtime Control of Ongoing Collection Sessions

Collection sessions may be initiated through configuration, but they are not always ideal to run continuously without adaptive control. In practical scenarios, UE power conditions, cell load, or uplink scheduling capacity may vary, requiring dynamic throttling or suspension of collection.

The current discussions in RAN2 offers only start and stop through configuration. This binary approach increases signaling and limits flexibility. A lighter-weight control interface is required, especially for cases where the network prefers to momentarily pause data collection due to transient congestion or to reallocate resources to delay-sensitive services.

Operators deploying AI/ML-based features at scale would be particularly concerned about the background nature of collection traffic overwhelming scheduling queues during peak hours. The RAN should be able to suppress low-priority collection during such periods — without needing to tear down or reconfigure the entire session.

Observation 5:
There is no support for temporary suspension or resumption of collection by NG-RAN without full reconfiguration.

Proposal 6:
Introduce a lightweight RRC signaling (or MAC CE if applicable) to suspend or resume AI/ML data collection at the UE. Define expected UE behavior: buffer ongoing data, avoid measurement transmission, and resume within the active configuration window.

Proposal 6-1: Allow the NG-RAN to broadcast a cell-level AIMLCollectionSuspensionIndicator applicable to all UEs using shared configurations. This provides coarse-grain control during load surges.

# Conclusion

In summary, we make the following proposals for UE Side Data collection:

UE-Initiated Collection Requests and NG-RAN Handling

Proposal 1: Define a new RRC message AIDataCollectionRequest containing optional IEs:

- associatedID: referencing a previously configured measurement set

- collectionIntent: indicating training or monitoring

- preferredWindow: a suggested start time or duration for collection

Proposal 1-1: Extend the response to include rejectionCause and optional retryWindow. If the NG-RAN declines the request (e.g., due to cell load), the UE should not retry immediately but follow the configured backoff.

Network-Controlled Constraints for UE-Side Data Collection

Observation 1:
The absence of network-defined constraints for collection frequency and timing may lead to uncontrolled uplink behavior and complicate scheduling.

Proposal 2:
Introduce a new IE AIMLDataCollectionConstraint in AIMLDataCollectionConfiguration message. This IE includes:

- maxSamplesPermeaurementtime: limiting per-UE contribution

collectionTimeWindow: defining allowed collection periods

Differentiation and Control of AI/ML Data on the User Plane

Observation 2:
AI/ML data flows are opaque to NG-RAN, and cannot be deprioritized or shaped unless they are carried in dedicated DRBs, which are not always justified.

Proposal 3:
Introduce a new SDAP-level marking for AI/ML traffic using existing SDAP header extension fields (TS 38.321). Define a trafficPurpose field with values such as:

- training

- inferenceFeedback

- diagnostic

This allows the NG-RAN to apply scheduling rules that distinguish between user traffic and internal UE-generated AI/ML data.

Scheduling Flexibility and Congestion Control for AI/ML Uploads

Observation 3:
There is no RRC-level mechanism for the NG-RAN to advise the UE when to upload AI/ML data, nor any congestion signaling specific to such traffic.

Proposal 4-1: Define a new optional IE AIMLUploadWindow in AIMLDataCollectionConfiguration, specifying preferred time windows or conditions for data upload. If omitted, the UE is free to upload at its discretion.

Proposal 4-2: Introduce a new RRC/MAC-level indication AIUploadThrottlingIndicator, allowing the NG-RAN to dynamically reduce AI/ML upload priority during congestion periods. This can be mapped to uplink scheduling behavior or DRB suspension.

Maintaining Data Collection Context Across Mobility and RRC state Transitions

Observation 4:
Data collection context is not preserved across intra- mobility events, risking session fragmentation and inconsistent training inputs.

Proposal 5:
Extend the handover preparation signaling to optionally include the active AIMLDataCollectionConfiguration context, including associatedID, measurement resources, and session timing. Upon successful handover, the target cell shall either continue the session or trigger a reconfiguration.

Proposal 5-1: If context transfer is not possible define a mandatory behavior for the UE to indicate its ongoing collection state via UAI or AIMLDataCollectionStatus immediately post-handover.

Runtime Control of Ongoing Collection Sessions

Observation 5:
There is no support for temporary suspension or resumption of collection by NG-RAN without full reconfiguration.

Proposal 6:
Introduce a lightweight RRC signaling (or MAC CE if applicable) to suspend or resume AI/ML data collection at the UE. Define expected UE behavior: buffer ongoing data, avoid measurement transmission, and resume within the active configuration window.

Proposal 6-1: Allow the NG-RAN to broadcast a cell-level AIMLCollectionSuspensionIndicator applicable to all UEs using shared configurations. This provides coarse-grain control during load surges.

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023

- RP-243307, “Way forward on AIML UE sided data collection,” Dec 2024

- RAN2#128 meeting report

- R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1