3GPP TSG-RAN WG2 Meeting #129bis	R2-2502908

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.1.3

Source:	Rakuten Mobile

Title:	Discussion on NW-Side AI/ML Data Logging Framework

Document for:	 Discussion and Decision

# 1. Introduction

This contribution discusses Rakuten’s proposals on Enhancements related to NW Side Data collection for AI/ML Air Interface WI.

# 2. Discussion

## 2.1 Logging Content and Sampling Behavior

### 2.1.1 Defining the Logging Entry Structure

While the triggering mechanism for logging has been standardized (i.e., based on L3 events such as serving cell RSRP crossing thresholds), what the UE logs once the trigger is active remains unspecified. Logs must include beam-specific measurements, timestamps, and association with resource configuration to ensure datasets are valid across both model and deployment boundaries.

Observation 1: Logging content per sampling interval is not defined, resulting in ambiguity across UE implementations.

Proposal 1-1: Define the following fields per log entry:

| Field | Description |
| --- | --- |
| Timestamp | Time of measurement (in ms or system SFN) |
| CSI-RS/SSB Resource ID | Identifier of the measured resource |
| L1-RSRP | Power level measured on the resource |
| Beam ID (optional) | Physical beam identifier if resolvable by the UE |
| Config Reference ID | Associated CSI-ReportConfig ID (or MeasObject ID) |

Proposal 1-2: Extend MDT’s LoggedMeasurementReport container to include these fields, ensuring reuse where possible but adding AI-relevant identifiers as needed.

### 2.1.2 Sampling Control Within Trigger Period

To provide balance between consistency and configurability, RAN2 should support both fixed sampling intervals and alignment with CSI-RS periodicity. The UE should default to CSI-RS alignment unless instructed otherwise.

Observation 2: Without standardized sampling intervals, training data becomes inconsistent across devices and configurations.

Proposal 2-1: Introduce loggingSamplingInterval in RRC configuration. Values may include:

| Value | Description |
| --- | --- |
| alignWithCSI_RS | One sample per CSI-RS transmission |
| fixed_20ms | Log every 20 ms |
| fixed_80ms | Log every 80 ms (etc., as needed by NW) |

Proposal 2-2: If the interval is not configured, UE defaults to CSI-RS alignment.

### 2.1.3 DL-PRS Logging for Positioning Model Training

While beam prediction has been the dominant use case driving logging discussions, LMF-side positioning models now introduce new requirements. Unlike beam management, positioning models depend heavily on precise timing offset measurements (e.g., RSTD) and PRS resource mapping. These are not captured by CSI-ReportConfig and cannot be expressed using current MDT log containers.

Moreover, the nature of PRS logging is inherently tied to downlink timing behavior and must reflect transmitter-specific metadata (e.g., TRP ID) to be useful for positioning inference.

Observation 2-3: Current log formats do not support DL-PRS measurements, limiting support for LMF-side model training.

Proposal 2-3: Define a DLPRSLoggingEntry structure with the following fields:

Proposal 2-4: Extend UEInformationResponse to optionally include a list of DLPRSLoggingEntry records, tagged with dataCollectionPurpose = positioning.

| Field | Description |
| --- | --- |
| PRS Resource ID | Identifier for the DL-PRS instance |
| Timing Offset | RSTD or equivalent timing delta |
| Signal Quality | RSRP or SINR on PRS resource |
| TRP ID (Optional) | Optional; TRP/Cell ID or PCI |
| Timestamp | Sampling time (SFN or subframe aligned) |

## 2.2 Beam Filtering and Granularity Control

### 2.2.1 Motivation for Beam Filtering

In realistic deployments, Set A/B configurations may include dozens of CSI-RS resources. Logging all beams per sampling interval can overwhelm the UE buffer. More importantly, many beams yield low-value data — e.g., those with low RSRP or low spatial diversity.

Training pipelines are more efficient when logs focus on dominant beams or those within a meaningful delta (e.g., within 3 dB of the top beam). Offloading this filtering to post-processing is inefficient; it makes more sense to filter at the point of capture.

Observation 3: Without beam filtering, logging volume becomes unbounded and model utility degrades.

Proposal 3-1: Introduce maxLoggedBeamsPerSample in RRC configuration to constrain the number of beam measurements the UE retains per sample.

Proposal 3-2: Configure the filtering mode:

| Filter Mode | Description |
| --- | --- |
| topKOnly | Retain only top-K beams by RSRP |
| topKPlusMargin | Retain beams within X dB of top beam (configurable delta) |
| configuredSubset | Log only beams explicitly identified by the NW |

## 2.3 Dataset Purpose and Multi-Use Case Labeling

### 2.3.1 Need for Purpose Tagging

The framework must support scenarios where a UE is logging data for multiple AI/ML use cases simultaneously — e.g., downlink beam management, positioning via PRS, or CSI reconstruction. These use cases differ in how logs are post-processed, labeled, and fed into model pipelines. Without tagging, the same log data could be routed to incompatible models, leading to poor accuracy or training failure.

MDT implicitly assumed a single OAM-related purpose. With AI/ML, the context must be explicit. We therefore propose adding a small, fixed-length field to tag each logging session with its intended training purpose.

Observation 4: Logs may be misrouted to the wrong model pipeline without dataset categorization.

Proposal 4: Introduce dataCollectionPurpose (4-bit) in logging config and report messages.

| Value | Purpose |
| --- | --- |
| 0000 | Beam Prediction |
| 0001 | Positioning |
| 0100 | CSI Feedback |
| 0110 | Multi-Purpose |
| 1xxx | Vendor-defined |

## 2.4 Session Management and Interruption Recovery

### 2.4.1 Anchoring and Resumption

RAN2 agreed that the UE may retain logs post-HO and report availability, but there is no way today to track or resume sessions cleanly. This presents a problem for AI pipelines where datasets must be stitched together and session-aware for time-sequence learning.

A simple solution is to introduce a session ID and allow the UE to mark resumption points explicitly, e.g., after recovery from IDLE or RLF. This avoids silent discontinuity and supports re-assembly of fragmented sessions.

Observation 5: No currently agreed mechanism tracks session continuity, leading to ambiguous or duplicated datasets.

Proposal 5-1: Introduce a loggingSessionID configured per session and included in all logs and reporting messages.

Proposal 5-2: UE shall insert a logResumeMarker in logs after RLF or HO, optionally with cause (e.g., HO,RLF, IDLE_RESUME).

### 2.4.3 Logging Capability Advertisement by the UE

Unlike legacy MDT, where capabilities were assumed uniform across UEs, AI/ML logging use cases demand a more flexible configuration approach. Some UEs can support multiple sessions; others cannot. Some can log at 20 ms intervals; others may only support 80 ms or longer. Without declaring these limits upfront, the network risks configuring sessions that fail silently.

Moreover, with beam-level filtering, multi-session logging, and data availability indicators now in play, it becomes increasingly important for the NW to adapt logging strategies based on UE capacity.

Observation 7: There is currently no way defined for the NW to query the UE’s logging-related limitations, which risks over-configuration and failure to capture datasets.

Proposal 7-1: Extend UE Capability signaling with the following fields:

| Capability Field | Description |
| --- | --- |
| maxConcurrentLoggingSessions | Maximum number of simultaneous logging sessions |
| loggingBufferSizeBytes | Max size of log buffer |
| supportedLoggingCategories | Bitmap of AI/ML training categories supported |
| supportedSamplingIntervals | List of supported sampling intervals |
| maxBeamsPerSample | Max number of beams per sampling interval |

### 2.4.4 Multi-Category Logging Support

As AI use cases multiply, it is increasingly likely — that a UE may be configured to log multiple datasets concurrently. For example, a UE may be configured to log:

- Beam power data for gNB-side model training

- PRS measurements for LMF-side positioning model training

- CSI feature sets for future channel reconstruction models

The current assumption that a single logging session suffices no longer holds. To maintain clarity in dataset management and model separation, RAN2 should support distinct configurations per dataset purpose.

Observation 8: There is no currently defined mechanism to configure or manage multiple AI/ML logging sessions concurrently in a scoped, clean way.

Proposal 8-1: Define LoggingConfigurationSet, allowing configuration of multiple logging sessions, each with:

| Field | Description |
| --- | --- |
| Session ID | Unique identifier for this configuration |
| dataCollectionPurpose | Tag for beam prediction, CSI, positioning |
| Sampling Interval | Per-session logging rate |
| Resource Scope | List of CSI-RS/SSBs to log, |

Example Table:

| Session ID | Purpose | Interval | Scope |
| --- | --- | --- | --- |
| 0x01 | Beam Prediction | 40 ms | Set A beams |
| 0x02 | Positioning | 80 ms | DL-PRS list |

Proposal 8-2: Extend UEInformationResponse and UEAssistanceInformation to include per-session report flags and availability indication.

## 2.5 Extensibility via Vendor Extension Container

Vendors may wish to log additional modem-specific fields, preprocessed metrics, or intermediate inference results as part of internal model training workflows. Rather than fragmenting the standard or forcing workarounds, we propose including an optional container in the logging structure reserved for vendor use.

To preserve interoperability, this container must be self-identifying and ignored by systems that do not recognize it.

Observation 9: Rigid logging formats discourage proprietary model development and experimentation.

Proposal 9-1: Define a vendorExtensionContainer in the log entry structure, which is optional and ignored unless recognized.

# 3. Conclusion

In this contribution, we provide our views for Rel-19 NW-side AI/ML Data Logging Framework, and the observations and proposal are summarized as follows:

Logging Content and Sampling Behavior

Observation 1: Logging content per sampling interval is not defined, resulting in ambiguity across UE implementations.

Proposal 1-1: Define the following fields per log entry:

| Field | Description |
| --- | --- |
| Timestamp | Time of measurement (in ms or system SFN) |
| CSI-RS/SSB Resource ID | Identifier of the measured resource |
| L1-RSRP | Power level measured on the resource |
| Beam ID (optional) | Physical beam identifier if resolvable by the UE |
| Config Reference ID | Associated CSI-ReportConfig ID (or MeasObject ID) |

Proposal 1-2: Extend MDT’s LoggedMeasurementReport container to include these fields, ensuring reuse where possible but adding AI-relevant identifiers as needed.

Observation 2-1: Without standardized sampling intervals, training data becomes inconsistent across devices and configurations.

Proposal 2-1: Introduce loggingSamplingInterval in RRC configuration. Values may include:

| Value | Description |
| --- | --- |
| alignWithCSI_RS | One sample per CSI-RS transmission |
| fixed_20ms | Log every 20 ms |
| fixed_80ms | Log every 80 ms (etc., as needed by NW) |

Proposal 2-2: If the interval is not configured, UE defaults to CSI-RS alignment.

Observation 2-2: Current log formats do not support DL-PRS measurements, limiting support for LMF-side model training.

Proposal 2-3: Define a DLPRSLoggingEntry structure with the following fields:

Proposal 2-4: Extend UEInformationResponse to optionally include a list of DLPRSLoggingEntry records, tagged with dataCollectionPurpose = positioning.

| Field | Description |
| --- | --- |
| PRS Resource ID | Identifier for the DL-PRS instance |
| Timing Offset | RSTD or equivalent timing delta |
| Signal Quality | RSRP or SINR on PRS resource |
| TRP ID (Optional) | Optional; TRP/Cell ID or PCI |
| Timestamp | Sampling time (SFN or subframe aligned) |

Beam Filtering and Granularity Control

Observation 3: Without beam filtering, logging volume becomes unbounded and model utility degrades.

Proposal 3-1: Introduce maxLoggedBeamsPerSample in RRC configuration to constrain the number of beam measurements the UE retains per sample.

Proposal 3-2: Configure the filtering mode:

| Filter Mode | Description |
| --- | --- |
| topKOnly | Retain only top-K beams by RSRP |
| topKPlusMargin | Retain beams within X dB of top beam (configurable delta) |
| configuredSubset | Log only beams explicitly identified by the NW |

Dataset Purpose and Multi-Use Case Labeling

Observation 4: Logs may be misrouted to the wrong model pipeline without dataset categorization.

Proposal 4: Introduce dataCollectionPurpose (4-bit) in logging config and report messages.

| Value | Purpose |
| --- | --- |
| 0000 | Beam Prediction |
| 0001 | Positioning |
| 0100 | CSI Feedback |
| 0110 | Multi-Purpose |
| 1xxx | Vendor-defined |

Session Management and Interruption Recovery

Observation 5: No current mechanism tracks session continuity, leading to ambiguous or duplicated datasets.

Proposal 5-1: Introduce a loggingSessionID configured per session and included in all logs and reporting messages.

Proposal 5-2: UE shall insert a logResumeMarker in logs after RLF or HO, optionally with cause (e.g., HO,RLF, IDLE_RESUME).

Observation 7: There is currently no way for the NW to query the UE’s logging-related limitations, which risks over-configuration and failure to capture datasets.

Proposal 7-1: Extend UE Capability signaling with the following fields:

| Capability Field | Description |
| --- | --- |
| maxConcurrentLoggingSessions | Maximum number of simultaneous logging sessions |
| loggingBufferSizeBytes | Max size of log buffer |
| supportedLoggingCategories | Bitmap of AI/ML training categories supported |
| supportedSamplingIntervals | List of supported sampling intervals |
| maxBeamsPerSample | Max number of beams per sampling interval |

Observation 8: There is no current mechanism to configure or manage multiple AI/ML logging sessions concurrently in a scoped, clean way.

Proposal 8-1: Define LoggingConfigurationSet, allowing configuration of multiple logging sessions, each with:

| Field | Description |
| --- | --- |
| Session ID | Unique identifier for this configuration |
| dataCollectionPurpose | Tag for beam prediction, CSI, positioning |
| Sampling Interval | Per-session logging rate |
| Resource Scope | List of CSI-RS/SSBs to log |

Proposal 8-2: Extend UEInformationResponse and UEAssistanceInformation to include per-session report flags and availability indication.

Extensibility via Vendor Extension Container

Observation 9: Rigid logging formats discourage proprietary model development and experimentation.

Proposal 9-1: Define a vendorExtensionContainer in the log entry structure, which is optional and ignored unless recognized.

# 4 References