3GPP TSG-RAN2 Meeting #129-bis	R2-2502911

Wuhan, China, 7 - 11 April 2025 	Revision of R2-2501094

Agenda Item:	8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS and RRM measurements

Document for:	Discussion and Decision

# Introduction

In this contribution the open issues for LP-WUS and RRM measurements are discussed further.

For the RAN2 and RAN4 agreements in previous meetings the reader is referred to the Annex.

# Discussion

## MR serving cell offloading and further relaxation

RAN4 agreed to study different use cases for MR serving cell offloading (case #1) and relaxation (case #2 and #3):

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

The WID objective also includes further MR serving cell relaxation and offloading (RP-241824):

- Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

Case 1 (MR serving cell fully offloading)

RAN2 has agreed that for the entry condition of fully offloading (RAN4 case #1) an MR threshold is configured, and optionally an LR threshold (see annex). But RAN2 did not explicitly agree that when the entry condition is fulfilled and the LR is performing serving cell measurements, that the MR is then not required to perform serving cell measurements. This aspect is essential and needs to be captured in the specification:

When the LR entry condition for serving cell offloading is fulfilled and the LR performs serving cell measurements, then the MR is not required to perform serving cell measurements (i.e. RAN4 case #1 is supported).

Case 3 (MR serving cell relaxation with MR neighbour cell relaxation)

Similar as with RAN4 case #2, the relaxed MR serving cell measurements could be used as a safety precaution if WUR accuracy is too low. I.e. less frequent exit-condition checks are done by MR to ensure the UE has not moved to too poor coverage to be reachable by LP-WUS when it is only monitoring LP-WUS (and hence becomes unreachable).

If such safety precaution is not considered, the serving cell exit condition is solely dependent on LR measurements, i.e. does not require MR measurements. When the MR performs relaxed intra-frequency measurements, i.e. MR serving cell measurements come for free in such case. But this is not the case for relaxed inter-frequency neighbour cell measurements.

There can be use cases where the MR and LR have to perform serving cell measurements simultaneously. For example, when MR measurements are used to retune the LR timing/frequency e.g. when the LR quality is bad, or when both MR and LR measurements are used for the entry condition. But in such cases the MR performs serving cell measurements continuously every DRX cycle, i.e. they are not relaxed.

Perhaps relaxed MR serving cell measurements performed during intra-frequency neighbour cell measurements can be used to assist the LR exit condition. How to use these MR serving cell measurements can be left to UE implementation. Furthermore, it is assumed that cell reselection is performed based on MR measurements only and for cell reselection both serving and neighbour cell measurements are required.

RAN2 to discuss the use of relaxed MR serving cell measurements of RAN4 case#3 (e.g. to assist with exit condition evaluation).

RAN2 to confirm that cell (re-)selection is based on MR measurements.

## Entry/exit condition for MR serving cell offloading

RAN2 has been discussing entry/exit condition for MR serving cell offloading separately from entry/exit condition for LP-WUS monitoring. In both cases, MR and optionally LR measurements are used for entry. And in both cases LR measurements are used for the exit condition.

There is no power saving gain when the entry condition for RRM offloading is fulfilled but the entry condition for LP-WUS monitoring is not fulfilled, i.e. the MR still has to wake-up every DRX cycle to monitor paging. The same applies when entry for LP-WUS monitoring is fulfilled, but not for serving cell offloading.

There is no good reason for why the entry/exit condition for offloading and LP-WUS monitoring should be different, i.e. the same type of physical signal is used (LP-SS and LP-WUS):

The same entry/exit condition is used for both serving cell measurement offloading and LP-WUS monitoring.

RAN2 has not agreed explicitly on the UE requirements when the exit condition is fulfilled:

When the LR exit condition is fulfilled, then the UE shall stop using LR measurements and the UE is required to start MR serving cell measurements.

The NW can optionally configure an LR threshold in addition to the MR threshold for entry. In case an LR threshold is configured it can be left to UE implementation when to start/stop these LR measurements:

If an LP-SS threshold is configured for the entry condition, it is left to UE implementation when to start/stop the LP-SS measurements.

## Further MR neighbour cell relaxation

RAN2 agreed to use the Rel-16 relaxation criteria (“not at cell edge” and “low mobility”) as a baseline for further neighbour cell relaxation in Rel-19:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

With Rel-16 neighbour cell relaxation a scaling factor of 3 is used. RAN4 agreed to use a scaling factor ≥ 4 in Rel-19 for further MR neighbour cell relaxation, see the annex.

### The need for intra-frequency neighbour cell measurements

In legacy the neighbour cell measurements are configured using intra-frequency (Sintrasearch) and inter-frequency (SnonIntraSearch) measurement thresholds. Typically the UE has to perform intra-frequency neighbour cell measurements in the complete cell and inter-frequency measurements are only triggered when the UE is near the cell border:

Practically the cell border is determined when the UE performs intra-frequency cell reselection, e.g. a neighbour becomes 4 dB better than the serving cell. In case the UE does not find a better intra-frequency neighbour and ends up in poor coverage, then the UE is triggered to perform inter-frequency measurements and find a suitable cell on another frequency. When the UE finds a suitable cell on a higher priority frequency during background measurements, then the UE can leave the serving cell at any time.

It is preferred that the UE performs intra-frequency cell reselection for mobility (shorter interruption and missed paging probability) and that inter-frequency measurements (which take longer time and consume more power) are deferred after intra-frequency measurement attempts are “exhausted”.

In urban environments the intra-frequency (Sintrasearch) cannot be configured close to the inter-frequency (SnonIntraSearch) measurement threshold, because due to the radio propagation conditions a stronger intra-frequency neighbour cell may be detected close to the cell center:

In urban environments the radio conditions may also change rapidly and dramatically, e.g. when you turn a corner. In case of multi-beam operation the cell coverage may also be beam dependent, i.e. beams of the same cell provide different quality, which may trigger intra-frequency cell reselection:

There can be scenarios where the UE has to measure intra-frequency neighbour cells most of the time and these intra-frequency measurements cannot be relaxed.

For this reason it is beneficial to discuss neighbour cell measurements performed by (OFDM-based) LR, i.e. for the use case when intra-frequency neighbour cell measurements are needed and cannot be relaxed. If the LR performs intra-frequency measurements and ranking then the MR is only woken up when cell reselection is initiated, i.e. the MR is only activated when an intra-frequency neighbour is Q-hyst (db) better than the serving cell based on LR measurements:

RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by (OFDM-based) LR in scenarios where the UE continuously needs to perform intra-frequency measurements and these measurements cannot be relaxed.

### Rel-16 relaxed neighbour cell measurements

Neighbour cell relaxation and LP-WUS coverage

Using the legacy measurements thresholds (SIntraSearch, SNonIntraSearch) and Rel-16 threshold (Not-at-cell-edge) different areas can be identified with different measurement requirements, which also depend on the UE mobility (lowMobility):

The LP-WUS coverage depends on the entry/exit conditions which are independent from the neighbour cell measurements criteria and areas depicted above. The LP-WUS coverage can cover the complete cell or only a small portion in the cell centre independent from the neighbour cell measurement areas.

It is obvious that the legacy neighbour cell measurement requirements apply when the LP-WUS UE is outside LP-WUS coverage or the cell does not support LP-WUS. But also when the LP-WUS UE is inside the LP-WUS coverage area the legacy neighbour cell measurement requirements apply, because having an LR that can perform serving cell measurements does not justify that the LP-WUS UE can (further) relax neighbour cell measurements:

The neighbour cell measurement requirements are decoupled from the LP-WUS entry/exit conditions and LP-WUS coverage area.

More specifically this means: when the UE that is within LP-WUS coverage but the UE is moving around, especially towards the cell border, then the UE is not allowed to relax neighbour cell measurements and benefit from LP-WUS power savings.

Neighbour cell relaxation and LR serving cell measurements

In case the serving cell offloading entry condition is fulfilled and only the LR is performing serving cell measurements, then the UE determines, based on the LR serving cell measurements, when the MR has to trigger neighbour cell measurements again, i.e. when low mobility and/or not-at-cell-edge criterion are no longer fulfilled:

When the MR serving cell offloading entry condition is fulfilled, the UE determines based on the LR serving cell measurements whether low mobility and/or not-at-cell-edge criterion are fulfilled.

The low mobility and/or not-at-cell-edge criterion are not part of the LR serving cell exit condition, i.e. the UE is allowed to continue offloading the serving cell measurements from MR to LR when the MR is performing neighbour cell measurements.

The OFDM-based serving cell measurements using PSS/SSS are quite in performance compared to the MR serving cell measurements, if not identical:

Ask RAN4 whether the Rel-16 neighbour cell relaxation configuration (relaxedMeasurement-r16) can be re-used with (OFDM-based) LR serving cell measurements. Or if separate parameter value(s) need to be configured with (OOK-based) LR serving measurements.

### Rel-19 further relaxed neighbour cell measurements

RAN4 started to discuss scaling factors for neighbour cell measurements larger than 3 used in Rel-19. For example scaling factors ≥ 8, which allow the MR to enter ultra-deep sleep.

In the figures below it is exemplified how Rel-19 relaxation (i.e. new scaling factors) fits into the legacy measurement requirements, for the case when Rel-19 relaxation is configured above and below the inter-frequency measurement threshold:

However, it is not clear what the added value of Rel-19 further relaxation is because:

When the UE is stationary and above the not-at-cell-edge threshold (Rel-16) then the UE is not required to measure intra- or inter-frequency neighbour cells, i.e. the scaling factor is infinite, i.e. much better than the Rel-19 scaling factor.

And as discussed before in certain scenarios further relaxation is not feasible. For example, in an urban environment where the coverage is very unpredictable and irregular and the propagation conditions can suddenly dramatically change. In such an environment relaxation can only be allowed for a stationary UE, that while it was mobile did not find a better intra-frequency neighbour cell:

There is a risk that paging/cell reselection is impacted when a moving UE does not measure neighbour cells for 10-20 seconds or more (i.e., scaling factor between 8 and 16 with DRX cycle 1.28 sec).

It is not clear why neighbour cell measurements can be relaxed for such a long time for a mobile UE, except that such a long time is needed to support ultra-deep sleep and for LP-WUS to be beneficial. In certain scenarios a moving UE cannot be allowed to relax neighbour cell measurements for such a long time:

It is up to the network implementation to configure LP-WUS with or without Rel-19 further neighbour cell relaxation for a moving UE. The Rel-19 scaling factors are further discussed in RAN4.

Higher priority frequency measurements are essential for load balancing, and they can already be relaxed up to 1 hour with Rel-16 relaxation:

Higher priority frequency measurements are not further relaxed.

## LP-SS measurement accuracy and latency

In legacy the UE typically measures the serving cell every DRX cycle (e.g. DRX cycle 1.28 sec), and it takes the average of 2 RSRP/RSRQ measurement samples made during consecutive DRX cycles, see 38.133:

4.2.2.2	Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

-	otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

Thus the DRX cycle determines the latency in which the UE detects when the serving cell exceeds or drops below a threshold.

For LP-SS measurements RAN1 agreed that the accuracy requirements should be met within 1.28 sec:

Agreement

For LP-SS design from RAN1 perspective, consider at least the following as the design target:

- For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s.

  - FFS: X

  - Note: Y is chosen for evaluating LP-SS design.

  - Network overhead and network power consumption are to be considered

In case of OOK-based WUR and LP-SS the measurement latency is independent of the DRX cycle. The assumption is that the UE is required measure 5 samples of LP-SS measurements that are spaced 320 ms apart:

This means that if the DRX cycle is 320 ms, then the LR measurements are 2x slower compared to the MR measurements. RAN1 agreed to support 320 ms periodicity, and other periodicities are FFS. And RAN4 agreed that the measurement delay depends on the LP-SS periodicities:

Agreement

For LP-SS periodicity, support at least 320ms.

- FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Agreement:

Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

RAN4 has been discussing how to capture the LP-SS measurement accuracy requirements, e.g. as dedicated accuracy requirements, potentially including test cases, or as a cell reselection margin (see legacy 3dB margin below) to the core requirements:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

In legacy the UE is required to be able to determine that an intra-frequency neighbour cell is 3 dB better within Tevaluate,NR_Intra in FR1. For LP-WUS a larger margin would need to be defined, and this margin may depend on the LP-SS periodicity. A larger cell reselection margin implies that cell reselection is delayed:

RAN2 to discuss the impact of an increased cell reselection margin when LP-SS measurements or Rel-19 further RRM relaxation are configured.

# Summary

Observation 1	There can be scenarios where the UE has to measure intra-frequency neighbour cells most of the time and these intra-frequency measurements cannot be relaxed.

Observation 2	When the UE is stationary and above the not-at-cell-edge threshold (Rel-16) then the UE is not required to measure intra- or inter-frequency neighbour cells, i.e. the scaling factor is infinite, i.e. much better than the Rel-19 scaling factor.

Observation 3	There is a risk that paging/cell reselection is impacted when a moving UE does not measure neighbour cells for 10-20 seconds or more (i.e., scaling factor between 8 and 16 with DRX cycle 1.28 sec).

Proposal 1	When the LR entry condition for serving cell offloading is fulfilled and the LR performs serving cell measurements, then the MR is not required to perform serving cell measurements (i.e. RAN4 case #1 is supported).

Proposal 2	RAN2 to discuss the use of relaxed MR serving cell measurements of RAN4 case#3 (e.g. to assist with exit condition evaluation).

Proposal 3	RAN2 to confirm that cell (re-)selection is based on MR measurements.

Proposal 4	The same entry/exit condition is used for both serving cell measurement offloading and LP-WUS monitoring.

Proposal 5	When the LR exit condition is fulfilled, then the UE shall stop using LR measurements and the UE is required to start MR serving cell measurements.

Proposal 6	If an LP-SS threshold is configured for the entry condition, it is left to UE implementation when to start/stop the LP-SS measurements.

Proposal 7	RAN2 to discuss the need for intra-frequency neighbour cell measurements performed by (OFDM-based) LR in scenarios where the UE continuously needs to perform intra-frequency measurements and these measurements cannot be relaxed.

Proposal 8	The neighbour cell measurement requirements are decoupled from the LP-WUS entry/exit conditions and LP-WUS coverage area.

Proposal 9	When the MR serving cell offloading entry condition is fulfilled, the UE determines based on the LR serving cell measurements whether low mobility and/or not-at-cell-edge criterion are fulfilled.

Proposal 10	The low mobility and/or not-at-cell-edge criterion are not part of the LR serving cell exit condition, i.e. the UE is allowed to continue offloading the serving cell measurements from MR to LR when the MR is performing neighbour cell measurements.

Proposal 11	Ask RAN4 whether the Rel-16 neighbour cell relaxation configuration (relaxedMeasurement-r16) can be re-used with (OFDM-based) LR serving cell measurements. Or if separate parameter value(s) need to be configured with (OOK-based) LR serving measurements.

Proposal 12	It is up to the network implementation to configure LP-WUS with or without Rel-19 further neighbour cell relaxation for a moving UE. The Rel-19 scaling factors are further discussed in RAN4.

Proposal 13	Higher priority frequency measurements are not further relaxed.

Proposal 14	RAN2 to discuss the impact of an increased cell reselection margin when LP-SS measurements or Rel-19 further RRM relaxation are configured.

# Annex: Agreements

## RAN2 agreements (selection)

On serving cell RRM relaxation criteria

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements.

If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

## RAN4 agreements (selection)

Issue 1-1-3-1: On RRM requirements to be specified for OOK-based LP-WUR based on LP-SS with overlaid OFDM sequence

Agreement:

- In Rel-19, the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not.

Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence

Agreement:

- Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS

  - Reuse the same requirement for LP-SS OOK based

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Agreement:

At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

Agreement:

When both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately.

MR and LR evaluation requirements are defined separately.

Issue 1-2-4-2-2: On single or multiple LR evaluation requirements

Agreement:

Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases.

Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Agreement:

LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y.

Number x and y are defined as measurement requirement as  [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

Sub-topic 1-3 MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Agreement:

- MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

Sub-topic 2-1 On simulation assumptions and parameters

Issue 2-1-1: SINR setting

Agreement:

Include -6 dB SINR in simulation assumptions SINR and -6 dB SINR is low priority and depending on company input;

Treat -0.5dB and 2dB SINR as low priority cases;

Issue 2-1-5: On ideal LP-RSRP/RSRQ in simulation

Agreement:

Use on method 1 in P2 as a base further check the difference between method 1 and method 2 in P2 and check the channel estimation assumption in ideal RSRP for OOK based LR

Issue 2-1-6: On SCS in simulation

Agreement:

Using SCS = 30KHz as the base in the simulation

Issue 2-1-7: On OOK interference

Agreement: Define a specific LP-SS binary sequences of the serving and  interference cell for simulation alignment.