3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502915

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SSB SCell operation

WID/SID:	Netw_Energy_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we will discuss the aspects related to the on-demand SSB SCell operation which is one of the objectives of the R19-network energy savings work item

# 2	On-demand SSB SCell Operation

## 2.1	Scenarios and signalling

RAN1 has made following agreements regarding on-demand SSB:

Agreement RAN1#116 Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement RAN1#116bis For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signaling are applicable

Based on the agreements in RAN1#117, RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission. Our current understanding is these signaling are applicable to Scenario#2, Scenario#2A as depicted in the figure 1 below, where:

- For Scenario#2: the on-demand SSB transmission can be indicated by gNB at the time instance T1 (when UE receives SCell configuration via RRC message) or during the time period between T1 and T2.

- For Scenario#2A: the on-demand SSB transmission can be indicated by gNB at the time instance T2 (when UE receives SCell activation via MAC CE command). From RAN1 discussion it is not clear whether this would include scenario where SCell is directly activated when configured.

For the RAN2 discussion we prefer to consider Scenario#2 and Scenario#2A for both Case#1(without always-on SSB) and Case#2(with always-on SSB).

Figure 1: Scenarios and indication for on-demand SSB transmission.

RAN2 could possibly start the discussion on how the on-demand SSB configurations that can be provided to the UE to support these scenarios.

Observation 1: During the RAN1#117 meeting, both RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

## 2.2	On-demand SSB configuration

Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

During the RAN1#120, it has been agreed to introduce higher layer parameters for on-demand SSB configuration both for Case#1 and Case#2.

In general, on-demand SSB configuration may be Cell-specific and provided via the Serving Cell Configuration. As per our understanding, the set of parameters required for the on-demand SSB operation may be kept minimal. As indicated in Figure 3 two options may be considered.

- the minimal set consisting of mandatory parameters for OD-SSB operation that can work on top of the legacy SSB configuration.

- the full set of parameters for OD-SSB operation that would replace the legacy SSB configuration.

Figure 2: On-demand SSB configuration structure

For example, for Case#1 where no always-on SSB is configured, the OD-SSB configuration may contain full set of parameters. Whereas for Case#2, OD-SSB configuration may contain minimal set and rest of the parameters will be same as the always-on SSB configuration.

Also, the configuration for OD-SSB is provided via ServingCell configuration which already carried the Physical Cell ID, we do not see a need to have physical cell ID to be provided as part of the OD-SSB configuration. It may be difficult in RAN WGs to limit which parameters can have different values thus it seems most secure way to start RAN2 signaling so that for each instance of OD-SSB can signal all the parameters and then just have those as optional and leave it to network to decide which parameters are feasible to be changed.

RAN1 is discussing the parameters that may be provided as a list – This seems to be merely a signaling optimization thought in RAN1 and does not actually relate to actual UE behaviour. Such a discussions should take place in RAN2 – not RAN1. From the RAN2 perspective, it might be simpler to support multiple of OD-SSB configuration per SCell where each configuration may contain any combination of parameters. The identifier of this configuration may be used in the MAC-CE, this would reduce the MAC-CE size.

SSB-Pattern-Config :: SEQUENCE {

pattern-id INTEGER (1..M)

ssb-Periodicity-r19 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1},

activeDuration ENUMERATED {sf5, sf10, sf20, sf40, sf80, sf160, sf320, sf640} OPTIONAL,

numberOfBurst INTEGER (1..N) OPTIONAL

ssb-Frequency ARFCN-ValueNR,

subcarrierSpacing SubcarrierSpacing,

ssb-PositionsInBurst CHOICE {

shortBitmap BIT STRING (SIZE (4)),

medium Bitmap BIT STRING (SIZE (8)),

longBitmap BIT STRING (SIZE (64))

} OPTIONAL,

ss-PBCH-BlockPower INTEGER (-60..50) OPTIONAL,

sfn-Offset INTEGER (0..1023)  OPTIONAL,

halfFrameIndex ENUMERATED {zero, one} OPTIONAL,

}

As an alternative, the parameters that may not change example frequency, SS-PBCH-Power, sfn-Offset, subcarrierSpacing may be considered common across multiple configurations and the parameters that may be configured with multiple values may be grouped and indicated using an identifier. But for simplicity it seems easiest that we start with assumption that each set of SSB configurations have each value present.

Proposal 1 : RAN2 starts signaling design so that each parameter listed by RAN1 can be signalled for any instance of OD-SSB configuration.

If there is need to optimize signaling overhead – which does not seem critical – we can later consider if we e.g. can signal delta configuration to previous instance of OD-SSB configuration.

Proposal 2: Possible signaling optimizations can be down prioritized in RAN2 work.

It should be though noted that in the RAN1 parameter list they have indicated e.g. PCI of the cell which is implicitly covered by RAN2 signaling – assuming that OD-SSB configuration is cell specific. Thus:

Proposal 3 : OD-SSB configuration is Cell-specific and Physical cell id need not be configured for OD-SSB and follows the serving cell configuration.

It is worth noting that the objective for SSB adaptation also considers the RRC configuration enhancements and it may be possible that the on-demand SSB configuration framework is also applicable to the SSB adaptation. RAN2 may take this into account while discussing the RRC configurations for on.demand SSB.

## 2.3	On-demand SSB indication via MAC-CE

From previous RAN2 discussions, we have the following agreement:

Agreements on OD-SSB Scell (RAN2 #127) OD-SSB transmission indication: - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Agreements on OD-SSB Scell (RAN2 #127-bis) - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE

Further for MAC CE design, RAN1 has agreed to support OD-SSB activation before SCell activation as well as upon SCell activation and also the possibility of configuring multiple configurations at least containing periodicity of the OD-SSB, sfn-offset, half-frame-index, but whether also for any other parameters is still FFS.

From RAN2 signalling point of view, the simplest way would be to indicate in the MAC CE the SSB configuration IDs configured in RRC and the RRC configuration can accommodate any parameter that RAN1 concludes to support multiple candidate values.

Similar to other MAC CEs with SCell bitmap, two MAC CEs with one octet bitmap and four octet bitmap can be defined with different eLCID. Which MAC CE to be used could be depending on whether the highest SCell index is larger than 8 or not.

Proposal 4: The different periodicities (and any other parameters that may be adaptive) are configured as different OD-SSB configurations in RRC and MAC indicates which OD-SSB configuration ID to be activated, which is forward compatible if other parameters also introduced in RAN1.

Proposal 5: The new flexible size MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated SSB configuration ID for each SCell.

It should be also noted that in AI 8.5.4 for SSB adaptation in time domain, quite similar behaviour can be considered as for OD-SSB i.e. by merely just changing the periodicity of the SSB burst and not fully turning on/off the SSB. From RAN2 perspective these two features could be rather same in the end i.e., different SSB periodicity is activated with MAC CE thus it might be good idea to consider these two features simultaneously when designing MAC CE.

Proposal 6: Take into account in MAC CE design for OD-SSB also SSB adaptation in time domain feature for activating different periodicities

## 2.4	On-demand SSB indication via RRC

Agreement RAN1#118 For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer Agreement RAN1#120 For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

As per the RAN1 agreements, it should be possible to indicate the (de)activation of OD-SSB transmission via RRC. Our understanding is an additional parameter may be required to indicate the activation status especially when multiple OD-SSBs may be configured.

Proposal 7 : Include additional activation status parameter to indicate if OD-SSB configuration is activated or not when SSB is configured.

As per the above RAN1 agreements, our understanding is that direct SCell activation with on-demand SSB is supported.

Proposal 8 : RAN2 to confirm that direct SCell activation with on-demand SSB activation via RRC is supported.

On L3 measurements based on on-demand SSB, RAN1 thinks that RAN2 should lead the discussions. Some agreements were made during RAN2#127 meeting and RAN2 is required to study how UE performs L3 measurements for OD-SSB.

Agreements on OD-SSB Scell (RAN2 #127) Measurement on OD-SSB: - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration

Our understanding is that on-demand SSB based L3 measurements and reporting may depend on the on-demand SSB pattern.

## 2.5	On-demand SSB and SMTC

The MeasConfig framework, shown in Figure 3, configures the MeasObject. This object defines the SSB frequency to measure and the timing for the measurement is provided by the SMTC1.

For case#2, when always-on SSB and on-demand SSB are transmitted on the same frequency, the UE measures ssbFrequency in SMTC1 as per the legacy behaviour. When on-demand SSB is activated via MAC-CE, the SMTC associated with the on-demand SSB configuration may be used for the measurement.

For case#1, if the MeasObject is configured, as per the legacy behaviour, it should be ensured that RAN2 agreement not measuring non activated SSBs is achieved in the signaling as well. Upon activation of on-demand SSB via MAC-CE, the new SMTC may be applied.

Figure 3: MeasConfig framework

This requires to align the SMTC window with the on-demand SSB transmission.. The SMTC configuration may be explicitly configured for each on-demand SSB configuration or derived based on the on-demand SSB periodicity.

Figure 4: on-demand SSB and SMTC

Proposal 9: Consider dedicated SMTC configuration for each on-demand SSB configuration.

Proposal 10: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.

## 2.5	On-demand SSB and ServingCellMO

In existing spec, servingCellMO is associated with one MO which includes one ssb-Frequency i.e. AFCRN value. UE is required to perform measurement on the MO indicated by servingCellMO for serving cell measurement. As specified in TS 38.331 clause 5.5.3, the UE will derive the serving cell measurement result based on the SSB or CSI-RS resources in the MO indicated by servingCellMO.

For the on-demand SSB operation within SCell, network may configure at least one OD-SSB in a SCell with or without always-on SSB. Network may indicate in the OD-SSB activation command e.g. MAC CE or RRC signaling (or DCI for SSB adaptation) when the OD-SSB transmission is activated. That is, UE is able to know which OD-SSB to receive/measure when OD-SSB is activated, and it was agreed UE shall measure OD-SSB only when it is activated.

As the measurement is based on the activated OD-SSB, network may configure separate MOs for the OD-SSB configuration e.g. in case OD-SSB has different frequency location compared to “regular” SSB. And UE will measure corresponding MO when the OD-SSB in the MO is activated.  As the existing servingCellMO can only point to one MO it seems that then OD_SSB cannot be used to derive the serving cell measurement e.g.servingCellMO is configured to one of the measurement object ID e.g. MeasObjectID=1 as in current specification, the UE will not be able to derive the serving cell measurement if the OD-SSB corresponding to the MeasObjectId=1 is not activated

Proposal 11 : It should be possible to configure dedicated servingCellMo for each SSB configuration

And as we discuss in our R2-24xxxxx for SSB adaptation there seems to be lots of commonalities between two features especially in RAN2. Thus it seems logical that whenever RAN2 makes designs for MAC/RRC they should be done by jointly taking both objectives into account to avoid unnecessary complexity/duplication of behaviour in RAN specifications.

Proposal 12: When designing on-demand SSB and SSB adaptation activation MAC CE/RRC signaling they should be done jointly in RAN2

## 2.6	On-demand SSB and L1 measurement configuration

During the RAN1#118 meeting it has been agreed that L1 measurement based on on-demand SSB may be supported. The L1 measurements are configured using the CSI-reporting framework where the resources to be measured are included. The UE may be configured with CSI-ResourceConfig that contains,

- A list of NZP CSI-RS resource set and/or SSB resource set provided by nzp-CSI-RS-ResourceSetList and/or csi-SSB-ResourceSetList

- Or a list of CSI-IM resource set provided by csi-IM-ResourceSetList

The SSB resources are provided as the SSB-Index that maps to a SSB configuration.

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17        SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF ServingAdditionalPCIIndex-r17  OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

It is uncertain whether UE would measure single SSB(e.g always-on) or multiple SSBs(on-demand SSB and always-on SSB) when on-demand SSB based L1 measurements enabled. This could affect the current CSI reporting framework and needs to be discussed in RAN1 before RAN2 can consider signaling enhancements.

Proposal 13: RAN2 to wait for RAN1 decision in the signaling enhancement needed for the CSI-Reporting framework.

# 3	Conclusion

Observation 1: During the RAN1#117 meeting, both RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

Proposal 1 : RAN2 starts signaling design so that each parameter listed by RAN1 can be signalled for any instance of OD-SSB configuration.

Proposal 2: Possible signaling optimizations can be down prioritized in RAN2 work.

Proposal 3 : OD-SSB configuration is Cell-specific and Physical cell id need not be configured for OD-SSB and follows the serving cell configuration.

Proposal 4: The different periodicities (and any other parameters that may be adaptive) are configured as different OD-SSB configurations in RRC and MAC indicates which OD-SSB configuration ID to be activated, which is forward compatible if other parameters also introduced in RAN1.

Proposal 5: The new flexible size MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated SSB configuration ID for each SCell.

Proposal 6: Take into account in MAC CE design for OD-SSB also SSB adaptation in time domain feature for activating different periodicities

Proposal 7 : Include additional activation status parameter to indicate if OD-SSB configuration is activated or not when SSB is configured.

Proposal 8 : RAN2 to confirm that direct SCell activation with on-demand SSB activation via RRC is supported.

Proposal 9: Consider dedicated SMTC configuration for each on-demand SSB configuration.

Proposal 10: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.

Proposal 11 : It should be possible to configure dedicated servingCellMo for each SSB configuration

Proposal 12: When designing on-demand SSB and SSB adaptation activation MAC CE/RRC signaling they should be done jointly in RAN2

Proposal 13: RAN2 to wait for RAN1 decision in the signaling enhancement needed for the CSI-Reporting framework.