3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502917

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On Adaptation of common signal/channel transmissions for NES

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

# 2	Adaptation of SSB in time domain

During the previous meeting, it was agreed in RAN2 that.

- Common signaling approach for OD-SSB and SSB adaptation would be followed

- SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell

- Additional SMTC configuration according to SSB adaptation would be introduced for serving cell

- Neighbor cell measurements are based on legacy SMTC.

Scenarios:

Figure 1: Timeline of OD-SSB SCell activation scenario with SSB adaptation.

From a scenario perspective, adaptation of the SSB burst periodicity can be applied for activation of an SCell, which is also studied extensively under Objective 1 with OD-SSB. Considering the scenarios studied in Objective 1, the more relevant scenarios are Scenario #2 and Scenario #2A.

For Objective 3, it is also relevant to consider SSB burst periodicity adaptation in the post-SCell activation phase, i.e., in Scenario #3B of Objective #1.

Exemplary illustration of the OD-SSB scenarios with SSB burst periodicity adaptation are provided in Figure 1.

A use case specifically for Scenario #3B, is if the network operates with long SSB burst periodicity after completion of SCell activation, i.e., time point reference T3 in Figure 1. Then, it would be useful for UEs in the cell to measure more SSBs, e.g., for improved time and frequency synchronization or to use those SSBs as reference for more frequent CSI reporting. In these cases, the network may adapt the SSB burst periodicity value to a value smaller than the prior legacy one.

If all the UEs in the SCell are R19 UEs supporting SSB adaptation in time domain, then it is also possible that the network adapts the SSB burst periodicity to longer periodicity than the default configured SSB burst periodicity and obtain NES gains.

Proposal 1: Consider SSB burst periodicity adaptation at least for Scenarios #2, #2A and #3B of Objective 1.

## 2.1	Configuration for SSB adaptation in time domain

During RAN1#120 meeting, RAN1 sent an LS to RAN2 and RAN3 to consider the design of the corresponding higher-layer parameters in Rel.19.

For SSB adaptation in time domain, SSB burst periodicity value can be provided in Serving Cell Configuration. Two options are considered:

- Option 1: Dedicated SSB burst periodicity parameter for adaptation, to apply instead of legacy one

- Option 2: SSB burst periodicity parameter for adaptation configured as OD-SSB configuration, with OD-SSB in the same frequency as AO-SSB

To ensure alignment with Objective 1 and avoid unnecessary complexity, we prefer Option 2. In any of the options, to ensure compatibility with legacy SSB burst periodicity, the number of supported periodicity values for the adapted SSB burst periodicity should be either equal or a subset of the legacy ones – for simplicity we would consider it useful to have exactly the same values.

Proposal 2: Consider similar signaling structure for SSB adaptation as for OD-SSB to avoid unnecessary complexity.

## 2.2	Indication for SSB adaptation in time domain

During the RAN1#120 meeting, the following agreements were reached, regarding the SSB adaptation in time domain:

Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signaling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signaling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signaling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Based on the RAN1#120 agreements, it is up to RAN2 to decide on the MAC-CE signaling design.

For configuring the adapted SSB burst periodicity the MAC-CE currently discussed under Objective 1 for realization of OD-SSB could be reused to enable SSB adaptation in time domain. However, the UE may need to distinguish between OD-SSB aspects and SSB adaptation aspects within the MAC-CE design, if there are different RAN4 requirements defined for the SSB adaptation vs. OD-SSB. If this is required this can be achieved in the RRC signaling easily by having specific parameter indicating which requirements apply or implicitly by presence of parameters that are only applicable for a use case.

The overall MAC-CE design for enabling SSB adaptation in the time domain would still be similar to the MAC-CE design for OD-SSB of Objective 1. For this reason, we believe that at least RRC signaling and MAC-CE aspects to indicate SSB adaptation in time domain can be treated in conjunction with the respective designs discussed/agreed in Objective 1.

For both OD-SSB and SSB adaptation in time domain, it should be possible to “deactivate” adaptation/OD-SSB to fall back to normal always-on SSB behavior.

Proposal 3: When designing on-demand SSB and SSB adaptation activation, MAC CE/RRC signaling should be done jointly in RAN2.

Proposal 4: It should be possible to deactivate OD-SSB/SSB adaptation with the signaling (RRC/MAC CE) as well (likely this comes for free when designing signaling)

It is noted that RAN4#114 discussed the scenario for OD-SSB based direct SCell activation. The following agreement was achieved:

Issue 6-1: OD-SSB based direct SCell activation - Background RAN1 #118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.. Agreement: - RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion.

Direct SCell activation should be considered with SSB adaptation in time domain too. To that end, an additional bit can be added in the signaling command, MAC-CE or RRC signaling to activate/deactivate SSB adaptation with SCell activation.

Proposal 5: Support direct SCell activation with SSB adaptation (and also OD-SSB).

## 2.3	Measurements

Dynamic adaptation of this SSB transmission may have an impact on L3 measurements performed by the UE, e.g., with the UE performing excess measurements when there is no SSB burst transmission or e.g., with the UE missing transmission occasions of SSB bursts.

A way to address these issues would be to align, the SSB adaptation in time domain, e.g., burst periodicity, with a dedicated SMTC as well. To that end, a dedicated SMTC parameters can be used to enable measurements of adapted SSBs. The configuration of an SMTC can be aligned with Objective 1 that focuses on “On-demand SSB SCell operation. For example, one could configure together with SSB configuration a SMTC configuration that is then activated whenever corresponding SSB configuration is activated.

Proposal-6: When designing on-demand SSB and SSB adaptation activation, SMTC configuration for the two features should be done jointly in RAN2

Proposal 7: When configuring SSB (OD-SSB/SSB adaptation) one should simultaneously configure corresponding SMTC which is activated once the SSB configuration is activated.

Moreover, in the RAN2#129 meeting, it was agreed that L3 neighbor cell measurements are based on legacy SMTC and adapting SSB burst periodicity in the SCell should not impact them. Hence, the adapted SSB periodicity in the SCell should not be larger than the legacy SMTC periodicity.

Observation 1: Adapted SSB periodicity in SCell is not larger than the legacy SMTC periodicity.

Changes in SSB periodicity may affect the frequency and accuracy of measurements used within the ServingCellMO, such as RSRP and RSRQ, which are based on SSBs.

In existing specifications, servingCellMO is associated with one MO which includes one ssb-Frequency i.e. AFCRN value. As specified in TS 38.331 clause 5.5.3, the UE will derive the serving cell measurement result based on the SSB or CSI-RS resources in the MO indicated by servingCellMO.

SSB adaptation in time domain though refers to adaptation of SSB burst periodicity and the frequency location of the adapted SSBs remains unchanged in comparison to initial SSB configuration. Thus, the existing servingCellMO can be used to derive the serving cell measurement.

Proposal 8: RAN2 to confirm that the existing servingCellMO can be used to derive the serving cell measurement in case the frequency of SSB configuration is same as for always on SSB (always the case with SSB adaptation in time domain).

## 2.4	On BFD and impact on RAN2/BFR

On the potential impact on BFR, RAN4 has agreed in the previous meeting that SSB adaptation would not impact on SCell BFD since the BFD is done on CSI-RS for SCell [R4-2502689]. Requirements for candidate beam are still to be discussed in RAN4. Based on the RAN4 agreements, no impact is foreseen to RAN2 BFR procedure.

Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues.

Proposal 8: SSB adaptation has no impact on BFR procedure for SCell.

# 3		PRACH Adaptation in time domain

As per the latest RAN1#117 meeting outcome, it was agreed to support at least the PRACH adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any). It was also noted that NES-capable UEs can use both additional PRACH resources and PRACH resources configured for legacy UEs.

From RAN2 point of view, we do not see much impact to support PRACH adaptation for 2-step RACH as well, thus it could be left to RAN1 to decide. . .

Proposal 9: it is up to RAN1 to decide whether PRACH adaptation is supported for the 2-step as well.

In the previous meeting, RAN1 agreed to support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.UE behavior is still to be defined wrt. the application resources for PRACH mask index when additional PRACH resource(s) are indicated as available, which more for RAN1 scope.

We do not see any difference for CBRA and CFRA on how the UE decides the available PRACH resources, i.e. the legacy resources and the additional one if activated. Thus From RAN2 point of view, no impact is foreseen for now to support PDCCH order based CFRA, unless RAN1 requires new PRACH mask table.

Proposal 10: For both CBRA and CFRA, no distinguishment of legacy RO resources and additional ones if indicated as available.

One potential implication on RAN2 could be on the Conditional handover (CHO) procedure as the NW does not know the time instant when UE initiates CHO and the UE can’t be aware in advance about the potential activation/deactivation of additional PRACH resources at one or more potential target cell(s). So RAN2 should consider whether it is feasible to support additional PRACH resources for CHO and if they want to be supported then how it can be achieved.

Proposal 11: Study whether further enhancements are needed for the conditional handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Another implication on RAN2 will be the beam failure recovery configuration as for beam failure recovery whether the additional PRACH resources can be used for beam failure recovery. We do not see strong reason to restrict. It could be left to NW implementation if it intends to configure the additional PRACH resources for BFR.

Proposal 12: Additional PRACH resources can be configured for beam failure recovery procedure and the UE considered as available if the resource is activated when BFR is triggered.

From our perspective, it is natural to expect that additional PRACH resources are configured semi-statically in Rel-19. Whether adaptation of the resources will be performed only via at least DCI format 1_0, as per RAN1#118bis agreements, or also via semi-static signalling will need to be further discussed. Indeed, system Information (SI), e.g. SIB1, could also be considered for indicating PRACH adaptation to UEs that have missed the DCI-based indication of additional PRACH resources for NES-capable UEs. The SIB1 would then include the information about the configuration of additional PRACH resources and the activation status. Such procedure would not require any SI change.

Proposal 9: Higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation could be considered, in addition to the DCI-based adaptation.

Proposal 13: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation.

## PRACH Adaptation in time domain in conjunction with adaptation of paging occasions

A key open aspect is how to handle the potential PRACH load increase, especially for Ns=8 or larger, when the POs are clustered in time as agreed by RAN2.

The PRACH load level will inherently increase if the paging occasions are confined in time domain. This is because the UEs will have to be paged during such confined time period, thereby causing the UEs to respond to the paging also in a confined time interval. So it should be possible to temporarily increase the amount of PRACH resources in order to accommodate more clustered paging responses from UEs. The legacy approach of configuring a high amount/frequently occurring PRACH resources will impact the network’s resource availability and RO monitoring energy consumption. Thus, one approach which can be considered by RAN2 is to temporarily make additional PRACH resources available, when one or more UEs are paged. Such additional PRACH resources would not be available, when no UEs are paged, and thus the PRACH resource can be repurposed and the RO monitoring omitted.

Proposal 14: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.

# Adaptation of paging occasions

In RAN2 #127bis the following was agreed:

Agreements on paging adaptation - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

In RAN2 #128 it was further agreed:

Agreements on paging adaptation - Introduce value for Ns=8. FFS on 16. - Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In RAN2 #129 the following was agreed:

Agreements on paging adaptation 1. For N, values smaller than T/32 are not supported. 2. The maximum possible value for Ns is 8. 3. Introduce a separate PEI configuration. 4. Paging adaptations are configured semi-statically and updated via system information update notification. 5. A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

In release 17, 3GPP specified the Paging Early Indication (PEI). The PEI is provided via DCI format 2_7 and consists of a paging indication field, which is the product of the number of POs indicated by the PEI and the number of subgroups per PO, and a TRS availability indication bitmap. Currently, the number of POs per PEI is in the range [1, 2, 4, 8] and since the legacy number of POs per PF is 4, the configuration of 8 POs per PEI implies two PFs are addressed by the PEI. This seems unlikely in the low paging load scenario currently targeted for paging enhancements by RAN2, because the PF interval would be long, which would then result in a very long time-distance between the PEI and the second PF. Therefore, we expect the number of POs per PEI to be configured as maximum 4.

As noted above, at the RAN2 #129 meeting it was agreed to introduce a separate PEI configuration. However, it is still left for further discussion whether the separate configurations for legacy and release 19 UEs are applicable to the same PEI, using DCI format 2_7, or whether 2 separate PEIs will also be transmitted.

According to our understanding, the network energy saving potential is maximized if the network can address both legacy POs and additional release 19 POs using a single PEI with DCI format 2_7. If the network is required to transmit two DCIs using format 2_7 it will increase both the physical resource consumption and impact the network sleeping opportunities. Furthermore, if the network is required to transmit two DCIs it will limit the network scheduling flexibility as two PDCCH monitoring occasions will be consumed, which essentially means less scheduling options for other use cases.

Observation 2: The network energy saving potential is maximized if the network can address both legacy POs and additional release 19 POs using a single PEI with DCI format 2_7.

In our view a good option is to define a subset of the additional release 19 POs are multiplexed with the legacy POs in the legacy DCI format 2_7 bit positions, while the remaining release 19 POs are not multiplexed and instead allocated the legacy spare bits, such that all addressing is done in a single DCI format 2_7.

For example, assuming the legacy number of POs is 4 (0-3) and the additional release 19 POs is 8 (0-7), it would be feasible to address the first 4 (0-3) of the additional release 19 PO together with the legacy POs, while the remaining 4 POs (4-7) can be addressed in the bits that appear as spare bits to the legacy UEs. If the number of POs per PEI is 4 there may be spare bits in the DCI format 2_7, because it has a maximum size of 43 bits including up to 6 bits for the TRS availability indication. Assuming 4 subgroups per PO, up to 4*4+6=22 bits will be used.

With spare 22 bits the network can for example

- accommodate 8 additional release 19 POs each having 2 subgroups per PO, in total 16 bits, or

- accommodate the additional release 19 POs #5-8 each having 4 subgroups PO, in total 16 bits, while the additional release 19 POs #1-4 can be multiplexed with the legacy POs.

Observation 3: There may be more than 20 spare bits in the PEI DCI format 2_7, when the number of legacy POs per PEI is 4.

It is still an open question whether the legacy POs and the first 4 additional release 19 POs (0-3) correspond to the same PDCCH monitoring occasions for the DCI format 1_0 scrambled with P-RNTI. Independently of this aspect we do not see a problem in multiplexing the 4 legacy POs and the first 4 additional release 19, because the paging enhancement is assumed to be applied in a low load scenario and therefore the impact to the false alarm rate will be limited.

Observation 4: Multiplexing legacy POs and additional release 19 POs is not problematic for the false alarm rate since the paging load is low.

Proposal 15: A single PEI with DCI format 2_7 can address both legacy POs and additional release 19 POs using a mixture of multiplexing and separate signalling.

Based on this analysis, we therefore consider it feasible to have a PEI configuration for the legacy UEs and a new, separate PEI configuration for the release 19 UEs, and that both configurations define bits in the same PEI using DCI format 2_7. The separate configuration can indicate which of the additional release 19 POs are allocated to the spare bits.

Proposal 16: A separate PEI configuration can be used for release 19 UEs to define how the release 19 POs are mapped to the same DCI format 2_7 used by legacy UEs for PEI.

As previously discussed by RAN2 the PF & PO calculations in TS 38.304 also apply to the new Ns and N values. However, the new Ns and N values will also impact the paging configuration in TS 38.331 (in PCCH-Config). The firstPDCCH-MonitoringOccasionOfPO information element may be provided per PO to define the first PDCCH monitoring occasion. According to the running CR for the RRC it is FFS how to configure & describe the firstPDCCH-MonitoringOccasionOfPO.

With Ns = 8 the number of firstPDCCH-MonitoringOccasionOfPO information elements will double compared to Ns = 4. The number of bits used per information element depends on the subcarrier space used in the corresponding bandwidth part and the N. For example, the combination of 15 kHz SCS and T/1 results in a PDCCH monitoring range of 0-139 i.e. 8 bits are required. For the same SCS and T/16 the range is 0-2239, which will require 12 bits. Since N is increased to T/32 an additional bit is required. In summary, the doubling of Ns and halving of N may require more than double the number of bits to transmit the firstPDCCH-MonitoringOccasionOfPO. Such a significant increase in signaling overhead needs to be addressed by RAN2 as already agreed in RAN2 #127bis.

Observation 5: The use of Ns=8 and N=T/32 will result in more than double number of bits required for firstPDCCH-MonitoringOccasionOfPO as compared to the legacy signaling.

The signaling of the firstPDCCH-MonitoringOccasionOfPO concerning the first 4 POs (i.e. the maximum for legacy UEs) cannot be optimized due to the requirement of backwards compatibility, but we think it is beneficial for RAN2 to further discuss how to efficiently configure the firstPDCCH-MonitoringOccasionOfPO for the additional for POs of a PF.

In our view, the current firstPDCCH-MonitoringOccasionOfPO provides a very high degree of flexibility to the network scheduler because the PDCCH MO can be indicated for any symbol in the paging frame. One approach to minimize the signaling overhead can therefore be to transmit the additional firstPDCCH-MonitoringOccasionOfPO with a reduced granularity. Another implication of the current ability to indicate any symbol in the paging frame is that a second PO’s PDDCH MO may be defined to occur before a first PO’s PDCCH MO. To reduce the signaling RAN2 could instead discuss to signal the second PO’s PDCCH MO to be relative to the first PO’s PDCCH MO.

Proposal 17: RAN2 to discuss how to efficiently transmit firstPDCCH-MonitoringOccasionOfPO for the 4 additional POs of a PF e.g. by reducing granularity or delta to previous PO signaling.

# Conclusion

Proposal 1: Consider SSB burst periodicity adaptation at least for Scenarios #2, #2A and #3B of Objective 1.

Proposal 2: Consider similar signaling structure for SSB adaptation as for OD-SSB to avoid unnecessary complexity.

Proposal 3: When designing on-demand SSB and SSB adaptation activation, MAC CE/RRC signaling should be done jointly in RAN2.

Proposal 4: It should be possible to deactivate OD-SSB/SSB adaptation with the signaling (RRC/MAC CE) as well (likely this comes for free when designing signaling)

Proposal 5: Support direct SCell activation with SSB adaptation (and also OD-SSB).

Proposal-6: When designing on-demand SSB and SSB adaptation activation, SMTC configuration for the two features should be done jointly in RAN2

Proposal 7: When configuring SSB (OD-SSB/SSB adaptation) one should simultaneously configure corresponding SMTC which is activated once the SSB configuration is activated.

Observation 1: Adapted SSB periodicity in SCell is not larger than the legacy SMTC periodicity.

Proposal 8: RAN2 to confirm that the existing servingCellMO can be used to derive the serving cell measurement in case the frequency of SSB configuration is same as for always on SSB (always the case with SSB adaptation in time domain).

Proposal 8: SSB adaptation has no impact on BFR procedure for SCell.

Proposal 9: it is up to RAN1 to decide whether PRACH adaptation is supported for the 2-step as well.

Proposal 10: For both CBRA and CFRA, no distinguishment of legacy RO resources and additional ones if indicated as available.

Proposal 11: Study whether further enhancements are needed for the conditional handover procedure when the additional PRACH resources are activated at one or more candidate target cells.

Proposal 12: Additional PRACH resources can be configured for beam failure recovery procedure and the UE considered as available if the resource is activated when BFR is triggered.

Proposal 9: Higher-layer signalling based on SI, e.g. SIB1, for PRACH adaptation could be considered, in addition to the DCI-based adaptation.

Proposal 13: RAN2 needs to wait for more RAN1 inputs before we can progress detailed signaling of PRACH adaptation.

Proposal 14: PRACH adaptation in time domain is supported to temporarily increase the PRACH capacity to be aligned with clustered paging and spread the PRACH load when paging occasion adaptation is used.

Observation 2: The network energy saving potential is maximized if the network can address both legacy POs and additional release 19 POs using a single PEI with DCI format 2_7.

Observation 3: There may be more than 20 spare bits in the PEI DCI format 2_7, when the number of legacy POs per PEI is 4.

Observation 4: Multiplexing legacy POs and additional release 19 POs is not problematic for the false alarm rate since the paging load is low.

Proposal 15: A single PEI with DCI format 2_7 can address both legacy POs and additional release 19 POs using a mixture of multiplexing and separate signalling.

Proposal 16: A separate PEI configuration can be used for release 19 UEs to define how the release 19 POs are mapped to the same DCI format 2_7 used by legacy UEs for PEI.

Observation 5: The use of Ns=8 and N=T/32 will result in more than double number of bits required for firstPDCCH-MonitoringOccasionOfPO as compared to the legacy signaling.

Proposal 17: RAN2 to discuss how to efficiently transmit firstPDCCH-MonitoringOccasionOfPO for the 4 additional POs of a PF e.g. by reducing granularity or delta to previous PO signaling.