3GPP TSG-RAN WG2 Meeting #129bis	R2-2502918

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.11.3

Source:	Nokia

Title:	Other Aspects of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved [1]. The work item description has been updated in RAN#104 meeting and the objectives are mentioned in [RP-241614].

In this contribution we discuss RAN2-aspects relevant to SBFD which are not covered in the other SBFD agenda item on Random Access considering emergency call handling and CSI-RS measurements as well as also issues brought by rapporteur in the email discussion [Post129][217][SBFD] List of open issues of RRC impact (Huawei)

# 2	SBFD signalling aspects

So based on the decision in RAN2 and RAN1 we should add signaling of SBFD symbols within TDD-UL-DL pattern configuration in SIB1 – and this will naturally mean that also the corresponding information elements are included in the “common” signaling part of dedicated signaling to allow RRC_CONNECTED mode UEs to be signalled SBFD parameters without requirement of reading SIB1 before utilizing RACH.

Proposal  1: RAN2 needs to add signaling of SBFD parameters (up to RAN1) within SIB1 which implicitly also means they should be added to ServingCellConfigCommon

# 3	RLM handling

There was discussion in the RAN2 list of open issues email discussion regarding whether network needs/can configure the valid symbol type for CSI-RS based RLM/BFD/CBD measurement.

In the last RAN1 meeting, an LS was sent to RAN4 (R1-2501560), where in Issue 2 RAN1 answered that configuration 1 and configuration 2 are not applicable to CSI-RS configuration for RLM/BFD. The configuration of a valid symbol type is applicable only to L1-RSRP and SINR measurement reporting. Therefore, we believe that this issue should be discussed in RAN1 and RAN4.

Proposal 2: It is up to RAN1 and RAN4 to discuss the configuration of a valid symbol type for RLM/BFD measurements

# 3	Mobility aspects of SBFD

Mobility measurements, e.g. RSRP and RSSQ, can be based on SBFD or CSI-RS signals. CSI-RS are impacted by SBFD operation as noted in the following agreements made by RAN1:

RAN1#117: Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Furthermore, in the email discussion “[Post129][217][SBFD] List of open issues of RRC impact (Huawei)” it was also discussed whether RAN2 should consider to support separate Layer3 measurement report for CSI-RS resources in SBFD symbol. Related to this issue, we think that RAN2 should first discuss whether the time-frequency resource configuration of the neighbour cell is known by the UE performing the layer 3 measurements, and then it can be up to RAN1 or RAN4 to decide whether separate Layer3 measurements are needed.

On providing information to the UEof the SBFD configuration of neighbouring cells. If the neighbour cell operates with SBFD and its CSI-RS transmission takes place in a SBFD symbol, the CSI-RS would be punctured and this needs to be known by the UE to ensure accurate e.g. RSRP or RSRQ measurement. RAN1 has not made any distinction between serving-cell or inter -cell CSI-RS based measurements, so it is assumed that RAN1 agreements apply to both cases.

The following agreements have been reached in the previous RAN2 meeting regarding this issue:

Prioritization of SBFD cells / frequencies during cell reselection is not considered. RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

Observation 1: According to RAN1 agreements, contiguous CSI-RS resources are configured to the UEs, but only the CSI-RS resources in the DL subband are measured by the SBFD aware UEs. The CSI-RS resources in the UL subband and guardbands are assumed not to be present, i.e. not transmitted by the gNB. RAN1 has not made any distinction between serving-cell or inter-cell CSI-RS based measurements.

Observation 2: For mobility measurements, if the neighbour cell operates with SBFD and its CSI-RS transmission takes place in a SBFD symbol, the CSI-RS would be punctured and this needs to be known by the UEs to ensure accurate e.g. RSRP or RSRQ measurement.

Proposal 3: For inter-cell CSI-RS measurements, SBFD aware UEs are provided with information of the SBFD configuration of neighbouring cells.

As the legacy UEs cannot be aware where SBFD symbols are present, it is then up to the network to ensure that for such UEs, a very conservative approach is to be taken so that even for the neighbour cell measurement, CSI-RS are configured in such a way that they do not occur in SBFD symbols.

Proposal 4: Network needs to ensure that CSI-RS measurements do not overlap with SBFD symbols for UEs that are not SBFD aware.

# 4	Emergency call handling

Emergency calls are critical, thus it is imperative to minimize the risk of interruptions or temporarily too poor quality in the voice transmission. In cells utilizing SBFD this can be an issue if not taken into account when allocating resources for emergency calls or mission critical services with a low data rate. This is particularly important for non SBFD aware UEs. Assuming that the cell operates with a mix of DL-only, UL-only and SBFD slots, the DL resource allocation for a UE making an emergency call should take place in DL-only slots, and the UL resource allocation should take place in UL-only slots for legacy UEs but likely also for SBFD aware UEs as the NW may not be aware of the UEs' SBFD capability at the call setup procedure.

Proposal 5: The NW should ensure that when establishing voice calls and especially emergency calls (at least for non SBFD aware UEs), resources are not allocated to SBFD symbols (i.e. resources should be DL-only or UL-only symbols).

# 5	Conclusion

Proposal  1: RAN2 needs to add signaling of SBFD parameters (up to RAN1) within SIB1 which implicitly also means they should be added to ServingCellConfigCommon

Proposal 2: It is up to RAN1 and RAN4 to discuss the configuration of a valid symbol type for RLM/BFD measurements

Observation 1: According to RAN1 agreements, contiguous CSI-RS resources are configured to the UEs, but only the CSI-RS resources in the DL subband are measured by the SBFD aware UEs. The CSI-RS resources in the UL subband and guardbands are assumed not to be present, i.e. not transmitted by the gNB. RAN1 has not made any distinction between serving-cell or inter-cell CSI-RS based measurements.

Observation 2: For mobility measurements, if the neighbour cell operates with SBFD and its CSI-RS transmission takes place in a SBFD symbol, the CSI-RS would be punctured and this needs to be known by the UEs to ensure accurate e.g. RSRP or RSRQ measurement.

Proposal 3: For inter-cell CSI-RS measurements, SBFD aware UEs are provided with information of the SBFD configuration of neighbouring cells.

Proposal 4: Network needs to ensure that CSI-RS measurements do not overlap with SBFD symbols for UEs that are not SBFD aware.

Proposal 5: The NW should ensure that when establishing voice calls and especially emergency calls (at least for non SBFD aware UEs), resources are not allocated to SBFD symbols (i.e. resources should be DL-only or UL-only symbols).