3GPP TSG-RAN WG2 Meeting #129bis	R2-2502925

Wuhan, China, 7 – 11 April 2025

Agenda Item:		8.3.5

Source:				ETRI

Title:					Discussion on LCM and spec impact

Document for:		Discussion and Decision

1. Introduction

In this contribution, we discuss LCM and potential specification impact for AI/ML mobility.

2. Discussion

2.1 General AI/ML framework

As stated in the TR [1], for LCM and potential specification impact for AI/ML mobility, RAN2 can reuse LCM and the protocol aspects captured in section 7.2 of TR 38.843 [2] and the agreement concluded under WID NR_AIML_air-Core [3] in principle. Anything mobility specific can be added on top of that. By the same reasoning, general AI/ML framework in chapter 4 of TR 38.843, e.g., Life Cycle Management in section 4.2 and Functional framework details in section 4.4, can be reused for general AI/ML framework for mobility.

Proposal 1: General AI/ML framework in chapter 4 of TR 38.843, e.g., Life Cycle Management in section 4.2 and Functional framework details in section 4.4, can be reused for general AI/ML framework for mobility.

2.2 RRM measurement prediction

For model training of RRM measurement prediction with UE-sided models, the AI/ML model training at UE side is at least supported. Model training at NW side may be further studied based on the support of model transfer. For model training at NW side, the AI/ML model training at UE-side OTT server can be supported and RAN2 can further discuss model training at gNB, OAM or Core Network. Therefore, for model training of RRM measurement prediction with UE-sided models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server.

Proposal 2: For model training of RRM measurement prediction with UE-sided models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server.

For model training of RRM measurement prediction with NW-sided models, the AI/ML model training at gNB side is at least supported. Model training at NW side other than gNB side may be further studied based on the support of model transfer. For model training at NW side other than gNB side, the AI/ML model training at OAM can be supported and RAN2 can further discuss model training at Core Network or OTT server. Therefore, for model training of RRM measurement prediction with NW-sided models, training data can be generated by the gNB or UE, while the termination point for training data may include the gNB or OAM.

Proposal 3: For model training of RRM measurement prediction with NW-sided models, training data can be generated by the gNB or UE, while the termination point for training data may include the gNB or OAM.

For inference of RRM measurement prediction with UE-sided models, input data is internally available at UE, where the inference process is performed.

Proposal 4: For inference of RRM measurement prediction with UE-sided models, input data is internally available at UE, where the inference process is performed.

For inference of RRM measurement prediction with NW-sided models, the UE can generate the necessary input data while the termination point for this input data lies within the gNB, where the inference process is performed.

Proposal 5: For inference of RRM measurement prediction with NW-sided models, the UE can generate the necessary input data while the termination point for this input data lies within the gNB, where the inference process is performed.

For management of RRM measurement prediction with UE-sided models, the network can initiate performance monitoring, and can make management decisions based on the performance monitoring results. The UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination point for these is the gNB.

Proposal 6: For management of RRM measurement prediction with UE-sided models, the network can initiate performance monitoring, and can make management decisions based on the performance monitoring results. The UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination point for these is the gNB.

For management of RRM measurement prediction with NW-sided models, the gNB is responsible for performance monitoring. UE will not be involved in management decisions (e.g., selection, (de)activation, switching etc.) of gNB-side model, except for being configured to provide the required measurement/data. Additionally, the UE will not be informed about any gNB-side management decision.

Proposal 7: For management of RRM measurement prediction with NW-sided models, the gNB is responsible for performance monitoring. UE will not be involved in management decisions (e.g., selection, (de)activation, switching etc.) of gNB-side model, except for being configured to provide the required measurement/data. Additionally, the UE will not be informed about any gNB-side management decision.

For WI on AI/ML for NR air interface [3, 5], related to the topic of NW-side data collection, RAN2 can continue the discussion on configuration and reporting mechanisms to enable NW-side data collection, focusing on the FFS agreed during RAN2#129. Further, RAN2 should start the discussion on how to capture in normative procedures the agreements reached so far on this topic, with focus on the running CRs for TS 38.300 and TS 38.331. Related to the topic of UE-side data collection, RAN2 can continue the discussion on the mechanisms to configure the UE (with or without UE request) for the UE-side data collection. Further, RAN2 should start the discussion on how to capture in normative procedures the agreements reached so far on this topic, with focus on the running CRs for TS 38.300 and TS 38.331. Therefore, discussion on data collection for SI on AI/ML for Mobility can be postponed until further progress for the data collection for WI on AI/ML for NR air interface is achieved.

Proposal 8: Discussion on data collection for SI on AI/ML for Mobility can be postponed until further progress for the data collection for WI on AI/ML for NR air interface is achieved.

For inference output of RRM measurement prediction with UE-sided models, the UE can report RRM measurement prediction results to the gNB using legacy Measurement Report procedures. The gNB can configure RRM measurement prediction and the conditions to trigger the report. FFS if periodical reporting is needed. Possible reporting enhancements (e.g., prediction time information, etc.) are left to the normative phase discussion.

Proposal 9: For inference output of RRM measurement prediction with UE-sided models, the UE can report RRM measurement prediction results to the gNB using legacy Measurement Report procedures. The gNB can configure RRM measurement prediction and the conditions to trigger the report. FFS if periodical reporting is needed. Possible reporting enhancements (e.g., prediction time information, etc.) are left to the normative phase discussion.

2.3 Measurement event prediction

In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model, as stated in the TR [1]. Therefore, model training and inference function for RRM measurement prediction with UE-sided models can be reused for model training and inference function for indirect measurement event prediction with UE-sided models.

Proposal 10: Model training and inference function for RRM measurement prediction with UE-sided models can be reused for model training and inference function for indirect measurement event prediction with UE-sided models.

For model training of direct measurement event prediction with UE-sided models, the AI/ML model training at UE side is at least supported. Model training at NW side may be further studied based on the support of model transfer. For model training at NW side, the AI/ML model training at UE-side OTT server can be supported and RAN2 can further discuss model training at gNB, OAM or Core Network. Therefore, for model training of direct measurement event prediction with UE-sided models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server.

Proposal 11: For model training of direct measurement event prediction with UE-sided models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server.

For inference of direct measurement event prediction with UE-sided models, input data is internally available at UE, where the inference process is performed.

Proposal 12: For inference of direct measurement event prediction with UE-sided models, input data is internally available at UE, where the inference process is performed.

For management of indirect/direct measurement event prediction with UE-sided models, the network can initiate performance monitoring, and can make management decisions based on the performance monitoring results. The UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination point for these is the gNB.

Proposal 13: For management of indirect/direct measurement event prediction with UE-sided models, the network can initiate performance monitoring, and can make management decisions based on the performance monitoring results. The UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination point for these is the gNB.

For inference output of indirect/direct measurement event prediction with UE-sided models, the UE can report measurement event prediction results to the gNB using legacy Measurement Report procedures. The gNB can configure measurement event predictions and the conditions to trigger the report. FFS for direct measurement event prediction if a new reporting procedure is needed. Possible reporting enhancements (e.g., prediction time information in indirect/direct case, probability in direct case, etc.) are left to the normative phase discussion.

Proposal 14: For inference output of indirect/direct measurement event prediction with UE-sided models, the UE can report measurement event prediction results to the gNB using legacy Measurement Report procedures. The gNB can configure measurement event predictions and the conditions to trigger the report. FFS for direct measurement event prediction if a new reporting procedure is needed. Possible reporting enhancements (e.g., prediction time information in indirect/direct case, probability in direct case, etc.) are left to the normative phase discussion.

2.4 RLF/HOF prediction

In RAN2#129 meeting [4], RAN2 agreed that RLF prediction will not be studied in Rel-19. Therefore, it is natural that LCM and potential specification impact for RLF/HOF prediction will not be discussed in Rel-19.

Proposal 15: LCM and Potential specification impact for RLF/HOF prediction will not be discussed in Rel-19.

3. Conclusion

Based on the discussion in Section 2, we propose the following:

Proposal 1: General AI/ML framework in chapter 4 of TR 38.843, e.g., Life Cycle Management in section 4.2 and Functional framework details in section 4.4, can be reused for general AI/ML framework for mobility.

Proposal 2: For model training of RRM measurement prediction with UE-sided models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server.

Proposal 3: For model training of RRM measurement prediction with NW-sided models, training data can be generated by the gNB or UE, while the termination point for training data may include the gNB or OAM.

Proposal 4: For inference of RRM measurement prediction with UE-sided models, input data is internally available at UE, where the inference process is performed.

Proposal 5: For inference of RRM measurement prediction with NW-sided models, the UE can generate the necessary input data while the termination point for this input data lies within the gNB, where the inference process is performed.

Proposal 6: For management of RRM measurement prediction with UE-sided models, the network can initiate performance monitoring, and can make management decisions based on the performance monitoring results. The UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination point for these is the gNB.

Proposal 7: For management of RRM measurement prediction with NW-sided models, the gNB is responsible for performance monitoring. UE will not be involved in management decisions (e.g., selection, (de)activation, switching etc.) of gNB-side model, except for being configured to provide the required measurement/data. Additionally, the UE will not be informed about any gNB-side management decision.

Proposal 8: Discussion on data collection for SI on AI/ML for Mobility can be postponed until further progress for the data collection for WI on AI/ML for NR air interface is achieved.

Proposal 9: For inference output of RRM measurement prediction with UE-sided models, the UE can report RRM measurement prediction results to the gNB using legacy Measurement Report procedures. The gNB can configure RRM measurement prediction and the conditions to trigger the report. FFS if periodical reporting is needed. Possible reporting enhancements (e.g., prediction time information, etc.) are left to the normative phase discussion.

Proposal 10: Model training and inference function for RRM measurement prediction with UE-sided models can be reused for model training and inference function for indirect measurement event prediction with UE-sided models.

Proposal 11: For model training of direct measurement event prediction with UE-sided models, training data can be generated by the UE, while the termination point for training data may include the UE or a UE-side OTT server.

Proposal 12: For inference of direct measurement event prediction with UE-sided models, input data is internally available at UE, where the inference process is performed.

Proposal 13: For management of indirect/direct measurement event prediction with UE-sided models, the network can initiate performance monitoring, and can make management decisions based on the performance monitoring results. The UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination point for these is the gNB.

Proposal 14: For inference output of indirect/direct measurement event prediction with UE-sided models, the UE can report measurement event prediction results to the gNB using legacy Measurement Report procedures. The gNB can configure measurement event predictions and the conditions to trigger the report. FFS for direct measurement event prediction if a new reporting procedure is needed. Possible reporting enhancements (e.g., prediction time information in indirect/direct case, probability in direct case, etc.) are left to the normative phase discussion.

Proposal 15: LCM and Potential specification impact for RLF/HOF prediction will not be discussed in Rel-19.

4. References

TR 38.744 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

R2-25xxxx Introduction of AI/ML for NR Air interface feature in TS38.300 vivo

RAN2#129 Meeting Report

RP-250310	Status report for WI on AI/ML for NR air interface	Qualcomm Innovation Center Inc