3GPP TSG-RAN2 Meeting # 129bis	R2-250292

# Wuhan, CN, 07th – 11th Apr. 2025

Agenda Item:	8.10.4

Title:	MRO and MDT for NTN

Source:	ZTE Corporation, Sanechips

Document for:	Discussion, Agreement

Introduction

Latest LS from RAN3 with a compilation of intermediate conclusions [1], tasks RAN2 to work on the MRO/MDT for NTN. In this contribution, we focus on the following aspects (some stems from the LS, some not):

# MRO,

How RACH-less handover for NTN impacts MRO.

report enhancement.

# And MDT,

How should we define the new area scope for MDT of NTN? We made our suggestion and draft a reply LS at the end of this contribution.

- MRO for NTN

RACH-less HO in NTN

RACH-less HO was supported for HO in NTN, and in other scenarios (e.g., LTM, mIAB or any other scenarios when needed). As an essential mechanism to reduce the HO interruption, RACH-less HO should be in the scope of MRO. The RACH-less HO in NTN works as following:

UE can be optionally configured with RACH-less HO by rach-LessHO and configured grant.

If the condition of configuration grant is not met, UE “falls back” to apply RACH based HO. “The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell.” (38.300 i30: 9.2.3.6 RACH-less handover)

There could be following 4 cases if RACH-less HO was configured:

|  | Procedure. | Spec impacts analysis. |
| --- | --- | --- |
| Case 1. | 1. RACH-less configured and successfully executed | Open question: - Do we need to enhance Random Access report, even for a RACH-less HO? Maybe not, a single RACH-less related without a context, e.g., HO, might not benefit network much. |
| Case 2. | 1. RACH-less configured and execution fails. | Open question: - Do we need to enhance RLF Report, therefore, to facilitate network to be aware of the issues on RACH-less related configuration? Maybe yes. Network can further optimize a RACH-less related configuration. - Where to include the RACH-less related configuration can be discussed later, e.g., inside or outside of ra-InformationCommon. |
| Case 3. | 1. RACH-less configured but UE falls back to RACH-based HO. 2. The RACH-based HO successes. | Open question: - Do we need to enhance Random Access report, for a supposedly RACH-less which falls back to RACH based? Since there is no way for network to be aware as there is no failure related report, this might be needed. |
| Case 4. | 1. RACH-less configured but UE falls back to RACH-based HO. 2. The RACH-based HO fails. | Open question: - Surely, we need to record the RACH information in RLF report for a RACH-based HO, but do we need to enhance RLF report for a supposedly RACH less which falls back to RACH based? Or we put this into the Random Access report? |

RAN2 to discuss for the RACH-less HO, whether to report to network a supposedly RACH-less which falls back to RACH based, and in which report (e.g., Random Access report or rlf-Report.)

report enhancement

Based on the discussion in RAN2#129 meeting, following agreements on MRO for NTN have been achieved:

Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

Although RAN2 has agreed to reuse the RLF report for reporting conditions that meet the CHO trigger conditions, compared to traditional RLF reports, NTN may present specific issues that could potentially lead to radio link failures.

We believe that providing additional information in the report (absolute time/absolute locate/speed information when UE triggers CHO or RLF) would not only help quickly identify and locate fault causes but also provide comprehensive data support for dynamic network optimization. For example:

In NTN, satellite positions dynamically change over time. the absolute time and location when UE triggers CHO or RLF can help determine whether uplink failures are caused by ephemeris prediction errors(e.g., beam switching delays, coverage blind spots).

Issues such as link interruptions and insufficient power compensation potentially caused by high-speed mobility can be more intuitively reflected through speed data, offering references for network optimization.

Therefore, it is necessary to introduce additional NTN-related failure information in NTN RLF reports, such as absolute time, absolute location, and speed information when UE triggers CHO or RLF.

Compared to traditional RLF reports, NTN may present specific issues that could potentially lead to radio link failures.

Introduce absolute time, absolute location, and speed information in NTN RLF reports when UE triggers CHO and upon RLF.

- MDT for NTN

background

In legacy, UE applies the MDT based on the configuration and the cell it camps on, e.g., if the cell it camps on belongs to the Area Scope configured by network, MDT is on. However, the cell UE sees may be larger than the MDT aims to cover.

Figure 3.1-1. MDT happens only in specific area operator is interested in, instead of the whole region a beam covers.

RAN3 came up with an idea to optimize the MDT procedure for NTN in which a cell UE sees Therefore, less efforts are needed to carry out the MDT, it only happens where operator wants it to be. RAN3 asks RAN2 about the MDT for NTN, on the definition of area scope.

On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP. … 2. Actions: To RAN2 group. ACTION: RAN3 kindly asks RAN2 to take the above into consideration and update their specifications as needed. RAN3 also kindly requests RAN2 to provide feedback to the question on MDT for NTN. …

  - solutions

RAN3 proposed two solutions, i.e., it is assumed that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP.

For geographical area, it is an intuitive solution. And in existing spec, there is already well-defined IE to capture an area, e.g., a circle characterized in SIB25 for an NTN UE to skip the neighbor measurement of a TN cell. This IE could be reused.

As for the mapped cell ID, it is used between CN and NTN RAN to maintain the concept of a logical cell that does not move along with the satellite. However, such concept is unknown/transparent to UE. The mapped cell ID known to UE, aware to UE is not enough. A UE still needs to find out what is the mapped geographical area behind it.

Mapped cell ID is transparent (i.e., invisible) to UE.

If Mapped cell ID is to be used for UE to identify an absolute geographic area, the mapping information needs to be indicated to UE, i.e., Mapped cell ID exposed to UE alone does not help.

Whichever ID applies, eventually UE needs to be aware the geographic information to apply MTD.

Additionally, some companies think that geographic area-based schemes cannot avoid high energy consumption for RRC_IDLE/RRC_INACTIVE UEs. However, in our understanding, NTN UEs in RRC_IDLE/RRC_INACTIVE states must maintain location information to support cell reselection (TS 38.304 5.2.4.2). UEs continuously acquire their location via GNSS or network assisted information (ephemeris information in SIB19, reference location information) to ensure mobility management under dynamic coverage. Therefore, to some extent, geographic area-based MDT measurements can reuse location information already maintained by NTN UEs for other causes.

Geographic area-based MDT measurements can reuse location information already maintained by NTN UEs for other causes (e.g., cell reselection).

It is then proposed that RAN2 agree to adopt the absolute geographic area for NTN MDT.

Adopt geographic area as the area scope information for NTN MDT.

Meanwhile, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN. Well, this is a stage 3 issue, and I guess RAN2 should be fine to investigate into this. As indicated earlier, we had already something to reuse in SIB25. Not sure whether RAN2 will adopt the same IE in SIB25 for the NTN MBS case, but it is a good idea to be aligned.

A geographic area is already well defined in 3GPP.

Reuse the geographic area defined in SIB25, for the absolute geographic area as the area scope information for NTN MDT. Can coordinate with Rel-19 NTN Working Item during stage 3 phase.

LS back to RAN3 on above agreements.

- Conclusion

Based on above analysis, we make the following suggestions to RAN2:

# for MRO

- RAN2 to discuss for the RACH-less HO, whether to report to network a supposedly RACH-less which falls back to RACH based, and in which report (e.g., Random Access report or rlf-Report.)

Introduce absolute time, absolute location, and speed information in NTN RLF reports when UE triggers CHO and upon RLF.

# for MDT

- Adopt geographic area as the area scope information for NTN MDT.

Reuse the geographic area defined in SIB25, for the absolute geographic area as the area scope information for NTN MDT. Can coordinate with Rel-19 NTN Working Item during stage 3 phase.

LS back to RAN3 on above agreements.

- Reference

R3-247908 LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT

- Annex

      - 5.2.4.2	Measurement rules for cell re-selection

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN (quasi-)Earth-fixed cell and has obtained its location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements;

-	else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving cell and has obtained its location information:

-	if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	else, the UE shall perform intra-frequency measurements;

-	Else, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN (quasi-)Earth-fixed cell and has obtained its UE location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8];

-	else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving cell and has obtained its location information:

-	if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8];

-	Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

-	For UE camping on NTN cell, if the UE supports skipping TN measurement, and the UE has obtained its location information, and if coverageAreaInfoList and tn-AreaIdList are broadcast in system information, the UE may not perform measurements of a TN frequency when UE is not in the coverage of that frequency provided via tn-AreaIdList, regardless of the frequency priority.

- - SIB25

SIB25 contains TN coverage information to assist neighbour cell measurements for the UEs in an NTN cell.

SIB25 information element

-- ASN1START

-- TAG-SIB25-START

SIB25-r18 ::= SEQUENCE {

coverageAreaInfoList-r18 CoverageAreaInfoList-r18 OPTIONAL, -- Need R

lateNonCriticalExtension OCTET STRING OPTIONAL,

...

}

CoverageAreaInfoList-r18 ::= SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF CoverageAreaInfo-r18

CoverageAreaInfo-r18 ::= SEQUENCE {

tn-AreaId-r18 TN-AreaId-r18,

tn-ReferenceLocation-r18 ReferenceLocation-r17,

tn-DistanceRadius-r18 INTEGER(0..65535)

}

-- TAG-SIB25-STOP

-- ASN1STOP

SIB25 field descriptions

coverageAreaInfoList Contains a list of TN coverage area's information to assist skipping TN measurements for NTN UEs in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

tn-DistanceRadius Distance from the TN coverage area reference location. It is used for skipping TN measurements in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 1m.

- Draft LS to RAN3

3GPP TSG-RAN WG2 Meeting #129bis	R2-250xxxx

Wuhan, CN, 7th – 11th Apr 2025

Title:	[DRAFT] Reply LS on area scope of MDT in NTN

Response to:	R3-247892

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	ZTE [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks RAN3 for providing the latest progress. For the following enquiry:

- “On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.”

RAN2 discussed and concluded that geographical area can be used for the Area Scope for logged MDT over Uu for NTN. RAN2 further agreed to reuse the geographic area defined in SIB25, which however needs coordination with another Working Item (i.e., NTN in Rel-19) in stage 3 phase.

2. Actions:

To RAN3

ACTION: 	RAN2 respectfully asks RAN3 to take the above responses into account.

3. Date of Next TSG-RAN WG2 Meetings:

Please refer to http://portal.3gpp.org/