3GPP TSG-RAN2 Meeting #129bis	R2-2502936

# Wuhan, China, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5331 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on sidelink relay RRC specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. the RRC connection initiation/resume conditions for U2U relay discovery is missing. 2. N3C MP related UE capabilities are also captured in SidelinkParameters, but N3C feature is missing in corresponding description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. In clause 5.3.3.1a/5.3.13.1a, add the new RRC connection initiation/resume conditions for U2U relay discovery. 2. In clause 6.3.3, add the N3C MP in description of SidelinkParameters. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR SL U2U relay, N3C MP Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. UE can not initiate/resume the RRC connection if network indicates support of U2U discovery and corresponding pool is not configured. 2. N3C feature is missing in description of SidelinkParameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1a, 5.3.13.1a, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

5.3.3.1a	Conditions for establishing RRC Connection for NR sidelink communication/discovery/V2X sidelink communication/MP operation

For NR sidelink communication/discovery, an RRC connection establishment is initiated only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency;

1>	if configured by upper layers to transmit NR sidelink discovery and related data is available for transmission:

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2U relay discovery messages and sl-L2-U2U-Relay-r18 is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2U relay discovery messages and sl-L3-U2U-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages and sl-L3U2N-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink non-relay discovery messages and sl-NonRelayDiscovery is included in SIB12:

3>	if the frequency on which the UE is configured to transmit NR sidelink discovery is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 includes neither sl-DiscTxPoolSelected nor sl-TxPoolSelectedNormal for the concerned frequency;

For L2 U2N Relay UE in RRC_IDLE, an RRC connection establishment is initiated in the following cases:

1>	if any message is received from a L2 U2N Remote UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4; or

1>	if RemoteUEInformationSidelink containing the connectionForMP is received from a L2 U2N Remote UE as specified in 5.8.9.8.3;

For V2X sidelink communication, an RRC connection is initiated only when the conditions specified for V2X sidelink communication in clause 5.3.3.1a of TS 36.331 [10] are met.

NOTE 1:	Upper layers initiate an RRC connection (except if the RRC connection is initiated at the L2 U2N Relay UE upon reception of a message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1, or upon reception of RemoteUEInformationSidelink message containing the connectionForMP). The interaction with NAS is left to UE implementation.

For N3C relay UE in RRC_IDLE, an RRC connection establishment is initiated when a N3C remote UE indicates it to enter RRC_CONNECTED state.

NOTE 2:	How/when the N3C remote UE to indicate N3C relay UE to enter RRC_CONNECTED state is left to UE implementation, e.g. before reporting relay UE information with non-3GPP connection(s).

Next change

5.3.13.1a	Conditions for resuming RRC Connection for NR sidelink communication/positioning/discovery/V2X sidelink communication

For NR sidelink communication/positioning/discovery an RRC connection is resumed only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency;

1>	if configured by upper layers to transmit NR sidelink discovery and related data is available for transmission:

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2U relay discovery messages and sl-L2-U2U-Relay-r18 is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2U relay discovery messages and sl-L3-U2U-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages and sl-L3U2N-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink non-relay discovery messages and sl-NonRelayDiscovery is included in SIB12:

3>	if the frequency on which the UE is configured to transmit NR sidelink discovery is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-DiscTxPoolSelected or sl-TxPoolSelectedNormal for the concerned frequency;

1>	if configured by upper layers to perform NR sidelink positioning and indicated by upper layers to transmit SL-PRS:

2>	if the frequency on which the UE is configured to transmit SL-PRS is included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-PRS-ResourcesSharedSL-PRS-RP-r18 in sl-TxPoolSelectedNormal for the concerned frequency; or

2>	if the frequency on which the UE is configured to transmit SL-PRS is included in sl-PosFreqInfoList within SIB23 provided by the cell on which the UE camps; and if the valid version of SIB23 does not include sl-PRS-TxPoolSelectedNormal for the concerned frequency;

For L2 U2N Relay UE in RRC_INACTIVE, an RRC connection establishment is resumed in the following cases:

1>	if any message is received from the L2 U2N Remote UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4; or

1>	if RemoteUEInformationSidelink containing the connectionForMP is received from a L2 U2N Remote UE as specified in 5.8.9.8.3;

For V2X sidelink communication an RRC connection resume is initiated only when the conditions specified for V2X sidelink communication in clause 5.3.3.1a of TS 36.331 [10] are met.

NOTE:	Upper layers initiate an RRC connection resume (except if the RRC connection resume is initiated at the L2 U2N Relay UE upon reception of a message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1, or upon reception of RemoteUEInformationSidelink message containing the connectionForMP). The interaction with NAS is left to UE implementation.

Next change

6.3.3	UE capability information elements

*********irrelevant text omitted************

#### –	SidelinkParameters

The IE SidelinkParameters is used to convey capabilities related to MP using N3C link, NR and V2X sidelink communications/positioning.

*********irrelevant text omitted************

End of the change