3GPP TSG-RAN WG2 Meeting #129-bis					R2-2502937

Wuhan, China, April. 7–11, 2025

Source:	Continental Automotive

Title:                       Discussion on NW side data collection aspects

Agenda Item:         8.1.3

Document for:	Discussion and Decision

# Introduction

In the RAN2#129 meeting, the following agreements were made for network sided data collection [1].

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there are scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

In this document, we will further discuss the importance of trigger mechanism for the available data which is collected for NW side model.

# Discussion

According to the RAN2#129 agreement, it is agreed that there should be an indication to indicate to the network that there is some data available for the NW side model. We agree that this is crucial for the network to understand that there is data available at the UE which needs to be transferred to the network. However, the mechanism of triggering the data availability is open for discussion. The current discussion agrees that a data availability indication from the UE to the network is necessary for efficient network-side data collection.

However, the optimal trigger(s) for this indication remain undefined. Triggering too frequently can cause unnecessary signalling overhead, while delaying the indication can lead to data loss or delayed retrieval, hindering the network's ability to optimize AI/ML models. To address the problem of trigger for data there could be a couple of possibilities. The network is aware of the capabilities of the UE, the AI functionality that the UE is serving based on the UE’s capability and functionality reporting. Hence, the network may configure a threshold for the UE side data availability. The UE monitors the configured threshold and which when crossed the UE provides the data which is available for the NW side model.

The threshold can be configured in multiple ways, it could be configured based on AI functionality, dataset content, UE capabilities, etc. There are certain benefits of using threshold as a trigger for data reporting to the network. It helps in reducing the overhead for obtaining the dataset for model training/inference as well as for energy constrained devices the threshold-based technique may help save the battery given that the data is only transferred when the threshold is met.

Additionally, there could be another method where the network configures a timer in addition to threshold for UE side dataset availability. The timer may act as a timebound for providing the collected dataset to the network. The network configures the timer periodically or can be event triggered based. The UE starts collecting the data when the timer starts and when the UE timer stops UE stops collecting the dataset and provide the collected dataset to the network.

The configuration of the timer may be based on the UE’s AI functionality it is serving or based on the UE’s capability. The timer activation may follow the configured threshold, or the network may configure it periodically based on the UE’s AI functionality or capability.

Fig: Process flow for the proposed method

Proposal 1: The network may configure a threshold for the UE side data availability based on which the UE may report the collected data to the network.

Proposal 2: A timer is beneficial for the device to determine the period for which it should collect the data in addition to the configured threshold.

# Conclusions

In this document, we will further discuss the trigger mechanism for the available data which is collected for NW side model.  Additionally, we request RAN2 to discuss the following proposals:

Proposal 1: The network may configure a threshold for the UE side data availability based on which the UE may report the collected data to the network.

Proposal 2: A timer is beneficial for the device to determine the period for which it should collect the data in addition to the configured threshold.

# References

[1] RAN2#129 meeting chair notes.