3GPP TSG-RAN2 Meeting #129bis	R2-2502938

Wuhan, China, 7th-11th April, 2025

Agenda item:	8.2.2

Title:	AIoT Paging: Handling a New Service Request

Source:	Philips International B.V.

Document for:	Discussion

# Introduction

RAN#106 approved WI for Ambient IoT in [1]. One of the objectives for RAN2 is listed as follows:

- Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

In RAN2#129 [2], the following agreements were made:

Agreements: 1. Parallel service requests by the same reader is not supported. 2. The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). 3. The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID 4. 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. 5. RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

To allow further discussion of the FFD items, RAN2#129 allocated an email discussion [2]:

[POST129][035][AIoT] Paging (Qualcomm) Intended outcome: Discuss and address the remaining paging FFSs considering the agreements this meeting: FFS device behaviour if multiple requests are received in parallel (if needed), FFS how reader will generate “transaction ID”, FFS the size of transaction ID. Deadline: Long

The report from this email discussion makes a number of proposals. This contribution discusses some of those proposals

# Discussion

The report of the email discussion concluded with a number of proposals for discussion in RAN2#129bis sessions. Some proposals relating to the expected behaviour of the device when it receives a new service request while processing an earlier one are quoted below:

Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/8/4) Proposal 2: Discuss and decide how a device determines a procedure is ‘ongoing’ or not. Proposal 3: If option (a) is to be agreed in P1: Discuss and decide how a device determines whether a R2D message from Reader is associated with the ongoing procedure or not. Proposal 4: Discuss and decide whether the device needs to distinguish and behave differently between when the same service request is received again from the same reader vs a different reader (yes/no/ffs = 14/12/2). Proposal 5: Discuss and decide whether RAN2 needs to consider the case where the device has already successfully responded to the same service request but now received the same request from a different reader e.g. for location use case (yes/no/unclear = 9/16/3).

In this contribution, we discuss issues raised by these proposals.

## Determining whether a procedure is ongoing or not

Perhaps the key question is contained in proposal 2. Taking into account the agreement from RAN2#129 that, “The device is expected to only perform one procedure at a time,” we might consider a simplified state diagram of the device operation as follows:

Figure 2.1-1: Simplified state diagram of AIoT device

In State 0, there is no ongoing service request at the device. When the device successfully receives a new service request with a transaction identifier, it advances to State 1 and stores the transaction identifer. If the device determines that it is addressed by the service request and therefore must respond, it advances further to State 2 and participates in the access process. On successfully responding to the service request, it drops back into State 1. Alternatively, if the device is not addressed by the service request, it remains in State 1 without entering State 2.

In State 1, the device knows that it should not respond to further paging messages carrying the same transaction identifier. This is particularly important if it has responded to the service request in State 2.

When the reader has completed the service request, the device could return to State 0 and discard the transaction identifier. However, in the absence of an explicit ‘Service Request terminated’ message, the device has to determine itself when the service request has terminated; this process is not well-defined.

Observation 1: The process by which a device determines that a service request has terminated is not well-defined

In this context, in controlled environments, the first indication for a device that the current service request has indeed been terminated may the arrival of a new service request.

Observation 2: In controlled environments, the first indication for a device that the current service request has indeed been terminated may be the arrival of a new service request

We discuss this further in the next section.

## Device behaviour in controlled environments

In certain environments, like factories or warehouses, the network installation can be arranged so that the whole plant or well-defined areas can be covered by a single base station, which may, nevertheless support multiple readers covering the same or overlapping coverage zones.

### New service request from the same reader

Based on the RAN2#129 agreements, “Parallel service requests by the same reader is not supported,” a reader will not issue a new service request until it considers that the current service request has terminated. If, for some reason, it is necessary to override the current service request with a new one, then the reader shall terminate the current service request before launching the new one.

Observation 3: In accordance with agreements from RAN2#129, the reader will not launch a new service request until the current one has terminated

Given this and the observations made in the previous section, when a device receives a new service request from the same reader, the most appropriate behaviour is for it to abandon the current service request, at whatever stage it has reached, and respond as required to the new service request.

Normally, a device should be in State 1 or State 0 when a new request is received. It is possible that the device is in State 2, attempting to respond to the current service request when it is terminated by the reader. However, even in State 2, if a new service request is received, the device should abandon the current service request.

Proposal 1: When a device receives a new service request from the same reader, it abandons the current service request and responds as required to the new service request

### New service request from a different reader

This can be extended easily to the multi-reader scenario when all readers are managed by the same base station. By extension, there should only be one ongoing service request across all readers covering the same coverage zone.

Observation 4: In a controlled environment, there should only be one ongoing service request across all readers covering the same coverage zone

Given this, a new service request issued by a new reader implies that any ongoing service request is terminated. Therefore, the device’s expected behaviour should be the same.

Proposal 2: When a device receives a new service request from a different reader belonging to the same base station, it abandons the current service request and responds as required to the new service request

### Same service request from a different reader

Proposals 4 and 5 of the email discussion ask about the expected device behaviour when the same service request is received from the same reader v. a different reader.

One reason that a base station might send the same service request via a different reader is to solicit a response from devices that are part of the group but are inaccessible to the first reader.

Observation 5: A base station might send the same service request via a different reader is to solicit a response from devices that are part of the group but are inaccessible to the first reader

In this case, it is an ongoing service request and there is no reason for a device to treat this any differently regardless of the fact that it comes from a different reader. Thus, if it has already responded to the service request, it need not respond to the second reader; if it has not responded to the first reader, it may respond to the second reader.

Proposal 3: A device receiving the same service request via a different reader shall treat it as an ongoing service request and respond as necessary

If the base station is attempting to solicit new responses from devices via different readers for positioning purposes then a new service request should be used.

Proposal 4: If the base station is attempting to solicit new responses from devices via different readers for positioning purposes then a new service request should be used

## Operation in uncontrolled environments

Future applications of AIoT could include scenarios in which readers with overlapping coverage areas are operated independently. In such environments, it is possible that a second service request could arrive from a second, independent reader. If this second reader is not co-ordinated with the current reader, then there is the possibility that two access procedures will operate in parallel. This is potentially problematic because both readers will transmit access occasions, which may interfere with each other; even if not, the device might not be able determine in which access occasion it should respond.

Observation 6: In uncontrolled environments, two or more service requests could become active simultaneously via independent readers, causing problems for the device

To mitigate these problems, enhancements to the signalling and to the device handling could be considered to provide some robustness against these situations. During the email discussion, at least one company suggested that access occasions sent by the reader might also need to be labelled with the transaction identifier. It might also be more appropriate for the device to ignore an incoming service request if one is already ongoing; appropriate signalling might help the device decide the most appropriate course of action.

While the details of this discussion are out of scope for now, RAN2 should nevertheless allow for future enhancements to the signalling and device handling to provide operational robustness in uncontrolled environments.

Proposal 5: RAN2 should allow for future enhancements to the signalling and device handling to provide operational robustness in uncontrolled environments

# Conclusion

In this paper, we make the following observations:

Observation 1: The process by which a device determines that a service request has terminated is not well-defined.

Observation 2: In controlled environments, the first indication for a device that the current service request has indeed been terminated may be the arrival of a new service request

Observation 3: In accordance with agreements from RAN2#129, the reader will not launch a new service request until the current one has terminated

Observation 4: In a controlled environment, there should only be one ongoing service request across all readers covering the same coverage zone

Observation 5: A base station might send the same service request via a different reader is to solicit a response from devices that are part of the group but are inaccessible to the first reader

Observation 6: In uncontrolled environments, two or more service requests could become active simultaneously via independent readers, causing problems for the device

From these observations, we derive the following proposals:

Proposal 1: When a device receives a new service request from the same reader, it abandons the current service request and responds as required to the new service request

Proposal 2: When a device receives a new service request from a different reader belonging to the same base station, it abandons the current service request and responds as required to the new service request

Proposal 3: A device receiving the same service request via a different reader shall treat it as an ongoing service request and respond as necessary.

Proposal 4: If the base station is attempting to solicit new responses from devices via different readers for positioning purposes then a new service request should be used

Proposal 5: RAN2 should allow for future enhancements to the signalling and device handling to provide operational robustness in uncontrolled environments

# References

[1]	RP-243326, Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106, Dec 2024

[2]	Draft minutes of RAN2#129

[3]	Draft report of email discussion [POST129][035][AIoT] Paging (Qualcomm)