3GPP TSG-RAN WG2 Meeting #129bis	R2-2502947

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.8.2

Source:	NERCDTV

Title:			  Further discussion on NTN DL coverage enhancements

WI Code:       NR_NTN_Ph3-Core

Document for:	Discussion and Decision

- Introduction

According to the updated WID for R19 NTN, the objective of downlink coverage enhancements are as follows [1]:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

And in the meeting of RAN2 128, the following agreements were made for downlink coverage enhancements [2]:

Agreements: 1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.) 2. As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs) 3. RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells 4. For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

Further, in the RAN2 129 meeting, it has been agreed for downlink coverage enhancements that [3]:

Agreements: - RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer - RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame - RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) - The number of SMTC/gaps a UE needs to consider at any time will not be increased further - From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1. - RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). - RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

In this contribution, we will further discuss the downlink coverage enhancements including possible SMTC enhancements and access control.

- Discussion

Possible SMTC enhancements

One objective of downlink coverage enhancements for R19 NTN is to define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. And in the last meeting, it was agreed to support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC, and if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient.

Although different SMTCs may be applied for different cells, the UE only needs to consider some of the SMTCs according to its location considering the aspect of power saving. In our point of view, UE may only need to select the SMTCs of some neighbour cells close to the UE and then perform measurements using the SMTCs selected.

Observation 1: UE only needs to consider some of the SMTCs according to its location.

However, if the UE selects SMTCs for measurements based on its accurate location, it needs to perform positioning and obtain its location, which will introduce power consumption and complexity if it acquires its location frequently.

Considering the additional power consumption and complexity above, we propose an alternative solution to use the SSB to indicate UE location at a large granularity. UE can determine the SMTCs used for measurements according to the serving SSB where the UE is located currently as shown in the following figure. When the serving SSB changes, e.g. due to the movement of UE or satellite, the SMTCs selected may change accordingly.

Figure 1 SSB based SMTC selection

Proposal 1: Introduce SSB based SMTC selection.

Cell barring

According to the updated WID agreed in RAN plenary #106 meeting, the R18 network energy saving will not be considered for R19 NR NTN. Besides, it was also agreed that the maximum of the additional default value of extended SSB periodicity assumed by UE during initial access is 160ms. On the other hand, there may be no impact on legacy UEs considering the link level enhancements discussed so far. Therefore, legacy UEs should not be barred from accessing the DL CE cells since there are no issues of backward compatibility.

Proposal 2: Legacy UEs should not be barred from accessing the DL CE cells.

3. Conclusion

Based on the above discussion, we give the following observation and proposals:

Observation 1: UE only needs to consider some of the SMTCs according to its location.

Proposal 1: Introduce SSB based SMTC selection.

Proposal 2: Legacy UEs should not be barred from accessing the DL CE cells.

References

[1] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

[2] RAN2#128 chairman’s notes.

[3] RAN2#129 chairman’s notes.