3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502948

Wuhan, PRC, 07 – 11 April 2025

Agenda item:	8.7.4.1

Source:	Samsung

Title:	Finalising LCP design for XR Ph3

Document for:	Discussion & Decision

# Introduction

In the RAN2#128 meeting (November 2024), RAN2 agreed the following for LCP enhancements for Rel-19 XR:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

At the subsequent and most recent RAN2 meeting (RAN2#129, February 2025), the following was additionally agreed:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

In this paper, we discuss the key remaining issues of the operations related with the additional LCH priority, and the impact of agreements made. We look at the details of intra-UE prioritization and its specification impact, UE behaviour when Bj is negative, enhancements to MAC PDU assembly, as well as handling of other priority traffic e.g. SRBs in light of the additional LCH priority for XR traffic.

# Discussion

## Additional LCH priority in other procedures: high-level spec impact

For overlapping grants, 38.300 currently states that decision on which one to transmit is based on “comparison between the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDUs associated with the overlapping resources”. For all cases other than autonomousTx and HARQ retransmission, “priority” used in this comparison is based on data which can be multiplexed in the MAC PDUs, and not necessarily data that will eventually end up in the MAC PDU which will be sent using this grant (we expand upon this further in Section 2.2). In terms of the spec impact, the key issue is whether we make it clear in the spec that “priority” is the adjusted priority (if configured, and if threshold is breached), e.g. add to 38.300 the underlined bit: “...comparison between the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDUs associated with the overlapping resources (where the priority is the adjusted priority if the conditions specified in clause 5.4.3.1.3 in TS 38.321 [6] are met)”. Our proposal is to indeed do this.

Proposal 1. RAN2 to agree to clarify by addition of text to intra-UE prioritization procedure in TS 38.300 that the priority used for clashing grants comparison is the adjusted priority if the conditions specified in clause 5.4.3.1.3 in TS 38.321 [6] are met.

## The intra-UE prioritization case and the additional LCH Priority: additional spec impact

Looking into the intra-UE prioritization further, as a reminder, according to 38.300, “in case a configured uplink grant transmissions or a dynamically allocated uplink transmission overlaps in time with a scheduling request transmission, the UE prioritizes the transmission based on the comparison between the priority of the logical channel which triggered the scheduling request and the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDU associated with the overlapping resource”. Therefore, in this legacy intra-UE prioritization, when more than one UL grant(s) and/or SR(s) overlap in time domain, a priority-based comparison among the overlapped UL grant(s) and/or SR(s) is conducted to determine which UL grant or SR should be transmitted. The priority-based comparison is based on legacy (default) LCH priority, i.e., the priority of UL grant is defined as the highest (legacy) priority of LCHs multiplexed or to be multiplexed in the corresponding MAC PDU, and the priority of SR is defined as the (legacy) priority of the LCH triggering the SR.

Based on 38.321, we also have the following:

“For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.”

The following should therefore be noted:

Observation 1. For the cases of autonomousTx and HARQ retransmission, priority of an uplink grant is determined following the MAC PDU assembly; in all other cases, it is determined prior to the actual MAC PDU assembly and based on data available that can be multiplexed (and not necessarily data that will actually end up in the MAC PDU which will be sent using this grant).

What is different in the case of intra-UE prioritization linked to logical channels that have LCH priority-adjusted data to be transmitted and which are multiplexed or can be multiplexed in MAC PDUs associated with the overlapping resources, is whether an update to the determined uplink grant priority (for purposes of intra-UE prioritization) is needed following the MAC PDU assembly, e.g. in cases where all of the LCH priority-adjusted data was inserted into the MAC PDU.

(Similar considerations apply for the case of intra-UE overlapping resources prioritization linked to scheduling request triggered by logical channels that have LCH priority-adjusted data to be transmitted, where RAN2 should discuss whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority.)

The key question in our view is whether we additionally update the determined uplink grant priority following the MAC PDU assembly (i.e. possibly revise the decision on which grant to transmit – although this could increase complexity from implementation point of view); this could be relevant in a case where some high priority data did not actually make it into the MAC PDU. This should not happen often – it would not happen often that highest priority LCH data at time of grant priority determination, did not actually end up in the MAC PDU. What is more likely, however, is that some data packets may have expired by the time of transmission and in this case we may wish to revise the decision. In other words, if the priority-adjusted data that had been included in the MAC PDU loses the need for special handling, the justification for continuing with the additional priority in the intra-UE prioritization becomes weaker.

Based on the above discussion, we propose the following:

Proposal 2. For the case of intra-UE prioritization other than autonomousTx and HARQ retransmission case, which is linked to logical channels that have LCH priority-adjusted data to be transmitted and which can be multiplexed in MAC PDUs associated with the overlapping resources, RAN2 to discuss whether an update to the determined uplink grant priority is needed following the MAC PDU assembly e.g. at time of MAC PDU transmission.

Priority of an SR is the priority of the LCH that triggered the BSR which could not be sent (and therefore triggered said SR). The question is whether something could happen to make this priority no longer relevant (when determining whether to send this SR or a grant that clashes with it). It is possible that – by the time of the MAC PDU transmission – data in this LCH has ‘expired’ and the SR no longer needs to be sent. It is also possible that data linked to the grant which clashes with the SR has expired. Therefore we propose for RAN2 to discuss whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission. One additional consideration is whether we treat non-BSR (and non-DSR) triggered SR differently.

Proposal 3a. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request triggered by logical channels that have LCH priority-adjusted data to be transmitted, RAN2 to discuss whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission.

Proposal 3b. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request, RAN2 to discuss whether to capture the cause of SR triggering in the specifications i.e. make it clear that SR in question was triggered according to the BSR/DSR procedures (as the additional priority would only be relevant in this case).

## Case of an LCH with an upgraded priority with negative Bj

Currently, the LCHs are served in the first round of LCP procedure only if corresponding Bj>0. If corresponding Bj becomes negative, it will in any case be incremented by PBR × T before every instance of the LCP procedure. Therefore we do not foresee a case where Bj would stay negative for long enough to have any substantial impact; the only case could be where PBR is set very low (and it is unclear why the network would do that for XR data), or if T is very short i.e. if the LCH in question is selected for MAC PDU assembly very frequently – but if this happens, presumably this channel has high priority and will hence be served in 2nd rounds of these ostensibly very frequent LCP procedures. As a result, we do not envisage a use case for Bj becoming negative – and staying negative – for too long for XR data.

Proposal 4. Do not allow an LCH with an upgraded priority to be transmitted in the 1st round of LCP when Bj is negative i.e. stick to legacy behaviour.

## Data prioritization over comparatively less urgent MAC CEs

We now turn our attention to the potential use-case of prioritization of data over MAC CEs, which for LCHs with priority-adjusted data, is potentially very relevant as the in-time delivery is important. So far, dedicated configured grant or resource separated by LCP have been introduced over multiple releases of NR to aid in-time delivery. In Rel-18, multiple CG occasions, DSR reporting and discard mechanism were introduced to support in-time delivery with higher resource efficiency. However, most of the MAC CEs, e.g., BSR/DSR/PHR, etc., still have higher absolute priorities over any of XR data, and those MAC CEs can use any type of uplink grants. This could make unexpected interruption during LCP and MAC PDU generation, resulting in LCH priority-adjusted data being delayed due to such MAC CEs. Therefore, we see value in data prioritization over some less important/urgent MAC CEs, and propose the following:

Proposal 5. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.

As an illustration of some additional underlying issues, and using the BSR MAC CE as an example, a BSR is triggered and LCH#1 uses new/adjusted priority P_a as it contains SDUs close to expiry. By the time MAC PDU assembly starts, data arrives into LCH#2 which has priority P_b1 > P_a (and this was not the case when BSR was triggered, where LCH#2 had a priority P_b<P_a). If we now want to prioritize BSR MAC CE over data or not (in legacy systems we would prioritize the BSR MAC CE, but for XR data we may wish to prioritize data), it is unclear whether we should compare P_b and P_a, or P_b1 and P_a.

Observation 2: Re-calculating LCH priorities at time of MAC PDU assembly relative to priorities at the time of BSR triggering could cause issues in cases where we want to prioritize data over a BSR MAC CE.

Proposal 6. Assuming RAN2 agrees to prioritizing LCH with LCH priority-adjusted data over BSR MAC CEs where beneficial, when determining the priority of the BSR MAC CE relative to data in order to compare it with the priority of the data, the priority should be that of the logical channel that resulted in the most recent BSR trigger prior to MAC PDU assembly, and whose data cannot all be included in the MAC PDU.

## Handling of SRBs

In the past meetings, certain submissions discussed the perceived impact of the additional LCH priority on timely transmission of signaling data. In our view, the handling of this case is best left to network implementation:

Proposal 7. Handling of any concerns to do with XR traffic potentially delaying SRBs traffic is left to network implementation.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. RAN2 to agree to clarify by addition of text to intra-UE prioritization procedure in TS 38.300 that the priority used for clashing grants comparison is the adjusted priority if the conditions specified in clause 5.4.3.1.3 in TS 38.321 [6] are met.

Proposal 2. For the case of intra-UE prioritization other than autonomousTx and HARQ retransmission case, which is linked to logical channels that have LCH priority-adjusted data to be transmitted and which can be multiplexed in MAC PDUs associated with the overlapping resources, RAN2 to discuss whether an update to the determined uplink grant priority is needed following the MAC PDU assembly e.g. at time of MAC PDU transmission.

Proposal 3a. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request triggered by logical channels that have LCH priority-adjusted data to be transmitted, RAN2 to discuss whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission.

Proposal 3b. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request, RAN2 to discuss whether to capture the cause of SR triggering in the specifications i.e. make it clear that SR in question was triggered according to the BSR/DSR procedures (as the additional priority would only be relevant in this case).

Proposal 4. Do not allow an LCH with an upgraded priority to be transmitted in the 1st round of LCP when Bj is negative i.e. stick to legacy behaviour.

Proposal 5. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.

Proposal 6. Assuming RAN2 agrees to prioritizing LCH with LCH priority-adjusted data over BSR MAC CEs where beneficial, when determining the priority of the BSR MAC CE relative to data in order to compare it with the priority of the data, the priority should be that of the logical channel that resulted in the most recent BSR trigger prior to MAC PDU assembly, and whose data cannot all be included in the MAC PDU.

Proposal 7. Handling of any concerns to do with XR traffic potentially delaying SRBs traffic is left to network implementation.