3GPP TSG-RAN2 #129bis	R2-2502950

Wuhan, China,Apr. 7th – 11st, 2025

Agenda item:	8.6.3 (NR_Mob_Ph4-Core)

Source:	LG Electronics Inc.

Title: 	L1 event triggered measurement report

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss issues related to event triggered LTM measurement report.

- Agreements of RAN2#128 - TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - SSBRI and CRI is used to represent the candidate beam ID in L1 MR MAC CE. - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Revisit it once we can have better understanding on MR MAC CE format and how much bits actually can be saved from differential RSRP values. - Support the truncated measurement report MAC CE. - Agreements of RAN2#129 - Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused". - Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide). - Do not create a new resource configuration just for the event triggered measurements. - Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused". - Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report. - Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue. - TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval. - Cell specific offset is not supported for LTM L1 event evaluation. - The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity. - Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. - Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam. - R18 differential RSRP encoding is used as baseline. - LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant. - Truncated MR MAC CE includes the triggered beam information: - One report config ID (event config ID) - At least one triggered beam id (SSBRI or CRI) + L1 RSRP - Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur. - For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

2. Discussion

2.1 Reduction of successive triggering of multiple L1 MR MAC CEs

Since TTT is applied per beam, it may happen that two or more beams successively satisfy the entering condition of the same report configuration. In this case, each TTT corresponding to different beam may expire successively, and consequently multiple L1 MR MAC CEs are triggered successively, as shown in Figure 1.

Figure 1. Successive triggering of L1 MR MAC CEs for the same report config

Note that such successive reporting can already occur in event triggered L3 MR. If two different cells of the same measurement object satisfy the entry condition of the same event at different times (say, cell A first and then cell B) and they keep satisfying the entry condition for TTT respectively, the cell A triggers an initial measurement reporting upon expiry of TTT for cell A and the cell B shortly triggers a subsequent measurement reporting upon expiry of TTT for cell B. If the second measurement reporting is triggered, the numberOfReportsSent is reset but the existing cellsTriggeredList is not reset but updated by the cell B (i.e., cellB is included into the cellsTriggeredList).

If triggering of MR MAC CE follows event triggered L3 MR approach, the reporting may be more frequent. For example, if the acquisition times of beam measurement results for different beams are slightly different, or two different beams satisfy the entering condition at slightly different times, each beam will trigger reporting successively. Higher dynamics of beam quality also contributes to a frequent subsequent LTM reporting, compared to L3 cell quality, provided that L2 filtering of L1 measurement results is not introduced. Considering that each of the triggered reporting would also lead to periodical reporting, the total  number of resulting L1 reporting could be very high.

To reduce successive triggering of L1 MR MAC CEs within a short period of time, it is beneficial to introduce some restrictions in triggering L1 MR MAC CEs. For example, when the L1 MR MAC CE is triggered at t1 by the beam1 for an LTM event, if there is other beam (beam2 and beam3) for which TTT is running for the same event, the L1 MR MAC CE at t1 includes the beam1 as triggering beam, and also includes beam2 and beam3. Since the L1 MR MAC CE includes beam1, beam2, and beam3 with beam1 as triggering beam, the necessity of the triggering subsequent L1 MR MAC CE by beam2 and beam3 at t2 and t3 becomes weakened. Then, UE can refrain from triggering the subsequent L1 MR MAC CEs, regardless of the TTT status for these beams, as shown in Figure2. The details of how to refrain from triggering the subsequent L1 MR MAC CEs can be further discussed at the next meeting once RAN2 agrees to introduce some restrictions in triggering L1 MR MAC CEs.

Figure 2. Reduction of successive triggering of L1 MR MAC CEs for the same report config

Proposal 1: When L1 MR MAC CE is triggered by the beam (i.e., triggering beam) for a LTM report config ID, the L1 MR MAC CE also include other beam(s) for which TTT is running for the same report config ID as TTT-running beam(s).

Proposal 2: When L1 MR MAC CE is triggered by a beam (beam1) for a report configuration, if there is other beam (beam2) for which TTT is already running for the same report configuration, the UE refrains from triggering another L1 MAC CE for the beam2, if configured to do so by network.

2.2 Handling of multiple pending L1 MR MAC CEs

When UE receives UL grant, UE may already have more than one pending L1 MR MAC CEs to be transmitted. Then, UE should be able to determine whether UE should include all the pending L1 MAC CEs in MAC PDU to construct and transmit. If the UL grant is insufficient to transmit all the pending L1 MR MAC CEs, UE should select which L1 MR MAC CE MR(s) to transmit first, and it should be able to also determine how to handle remaining MAC CE(s) (e.g., discard or keep them). The expected UE behaviors may be different depending on whether those pending L1 MR MAC CEs are triggered by the same or different event IDs.

Let us consider the following case first where multiple L1 MR MAC CEs are triggered by the same report configuration.

Case (a) Consider the case that two L1 MR MAC CEs were triggered by different LTM report config IDs at and these two MAC CEs are pending, and UE now receives UL grant. Figure 3 illustrates this case.

If the UL grant is sufficient to transmit both L1 MR MAC CEs, UE should send both MAC CEs in one MAC PDU, as shown in the Figure 3.

Figure 3. L1 MR MAC CEs are triggered by beam1 and beam2 for different events successively at t1 and t2. While two L1 MR MAC CEs are pending, UE receives UL grant sufficient to carry two L1 MR MAC CEs. Then UE transmits MAC PDU carrying the two L1 MR MAC CEs.

For the case (b), if the UL grant is insufficient to transmit both, UE should select only one to transmit with the UL grant. For the selection, if FIFO should be applied, UE should prioritize the L1 MR MAC CE#1 that was triggered earlier; if LIFO should be applied, UE should prioritize the L1 MR MAC CE#2 that was triggered later. Note, however, it is not clear which one of the two L1 MR MAC CEs is more urgent because there are triggered by different events. In other words, it does not really matter which principle among FIFO and LIFO is applied to this case. Therefore, we think it can be left to UE implementation how to select L1 MR MAC CEs to transmit first among L1 MR MAC CEs triggered by different LTM report config IDs, as shown in Figure 4, where UE arbitrarily selects L1 MR MAC CE triggered by reportConfig_A for MAC PDU transmission at t3.

Figure 4. L1 MR MAC CEs are triggered by beam1 and beam2 for different events successively at t1 and t2. While two L1 MR MAC CEs are pending, UE receives UL grant insufficient to carry two L1 MR MAC CEs. So UE arbitrarily selects one of the two L1 MR MAC CEs and transmits MAC PDU including the selected one at t3. Then UE receives another UL grant and transmits MAC PDU including remaining L1 MAC CE.

After transmission of the selected L1 MR MAC CE, UE should await another UL grant to transmit remaining (i.e., pending) L1 MR MAC CE. Since those two L1 MR MAC CEs were triggered by different LTM report config IDs, transmission of one L1 MR MAC CE should not affect transmission of the other L1 MR MAC CE. As shown in Figure 4, UE at t4 transmits remaining pending L1 MR MAC CE triggered by reportConfig_B.

Proposal 3: For UE having more than one pending L1 MR MAC CEs triggered by different LTM report config IDs, if UE receives UL grant which is insufficient to transmit all the pending L1 MR MAC CEs, it is up to UE implementation which L1 MR MAC CEs to select and transmit first.

Proposal 4: Upon transmitting the selected L1 MR MAC CE triggered by a LTM report config ID, UE does not discard other pending L1 MR MAC CEs triggered by different LTM report config ID.

Now we consider a different case below where multiple L1 MR MAC CEs are triggered by the same report configuration.

Case (b) Consider the case that two L1 MR MAC CEs are triggered by the same LTM report config IDs at slightly different times and these two MAC CEs are pending. Then, UE receives UL grant. Figure 4 illustrates this case.

In case UL grant is sufficient to transmit both, it is questionable if UE should transmit MAC PDU including both L1 MR MAC CEs in case these two MAC CEs were triggered by the same report config ID. From network point of view, as shown in Figure 4, since L1 MR MAC CE#2 includes both beam1 and beam2, UE’s transmission of L1 MR MAC CE#1 is not needed. In fact, if UE transmits both L1 MR MAC CEs, network would only get confused because it cannot know which one is the most recent one.

Figure 4. L1 MR MAC CEs are triggered by beam1 and beam2 for the same event successively at t1 and t2. While two L1 MR MAC CEs are pending, UE receives UL. UE needs not include L1 MR MAC CE#1 but only include L1 MR MAC CE#2 in MAC PDU, since the MAC CE#2 includes all triggered beams (beam1 and beam2).

We can avoid transmitting multiple L1 MR MAC CEs triggered by the same report config ID, two alternatives can be considered: alt1) if a L1 MR MAC CE is triggered by an LTM repot config ID, UE discards all the pending L1 MR MAC CEs previously triggered by the same LTM report config ID, and alt2) When UE constructs MAC PDU with multiple pending L1 MR MAC CEs triggered by the same report config ID, UE selects the most recent L1 MR MAC CE and then discard non-selected one(s) among them.

Proposal 5: To avoid transmitting multiple L1 MR MAC CEs triggered by the same report config Id, one of the followings are adopted:

- Alt1) If one L1 MR MAC CE is triggered by an LTM repot config ID, UE discards all the pending L1 MR MAC CEs previously triggered by the same LTM report config ID.

- Alt2) For UE having more than one pending L1 MR MAC CEs triggered by same LTM report config ID, UE selects latest L1 MR MAC CE(s) triggered by the LTM report config ID and transmits them. Upon transmitting the selected L1 MR MAC CE(s), UE discards other pending L1 MR MAC CEs previously triggered by the same LTM report config ID.

Proposal 6: In case two L1 MR MAC CEs are pending, if UL grant is sufficient for one regular L1 MR MAC CE or for two truncated L1 MR MAC CEs but insufficient for two regular L1 MR MAC CEs, UE is allowed to construct MAC PDU either by including one regular L1 MR MAC CEs or by including two truncated L1 MR MAC CEs, based on its implementation choice.

The proposal 6 addresses the case that two L1 MR MAC CEs are pending, but this can be generalized to cases with more than two pending L1 MR MAC CEs.

2.3 Format of L1 MR MAC CE

L1-RSRP for leaving beam

RAN2 agreed that for both normal L1 MR MAC CE and truncated L1 MR MAC CE, the beam information includes beam ID and L1-RSRP for triggered beams. Including L1-RSRP for beam fulfilling entering condition of the event is reasonable but including L1-RSRP for beam fulfilling entering condition of the event may be questionable for beam fulfilling leaving condition of the event for TTT, because network does not really care about the beam quality of the leaving beam. For this reason, one may consider to omit L1-RSRP for leaving beam(s). However, the saving of the signaling achievable by omitting L1-RSRP would be marginal considering differential L1-RSRP values, and omitting L1-RSRP for leaving beam complicates the format of L1 MR MAC CE. For simplicity of L1 MAC CE format, we see it more desirable to commonly include L1-RSRP values for entering beams and leaving beams.

Proposal 7: L1-RSRP is also included for leaving beam, i.e., the beam fulfills entering or leaving condition of the event for TTT duration.

Explicit indication of entering/leaving beam

Based on the intensive discussion at RAN2#129, RAN2 understood that network should be able to distinguish which beam(s) among the beams reported in the L1 MR MAC CE actually satisfy entering condition of the event. RAN2 also agreed to introduce some explicit indication, and detailed signaling is FFS. Regarding the format of the explicit indication, two options can be considered.

- Bitmap indication: different bit corresponds to the status of different beams included in the MAC CE.

- Bit per beam: one explicit beam is indicated per each beam included in the MAC CE

In both options, value one of the concerned bit indicates that the corresponding beam is an entering beam (i.e. the beam that fulfills entering condition of the event for TTT duration), and value zero of the concerned bit indicates that the corresponding beam is not an entering beam (i.e. not the beam that fulfills entering condition of the event for TTT duration)

Comparing both options, we note that bitmap indication occupies a fixed bit size, and the bit size should be the maximum number of beams to be included in the MAC CE. Considering how many beams can be included in the MAC CE may vary depending on UE measurement capabilities and the presence/quality of the beams, the bitmap approach may result in unused bits in the bitmap. On the other hand, bit per beam approach does not result in such unused bits since the bit is included only for include beams.

Observation: Bitmap indication occupies a fixed bit size that is equal to the maximum number of beams to be included in the MAC CE, which may result in some bits unused in cases reported number of beams is smaller than the maximum number.

Based on the observation, we propose to adopt an explicit per-beam indication.

Proposal 7: For each beam, introduce one bit to indicate whether the beam is a triggered beam for entering condition.

For the explicit indication for leaving beams, the same signaling format can be applied.

Proposal 8: For each beam, introduce one bit to indicate whether the beam is a triggered beam for leaving condition.

Selection/Ordering of beams in L1 MR MAC CE

RAN2 agreed to have a common format between regular L1 MR MAC CE and truncated MR MAC CE. Considering there may be multiple beams to include and truncated MAC CE can include a part of the beams based on UL grant size, RAN2 should determine how to select/order the beams for inclusion of the beams in the concerned L1 MR MAC CE.

Regarding the beams to include, we note that beams can be categorized as follows:

  - Entering beam, i.e., the beam is newly entering into a triggered beam list (entering condition of event has been met for the beam with TTT)

  - Leaving beam, i.e., the beam is leaving into a triggered beam list (leaving condition of event is met for the beam with TTT)

  - Other-triggered beam, i.e., the beam that has been already included in a triggered beam list (i.e., the beam was already reported as a triggered beam in previous reporting and the beam remains as a triggering beam)

  - non-triggered beam, i.e., the beam that is neither entering beam nor leaving beam nor other-triggered beam.

If there is at least one entering beam or leaving beam, it is important to not omit this beam in the transmitted L1 MR MAC CE, because this is the beam that has actually triggered the MAC CE. Provided that truncated L1 MR MAC CE is designed to include up to K beams for some K, the entering/leaving beam, if present, should be included first. Among entering beam and leaving beam, the ordering does not really matter. For simplicity, entering beam can be included first.

After including the entering/leaving beams, if any, in the MAC CE, other-triggered beam can be further included.

- In case regular L1 MR MAC CE is triggered, if the maximum size of the regular MAC CE can accommodate other-triggered beam(s), other-triggered beam is included in the MAC CE. If there is more than one other-triggered beams to be included in the MAC CE and the maximum MAC CE size cannot accommodate all of the other-triggered beam, beams with higher quality should be selectively included.

- In case truncated L1 MR MAC CE is triggered, if the maximum size of the truncated MAC CE, as determined by the UL grant size, can accommodate at least one other-triggered beam(s), the truncated MAC CE includes the other-triggered beam in the MAC CE. If there is more than one other-triggered beams to be included in the MAC CE and the UL grant size cannot accommodate all of the other-triggered beams to be included in the MAC CE, beams with higher quality should be selectively included, up to UL grant size.

After including other-triggered beam(s) in the MAC CE, non-triggered beams can be further included.

- In case regular L1 MR MAC CE is triggered, if the maximum size of the regular MAC CE can accommodate non-triggered beam(s), non-triggered beam is included in the MAC CE. If there is more than one non-triggered beam to be included in the MAC CE and the maximum MAC CE size cannot accommodate all of the non-triggered beams, beams with higher quality should be selectively included.

- In case truncated L1 MR MAC CE is triggered, if the maximum size of the regular MAC CE, as determined by the UL grant size, can accommodate non-triggered beam(s), non-triggered beam is included in the MAC CE. If there is more than one non-triggered beam to be included in the MAC CE and the UL grant size cannot accommodate all of the non-triggered beams to be included in the MAC CE, beams with higher quality should be selectively included.

Proposal 9: When L1 MR MAC CE is constructed, the following principles are applied for selection/ordering of the beams to be included in the L1 MR MAC CE:

- Inter-beam category ordering: entering beam(s), if any > leaving beam(s), if any > other triggered beam(s) than the entering beam(s) if any > non-triggered beams, up to maximum number of beam entries allowed for the L1 MR MAC CE

- Intra-beam category ordering: beam with higher quality is included first

- For truncated L1 MR MAC CE, the number of beams included in the MAC CE is maximized for the given UL grant.

Since differential L1-RSRP is supported, one of the beam should serve as reference. The easiest way is to take the best quality beam (i.e. the beam with highest L1-RSRP) among the beams included in L1 MR MAC CE as reference beam. For the reference beam, beam ID and non-differential L1-RSRP are included. Then, for any other beams, beam ID and differential L1-RSRP values can be included.

Proposal 10: For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as reference beam. For the reference beam, beam ID and non-differential L1-RSRP are included. For non-reference beam, beam ID and differential L1-RSRP value derived based on the non-differential L1-RSRP of the reference beam are included.

2.4 Truncated L1 MR MAC CE

Triggering of truncated L1 MR MAC CE

If UL grant is not sufficient for regular L1 MR MAC CE, UE has to consider transmitting truncated L1 MR MAC CE. Here, the question is what the necessary condition for transmitting truncated L1 MR MAC CE would be. For example, network may want UE to transmit truncated L1 MR MAC CE if all the entering/leaving beams, if any, are included in the truncated L1 MR MAC CE. Or network may want to UE to transmit truncated L1 MR MAC CE if all triggered beam, if any, are included in the truncated L1 MR MAC CE.

Proposal 11: To discuss if network should be able to enforce some restrictions to UE in transmitting truncated L1 MR MAC CE related to entering/leaving beams or triggered beams.

Normal L1 MR MAC CE after transmitting truncated L1 MR MAC CE?

Upon transmitting a truncated L1 MR MAC CE triggered by an event, it is currently unclear whether UE should keep attempting to transmit normal L1 MR MAC CE for the event or whether UE should refrain from transmitting normal L1 MR MAC CE.

Upon the reception of the truncated L1 MR MAC CE, different network implementations for subsequent behaviors seem possible. In one implementation, network may want to further receive regular L1 MR MAC CE for better understanding on overall beam status before making mobility decision. In other implementation, network may make decisions based on the received truncated L1 MR MAC CEs directly without waiting for regular L1 MR MAC CE. Provided that both implementations are possible, we see it reasonable for network to be able to configure UE whether to keep normal L1 MR MAC CE as pending, upon transmitting truncated L1 MR MAC CE.

Proposal 12: Network configures UE via RRC with whether UE should still consider triggered L1 MAC CE as pending or cancel it after transmitting truncated L1 MAC CE.

2.5 Restriction of evaluated RSs via candidate cell indication

For LTM, one or more beam sets are configured as common LTM configuration for candidate cells. The common configuration can be maintained and thus reused after LTM, which can reduce signaling of RRC reconfiguration upon event LTM execution.

LTM report configuration is configured within candidate cell configuration, and for each LTM report configuration, one of the common beam sets is configured as associated beam set. Then, when UE needs to perform event evaluation based on the LTM report configuration, UE needs to measure all the beams included in the associated beam set.

In the last RAN2 meeting, RAN2 discussed whether network should be able to further indicate a certain association between candidate cells and beams of the beam set. For example, if network configures a candidate cell ID within a report configuration for event-triggered beam reporting, UE only needs to focus on measurements of beams associated with the indicated candidate cell while excluding measurements of beams associated with other candidate cell(s). In our view, this measurement restriction would allow UE to skip unnecessary measurements, and such measurement reduction gain would be larger for a larger beam set commonly configured for multiple candidate cells.

Proposal 13: Network can optionally configure a candidate cell via RRC within report configuration of event LTM. If the candidate cell is configured, UE only evaluates RSs associated with the candidate cell among RSs associated with the event.

Event evaluation and UE measurement capability

In the MAC running CR, the following evaluation method is adopted for event evaluation, and MAC running CR rapporteur invites views.

- Method#1: for each reportID (associated with event), report is triggered if any beam (associated with this reportID) fulfils the event by evaluating all beams during TTT for each beam.

This approach is fine to us in general. But we wonder how this determines actual measurement requirements in relation to TTT. We think different UEs have different measurement processing capabilities. Then, for a given number of beams to be measured, different UE would perform different number of actual measurements for each beam during a given time duration. For a given UE, depending on the number of configured beams associated with an event, the number of actual measurements for a given beam for a given duration would be different, as shown in Figure 6. If actual number of measurements for each beam is different during TTT across UEs or across different beam set sizes, network has difficulty in deciding which TTT duration or beam set size is more proper.

Figure 6. Different number of actual measurements of each beam during TTT, depending on the UE measurement capabilities (figure 6a versus 6b) and the measurement beam set size (figure 6c versus 6d)

If UE can inform network of its capability related to how many beams it can evaluate within a unit time, network may be able to configure more proper value of TTT and proper number of beams for an event configuration. Since this is related to measurement capabilities, it would be better to send LS to RAN1 to ask for their feedback on the need of such capability.

Proposal 14: Adopt method1 in the MAC running CR for event evaluation description, and send LS to RAN1 to ask for feedback on the necessity of capability signaling on how many beams can be evaluated within a unit time.

3. Conclusion

Reduction of successive triggering of multiple L1 MR MAC CEs

Proposal 1: When L1 MR MAC CE is triggered by the beam (i.e., triggering beam) for a LTM report config ID, the L1 MR MAC CE also include other beam(s) for which TTT is running for the same report config ID as TTT-running beam(s).

Proposal 2: When L1 MR MAC CE is triggered by a beam (beam1) for a report configuration, if there is other beam (beam2) for which TTT is already running for the same report configuration, the UE refrains from triggering another L1 MAC CE for the beam2, if configured to do so by network.

Handling of multiple pending L1 MR MAC CEs

Proposal 3: For UE having more than one pending L1 MR MAC CEs triggered by different LTM report config IDs, if UE receives UL grant which is insufficient to transmit all the pending L1 MR MAC CEs, it is up to UE implementation which L1 MR MAC CEs to select and transmit first.

Proposal 4: Upon transmitting the selected L1 MR MAC CE triggered by a LTM report config ID, UE does not discard other pending L1 MR MAC CEs triggered by different LTM report config ID.

Proposal 5: To avoid transmitting multiple L1 MR MAC CEs triggered by the same report config ID, one of the followings are adopted:

- Alt1) If one L1 MR MAC CE is triggered by an LTM repot config ID, UE discards all the pending L1 MR MAC CEs previously triggered by the same LTM report config ID.

- Alt2) For UE having more than one pending L1 MR MAC CEs triggered by same LTM report config ID, UE selects latest L1 MR MAC CE(s) triggered by the LTM report config ID and transmits them. Upon transmitting the selected L1 MR MAC CE(s), UE discards other pending L1 MR MAC CEs previously triggered by the same LTM report config ID.

Format of L1 MR MAC CE:: L1-RSRP for leaving beam

Proposal 6: L1-RSRP is also included for leaving beam, i.e., the beam fulfills entering or leaving condition of the event for TTT duration.

Format of L1 MR MAC CE:: Explicit indication of entering/leaving beam

Observation: Bitmap indication occupies a fixed bit size that is equal to the maximum number of beams to be included in the MAC CE, which may result in some bits unused in cases reported number of beams is smaller than the maximum number.

Proposal 7: For each beam, introduce one bit to indicate whether the beam is a triggered beam for entering condition.

Proposal 8: For each beam, introduce one bit to indicate whether the beam is a triggered beam for leaving condition.

Format of L1 MR MAC CE:: Selection/Ordering of beams in L1 MR MAC CE

Proposal 9: When L1 MR MAC CE is constructed, the following principles are applied for selection/ordering of the beams to be included in the L1 MR MAC CE:

- Inter-beam category ordering: entering beam(s), if any > leaving beam(s), if any > other triggered beam(s) than the entering beam(s) if any > non-triggered beams, up to maximum number of beam entries allowed for the L1 MR MAC CE

- Intra-beam category ordering: beam with higher quality is included first

- For truncated L1 MR MAC CE, the number of beams included in the MAC CE is maximized for the given UL grant.

Proposal 10: For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as reference beam. For the reference beam, beam ID and non-differential L1-RSRP are included. For non-reference beam, beam ID and differential L1-RSRP value derived based on the non-differential L1-RSRP of the reference beam are included.

Truncated L1 MR MAC CE:: Triggering of truncated L1 MR MAC CE

Proposal 11: To discuss if network should be able to enforce some restrictions to UE in transmitting truncated L1 MR MAC CE related to entering/leaving beams or triggered beams.

Truncated L1 MR MAC CE:: Normal L1 MR MAC CE after transmitting truncated L1 MR MAC CE?

Proposal 12: Network configures UE via RRC with whether UE should still consider triggered L1 MAC CE as pending or cancel it after transmitting truncated L1 MAC CE.

Restriction of evaluated RSs via candidate cell indication

Proposal 13: Network can optionally configure a candidate cell via RRC within report configuration of event LTM. If the candidate cell is configured, UE only evaluates RSs associated with the candidate cell among RSs associated with the event.

Event evaluation and UE measurement capability

Proposal 14: Adopt method1 in the MAC running CR for event evaluation description, and send LS to RAN1 to ask for feedback on the necessity of capability signaling on how many beams can be evaluated within a unit time.