3GPP TSG-RAN WG2 #129-bis	R2-2502953

Wuhan, China, 7th – 11st April, 2025

Agenda Item:	8.1.5

Source:	Ericsson

Title:	On signalling feasibility of dataset and parameter sharing

Document for:	Discussion

# 1	Introduction

In R1-2410922 [1], RAN1 asked RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for the sharing NW-side model parameters and/or dataset to the UE or UE-side for the following options:

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

In the context of two-side models, the dataset/parameter sharing across UE-side and NW-side turns out to be an essential feature to enable training for a two-sided model. This paper focuses on the feedback that RAN2 could provide to RAN1 related to this matter, taking into account in particular the overhead and other feasibility aspects related to the various mechanisms that can be envisaged for the dataset/parameter sharing.

# 2	Discussion

The email discussion post RAN2#129 ([POST129][029][AI Phy] Model transfer) identified potential solutions for the delivery of the dataset/model parameters via over-the-air procedures (OTA) or non-over-the-air.

## 2.1	OTA solutions analysis

The dataset/model parameter size, as indicated by RAN1, exceeds the capacity of an RRC message. RAN1 in [1] indicates that the median dataset size may be 11.6MB in option 3a-1, and 225MB for option 4-1.

In the LS [1], RAN1 indicates to consider a dataset size between 11.6MB and 225MB for the dataset/parameter transfer.

The maximum allowable size for a dedicated RRC message conveyed in PDCP SDU is 9000 bytes and the DL RRC segmentation considers the support of up to five segments. Hence the amount of data that can be conveyed via RRC given the current specification is very far away from the amount of data requested by RAN1 for the delivery of the dataset/model parameters. Roughly 1200 – 25000 RRC segments would be required. Further, the UE will likely move before the gNB has been able to send all segments of a data set/model..

The maximum RRC message size cannot carry the large amount of data expected for dataset/model parameter delivery over-the-air.

Therefore, complex schemes would be needed to allow multiple gNBs to coordinate/cooperate in sending all segments. For example by partitioning the data into multiple subsets before transfer over the RRC protocol. This solution would imply that different gNBs (of the same vendor) would be in charge of delivering different portions of the dataset. However, this solution also introduces significant specification impacts and complexity:

- Over-the-Air Identification: A mechanism is needed to uniquely identify a dataset/model parameter and its corresponding partition IDs across different vendors. Without this, it would be unclear how the UE could correctly merge partitions to reconstruct the data. How these IDs are assigned at various network nodes of different vendors should be addressed in 3GPP, as well as in a real-world deployment. Related to this issue, we would need mechanism to identify UEs that have already received certain partitions in order to reduce the overhead.

- Increased Complexity: Both the NW and UE would face additional complexity in managing which parts of the dataset/parameters should be transmitted through specific gNBs and delivered to specific UEs in the network.

- Specification Changes: Updates would be required in the specifications to enable UEs to request specific partitions of datasets/model parameters, or to at least know which are the partition(s) that a certain network node can provide, so that the UE does not need to acquire again partition(s) already acquired. Further, how to signal any updates related to a specific partition is something that should be considered in the spec.

- Data Redundancy: In addition to the large Uu signaling overhead in the DL, as outlined in the previous observations, there would be the further unnecessary redundancy in sending datasets/model parameters back in the UL, from the UE to the UE-side training entity. This latter step is obviously needed because the training cannot happen locally in the UE, and the data must eventually be routed from the UE device to the training entity (e.g. an over-the-top (OTT) server). If we consider that in order to generate the dataset/model parameters, the network may also need to collect data from the UE in the UL (e.g. based on NW-side data collection solutions that we will specify in Rel.19), then the overall burden over the air interface in generating the two-sided model will become non-negligible. 
While alternative means such as Wi-Fi could be used for sending data from the UE to the training server, this cannot always be guaranteed.

- Service Assurance: Additional mechanisms are required to ensure that the information delivery will not impact normal services running at the network.

- Scalability: In order to bring benefits from the overhead point of view, the partitioning solution would require that many gNBs/cells share the burden of sending the overall dataset. However, this requires that many gNBs/cells share the same NW-side dataset/parameters (i.e. the same NW-side model). Assuming that one cell can deliver one or few dataset portions in RRC segments (each segment of 9000 bytes), to average out the overhead per cell, this would imply that the network would still need to rely on dozens of cells to deliver the 11.6MB dataset (which is the lightest dataset size according to RAN1), and hundreds of cells for the 225MB case (worst case). The issue would be even worse if SIB is used. Whenever such massive amount of sharing is not possible, e.g. the NW-model is shared by few cells/gNBs due to deployment reasons, then the benefits of this approach cannot be any longer exploited.

Partitioning the data into multiple subsets, and delivering the different partitions via different gNBs/cells poses the concerns of:

  - Over the air identification of the partitions

  - Increase complexity at UE/NW in handling the sharing of the transmission and reception of the various partitions in various gNBs/cells

  - Specification complexity

  - Data Redundancy

  - Scalability

The email discussion post RAN2#129 ([POST129][029][AI Phy] Model transfer) also addressed further OTA options that builds on data transfer over the control plane (CP) using RRC signalling from the gNB to the UE . However, the data originates from CN or OAM. For example,

- OTA solution 2: CN -> UE via gNB

- OTA solution 3: OAM -> UE via gNB

All the limitations and challenges associated with using RRC signalling from the gNB to the UE remain applicable to both OTA Solution 2 and OTA Solution 3. Furthermore, it is unclear how the CN would acquire the dataset or model parameters for transfer. Firstly, the CN has not been considered as an entity for training either one-sided or two-sided models in Rel-18/19. Secondly, assuming that the CN could collect dataset/model parameters from gNBs and then route them back via gNB(s) for over-the-air transfer to the UE would be excessive and an inefficient use of resources.

On the contrary, OAM has been considered a training entity for network-sided models. However, we do not see clear benefits in routing the dataset or parameters back to the RAN domain, especially given the challenges associated with using RRC signalling from the gNB to the UE.

OTA solution 3 depends on the feasibility of using RRC signalling from the gNB to the UE. Additionally, routing data back via gNB(s) for over-the-air transfer to the UE would be inefficient use of resources.

In the email discussion post RAN2#129 ([POST129][029][AI Phy] Model transfer), a UP solution was also discussed for over-the-air delivery. However, the UP solution does not solve the complexity concerns above. In particular, the gNB does not terminate the UP traffic, given the current NG-RAN architecture. Hence, it cannot be considered as feasible.

For two-sided model training, RAN2 concludes that standardizing OTA procedures over the UP/CP is not supported.

## 2.2	non-OTA analysis

For the reasons mentioned above, we believe that it is worth evaluating the feasibility of alternative approaches not based on over-the-air signalling. The email discussion post RAN2#129 ([POST129][029][AI Phy] Model transfer) summarized the candidate options into the following table. For further clarifications, we have introduced the term "gNB/OAM-centric data collection" to refer to the network entities that in which the data (dataset/parameter) can originate.

Table 1: non-OTA approaches for dataset/model paramter transfer according to email discussion post RAN2#129 ([POST129][029][AI Phy] Model transfer)

| Option | gNB/OAM centric data collection | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- | --- |
| 4) OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | Option 4a: gNB centric data collection, where the dataset/model parameter originates from the gNB and is collected at the OAM before it is further transferred to the UE-side training entity Option 4b: OAM centric data collection, where the dataset/model parameter originates from and is collected at the OAM before it is further transferred to the UE-side training entity | SA5 | Up to SA5 (any intermediate node, if any, between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| 5) CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | Option 5a: gNB centric data collection, where the dataset/model parameter originates from the gNB and is collected at the CN before it is further transferred to the UE-side training entity Option 5b: OAM centric data collection, where the dataset/model parameter originates from the OAM and is collected at the CN before it is further transferred to the UE-side training entity | SA2 | Up to SA2 (any intermediate node, if any, between CN and UE-side OTT server is up to SA2) |
| 6) gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | The dataset/model parameter originates from and is collected at the gNB and then transferred to the UE-training entity via OAM or CN. | RAN3, SA2, SA5 | Up to RAN3, SA2, SA5 (any intermediate node, if any, between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

Based on the explanation above, the redundancy between option 4/5 and option 6 is clear. Option 6 is already embedded in both option 4 and 5 when assuming gNB centric data collection. Therefore, to avoid any confusion, we propose to remove option 6 from the list of candidate options and replace it with further explanation of the how option 4 and 5 work in case of gNB or OAM centric data collection.

Option 6 should be removed from the list of candidate options for non-OTA for NW dataset/model parameter transfer as it is already embedded in option 4 and 5. Further clarifications can be added, if needed, to option 4 and 5 to emphasize the entity (gNB/OAM) from which the dataset/ model parameter originates.

In our view, the above alternative approaches, do not have any impact in the air interface and they are significantly more straightforward than over-the-air data delivery. To this end, we propose RAN2 informing SA groups about the above alternative approaches and assess their feasibility for transferring dataset/model parameters from the NW to the UE.

RAN2 respectfully asks SA2/SA3/SA5’s feedback on the feasibility of non-OTA options for the transfer of dataset/model parameters from the gNB to the UE-side for the purpose of two-side model training.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In the LS [1], RAN1 indicates to consider a dataset size between 11.6MB and 225MB for the dataset/parameter transfer.

Observation 2	The maximum RRC message size cannot carry the large amount of data expected for dataset/model parameter delivery over-the-air.

Observation 3	Partitioning the data into multiple subsets, and delivering the different partitions via different gNBs/cells poses the concerns of:

a.	Over the air identification of the partitions

b.	Increase complexity at UE/NW in handling the sharing of the transmission and reception of the various partitions in various gNBs/cells

c.	Specification complexity

d.	Data Redundancy

e.	Scalability

Observation 4	OTA solution 3 depends on the feasibility of using RRC signalling from the gNB to the UE. Additionally, routing data back via gNB(s) for over-the-air transfer to the UE would be inefficient use of resources.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For two-sided model training, RAN2 concludes that standardizing OTA procedures over the UP/CP is not supported.

Proposal 2	Option 6 should be removed from the list of candidate options for non-OTA for NW dataset/model parameter transfer as it is already embedded in option 4 and 5. Further clarifications can be added, if needed, to option 4 and 5 to emphasize the entity (gNB/OAM) from which the dataset/ model parameter originates.

Proposal 3	RAN2 respectfully asks SA2/SA3/SA5’s feedback on the feasibility of non-OTA options for the transfer of dataset/model parameters from the gNB to the UE-side for the purpose of two-side model training.

# References

R1-2410922, LS on signalling feasibility of dataset and parameter sharing,” RAN1#119.