3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502959

Wuhan, PRC, 07 – 11 April 2025

Agenda item:	8.1.5

Source:	Samsung

Title:	Outstanding issues on signalling feasibility of dataset and parameter sharing for CSI compression

Document for:	Discussion & Decision

# Introduction

RAN2 received from RAN1 an LS (in R2-2500015) on signalling feasibility of dataset and parameter sharing, which was discussed at RAN2#129. In the LS, RAN1 had outlined the then-most recent agreements on two-sided AIML CSI compression discussions, with specific focus on the dataset and/or parameter sharing solutions under consideration in RAN1. RAN1 had further asked RAN2 to evaluate the feasibility of signaling to support RAN1-identified solutions for the two-sided model case. Following the conclusion of RAN2#129, a RAN2 email discussion was held ([POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)) to discuss the key OTA and non-OTA options; outcome of this email discussion is captured by its rapporteurs in R2-2501939.

In this submission, we specifically discuss the key outstanding issues raised in the LS and in R2-2501939 which we feel would benefit from further discussion and/or a different/complimentary approach to the one proposed in R2-2501939. We provide a way forward for RAN2 comprising a number of proposals.

# Discussion – the OTA approach

In R2-2501939, 3 OTA solutions are identified:

-	OTA solution 1a: gNB -> UE via CP

-	OTA solution 2: CN -> UE via gNB

-	OTA solution 3: OAM -> UE via gNB

Rapporteurs additionally captured the perceived challenges as identified by individual companies related to each of the solutions, and the mitigating conditions for each of the solutions.

With regards to solution 1a, we note that the biggest perceived challenge was the feasibility of the RRC buffer size for the large quantities of data under consideration. Even smaller size of data to be transferred exceed the current maximum number of RRC segments. However, there appears to be a lack of common understanding in RAN2 on this aspect i.e. it is at present unclear what the data size threshold is where required RRC enhancements are no longer viable. There is even a potential contradiction with our AIML TR (the model transfer discussion), which acknowledges that the model size could be significant, and possibly comparable with what we are now discussing for data set/model parameter transfer, without claiming this to be a show-stopper for solution 1a (for the more general case of model transfer covered in the TR). We would therefore like to propose the following:

Proposal 1. RAN2 to consider updating the TR to clarify that viability of RRC enhancements needed for model transfer over CP is yet to be determined.

We note that the rapporteurs propose to preclude option 1b (i.e. gNB -> UE via UP) from further discussion. While we agree that there is uncertainty around the feasibility of option 1b, we note that it is not within RAN2’s remit to rule on this; in fact, option 1b (we refer here to the model transfer option 1b on the DL, rather than UE-side data collection option 1b on the UL) could be implemented with minimal impact on RAN2 specs, assuming a UP tunnel between UE and gNB can be developed. In line with this reasoning, we propose:

Proposal 2. RAN2 to consult SA2 on the viability of the option 1b (i.e. gNB -> UE via UP) for dataset transfer.

The time to complete the AIML PHY study is limited in RAN2, and even more so in RAN1. Assuming the cross-WG co-ordination still outstanding can be successfully done, it is still unlikely that down-selecting (when making recommendations for further work) can be done.

Proposal 3a. Given the limited time left for the Study, RAN2 should not preclude any of the data set transfer options.

Proposal 3b. Instead of recommending a specific option, RAN2 is to agree a TP to the TR capturing an analysis of the OTA options, including 1b (i.e. gNB -> UE via UP).

Returning to OTA solution 1a, we do agree that segmentation/concatenation of messages within a single data set needs to be discussed, to aid with the maximum number of RRC segments issue. More specifically, we need further clarification on how the segmentation/concatenation of messages within a single data set is to be done in UE server and whether assistance from the 3GPP network is needed for this. We therefore believe the following proposal is key:

Proposal 4. In the reply-LS to RAN1, RAN2 to ask RAN1 for their position on whether data segmentation/concatenation in support of OTA solution 1a (or more generally) should be supported, and what the required maximum data set size transmitted with one RRC message (transaction) would be.

And finally, on the topic of OTA solutions 2 & 3, the key open issue appears to be feasibility of having the training entity in the NW, which again we feel should be feasible so long as SA2 conclude the feasibility of options 2 & 3 for data collection transfer for the UE-side model case:

Proposal 5. RAN2 to assume it is feasible to have the training entity in the NW, as needed for OTA solutions 2 & 3, so long as SA2 conclude the feasibility of options 2 & 3 for data collection transfer for the UE-side model case.

# Discussion – the non-OTA approach

The RAN1 LS mentioned non-OTA and presumably this is why it formed part of the RAN2 discussion. However, we still maintain that RAN2 cannot make any definite suggestions on feasibility or otherwise of non-OTA solutions. The current statement from rapporteurs in P3 in R2-2501939 that “RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view” should therefore be changed to e.g. “RAN2 identified the following candidates and did not identify any technical showstopper from RAN2 point of view, as non-OTA approach is not expected to have any impact to RAN2 specification”.

Proposal 6. In its reply to RAN1, RAN2 to clarify that the non-OTA approach is not expected to have any impact to RAN2 specification, by making e.g. the following change to P3 of R2-2501939:

Proposal 3 [phase 2 10/14]: For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view as non-OTA approach is not expected to have any impact to RAN2 specification. Feasibility analysis of non-OTA solutions is required to be evaluated by RAN3, SA2, and SA5. It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

Proposal 7. Any statements or recommendations from RAN2 to RAN1 on which WGs are suitable to discuss non-OTA, should include SA3, by e.g. adding SA3 as concerned group in P3 and P8 of R2-2501939:

Proposal 3 [phase 2 10/14]: For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view. Feasibility analysis of non-OTA solutions is required to be evaluated by RAN3, SA2, SA3 and SA5. It does not preclude RAN3/SA2/SA5/SA3 to identify other candidate solutions beyond options listed below.

Proposal 8: RAN2 reply RAN1 LS based on based on outcome of Proposal 2/3/5/6 and to RAN3, SA2, SA3 and SA5 for further action on feasibility analysis of non-OTA solution and OTA solution 2 and 3.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. RAN2 to consider updating the TR to clarify that viability of RRC enhancements needed for model transfer over CP is yet to be determined.

Proposal 2. RAN2 to consult SA2 on the viability of the option 1b (i.e. gNB -> UE via UP) for dataset transfer.

Proposal 3a. Given the limited time left for the Study, RAN2 should not preclude any of the data set transfer options.

Proposal 3b. Instead of recommending a specific option, RAN2 is to agree a TP to the TR capturing an analysis of the OTA options, including 1b (i.e. gNB -> UE via UP).

Proposal 4. In the reply-LS to RAN1, RAN2 to ask RAN1 for their position on whether data segmentation/concatenation in support of OTA solution 1a (or more generally) should be supported, and what the required maximum data set size transmitted with one RRC message (transaction) would be.

Proposal 5. RAN2 to assume it is feasible to have the training entity in the NW, as needed for OTA solutions 2 & 3, so long as SA2 conclude the feasibility of options 2 & 3 for data collection transfer for the UE-side model case.

Proposal 6. In its reply to RAN1, RAN2 to clarify that the non-OTA approach is not expected to have any impact to RAN2 specification, by making e.g. the following change to P3 of R2-2501939:

Proposal 3 [phase 2 10/14]: For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view as non-OTA approach is not expected to have any impact to RAN2 specification. Feasibility analysis of non-OTA solutions is required to be evaluated by RAN3, SA2, and SA5. It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

Proposal 7. Any statements or recommendations from RAN2 to RAN1 on which WGs are suitable to discuss non-OTA, should include SA3, by e.g. adding SA3 as concerned group in P3 and P8 of R2-2501939:

Proposal 3 [phase 2 10/14]: For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view. Feasibility analysis of non-OTA solutions is required to be evaluated by RAN3, SA2, SA3 and SA5. It does not preclude RAN3/SA2/SA5/SA3 to identify other candidate solutions beyond options listed below.

Proposal 8: RAN2 reply RAN1 LS based on based on outcome of Proposal 2/3/5/6 and to RAN3, SA2, SA3 and SA5 for further action on feasibility analysis of non-OTA solution and OTA solution 2 and 3.