3GPP TSG RAN WG2#129bis  	R2-2502960

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.2.4

Source:	III

Title:	Discussion on A-IoT data segmentation and transmission

Document for:	Discussion

Introduction

In the previous RAN2#129 meeting, some related agreements were achieved in the following:

Agreements < How to assign AS ID > - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2 < Segmentation Indication> 1. To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient. 2. Segment retransmission is supported. 3. For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3 4. R2D segmentation is not supported for R19 A-IoT. < Procedures to consider> - From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”. < MAC PDU format design > 1. Aim to design simple MAC PDU format design 2. Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages 3. At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). FFS whether for D2R we need message type field, any length and need for padding 4. Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) Other message types are FFS. The message types may evolve based on functionality agreements.

The current RAN2 progress, this contribution discusses AS ID for CFRA and segmentation.

Discussion

AS ID for CFRA

The discussion about AS ID for CFRA had reached agreements in RAN2#129.

Agreements: For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: • Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. • Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from op-tion 2 • Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

In CFRA, only one device is paged at a time. The AS ID can be assigned by reader directly in message MSG2 whether RN16 is transmitted in MSG1 or not. Option 3 will introduce extra new MSG2 steps. In order to be consistent with CBRA, option 2 / option 4 is sufficient.

Observation 1: For CFRA case, option 2 / option 4 is sufficient.

Segmentation

In the MSG3 transmission of A-IoT random access procedure, to support data transmission under limited resource conditions, the A-IoT MAC layer has studied the segmentation mechanism to segment data blocks. Each segmentation contains corresponding control information to ensure that the reader can correctly assemble all segments to complete MSG3.

Two segmentation-based transmission processes are proposed:

- Unified response after full segment transmission: The device continuously transmits all segments data without waiting the response for each segment. Until all segments were transmitted, the device wait for the response from the reader. The response will indicate which segments should be re-transmitted.

- Send instant responses segment by segment: The device transmits each segment data and waiting the response for this segment. . The response will indicate this segments how to re-transmit.

Proposal 1: Two segmentation-based transmission processes are proposed:

- Unified response after full segment transmission,

- Send instant responses segment by segment.

- Conclusion

We have the following proposals:

Observation 1: For CFRA case, option 2 / option 4 is sufficient.

Proposal 1: Two segmentation-based transmission processes are proposed:

- Unified response after full segment transmission,

- Send instant responses segment by segment.