3GPP TSG-RAN2#129bis	R2-2502967

Wuhan, China, Apr 7th-11th, 2025

Agenda item:		8.11.2

Source:		Lenovo

Title:		 Random Access in SBFD

Document for:		Discussion and Decision

- Introduction

The Work Item for Sub-band non-overlapping full duplex (SBFD) has been started in RAN2. The following agreements were made at the previous meetings [1]:

Agreements at RAN2 #129: - When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. - When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation and does not apply the SBFD RACH configuration. - For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. - If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. - FFS whether RO type selection is performed before or after the RA type selection. - FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

This contribution looks at some of the aspects related to Random Access for SBFD including configuration of RO type, integration of RO selection into the RACH procedure and the fallback behaviour.

- Discussion

  - Configuration of RO Type

RAN2 has already agreed in the previous meeting that the network can indicate a SBFD aware UE with a RO type for initial RA transmission in the case of CBRA. This means that a SBFD aware UE may always have to initiate RA based on the network indication regardless of its measured metrics. For example, a UE at cell edge with high pathloss may be forced to initiate RACH on a SBFD RO if such was indicated by the network. Some companies mentioned in the last meeting that the benefit of such indication would be to enable load balancing. However, in some cases, this may lead to (1) a reduced probability of RACH success, due to high pathloss and inter-gNB CLI, and, more importantly, (2) increase of inter-UE CLI at a nearby “victim” UE receiving a downlink signal/channel, due to the preamble uplink transmission. In our view, some criteria need to be additionally defined so that the load balancing can be efficiently achieved without impacting the UE’s success probability for RACH or causing high inter-UE CLI at a nearby downlink UE. One approach is to allow UE to select an additional RO type if the measured SSB RSRP is above an SSB RSRP threshold, which would only allow UEs with good/high link budget (or low pathloss) to perform RA on an additional RO.

If the network indicates additional RO for initial RA in the case of CBRA, SSB RSRP threshold is configured as an additional criterion.

Moreover, to limit impact of inter-UE CLI, one approach would be to indicate/configure a different SSB RSRP threshold for each SSB or a group of SSBs. For example, a high SSB RSRP threshold can be indicated/configured for an SSB (or a group of SSBs) providing coverage for a hotspot area (e.g., a shopping mall or an indoor office). Such an approach will allow only a UE with a very good/high link budget (or very low pathloss) to select additional RO type. In such case, the UE would use a low PRACH power that incur/cause low inter-UE CLI. On the other hand, a low SSB RSRP threshold can be indicated/configured for an SSB (or a group of SSBs) providing coverage for a non-hotspot area, which would allow more UEs to select additional RO type, eventually achieving a desired load balancing between additional RO type and legacy RO type. There was also the following agreement in the last meeting: If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold, and it was FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. In our view, an additional NW indication seems unnecessary. Based on the above reasoning, we think it is sufficient for the NW to configure the SSB RSRP threshold for using additional RO, i.e., a SBFD aware UE will select additional RO if its measured SSB RSRP is above the configured threshold.

RAN2 to agree that a different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs to limit inter-UE CLI and achieve a better load balancing between additional RO type and legacy RO type.

If no NW indication is provided for RO type, UE selects additional RO if the measured SSB RSRP is above the configured SSB RSRP threshold.

Additionally, another aspect that was still FFS was how this indication is signalled. In our view, the most straight forward solution is to include this indication in the system information signalling (e.g., SIB1) in order to prevent the additional overhead and power consumption required for dedicated signalling in Idle Mode.

For the case of initial RA transmission in CBRA, the network indication of RO type is provided in SIB.

  - RO Selection in the RACH procedure

During the previous RAN2 meetings, there were discussions regarding where the RO selection needs to be done in the RACH procedure. In our view, it is a cleaner solution to select the RO type after the RA type has been selected since the RO type selection should be based on the RA resources available to the UE. Additionally, since the same is followed for CFRA type of RACH, we think this can also be extended for CBRA to maintain alignment across both types of RACH. Furthermore, if SBFD is not supported for 2-step RACH and 2-step RA type is selected, the RO type selection step can be skipped, and the RACH can be performed in 2 steps. Whereas, if the RO type selection is done first and SBFD RO is selected, 2-step RACH cannot be performed which means that the UE will have to perform 4-steps before RACH is completed which does not bring the same benefit as in the former case. Hence, we think that the RO type selection should be performed after the RA type selection.

RO type selection is performed after the RA type selection.

  - Fallback Mechanism

There was a lot of discussion in the previous RAN2 meeting with respect to fallback of PRACH resource selection. Some companies brought up that a SBFD aware UE may benefit if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported. Additionally, most companies mentioned that if such fallback is supported then the fallback only happens once to avoid ping-pong. In our view, the exact benefit of supporting such fallback from non-SBFD symbol to SBFD symbol is not clear. If legacy RO is selected for initial RACH transmission and it fails, the chances of RACH transmission being successful in the SBFD RO is less likely since SBFD ROs are faced with higher CLI than legacy ROs. Furthermore, although companies mention that the fallback to SBFD RO is only restricted to one time, it is unclear how the UE behaves if the RACH attempt on the fallback SBFD RO also fails. That is, either the UE will not fallback again to legacy RO and continue to perform RACH on SBFD RO leading to additional delays and power consumption to combat for the higher CLI, or the UE falls back once more to legacy RO which will also cause additional latency due to such switching: Hence, in our view, the UE can only fall back in one direction, i.e., from SBFD RO to non-SBFD RO type.

- Proposal 6 Switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is not supported.

- Conclusion

This document discusses some aspects of Random Access for SBFD. Following are the proposals and observations made in this document:

- Proposal 1: If the network indicates additional RO for initial RA in the case of CBRA, SSB RSRP threshold is configured as an additional criterion.

- Proposal 2: RAN2 to agree that a different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs to limit inter-UE CLI and achieve a better load balancing between additional RO type and legacy RO type.

- Proposal 3: If no NW indication is provided for RO type, UE selects additional RO if the measured SSB RSRP is above the configured SSB RSRP threshold.

- Proposal 4: For the case of initial RA transmission in CBRA, the network indication of RO type is provided in SIB.

- Proposal 5: RO type selection is performed after the RA type selection.

- Proposal 6: Switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is not supported.

- References

[1] R2-2501332: MIMO, LP-WUS, SBFD Chair’s Notes