3GPP TSG-RAN2 Meeting #129bis	R2-2502968

Wuhan, China, Apr. 7th– 11th, 2025

Source: 			ZTE Corporation

Title: 	Discussion on measurement event prediction

Agenda item:	8.3.3

Document for: 	Discussion and Decision

- Introduction

In the last meeting, we have achieved the agreements about the simulation methodology and new metrics, as follows [1]:

- For evaluation purpose, it is assumed PW length = TTT length =320ms - For evaluation purpose, Option 2 is agreed i.e. each counter can be updated only one time for related predicted and/or ground truth measurement event. - To introduce “Total number of HOF per UE per second” as SLS metric

In this contribution, we’d like to provide the simulation results for measurement event prediction and system level simulation (SLS), and give the corresponding analysis.

- Discussion

## 2.1 Evaluation results for measurement event prediction

In this contribution, we study the indirect measurement event prediction simulation for intra-freq temporal domain case A in FR2 scenario, which is high priority based on the previous agreements. Specifically, future measurement results of serving cell and neighbor cells are predicted by temporal domain prediction case A at first. Afterwards, predicted and actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model, as shown in Figure 1:

Figure 1: Indirect measurement event prediction model

The table 1 below show some important simulation assumptions in our simulation. The details please see our simulation results table in the FTP.

Table 1: Simulation assumptions for measurement event prediction and SLS

| Parameters | Value |
| --- | --- |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 90 |
| OW length (ms) | 800 |
| PW length (ms) | 320 |
| Max ETD (ms) | 80 |

In our simulation, the measurement event prediction starts when entering condition of measurement event is fulfilled based on the actual measurement results. And then the prediction is performed every measurement periodicity until the measurement event is predicted to trigger or the leaving condition is fulfilled based on the actual measurement. In this way, for a real actual measurement event, there are multiple time instances that can predict the measurement event. The simulation results are shown in the Table 2.

Table 2: Simulation results for measurement event prediction

| ## Precision | ## Recall | ## F1_score | ## Time advance |
| --- | --- | --- | --- |
| ## 0.8280 | ## 0.9942 | ## 0.9026 | ## 297.55ms |

Based on the simulation results, we find that the recall is near to 1, which means that almost all actual measurement event can be predicted in advance. This is because for one real measurement event, there are more than one time instance that the measurement event can be predicted. And with the increase of the time, the time distance between the time instance when the inference is performed and the time instance that actual event is triggered reduces, the prediction accuracy improves.

Observation 1: For measurement event prediction with temporal domain case A, the metric recall is very high, which is near to 1.

However, the precision is about 0.8280, which means that about 18% predicted measurement event is not true. In our understanding, this is because the instantaneous fluctuations of the channel quality and/or prediction error of the AI model can cause false event prediction. This may further result in too-early-handover, and may impact system level performance (e.g. HOF). In our simulation, the F1_score is 0.9026, which shows good performance.

Observation 2: For measurement event prediction with temporal domain case A, the metric precision is 0.8280, which is smaller than the recall.

Besides the precision, recall and F1 score, a new metric ‘time advance’ is proposed in the email discussion, which is defined as the time distance between 1st time instance of positive true prediction and corresponding ground truth label. This metric reflects that how far in advance the UE can predict a measurement event. In our simulation, the time advance is 297.55ms, which refers that majority of the measurement event can be predicted when the entering consider is fulfilled based on the actual measurement results.

Observation 3: For measurement event prediction with temporal domain case A, the time advance is 297.55ms, which nears to TimeToTrigger.

Proposal 1: Regarding measurement event prediction, capture observation 1 ~3 into the TR.

## 2.2 Evaluation results for system level simulation

The simulation assumptions for SLS reuses that for measurement event prediction. Handover option 2 (i.e. the handover command is relying on the legacy measurement event) is adopted in our simulation, as follows:

Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time.

Besides, in order to study the gain that AI provides, we also study the handover performance in the baseline scenario. Our simulation results are as follows:

Table 2: the handover failure rate in the simulation with AI and in baseline scenario

| Simulation case | HO failure rate (%) | Total number of HO attempts per UE per second | Total number of HOF per UE per second |
| --- | --- | --- | --- |
| Baseline | 7.56% | 0.14 | 0.0106 |
| AI | 6.86% | 0.14 | 0.0096 |

It is observed that the handover failure rate for SLS with non-AI is 7.56%, while the handover failure rate for SLS with AI is 6.86%, the handover failure rate reduces 0.7%. For total number of HOF per UE per second, with AI, the total number of HOF per UE per second reduces 0.001. So, the handover performance for SLS with AI outperforms than that with non-AI.

This is because with handover model option 2, the handover preparation time are saved if the prediction is accurate, which may avoid too-late-handover. If the prediction is inaccurate, the handover command is based on the legacy measurement event, in this way, the handover performance is same as the handover performance with non-AI. However, on the other hand, the benefit that the AI provides is limited, this is because only handover preparation time (i.e. 40ms) can be saved.

Observation 4: With handover model option 2, the handover performance for SLS with AI outperforms than that with non-AI in the terms of handover failure rate and total number of HOF per UE per second.

Proposal 2: Regarding system level simulation, capture Observation 4 into the TR.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

Measurement event prediction:

Observation 1: For measurement event prediction with temporal domain case A, the metric recall is very high, which is near to 1.

Observation 2: For measurement event prediction with temporal domain case A, the metric precision is 0.8280, which is smaller than the recall.

Observation 3: For measurement event prediction with temporal domain case A, the time advance is 297.55ms, which nears to TimeToTrigger.

Proposal 1: Regarding measurement event prediction, capture observation 1 ~3 into the TR.

System level simulation:

Observation 4: With handover model option 2, the handover performance for SLS with AI outperforms than that with non-AI in the terms of handover failure rate and total number of HOF per UE per second.

Proposal 2: Regarding system level simulation, capture Observation 4 into the TR.

- Reference

- Report of RAN2 129 meeting.