3GPP TSG-RAN2 Meeting #129bis	R2-2502969

Wuhan, China, Apr. 7th – 11th, 2025

Source: 			ZTE Corporation

Title: 	Discussion on model generalization of RRM measurement prediction

Agenda item:	8.3.2.2

Document for: 	Discussion and Decision

- Introduction

In the last meeting, we have agreed to study the generalization over different cell configurations, as follows [1]:

- Two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study. - FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2). - Agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2).

In this document, we’d like to provide our simulation results, and share the corresponding views.

- Discussion

In this section, we’d like to show our evaluation results for model generalization over cell configurations with temporal domain case B. Some of the simulation assumptions are shown in the table 1 and table 2:

Table 1: Simulation assumptions for model generalization simulation of cell setting configurations

| Parameters | Value |
| --- | --- |
| MRRT | 50% |
| PW length (ms) | 200 |
| OW length (ms) | 600 |
| RRM sub use case | Option 2 |
| Filtering option | Option 3 |
| UE speed | 30km/h |
| Skipping pattern |  |

Table 2: The generalization parameters for FR1

| Parameters | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | Uma |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

The simulation results are shown in the table 3. Case 1 and case 2 in the table 3 means a combination that AI model is tested using the dataset with the same cell configuration.

Table 3: The evaluation results for model generalization over cell configurations in FR1 scenario

| Simulation Case | Training/Inference | Average L3 cell level RSRP difference (dB) |
| --- | --- | --- |
| Case 1 (test @ configuration 1) | Baseline Training @ configuration 1+Test @ configuration 1 | 0.577 |
|  | GC #1 Training @ configuration 2+Test @ configuration 1 | 0.616 |
|  | GC #2 Training @ configuration 1+ configuration 2+Test @ configuration 1 | 0.585 |
| Case 2 (test @ configuration 2) | Baseline Training @ configuration 2+Test @ configuration 2 | 0.521 |
|  | GC #1 Training @ configuration 1+Test @ configuration 2 | 0.548 |
|  | GC #2 Training @ configuration 1+ configuration 2+Test @ configuration 2 | 0.533 |

Based on the simulation results, we observe that the AI model that is trained and tested using the dataset with the same cell configurations (i.e. baseline scenario) can provide the best prediction accuracy. And, for the same test configurations, the AI model is trained using a mixed dataset with configuration A and configuration B (i.e. GC#2) has better prediction accuracy compared to the AI model that is trained using configuration A but tested using configuration B (i.e. GC#1), and has similar prediction accuracy compared to the baseline scenario. For example, if the AI model is tested using dataset with configuration 1, the average RSRP difference of GC#2 is only 0.008dB higher than that of baseline, which is acceptable. In our understanding, this is because from the mixed dataset, the AI model can learn the logic of both dataset with configuration 1 and configuration 2, and based on the AI model input, the AI model can identity which logic shall be used, so their prediction accuracy is similar.

While, the prediction accuracy of GC#1 is worse than that of GC#2 and baseline. For example, if the AI model is tested using dataset with configuration 1, the average RSRP difference of GC#1 is 0.616dB, which is 0.039dB higher than that of baseline and 0.031dB higher than that of GC#2; if the AI model is tested using the dataset with configuration 2, the average RSRP difference of GC#1 is 0.548dB, which is 0.022 dB higher than that of baseline and 0.015 dB higher than that of GC#2.

Observation 1: For model generalization over cell configurations in FR1, the AI model trained using mixed dataset (GC#2) can provide similar prediction accuracy compared to baseline case.

Observation 2: For model generalization over cell configurations in FR1, the prediction accuracy with the AI model trained with a different configuration (GC#1) is worse than the baseline case and GC#2 case.

Proposal 1: Regarding model generalization of cell configurations, capture observation 1~2 into the TR.

- Conclusion and proposals

In this contribution, we have the following observations and proposals:

Observation 1: For model generalization over cell configurations in FR1, the AI model trained using mixed dataset (GC#2) can provide similar prediction accuracy compared to baseline case.

Observation 2: For model generalization over cell configurations in FR1, the prediction accuracy with the AI model trained with a different configuration (GC#1) is worse than the baseline case and GC#2 case.

Proposal 1: Regarding model generalization of cell configurations, capture observation 1~2 into the TR.

- Reference

- Report of RAN2 129 meeting.