3GPP TSG-RAN WG2 #129bis	R2-2502979

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	7.2

Source:	Ericsson

Title:	Summary of RRC proposals for feMob

Document for:	Discussion

# 1	Introduction

In this contribution is provide a summary of the RRC proposals for FeMob with some possible conclusion for each of them. Please, note that MAC corrections are left out from this document.

# 2	Discussion

Hereafter the RRC related proposals submitted to AI 7.2 are listed and a possible outcome for what is proposed is suggested:

| Tdoc # | Company | Proposal(s) | Suggested conclusion |
| --- | --- | --- | --- |
| R2-2501904 | CATT | P1: Add condition to make sure only the SRB corresponding to the cell group that triggers the LTM execution applies the default SRB configuration upon LTM cell switch execution. | P1: To be discussed. It is true that if the LTM is at the SCG the UE will apply the default SRB1 and SRB2 configuration, but this may be correct in case of split SRB. Also, applying the default is not harmful as the network does not really need to provide new SRB1 and SRB2 configurations in this case. So, what we have now it works, even if this implies that fpr SRB1 and SRB2 the default configuration is always re-applied…but maybe this is a safe approach. |
| R2-2501932 | MediaTek | P1: Change capability description and signalling according to RAN4 LS in R2-2501737 | P1: Principle are okay, but Vivo CR is better. The proposals are fine and are in line which what the RAN4 LS is asking. However, for the change in RRC we need also to change the UE constant which is used in the inter-node RRC signalling to coordinate the L1 measurements. In this case Vivo CR is better. Suggestion to agree on CRs in R2-2502170 and R2-2502171. |
| R2-2501976 | Google | P1: RAN2 is requested to discuss the two opitons and decide option 1 or option 2 about a possible a procedural inconsistency about managing UE configuration between section 5.3.5.8.3 and section 5.3.5.18.6. | P1. Not Pursued. This is exactly the same case when there is a reconfiguration with sync with full configuration. With full configuration UE delete all the dedicated and common configuration, but then if the reconfiguration with sync fails, the UE needs to revert back to the previous configuration. In this case, there is nothing really new and we should not pursue this change. |
| R2-2502169 | vivo | P1: Remove the indication from MAC to RRC layer for the completion of RACH-based LTM. | P1. Not pursued. It seems that there is no issue in the procedural text point by vivo in RRC. The indentation 1> are all “or” and this means that the UE will only execute one of those. Also, even if the indication from lower layer is repeated in one indentation 2> we still think there is no issue and this does not mean the indication needs to be repeated twice. |
| R2-2502170 | Vivo, Ericsson, Xiaomi | P1: Align 38.306 according to the RAN4 LS received in R2-2501737 | P1: To be agreed. The CR looks fine and can be agreed (what RAN4 is asking is quite straightforward). |
| R2-2502171 | Vivo, Ericsson, Xiaomi | P1: Align 38.331 according to the RAN4 LS received in R2-2501737 | P1: To be agreed. The CR looks fine and can be agreed (what RAN4 is asking is quite straightforward). |
| R2-2502533 | Ericsson | P1: Add coordination between MN and SN about the TCI states, activation of TCI state, fast processing, and early TA acquisition across MN and SN | P1: To be discussed. This is something that was discussed in the last meeting and indeed there is a need for a coordination about the TCI states, activation of TCI state, fast processing, and early TA acquisition across MN and SN. This is because some of the capabilities are marked as “across all serving cell” including MN and SN serving cells. |
| R2-2502553 | Nokia | P1: Update the first sentence to "Indicates whether the UE supports early TA acquisition based on PDCCH ordered CFRA procedure before the cell switch command MAC CE”. Adopt the TP in Annex A for TS 38.306. P2: RAN2 to clarify the understanding on the timeframe associated with UE “maintaining” the UE-measured TA and to reflect this in 38.306. The following options can be considered as a starting point (other alternatives are not precluded): - UE measures and maintains the TA value starting from the moment when it is configured to perform UE-based TA measurement. - UE measures and maintains the TA value starting from the moment when it receives the CSC. P3: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells or/and TCI states for which early DL synchronization procedures can be performed. P4: MN informs SN about the number of cells which early UL synchronization procedures can be performed. P5: Change capability description and signalling according to RAN4 LS in R2-2501737 | P1. Not pursued. We understand the limitation comes from WGs which are not RAN2 and for this reason we cannot simply modify the meaning of the capability as we want (description which we believe is actually correct). P2. Not pursued. The UE-based TA is a UE-only feature and the intention is that the UE calculate itself the TA (if indicated by the network). When the UE calculate the TA and how to maintain it is up to the UE implementation and RAN4 did not specify any requirement for this feature. P3-P4: To be discussed. We in principle agree with the proposals and we believe this is indeed needed. However, the approach proposed in the TP is not very good as it would be good to group such new field same as we did for the L1 measurements (also in light of future extensions). For this, we can consider the CR in R2-2502533 as baseline. P5: Principle are okay, but Vivo CR is better. The proposals are fine and are in line which what the RAN4 LS is asking. However, for the change in RRC we need also to change the UE constant which is used in the inter-node RRC signalling to coordinate the L1 measurements. In this case Vivo CR is better. Suggestion to agree on CRs in R2-2502170 and R2-2502171. |
| R2-2502600 | Ofinno | P1: Proposal: To avoid the mismatch of UE-based TA measurements between the UE and the network, RAN2 agreed one of the following alternatives: - A UE determines to configure the lower layer for UE based TA measurements for each LTM candidate cell if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList - The ltm-CandidateToAddModList is mandatory present if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID | P1: Not pursued. The field description of ltm-UE-MeasuredTA-ID already handle the case described in the contribution: “the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID” |
| R2-2502971 | Huawei | P1: Capture that UE shall release PUCCH-CSI-Resources configured in LTM-CSI-ReportConfig when an indication from lower layers is received; | P1: To be discussed. We are wondering is this is really correct. For the CSI-ReportConfig of the serving cell this makes sense since the TAT is maintained towards the serving cell. However, LTM-CSI-ReportConfig provide resource for cells which are not “serving cell”, since these are candidate cells. So in principle the UE may still continue to use those PUCCH resources. |
| R2-2502943 | Huawei | Summary of MAC proposals |  |
| R2-2502966 | Samsung | P1: Clarify when the UE includes the reselectionMeasAvailable in RRCSetupComplete P2: Add that the validityStatus should also be included in case the UE has valid NR measurements in RRCResumeComplete P3: To be agreed Update procedural text to mention that certain UE actions are done according to UE capabilities. P4: Clarify that the IDs related to the UE variable is the one associated to the serving cell. | P1: Not Pursued. We think that the text proposal achieve exactly the same as the current text, or at least we fails to understand what the real difference is. P2: To be agreed. Indeed seems this case was overlooked in the procedural text and needs to be added. P3. Not Pursued. The proposed text is related to SIB11 and for this case UE will do something according to what is support. The existing text seems to be fine. P4: To be agreed. Is a small clarification but is good to have. Otherwise, a reader may think that is the serving cell ID, which is not. |

According to what is discussed above, the following proposals are made.

The following proposals are suggested to be agreed:

Agree CRs in R2-2502170 and R2-2502171

The Add that the validityStatus should also be included in case the UE has valid NR measurements in RRCResumeComplete.

Clarify that that the IDs related to the UE variable for L2 reset and UE-based TA is the one associated to the serving cell.

The following proposals are suggested to be discussed ONLINE:

Discuss whether the UE shall release PUCCH-CSI-Resources configured in LTM-CSI-ReportConfig when an indication from lower layers is received.

Discuss whether a coordination for the TCI states, activation of TCI state, fast processing, and early TA acquisition is needed between the MN and SN.

Discuss whether to add condition to make sure only the SRB corresponding to the cell group that triggers the LTM execution applies the default SRB configuration upon LTM cell switch execution.

The remaining proposals are suggested to be discussed OFFLINE:

Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

The following proposals are suggested to be agreed:

Proposal 1	Agree CRs in R2-2502170 and R2-2502171

Proposal 2	The Add that the validityStatus should also be included in case the UE has valid NR measurements in RRCResumeComplete.

Proposal 3	Clarify that that the IDs related to the UE variable for L2 reset and UE-based TA is the one associated to the serving cell.

The following proposals are suggested to be discussed ONLINE:

Proposal 4	Discuss whether the UE shall release PUCCH-CSI-Resources configured in LTM-CSI-ReportConfig when an indication from lower layers is received.

Proposal 5	Discuss whether a coordination for the TCI states, activation of TCI state, fast processing, and early TA acquisition is needed between the MN and SN.

Proposal 6	Discuss whether to add condition to make sure only the SRB corresponding to the cell group that triggers the LTM execution applies the default SRB configuration upon LTM cell switch execution.

The remaining proposals are suggested to be discussed OFFLINE:

Proposal 7	Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline.