3GPP TSG-RAN WG2 Meeting #129bis	R2-2503065

Wuhan, China, 7 - 11 Apr, 2025

Title:	Reply LS on soft satellite switch with re-sync

Response to:	R1-2501606/R2-2501717, R4-2502701/R2-2501741

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN2

To:	RAN1, RAN4

Cc:

Contact Person:

#### Name:	Lili Zheng

E-mail Address:	zhenglili4@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

RAN2 thanks RAN1 and RAN4 for the replies, and provide the following inputs after further RAN2 discussion:

RAN1 Question 1: Is RAN2 asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service, or whether RAN2 assumes that some RAN1 specification work may be needed?

RAN2 answer: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. If SFN is aligned at the uplink reference points for source and target satellites, the ssb-TimeOffset is given in number of subframes and the value can only be 5 or multiples of 5. RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213.

RAN1 Question 2: RAN1 assumes that the meaning of “the SSB pattern is changed” is that only the time offset is changed (may be determined by ssb-TimeOffset).

RAN2 answer: RAN2 confirms the only change is SSB time offset (determined by ssb-TimeOffset).

RAN1 Question 3: RAN1 assumes that the SIB19 of the source satellite includes ephemeris information of the target satellite, and that such information will not get outdated by the time the satellite switch occurs.

RAN2 answer: Starting from R17, it is up to UE implementation to reacquire SIB 19 before the end of serving cell validity duration, and RAN2 assumes in the soft satellite switch case it is also up to UE implementation to maintain a valid ephemeris information of target satellite and ensures it is not outdated at satellite switch.

RAN4 Question 1: RAN4 would like RAN2 to confirm which one of the following is the correct understanding of parameter ssb-TimeOffset,

-	Option 1: the relative offset between the SSB of the source satellite and the SSB of the target satellite, and the starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point

-	Option 2: the relative offset between the starting boundaries of SFN0 of the source satellite and the target satellite

RAN2 answer: Option 1 is aligned with RAN2 understanding.

RAN4 Question 2: Regarding the assumptions above, if option 1 is the correct understanding, RAN4 kindly ask RAN1/2:

1) Whether it is feasible to configure ssb-TimeOffset with values other than multiple of 5.

2) How the half-frame indication and SFN indication bits are configured during [t-ServiceStart, t-Service].

RAN2 answer: The starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point and ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Furthermore, RAN2 has agreed for Rel-18 RAN2 will not specify a specific solution but leave to NW implementation to reconfigure SMTCs as needed.

# 2. Actions:

To RAN1 and RAN4

ACTION: 	RAN2 kindly asks RAN1 and RAN4 to consider the above clarifications.

# 3. Date of Next RAN2 Meetings:

3GPP TSG RAN WG2#130	19 - 23, May, 2025		Malta, MT

3GPP TSG RAN WG2#131	25 - 29, Aug, 2025		India, IN