3GPP TSG-RAN WG2 #129bis	R2-2503080

# Wuhan, China, April 7th – 11th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0548 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the descriptions of nr-PeriodicOrOneShotTimeWindow and nr-StartSFN-TimeWindow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The meanings of IE nr-PeriodicityAndSlotOffsetTimeWindow-r18 and nr-OneShotSlotOffsetTimeWindow-r18 are separate according to the RAN1 parameter list. - nr-PeriodicityAndSlotOffsetTimeWindow-r18 is defined in RAN1 parameter list in R1-2312697 as below:Periodicity of window: Similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355 which is defined as below: dl-PRS-Periodicity-and-ResourceSetSlotOffset This field specifies the periodicity of DL-PRS allocation in slots configured per DL-PRS Resource Set and the slot offset with respect to SFN #0 slot #0 for a TRP where the DL-PRS Resource Set is configured (i.e. slot where the first DL-PRS Resource of DL-PRS Resource Set occurs). It means that the nr-PeriodicityAndSlotOffsetTimeWindow-r18 should follow the meaning of NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16 defined in LPP in Rel-16. Take an example here: startSFN =1, DurationTimeWindow =2, periodcity =4, slot offset =1 for the peridic window which is high light in dark green - nr-OneShotSlotOffsetTimeWindow-r18 is defined as the slot offset to the start SFN (nr-StartSFN-TimeWindow-r18) since there is no periodicity in nr-OneShotSlotOffsetTimeWindow-r18. Take an example here: startSFN =1, DurationTimeWindow =2, SlotOffset =1 for one shot window which is high light in purple So the description of nr-PeriodicityAndSlotOffsetTimeWindow-r18 should be revised to align with RAN1 RRC parameter list. The descriptions of IE nr-PeriodicityAndSlotOffsetTimeWindow-r18 and nr-OneShotSlotOffsetTimeWindow-r18 should be specified seperately. The reason why nr-StartSFN-TimeWindow-r18 is introduced can be found as below: Agreement(RAN1#113) To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: • The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index • The duration of the time window, which is given by a number of consecutive slots/symbols However the description of nr-StartSFN-TimeWindow doesn’t specify which TRP is based since there are multiple TRPs in assistance data, so which TRP is based for the nr-StartSFN-TimeWindow should be clarified in the description of nr-StartSFN-TimeWindow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1. Update description of nr-StartSFN-TimeWindow This field specifies the start of the time window in system frame number of the assistance data reference TRP in IE NR-DL-PRS-AssistanceData. #2. Add the description of nr-PeriodicityAndSlotOffsetTimeWindow This field specifies the periodicity of the periodic time window in slots and the slot offset with respect to SFN#0 slot#0 of the the assistance data reference TRP in IE NR-DL-PRS-AssistanceData. #3. Add the description of nr-OneShotSlotOffsetTimeWindow This field specifies the slot offset of the one-shot time window with respect to the SFN in IE nr-StartSFN-TimeWindow slot#0. #4. Delete the description of nr-PeriodicOrOneShotTimeWindow #5. Few editorial corrections Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Positioning time window config Inter-operability: If the UE supports the CR and the NW does not, the Network may use the nr-StartSFN-TimeWindow for the start of slot offset in nr-PeriodicityAndSlotOffsetTimeWindow-r18. If the NW supports the CR and UE does not, the UE may use the nr-StartSFN-TimeWindow for the start of slot offset in nr-PeriodicityAndSlotOffsetTimeWindow-r18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of nr-PeriodicityAndSlotOffsetTimeWindow-r18 doesn't align with the RAN1 parameter list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2501852 |  |  |  |  |  |  |  |  |

START OF CHANGE

6.4.3	Common NR Positioning Information Elements

*********************the unchanged part is kipped****************************************************

–	NR-DL-PRS-MeasurementTimeWindowsConfig

The IE NR-DL-PRS-MeasurementTimeWindowsConfig provides a set of time window(s) for the target device to perform measurements on indicated DL-PRS Resource Set(s) occurring.

-- ASN1START

NR-DL-PRS-MeasurementTimeWindowsConfig-r18 ::=

SEQUENCE (SIZE(1..2)) OF

NR-DL-PRS-MeasurementTimeWindowsConfigElement-r18

NR-DL-PRS-MeasurementTimeWindowsConfigElement-r18 ::= SEQUENCE {

nr-StartSFN-TimeWindow-r18		INTEGER (0..1023),

nr-PeriodicOrOneShotTimeWindow-r18 CHOICE {

nr-PeriodicityAndSlotOffsetTimeWindow-r18

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

nr-OneShotSlotOffsetTimeWindow-r18 CHOICE {

scs15-r18						INTEGER (0..10239),

scs30-r18						INTEGER (0..20479),

scs60-r18						INTEGER (0..40959),

scs120-r18						INTEGER (0..81919)

}

}																		OPTIONAL, -- Need ON

nr-SymbolOffsetTimeWindow-r18	INTEGER (0..13)							OPTIONAL, -- Need ON

nr-DurationTimeWindow-r18		ENUMERATED { n1, n2, n4, n6, n8, n12, n16, ... },

nr-SelectedDL-PRS-FrequencyLayerIndex-r18	INTEGER (0..nrMaxFreqLayers-1-r16)

OPTIONAL, -- Need ON

nr-SelectedDL-PRS-IndexListPerFreq-r18

SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-SelectedDL-PRS-IndexPerTRP-r18	OPTIONAL, -- Need OP

nr-MeasurementsToPerformInTimeWindow-r18			BIT STRING { rstd (0),

ueRxTx (1),

rsrp (2),

rsrpp (3),

rscp (4),

rscpd (5)

} (SIZE(1..16)) OPTIONAL, -- Need ON

...

}

NR-SelectedDL-PRS-IndexPerTRP-r18 ::= SEQUENCE {

nr-SelectedTRP-Index-r18				INTEGER (0..nrMaxTRPsPerFreq-1-r16),

dl-SelectedPRS-ResourceSetIndexList-r18	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

INTEGER (0..nrMaxSetsPerTrpPerFreqLayer-1-r16)		OPTIONAL, --Need OP

...

}

-- ASN1STOP

NR-DL-PRS-MeasurementTimeWindowsConfig field descriptions

nr-StartSFN-TimeWindow This field specifies the start of the time window in system frame number of the assistance data reference TRP in IE NR-DL-PRS-AssistanceData.

nr-PeriodicityAndSlotOffsetTimeWindow This field specifies the periodicity of the periodic time window in slots and the slot offset with respect to SFN#0 slot#0 of the the assistance data reference TRP in IE NR-DL-PRS-AssistanceData.

nr-OneShotSlotOffsetTimeWindow This field specifies the slot offset of the one-shot time window with respect to the SFN in IE nr-StartSFN-TimeWindow slot#0.

nr-SymbolOffsetTimeWindow This field specifies the symbol offset with respect to the slot offset in nr-PeriodicOrOneShotTimeWindow.

nr-DurationTimeWindow This field specifies the desired duration of a time window for the indicated DL-PRS Resource Set in unit of slots. Enumerated value 'n1' corresponds to 1 slot, n2 to 2 slots, n4 to 4 slots and so on.

nr-SelectedDL-PRS-FrequencyLayerIndex This field indicates the frequency layer provided in IE NR-DL-PRS-AssistanceData. Value 0 corresponds to the first frequency layer provided in nr-DL-PRS-AssistanceDataList in IE NR-DL-PRS-AssistanceData, value 1 to the second frequency layer in nr-DL-PRS-AssistanceDataList, and so on.

nr-SelectedDL-PRS-IndexListPerFreq This field provides the list of addressed TRPs of the selected frequency layer. If this field is absent, all DL-PRS Resources of all TRPs of the indicated frequency layer are addressed. The number of the indicated DL-PRS Resource Set(s) for all the selected TRPs in this list is the same.

nr-SelectedTRP-Index This field indicates the addressed TRP of the selected frequency layer. Value 0 corresponds to the first entry in nr-DL-PRS-AssistanceDataPerFreq provided in IE NR-DL-PRS-AssistanceData, value 1 corresponds to the second entry in nr-DL-PRS-AssistanceDataPerFreq, and so on.

dl-SelectedPRS-ResourceSetIndexList This field provides the list of addressed DL-PRS Resource Sets of the selected TRPs of the selected frequency layer. If this field is absent, all DL-PRS Resource Sets and Resources of the indicated TRP are addressed.

nr-MeasurementsToPerformInTimeWindow This field indicates the measurements that the target device shall perform in the provided time window. If multiple bits are set to 1, then the target device shall perform multiple measurements in the same time window.

END OF CHANGE