3GPP TSG-RAN WG2 Meeting #129bis	R2-2502983

Wuhan, China, Apr 7th – 11th, 2025

Agenda item:	9.3

Source: 	Session Chair (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN and IoT-NTN

Document for:	Approval

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT129bis][300] Organizational – NR-NTN and IoT-NTN session

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN and IoT-NTN

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday |  |  |  |  |
| 09:00 – 10:30 | [1], [2], [3], [7.0] R18 common (Diana) [7.0.1] UE capabilities Break out [8.0] NR19 General @NR151617 UP (Diana) [7.0.2] Other Rel-18 corrections cont | Breakout to start after completion of 7.0.1 [7.6] NR18 SL (Kyeongin) [8.5] NR19 NES @12:00 (if NR18 SL ends early) (Kyeongin) | Breakout to start after completion of 7.0.1 NRLTE151617 Pos (Nathan) [4.3] LTE positioning [5.3] NR Rel-16 and earlier [6.4] NR Rel-17 [7.1] NR18 Pos (Nathan) |  |
| 10:55 – 12:45 |  |  |  |  |
| 14:00 – 15:45 | [7.0.2] Other Rel-18 corrections cont [7.0.2.11] NR TEI18 [7.8] Other Rel-18 corrections | [7.2] NR18 Mob (Kyeongin) | [7.1] NR18 Pos (Nathan) [6.2][7.5] NR1718 SL Relay (Nathan) |  |
| 16:10– 18:00 | [8.2] NR19 Ambient IoT [2.5] (Diana) | [7.2] NR18 Mob continued if needed (Kyeongin) [8.6] NR19 Mob (if NR18 Mob ends early) (Kyeongin) | NR18 MIMO (Erlin) [7.0.2.13] [8.12] NR19 MIMO (Erlin) |  |
| 18:30 | Basketball match against RAN3 |  |  |  |
| Tuesday |  |  |  |  |
| 08:30 – 10:30 | [8.6] NR19 Mob [2] (Kyeongin) | [8.7] NR19 XR [2] (Dawid) | EUTRA&NR151617 (Mattias) Except NR17 NTN related Tdoc, which will be handled in Sergio´s session. [4.1] [5.1.1], [5.1.3.1], [5.1.3.2], [5.1.3.3] [6.1.1], [6.1.3], [6.1.3.1], [6.1.3.2], [6.1.3.3] |  |
| 11:00 – 13:00 | [8.1] NR19 AI/ML PHY [2.5] (Diana) | NR18 NTN NR /IoT(Sergio) [4.1] R17 IoT NTN corrections [6.1.1] Stage 2 R17 NTN corrections [6.1.2] UP R17 NTN corrections [6.1.3] CP R17 NTN corrections [7.0.2.17] R18 NR NTN corrections [7.0.2.18] R18 IoT NTN corrections [7.0.2.20] TEI18 (NTN related aspects) [8.9] NR19 IoT NTN [1] Sergio [8.9.3] Uplink Capacity Enhancements (if time allows) R2-2502773 Report of [Post129][307] | EUTRA&NR151617 (Mattias) cont’ if needed [7.0.2.11] NR18 SON/MDT [8.10] NR19 SONMDT [0.5] (Mattias) NR18 XR/MBS/QoE (Dawid) (if needed) [7.0.2.16] XR [7.0.2.14] MBS [7.0.2.15] QoE |  |
| 14:30 -16:30 | [8.2] NR19 Ambient IoT [2] (Diana) | [8.8] NR19 NR NTN (Sergio) [2] [8.8.1] Organizational [8.8.2] Downlink coverage enhancements [8.8.4] Support of Broadcast service | [8.10] NR19 SONMDT [0.5] (Mattias) cont’ if needed |  |
| 17:00– 19:00 | [8.3] NR19 AI/ML Mobility [2] (Diana) | [8.4] NR19 LP-WUS [1] (Erlin) | [8.13] NR19 SL Relay (Nathan) |  |
| Wednesday |  |  |  |  |
| 08:30 – 10:30 | [8.6] NR19 Mob [2] (Kyeongin) | [8.7] NR19 XR [2] (Dawid) | @9:30 [8.19] NR19 NR Other (Erlin) |  |
| 11:00 – 13:00 | [8.3] NR19 AI/ML Mobility [2] (Diana) | [8.5] NR19 Network Energy Saving [1] (Kyeongin) | [8.11] NR19 SBFD [0.75] (Erlin) |  |
| 14:30 -16:30 | [8.1] NR19 AI/ML PHY [2.5] (Diana) | [8.9] NR19 IoT NTN [1] Sergio [8.9.1] Organizational [8.9.3] Uplink Capacity Enhancements (cont) | [6.2][7.5] NR1718 SL relay CB (Nathan) [8.13] NR19 SL relay (Nathan) |  |
| 17:00– 19:00 | [8.18] TEI19 | [8.17] NR19 IoT NTN TDD mode [0.5] [8.8] NR19 NR NTN [2] (Sergio) [8.8.6] LTE to NR NTN mobility [8.8.3] Uplink Capacity/Throughput Enhancement [8.8.5] Support of regenerative payload (if time allows) | [7.1] NR18 Pos (Nathan) and TEI19 positioning @6pm Training on CR design (ASN.1 naming and other rules for Rel-19 rapporteurs) (Hakan) |  |
| Thursday |  |  |  |  |
| Colourful Polo day |  |  |  |  |
| 08:30 – 10:30 | @9:00 [8.1] NR19 AI/ML PHY [2.5] (Diana) CB time if need | [4.1], [7.0.2.18], [7.0.2.20] R18 IoT NTN, TEI18 (Sergio) - issues marked CB Thursday - outcome of [302] [8.9] R19 IoT NTN CB (Sergio) [8.9.2] Support of S&F | CB Nathan [7.1] NR18 Positioning [7.5] NR18/[8.13] NR19 SL relay (if needed) |  |
| 11:00 – 13:00 | [8.2] NR19 Ambient IoT [2.5] (Diana) | [8.9] R19 IoT NTN CB (Sergio) [8.9.3] outcome of [305] Ph1 [7.0.2.17] NR18 NR NTN (Sergio) - Outcome of [301] - issue marked CB Thursday [8.8] NR19 NR NTN CB (Sergio) - Outcome of [303], [304] [8.18] TEI 19 / [8.19] NR others (Sergio) (NTN related aspects) | CB Mattias CB EUTRA&NR151617 (Mattias) [8.10] CB SON/MDT R19 |  |
| 14:30 -16:30 | @14:30-15:30 CB AIoT (if needed) @15:30-16:30 CB NR19 AI/ML Mobility (if needed) | [7.6] CB NR18 SL (Kyeongin) [8.5] CB NR19 NES (Kyeongin) | CB Erlin @14:30 – 15:00 CB for R18 MIMOevo, NR19 MIMO (Erlin) @15:00 [8.4] CB NR19 LP-WUS (Erlin) |  |
| 17:00 – 19:00 | CB NR 18 and TEI19 Diana | [7.2] CB NR18 Mob (Kyeongin) [8.6] CB NR19 Mob (Kyeongin) | CB Dawid: NR18 XR/MBS/QoE [8.7] NR19 XR CB/continuation |  |
| Friday |  |  |  |  |
| 08:30 – 10:30 | CB Diana TBD @9:30-10:30 CB Ambient IoT | CB Sergio (from 9:00) [7.0.2.17] NR18 NR NTN (Sergio) - issues marked CB Friday [8.8] NR19 NR NTN CB - issues marked CB Friday - outcome of [304] [8.9] R19 IoT NTN CB [8.9.3] outcome of [305] Ph2 [8.9.4] Support of PWS [7.0.2.20] TEI18 - outcome of [302] [8.18] TEI 19 / [8.19] NR others (NTN related aspects) | CB Erlin NR19 MIMO if needed CB NR19 SBFD, NR19 others |  |
| 11:00 – 13:00 | CB Diana @11-12 R19 Ambient IoT Other CBs Reports from breakout sessions EoM |  |  |  |
| 14:30 – 16:00 |  |  |  |  |
| 16:00 – 17:00 |  |  |  |  |

Breaks

Morning coffee: 		10:30 to 11:00

Lunch: 			13:00 to 14:30 (except Monday)

Afternoon coffee:	16:30 to 17:00 (except Monday)

List and details of [AT129bis] offline discussions

NOTE:  No offline email discussions will be kicked off before Monday Apr 7th, 09:00 local time

[AT129bis][301][R18 NR NTN] Soft satellite switch with re-sync (Huawei)

Scope: Draft a reply LS to RAN1 and RAN4 and also a CR to capture the agreement on ssb-TimeOffset

Intended outcome: Draft reply LS and 38331 CR

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for Draft reply LS and CR:  Thursday 2025-04-10 08:00

[AT129bis][302][TEI18] In-band operation for NB-IoT (Ericsson)

Scope: discuss the actual change for stage 2 as well as possible stage 3 changes and also the reply LS to RAN4

Intended outcome: summary of the offline discussion and Draft reply LS

Deadline for companies' feedback:  Thursday 2025-04-10 20:00

Deadline for offline discussion summary and Draft reply LS:  Friday 2025-04-11 08:00

[AT129bis][303][R19 NR NTN] SMTC enhancements (Xiaomi)

Scope: discuss the wording of a LS to RAN4 regarding RAN2 intention to signalling support for different SMTC periodicity and offset in the same frequency layer, for both idle and connected mode. Also discuss other possible SMTC enhancements for connected/idle mode.

Intended outcome: summary of the offline discussion and Draft LS

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for offline discussion summary and Draft LS:  Thursday 2025-04-10 08:00

[AT129bis][304][R19 NR NTN] Service continuity (Apple)

Scope: discuss the proposal to provide intended service area IDs for each MBS broadcast service session in SIB21, or other alternative approaches to support service continuity

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for offline discussion summary:  Thursday 2025-04-10 08:00

[AT129bis][305][R19 IoT NTN] CB-msg3-EDT (Mediatek)

Scope:

Phase 1: discuss single v.s. multiple RNTI/window design

Phase 2: discuss remaining CB-msm33-EDT issues (mainly based on R2-2502771) and LS to RAN1 on CB-msg3-EDT shared resource configuration

Intended outcome: summaries of the offline discussion and Draft LS to RAN1

Deadline for companies' feedback (for phase2):  Thursday 2025-04-10 20:00

Deadline for offline discussion summary (for phase2) and Draft LS:  Friday 2025-04-11 09:00

## 4.1	EUTRA corrections Rel-17 and earlier

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

R2-2502561	Correction to explicit indication of epoch in SIB31	Nordic Semiconductor ASA	discussion	Rel-17

Alt1: the “next upcoming SFN” means any SFN after SFN n that is carrying last subframe of last repetition of message within SI window until SFN n+1023.

Alt2: the “next upcoming SFN” means any SFN after SFN n being the last subframe of SI window until SFN n+1023.

-	Xiaomi thinks a clarification is not needed

-	QC thinks interpretation 2 is correct but if we have a change we should go for a different formulation

-	MTK supports the intention and prefers to align to interpretation 2

-	Ericsson and Apple agree with Xiaomi that we don’t need further clarification. Apple thinks the same applies to NR as well.

-	Nordic suggests the following clarifications:

-	startSFN cannot be in the past from the UE point of view

-	startSFN can be in the future up to 1023 SFNs (according to E/// this was also RAN1 original intention)

-	proper network configuration solves the issue, i.e. startSFN is set to SFN of the last repetition within the SI window or a later SFN

-	QC thinks this could be solved by saying that the network never configures startSFN in the

Past

RAN2 understands that the network never configures startSFN in the past for the serving cell (can come back to check if a clarification in the spec is also useful)

## 6.1	Common

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

R2-2501704	LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 (C1-250715; contact: Qualcomm)	CT1	LS in	Rel-17	5GSAT	To:SA1	Cc:SA2, CT4, RAN2, RAN3

Noted

- QC thinks it would be good to clarify in Stage 2 that PWS is not supported in Rel-17 and Rel-18

We wait for SA1 feedback before deciding to have any clarification in Stage 2

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

R2-2502654	Corrections on Power saving features in NTN	Qualcomm Incorporated	discussion	Rel-17	NR_NTN_solutions-Core

Proposal 1	Add Rel-17 late noncritical extension to signal MAC-ParametersFRX-Diff-r16 in NTN. Text proposal is provided above.

- Vivo supports this

- Ericsson thinks we’d better say that we don’t support DCP in NTN. QC could agree but still thinks we should signal separately for NTN

- Huawei supports introducing mac-ParametersNTN

- Nokia agrees with Ericsson but could accept p1

DCP is not supported in NTN (at least up to Rel-19). FFS how/whether we clarify this in the specs

Proposal 2	Clarify that in NTN, if drx-onDurationTimer would not be running prior to symbol n (i.e., UE is in DRX sleep prior to symbol n), it is up to UE implementation whether to wakeup from C-DRX sleep prior to symbol n to report periodic CSI.

Can come back next time

### 6.1.3	Control Plane corrections

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

R2-2502249	Correction on carrier frequency information in RRCConnectionRelease.	CATT, Qualcomm Incorporated	CR	Rel-17	36.331	17.12.0	5106	-	F	NR_NTN_solutions-Core

- Samsung wonders if we need the first change. Ericsson agrees

- Samsung thinks we should rather have a clarification in Stage2. MTK and Oppo agree

- QC thinks a clarification is needed and having this in Stage 2 would also be ok

Not pursued

We introduce a clarification in Stage2

R2-2502250	Correction on carrier frequency information in RRCConnectionRelease.	CATT, Qualcomm Incorporated	CR	Rel-18	36.331	18.5.0	5107	-	A	NR_NTN_solutions-Core

Not pursued

R2-2502519	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.331	17.12.0	5303	-	F	NR_NTN_solutions-Core

- QC thinks this might also have RAN5 impacts. Also wonders if we need to update 304. ZTE thinks 304 is fine.

RAN2 confirms that location-based measurements for both quasi-earth fixed cell scenario and earth-fixed cell scenario is supported since R17

Agreed in principle (with the parenthesis after the dash: “(quasi-)”)

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

R2-2502520	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.306	17.12.0	1255	-	F	NR_NTN_solutions-Core

Agreed in principle (with the parenthesis after the dash: “(quasi-)”)

R2-2502521	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	38.306	18.5.0	1256	-	A	NR_NTN_solutions-Core

Agreed in principle (with the parenthesis after the dash: “(quasi-)”)

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 6

#### 7.0.2.17	NR NTN enhancements

(NR_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-232669)

RACH-less HO

R2-2501727	Reply LS on inter-gNB RACH-less HO in NTN (R3-250762; contact: Nokia)	RAN3	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN1

Noted

The related discussion is expected to happen in 7.0.2.1

R2-2502009	Correction to RACH-less handover for NTN	Xiaomi	CR	Rel-18	38.300	18.5.0	0977	-	F	NR_NTN_enh-Core

Not pursued

FR2-NTN UE capability

R2-2501747	Reply LS to RAN2 on UE capabilities for FR2-NTN (R4-2503039; contact: vivo)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN1

Noted

R2-2501782	Correction on NTN in FR2-NTN bands	vivo, ZTE Corporation, Ericsson, Sanechips	CR	Rel-18	38.306	18.5.0	1200	1	F	NR_NTN_enh-Core	R2-2409544

Agreed in principle

Soft satellite switch with re-sync

R2-2501717	Reply LS on soft satellite switch with re-sync (R1-2501606; contact: Huawei)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2

Noted

R2-2501741	reply LS on soft satellite switch with re-sync (R4-2502701; contact: Huawei)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2, RAN1

Noted

For Rel-18 RAN2 will not specify a specific solution but leave to NW implementation to reconfigure SMTCs as needed

Indicate RAN2 decision in the reply LS to RAN1 and RAN4

R2-2502842	Discussion on satellite switch with resync	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Add the following in the field description of ssb-TimeOffset: “It is given in number of subframes and the value can only be 5 or multiples of 5”.

- Ericsson thinks this is needed only if we need to shift back. Nokia agrees

- Oppo thinks that RAN2 can at least take an assumption

RAN2 understands that if SFN is aligned at the uplink reference points for source and target satellites, the ssb-TimeOffset is given in number of subframes and the value can only be 5 or multiples of 5.

Proposal 2: On Q1 from RAN1 LS, RAN2 answer is that: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. The ssb-TimeOffset will be configured in 5ms or multiples of 5ms, RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213, the SSB configuration is only “implicitly reconfigured” from RAN2 perspective.

On Q1 from RAN1 LS, RAN2 answer is that: RAN2 is asking whether a Rel-18 UE (based on current Rel-18 RAN1 specifications) can maintain the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. If SFN is aligned at the uplink reference points for source and target satellites, the ssb-TimeOffset is given in number of subframes and the value can only be 5 or multiples of 5. RAN2 does not intend to change the SSB pattern within half frame as defined in clause 4.1 of TS 38.213.

Proposal 3: On Q2 from RAN1 LS, RAN2 answer is that: RAN2 confirms the only change is SSB time offset (determined by ssb-TimeOffset).

Agreed

Proposal 4: On Q3 from RAN1 LS, RAN2 answer is that: Starting from R17, it is up to UE implementation to reacquire SIB 19 before the end of serving cell validity duration, and RAN2 assumes in the soft satellite switch case it is also up to UE implementation to maintain a valid ephemeris information of target satellite and ensures it is not outdated at satellite switch.

Agreed

Proposal 5: Regarding the two options of ssb-TimeOffset from RAN4 LS, RAN2 answer is that: Option 1 is aligned with RAN2 understanding.

Agreed

Proposal 6: Regarding RAN4 question on ssb-TimeOffset granularity and half-frame indication, RAN2 answer is that: ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Regarding RAN4 question on ssb-TimeOffset granularity and half-frame indication, RAN2 answer is that: if SFN is aligned at the uplink reference points for source and target satellites, ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.

Proposal 7: Option 3 is adopted, unless RAN1 raised further concern. The target satellite uses different SSB transmission pattern with source satellite during [t-Start, t-Service], and then change back to the source satellite SSB pattern after t-Service.

Proposal 8: Approve the reply LS in the Annex.

Draft a reply LS according to agreements above

[AT129bis][301][R18 NR NTN] Soft satellite switch with re-sync (Huawei)

Scope: Draft a reply LS to RAN1 and RAN4 and also a CR to capture the agreement on ssb-TimeOffset

Intended outcome: Draft reply LS and 38331 CR

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for Draft reply LS and CR:  Thursday 2025-04-10 08:00

R2-2503051	Draft Reply LS on soft satellite switch with re-sync	Huawei	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN4

Revise the last RAN2 answer as: “The starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point and ssb-TimeOffset can only be configured as 5ms or multiples of 5ms. RAN2 understanding is that SSB from the target satellite will indicate SFN and half frame bit corresponding to the actual time domain position as in legacy.”

Revised in R2-2503065

R2-2503065	Reply LS on soft satellite switch with re-sync	Huawei	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN4

Change “is” to “was” in the first RAN2 answer

R2-2503069	Reply LS on soft satellite switch with re-sync	Huawei	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN4

Approved

R2-2503052	Clarification on ssb-TimeOffset	Huawei	CR	Rel-18	38.331	18.5.0	XXXX	-	F	NR_NTN_enh-Core

Revised in R2-2503066 to clarify the description aligned to the response in the LS

R2-2503066	Clarification on ssb-TimeOffset	Huawei	CR	Rel-18	38.331	18.5.0	XXXX	1	F	NR_NTN_enh-Core

Agreed in principle

R2-2502681	LS response to RAN1 and RAN4 for soft satellite switch with re-sync	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	RAN2 to confirm there might be specification work in RAN1 when SSBs are shifted back upon t-Service for the soft satellite switch with resync.

Proposal 2	Confirm the second and the third assumptions from RAN1’s LS.

Proposal 3	UE initial access during soft satellite switch is not optimized in Rel-18 maintenance.

Proposal 4	Confirm that option 1 is the correct understanding of ssb-TimeOffset parameter.

Proposal 5	From RAN2 perspective, it is feasible to configure ssb-TimeOffset with values multiple of 5ms.

Proposal 6	The question on the half-frame indication and SFN indication bits touches upon a RAN1 procedure which restricts where an SSB can be placed within a half-frame. Thus, this question can be answered once RAN1 has finalized their discussions on this and other aspects, including the feasibility of supporting this feature accounting for the specification impacts.

R2-2502550	Further Considerations on SSBs from Source and Target Satellite in Soft Switching with Resynchronization	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 responds to RAN4 and states ssb-TimeOffset provided within SatSwitchWithReSync can be configured to other value than multiples of 5.

Proposal 2: RAN2 responds to RAN4 and indicates there is no separate half-frame or SFN indication for satellite switching with resynchronization which would apply during [t-ServiceStart, t-Service].

Proposal 3: RAN2 responds to RAN1 and indicates the intention was to check if the existing RAN1 specification supports such behaviour.

Proposal 4: RAN2 responds to RAN1 and indicates the “SSB pattern change” was supposed to refer to SSB shift in time by applying ssb-TimeOffset.

Proposal 5: RAN2 responds to RAN1 and indicates SIB19 comprises SatSwitchWithReSync, wherein ntn-Config associated with target satellite is conveyed. It is expected such information is also valid at the time of satellite switching.

Proposal 6: RAN2 considers a draft LS shown in Annex A when preparing a response LS to RAN1 and RAN4.

Proposal 7: RAN2 to consider relying on NCD-SSBs to mitigate the SSB overlapping issue for soft-satellite switching with resynchronization.

Proposal 8: RAN2 considers extending SatSwitchWithReSync in SIB19 by including NCD-SSB configuration which would be valid from t-ServiceStart to t-Service.

Proposal 9: RAN2 considers the NR RRC changes shown in Annex B.

R2-2501771	Discussion on RAN1 and RAN4 Reply LS on Soft Satellite Switch with Re-sync	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2502294	Discussion on the reply LS from RAN1/RAN4 on soft satellite switch	CATT	discussion

R2-2502327	Discussion on the satellite switch with sync	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2502523	Discussion on ssb-TimeOffset for soft satellite switch with resync	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

R2-2502539	Discussion on the reply LSs on the soft satellite switch with resynchronization	Xiaomi	discussion

R2-2502653	Way forward for soft satellite switch with resync	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2502740	Discussion on LS from RAN1,RAN4 on soft satellite switch with re-sync	CMCC	discussion	Rel-18	NR_NTN_enh-Core

Location/time-based CHO

R2-2502669	Correction to Rel-18 NR NTN CHO with only location/time-based trigger	Samsung	CR	Rel-18	38.331	18.5.1	5314	-	F	NR_NTN_enh-Core

- Xiaomi thinks there is no need for this as the NW can know the UE release and know whether the UE supports this. vivo also thinks this is not needed. Nokia agrees. Samsung this is not the general way we handle features

- QC agrees with Samsung. Ericsson agrees we should rely on capabilities and not UE releases. and supports the CR. CATT also agrees

Agreed in principle

R2-2502670	Correction to Rel-18 NR NTN CHO with only location/time-based trigger	Samsung	CR	Rel-18	38.306	18.5.0	1263	-	F	NR_NTN_enh-Core

Agreed in principle

#### 7.0.2.18	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN2; REL-18; WID: RP-223519)

R2-2501726	Reply LS on UE Location Information for NB-IoT NTN (R3-250761; contact Qualcomm)	RAN3	LS in	Rel-18	IoT_NTN_enh-Core	To:SA2	Cc:RAN2, CT1

RAN2 shares the same understanding as RAN3 (no need to further reply).

R2-2501968	Correction on SIB33 reception in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5105	-	F	IoT_NTN_enh-Core

- ZTE partially agrees with this but suggests to clarify the sentence and the description in the reason for change

- Samsung supports this but suggest to revise as:

“… where the UE is required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) and may acquire the SystemInformationBlockType33 (SystemInformationBlockType33-NB in NB-IoT).”

- ZTE also suggests to clarify the summary of change as:

“In 5.2.1.3, clarify that BL UEs, UEs in CE, or NB-IoT UEs may acquire the SIB33(-NB) in RRC CONNECTED after the expiry of T317”

Revise the CR as above in R2-2503061

R2-2503061	Correction on SIB33 reception in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5105	1	F	IoT_NTN_enh-Core

Agreed in principle

R2-2502522	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	36.300	18.4.0	1417	-	F	IoT_NTN_enh-Core

Remove the impacted architecture ffrom the cover sheet

Put the parenthesis after the –: “(quasi-)”

Revised in R2-2503062

R2-2503062	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	36.300	18.4.0	1417	1	F	IoT_NTN_enh-Core

Agreed in principle

R2-2502687	Correction on release procedures when HARQ feedback is not configured	Samsung	CR	Rel-18	36.331	18.5.0	5112	-	F	IoT_NTN_enh-Core

- Ericsson thinks the CR is not needed as the current text does not say how HARQ feedback is disabled. Xiaomi and Nokia agree with Ericsson. Vivo also agree

RAN2 understands that Note 0a also covers the case when HARQ feedback is disabled by DCI

Not pursued

#### 7.0.2.20	TEI18

R2-2501746	LS on NR NB-IoT in-band operation (R4-2503035; contact: Ericsson)	RAN4	LS in	Rel-19	TEI18	To:RAN2	Cc:RAN1

Noted

R2-2502682	In-band operation for NB-IoT	Ericsson	discussion	Rel-18	TEI18

Observation 1	RAN4's assumption is that UEs only need to support the same operation mode for anchor and non-anchor carriers which in principle indicates that mixed operation mode is not applicable to NTN in Release 18. This does not have impact in RAN2 specifications.

Observation 2	RAN4's assumption that RRC should indicate guardband-r13 in operationModeInfo IE in MIB-NB does not have an impact in RAN2 specifications.

Observation 3	A NB-IoT UE should not be able to access a cell with an operation mode it does not support.

Observation 4	The network is not expected to broadcast inband-SamePCI or inband-DifferentPCI for NTN in Release 18.

Proposal 1	Update stage 2 and add a note to state which combinations are allowed in NB-IoT NTN. Take the TP as a baseline.

- QC agrees but also thinks we need a new UE capability

- ZTE wonders how this in-band operation is expected to work in a NR band

- Nokia supports p1 but thinks we need to discuss p2

- Samsung thinks we just need Stage 2 update and a capability

Update stage 2 and add a note to state which combinations are allowed in NB-IoT NTN (can discuss the actual TP in offline 302)

Proposal 2	RAN2 to discuss whether signalling enhancements are needed to prevent UEs from access a NB-IoT NTN cell that sets operationModeInfo to inband-SamePCI or inband-DifferentPCI.

Continue in offline 302

[AT129bis][302][TEI18] In-band operation for NB-IoT (Ericsson)

Scope: discuss the actual change for stage 2 as well as possible stage 3 changes and also the reply LS to RAN4

Intended outcome: summary of the offline discussion and Draft reply LS

Deadline for companies' feedback:  Thursday 2025-04-10 20:00

Deadline for offline discussion summary and Draft reply LS:  Friday 2025-04-11 08:00

R2-2503053	Report of [AT129bis][302][TEI18] in-band operation for NB-IoT	Ericsson	discussion	TEI18

- ZTE and Nokia thinks we need to clarify this is about inband operation with NR.

- ZTE thinks we should add a new table with a new description title. Mediatek agrees

- Ericsson thinks a note to the existing table is sufficient as the table is already referring to coexistence with NR

- Nokia is fine with adding a new note to the existing table

Come back Friday regarding how to clarify this in Stage 2, e.g. note to existing Table or new table

Proposal 2	Introduce a new UE capability without signalling to indicate the support of in-band in NB-IoT NTN.

- ZTE thinks we don’t know what’s the feature

- HW thinks we can wait for RAN4 to decide on this and we can ask about this in the LS we send to them

We wait for RAN4 indication whether a new capability is needed

Proposal 3	From RAN2 perspective, the introduction of in-band support in NB-IoT NTN does not have any other Stage 3 impact.

Further continue the discussion offline

R2-2503063	Report of [AT129bis][302][TEI18] in-band operation for NB-IoT – Ph2	Ericsson	discussion	TEI18

Withdrawn

- Ericsson indicates that some companies need more time to check potential Ran2 impacts before replying to RAN4

- QC and thinks we still need to provide a reply LS to RAN4

Come back in the next meeting on possible impacts on RAN2 specs

Attempt to send a reply LS to RAN4 from this meeting, confirming the feasibility and indicating that RAN2 will continue to assess if any clarification in RAN2 specs is needed in the next RAN2 meeting

R2-2503054	Draft Reply on NR NB-IoT in-band operation	Ericsson	LS out	Rel-19	TEI18	To:RAN4	Cc:RAN1

Revised in R2-2503064 to reflect meeting agreements

R2-2503064	Draft Reply on NR NB-IoT in-band operation	Ericsson	LS out	Rel-19	TEI18	To:RAN4	Cc:RAN1

Discussed in [Post129bis][303]

[Post129bis][303][TEI18] LS to RAN4 on NR NB-IoT in-band operation (Ericsson)

Scope: Draft a reply LS to RAN4 on NR NB-IoT in-band operation

Intended outcome: Approved LS

Deadline: 1-week

R2-2501773	Discussion on RAN4 LS on IoT NTN and NR NTN Inband Operation.	vivo	discussion	Rel-18	TEI18

Observation 1: For NB-IoT NTN inband operation in Rel-18, the operational mode of anchor and non-anchor carriers will be the same (i.e. guard-band or standalone) by network implementation based on the current RRC signaling. There is no RAN2 stage-3 impact foreseen.

Observation 2: For NB-IoT NTN inband operation in Rel-18, guardband-r13 should be always indicated in operationModeInfo-r13 in MasterInformationBlock-NB. It can be supported by the current specification. There is no RAN2 impact foreseen.

Observation 3: To make sure a UE will not access a cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in operationModeInfo-r13 in MasterInformationBlock-NB, cellBarred-NTN shall be set to 'barred' on that cell. There is no RAN2 impact foreseen as the existing cell bar mechanism can fulfill this goal.

Proposal 1: RAN2 to capture the operational mode of anchor and non-anchor carriers will be the same for NB-IoT NTN inband operation in clause 5.5a of Rel-18 36.300.

Proposal 2: For NB-IoT NTN inband operation in Rel-18, it is RAN2’s understanding that cellBarred-NTN shall be set to 'barred' if the cell indicating inband-SamePCI-r13 or inband-DifferentPCI-r13 in MasterInformationBlock-NB (No spec change is required).

Proposal 3: For NB-IoT NTN inband operation in Rel-18, RAN2 responds to RAN4 that the RAN4 assumptions are aligned with RAN2 specification and no RAN2 stage-3 impact is foreseen.

R2-2502047	RAN2 impacts for NB-IoT operation as NR Inband	Nokia , Nokia Shanghai Bells	discussion

Proposal 1: RAN2 to discuss need for new UE capability in Rel-18 (without signalling) for the proposed NB-IoT Guard band mode operation for NB-IoT NR Inband deployment.

Proposal 2: Network configuration for NB-IoT operating mode for NR Inband should be  captured in RAN2 specification.

Proposal 3: No UE behaviour changes needed related to Inband configuration parameters in Rel-18 as this parameter is not applicable for IoT-NTN in NR Inband.

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243300)

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240924)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, etc.

For the LTE_TN_NR_NTN_mob WI, including endorsed draft CRs from the WI spec rapporteurs.

Rapporteur inputs do not count towards the tdoc limitation.

Running CRs

R2-2502192	Stage 2 Running CR for NR NTN phase 3	THALES	draftCR	Rel-19	38.300	18.5.0	B	NR_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2502676	Running RRC CR for NR NTN phase 3	Ericsson	CR	Rel-19	38.331	18.5.1	5315	-	B	NR_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2502511	Discussion on NR NTN UE capabilities	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Introduce an optional UE capability without signaling for ETWS geo-fencing.

- Vivo  thinks we can link to this the existing capability for ETWS, saying that the UE optionally supports also geo-fencing. Xiaomi agrees

- CATT thinks this is something new and then wonders if we can reuse the existing capability

- QC thinks this is also related to the discussion to support geo-fencing for TN

Come back in the next meeting

Proposal 2: Introduce an optional UE capability without signaling for intended service area provision for MBS broadcast service via NTN.

We add a sentence saying that the UE can optionally support intended service area provision for MBS broadcast service via NTN.

Proposal 3: No new UE capability is foreseen for regenerative payload.

Agreed

R2-2502512	Draft CR for Rel-19 NR NTN UE capabilities	Apple	draftCR	Rel-19	38.306	18.5.0	NR_NTN_Ph3-Core

To be reviewed in post meeting email discussion

Withdrawn

R2-2501770	Introduction of LTE TN to NR NTN Mobility UE Capability	vivo	CR	Rel-19	36.306	18.4.0	1900	3	B	LTE_TN_NR_NTN_mob-Core	R2-2501417

Agreements:

1.	We add a sentence saying that the UE can optionally support intended service area provision for MBS broadcast service via NTN.

2.	No new UE capability is foreseen for regenerative payload.

[Post129bis][304][R19 NR NTN] Stage2 CR (Thales)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][305][R19 NR NTN] RRC CR (Ericsson)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][306][R19 NR NTN] MAC CR (Interdigital)

Scope: discuss the running MAC CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][307][R19 NR NTN] 38.304 CR (ZTE)

Scope: discuss the running 38.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][308][R19 NR NTN] capability CR (Apple)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][314][LTE to NR NTN] RRC CR (CATT)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR

Deadline: long

### 8.8.2	Downlink coverage enhancements

Contributions should focus on RAN2 aspects of DL coverage enhancements due to extended SIB periodicity (up to 160ms), including e.g. possible SMTC impacts (while no contributions are expected on cell level / beam level DTX/DRX mechanism).

R2-2501798	Discussion on DL coverage enhancement	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

- Cell barring

Proposal 1: From SSB extension point of view, there is no need to bar legacy UE. From link level enhancement ( i.e. PDCCH CSS repetition, SIB1 PDSCH repetition, Msg4 PDSCH repetition) point of view, RAN2 can wait for RAN1 conclusion on whether the enhancements are backward compatible. Or RAN2 can send a LS to RAN1 on this.

-	Samsung thinks that link level enhancements will not require new access barring

-	Fujitsu thinks we should leave the possibility for the NW to bar legacy UEs in case of extended SSB periodicity

-	Samsung thinks that UEs not able to read SIB1 already assume the cell as barred. Furthermore there are no issues for UEs in connected mode.

From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

- Cell reselection enhancements

Proposal 2: Postpone the discussion on the mechanisms to prevent legacy UE from reselecting to a DL CE cell until RAN2/RAN1 concludes on whether link level enhancements are backward compatible or not.

- SMTC aspects

  - Whether to configure different SMTC periodicity for neighbour cells with different SSB periodicity.

Proposal 3: Ask RAN4 whether to support separate per SMTC periodicity configuration for SMTCs in smtc4list-r17, considering both idle/inactive and connected mode.

-	Google is not sure this is essential

-	HW generally supports this at least for connected mode and thinks that RAN2 can decide on this. LG agrees. Ericsson also agrees

-	CATT thinks this would have RAN4 impact on RRM requirements. QC agrees

-	Samsung thinks there is no need for this for idle mode.

-	Fujitsu thinks the enhancements should apply for both connected and idle mode

-	Google thinks this can be solved by UE implementation

-	Samsung thinks the NW could provide neighbour cells SSB periodicity. Huawei thinks the current behaviour is per frequency, not per cell and then does not like this solution.

RAN2 considers to introduce signalling support for different SMTC periodicity and offset in the same frequency layer, for both idle and connected mode. Ask RAN4 for confirmation

Continue in offline 303 to discuss the wording of a possible LS to RAN4. Also discuss other possible SMTC enhancements for connected/idle mode

  - Whether to increase the size of SMTC list beyond 4

Proposal 4: The list size of SMTC is not extended.

  - Enhancement to SMTC mechanism

Proposal 5: No enhancement on SMTC mechanism is needed.

  - LS to RAN4

Proposal 6: send LS to RAN4 regarding the agreement on SMTC for confirmation.

- Link level coverage enhancements

Proposal 7: If msg4 PDSCH repetition is agreed to support, Msg3 is used to carry the indication of msg4 PDSCH repetition capability/request.

Proposal 8: RAN2 to agree on adding the following codepoints:

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE;

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK & Msg4 PDSCH repetition of an eRedCap UE;

Proposal 9: RAN2 to agree on not adding the following codepoints:

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition for an eRedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

and send LS to RAN1 to confirm the RAN2 understanding that Msg4 PDSCH channel is always better than PUCCH with Msg4 HARQ-ACK channel.

[AT129bis][303][R19 NR NTN] SMTC enhancements (Xiaomi)

Scope: discuss the wording of a LS to RAN4 regarding RAN2 intention to signalling support for different SMTC periodicity and offset in the same frequency layer, for both idle and connected mode. Also discuss other possible SMTC enhancements for connected/idle mode.

Intended outcome: summary of the offline discussion and Draft LS

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for offline discussion summary and Draft LS:  Thursday 2025-04-10 08:00

R2-2503055	Report of [AT129bis][303][R19 NR NTN] SMTC enhancements	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 considers to take the following option for support for different SMTC periodicity and offset in the same frequency layer for both idle, inactive and connected mode, and asks RAN4 whether it is feasible to support this in Rel-19 time frame and provide feedback on the maximum number of periodicities, offsets, and together:

Option 1: support configuring different SMTC periodicities for SMTCs in one frequency layer.

RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode.

We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

Proposal 2: Network can provide assistant information on the association between SMTC and location for UE to perform SMTC selection for idle/inactive mode. FFS on the details of location, e.g. serving cell SSB index, reference location, cell direction indication, etc.

We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

R2-2503056	Draft LS on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Revised in R2-2503067

R2-2503067	Draft LS on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Add “, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases” in the introduction

Remove “but at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases.” From the first agreement

Change e.g. 6 to i.e. 6

Remove Draft

Revised in R2-2503068

R2-2503068	LS on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Approved

R2-2502524	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 discusses SMTC enhancements due to beam hopping for UE in all RRC states, and try to address this issue with a common solution if applicable.

Observation 3: Location and beam based SMTC selection achieve similar purpose of limiting UE used SMTC number, while beam based SMTC mechanism requires less signalling overhead and is more power saving friendly.

Observation 4: Opt3 (UE assistance information report) only works for connected mode, while different solutions are still needed for UE in idle/inactive mode for SMTC updates.

Observation 5: Original Opt4 assumes a common SMTC (e.g., same periodicity and offset) for all neighbors while UE measures each neighbor in different time instance which is only applicable when beam hopping mechanism is regular and periodically rotated among neighbor cells.

Observation 6: The same purpose of opt4 can be achieved with a more generic solution, i.e., time-based SMTC mechanism (opt4a), where a list of SMTC with different validity time to be used by UE in different time instance can be independently configured.

Observation 7: If UE based SMTC selection in connected mode is used, additional considerations on measurement gap/NW scheduling restrictions are needed.

Observation 8: For time-based SMTC selection, there is no issue for NW to provide the measurement gap configuration or make scheduling accordingly.

Observation 9: For beam based SMTC selection, there is no issue for NW scheduling restriction but the measurement gap configuration needs to configured in a beam specific method to match with the beam-specific SMTC configuration.

Observation 10: For location based SMTC selection, UE needs to report back the used SMTC in connected mode, which consumes more signalling overhead and delays in updating measurement gap for matching with the used SMTC at UE’ side.

Observation 11: Beam or time-based SMTC selection mechanism allows a unified solution for UE in all RRC states while the impact on measurement gap configuration in connected mode is more manageable.

Proposal 2: RAN2 performs down-selection of SMTC enhancements for all RRC states based on below options:

	Opt 2: Beam-based SMTC selection, where UE selects SMTCss based on used beams associated to SMTCs

	Opt4b: Time-based SMTC selection, where UE selects SMTCs based on measured time and the validity time of SMTCs.

Proposal 3: If beam-based SMTC selection is considered for connected mode, RAN2 considers enhancements on measurement gap configuration ( e.g., introducing beam-sepcific measurement gap configuration).

R2-2502678	DL coverage enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	RAN2 to discuss and clarify whether different SSB periodicities and SSB offsets are expected in the same frequency layer.

Proposal 2	Introduce a way to define 4 SMTCs with different periodicity and offset for a frequency layer.

Proposal 3	Discuss whether the new definition should be backwards compatible with pre-Rel-19 NTN UEs.

Proposal 4	Inter-frequency SMTC enhancements are not needed to support 160 ms default SSB periodicity.

Proposal 5	RAN2 to explore common solutions for RRC_IDLE/INACTIVE and RRC_CONNECTED so that the UE can be configured with the most appropriate measurement gaps and SMTCs for its current location.

Proposal 6	Cell DTX enhancements based on Rel-18 NES are not within the scope of Rel-19 downlink coverage enhancements.

R2-2502863	Discussion on Downlink Coverage Enhancements	CSCN	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: SSB index associated SMTC configuration at the cell level can be introduced for SMTC enhancement.

Proposal 2: SSB-specific SMTC configuration can be introduced to beneficial both UE and NW for measurement.

Proposal 3: If SSB-specific SMTC applied, UE should reacquire the SMTC configuration when SSB index changes.

Observation 1: Due to the SSB periodicity extension and beam hopping, gap configuration may be insufficient for DL-CE cells.

Proposal 4: RAN2 is recommended to discuss the method for accurate gap configuration for DL-CE cells.

Proposal 5: The gNB shall implement different configurations for UEs in different areas to avoid invalid configurations.

Proposal 6: A new NTN bar bit with the value {barred, notBarred} should be introduced to achieve selectively bar UEs supporting DL-CE.

Proposal 7: RAN2 may discuss on supporting beam level DTX and the UE behaviors in different period.

R2-2502513	DL coverage enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Network enables Msg4 PDSCH repetition via RSRP threshold configuration in SIB19.

-	Ericsson thinks this is also being discussed in RAN1

Proposal 2: As baseline, introduces extra LCID code points when the LX field is set to 1. UE indicates its capability and/or request on Msg4 PDSCH repetition by transmitting Msg3 with those LCID code points:

•	Case 1: If UE has the capability of Msg4 repetition and the measured UE’s DL reference signal strength is lower than RSRP threshold, UE indicates its capability and request for Msg4 PDSCH repetition.

•	Case 2: If UE has the capability of Msg4 repetition and the measured UE’s DL reference signal strength is higher than RSRP threshold, UE does not indicate its capability and/or request for Msg4 PDSCH repetition.

•	Case 3: If UE does not have the capability of Msg4 repeition, UE does not indicate its capability and/or request of Msg4 repetition.

Proposal 3: RAN2 aims to support joint indication to request for both Rel-18 UL coverage enhancement of PUCCH repetition for Msg4 HARQ-ACK and Rel-19 DL coverage enhancement of Msg4 PDSCH repetition.

-	Ericsson thinks this is also being discussed in RAN1

Come back in the next meeting after further RAN1 progress

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

R2-2501774	Discussion on DL Coverage in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2501804	Discussion on downlink coverage enhancement	LG Electronics Inc.	discussion	Rel-19

R2-2501842	Discussion on downlink coverage enhancement	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2501974	NTN downlink coverage enhancements	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

R2-2502038	Discussions on downlink coverage enhancement	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502048	Discussion on downlink coverage enhancements in NR NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502057	Further discussion on downlink coverage enhancements	CATT	discussion	Rel-19

R2-2502072	Discussion on NR NTN downlink coverage enhancements	DENSO CORPORATION	discussion	NR_NTN_Ph3-Core

R2-2502195	Discussion on cell barring for NR NTN downlink coverage enhancements	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502246	Further discussion of NR NTN coverage enhancement	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502315	Downlink coverage enhancement for NTN	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2502328	Discussion on DL coverage enhancement for NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502352	Further considerations on NR NTN DL-CE	Lenovo	discussion	Rel-19

R2-2502377	Discussion on Downlink Coverage Enhancements	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502397	Discussion on supporting location/time-based SMTC selection	ITRI	discussion	NR_NTN_Ph3-Core

R2-2502493	SMTC impacts due to NTN downlink coverage enhancements	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502502	Downlink coverage enhancement	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502629	Downlink coverage enhancements and different SMTCs	TOYOTA Info Technology Center	discussion	Rel-19

R2-2502652	Default extended SSB periodicity	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502667	Discussion on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502739	Analysis on DL coverage enhancements due to extended SSB periodicity	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502839	Discussion on DL coverage enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502870	Discussion on DL coverage enhancements	TCL	discussion	Rel-19

R2-2502947	Further discussion on NTN DL coverage enhancements	NERCDTV	discussion	Rel-19

### 8.8.3	Uplink Capacity/Throughput Enhancement

Contributions can be submitted on the possible RAN2 aspects of the agreements reached in RAN1.

R2-2502329	Discussion on Uplink Capacity Enhancement	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	OCC related parameters, at least including OCC index and OCC length are indicated by NW.

Proposal 2	For CG type 1, the OCC related configuration for PUSCH should be configured to UE via ConfiguredGrantConfig.

Proposal 3	For CG type 2 and dynamic grant, how to configure the OCC related configuration for PUSCH can wait for R1 progress.

Proposal 4	Define separate UE capabilities for OCC length 2 and OCC length 4 and the UE supporting the OCC length 4 shall support OCC length 2.

-	Ericsson thinks that for all the proposals in this AI we can wait for a RAN1 LS.

-	Xiaomi agrees. Samsung as well.

R2-2502525	Consideration on uplink capacity enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 designs higher layer signalling to provide OCC configuration for both DG and type 1/2 CG. Detailed OCC parameters (e.g., OCC sequence index) needed can wait for more input from RAN1.

Proposal 2: If OCC configurations are provided in RRC signalling:

- For DG, PUSCH-Config is used

- For type 1 CG, configuredGrantConfig is used

- For type 2 CG, configuredGrantConfig is used, except for DCI relevant configurations (if any), which is included in PUSCH-Config.

Proposal 3: Confirm RAN1 agreements that separate UE capabilities are introduced for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

R2-2502856	Discussion on UL Capacity and Throughput Enhancement	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Introduce separate UE capabilities for OCC with length 2 and OCC with length 4. The UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. The exact UE capability description can be finalized based on the RAN1 LS on feature list.

Proposal 2: The OCC sequence length and OCC sequence index should be indicated to UE via RRC and/or DCI. The exact solution can wait for further progress from RAN1.

Proposal 3: There is no need to introduce dedicated access control mechanism for OCC capable UEs.

-	HW agrees with p3. Xiaomi, Fujitsu and Oppo also agree

There is no need to introduce dedicated access control mechanism for OCC capable UEs

R2-2502699	Discussion on uplink capacity/throughput enhancement for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Kindly suggest RAN2 to discuss the access control mechanism for R19 UEs supporting OCC between the two options:

Option 1: introduce new bar bit in SIB1

Option 2: reuse the existing cellBarredNTN.

Proposal 2: Introduce 2 new UE capabilities to support OCC length 2 and OCC length 4 respectively, for OCC length 4, simultaneously supporting OCC length 2 is mandatory.

Proposal 3: It is proposed to introduce a new IE (e.g. occ-v19) including OCC parameters (e.g. occ-length and occ-index ) in PUSCH-Config to provide OCC related configuration for PUSCH as following:

R2-2502284	Discussion on Uplink Capacity/Throughput Enhancement for NTN	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2502612	Discussion on Uplink Capacity Enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Agreements;

1.	There is no need to introduce dedicated access control mechanism for OCC capable UEs

### 8.8.4	Support of Broadcast service

Contributions should address the signaling of the intended service area of a broadcast service.

R2-2502668	Discussion on Broadcast Service Area	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

- UE skipping MCCH acquisition

Proposal 1: Intended service areas of all MBS broadcast services of the current serving cell are included in the new SIBxx.

RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

Proposal 2: If UE is not in any intended service areas indicated in SIBxx, UE does not need to acquire MCCH.

If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

Observation 1: In case the intended service area of an MBS broadcast service covers the whole current serving cell, UE does not need to know the exact geographic area covered by the current cell, but only needs to know there are certain MBS broadcast service intended for the whole cell by reading the new SIBxx, and no matter where the UE locates in the cell, the UE acquires MCCH.

Proposal 3: In case the intended service area of an MBS broadcast service covers the whole current serving cell, discuss how to signal in SIBxx.

•	Option 1: signal the geographic area of the whole serving cell.

•	Option 2: indicate the corresponding intended service area by a one-bit flag.

If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

- Service continuity

Proposal 4: The new SIBxx includes ISAs of MBS broadcast services that are provided not only on the intra frequency but also on inter frequencies.

Proposal 5: For cell reselection, if UE can only receive an MBS broadcast service(s) by camping on a frequency and UE is in the ISA of that MBS broadcast service, UE may prioritize that frequency.

Proposal 6:  For cell reselection, if UE cannot receive an MBS broadcast service on a frequency or if UE not in the ISA of an MBS broadcast service which is provided on a frequency, UE may deprioritize that frequency.

Observation 2: In quasi-earth-fixed or earth-moving cells, the MBS broadcast service to an intended area is for a quite short duration due to satellite movement, which will lead to frequent updates of MBS broadcast service and the associate ISA and, thus, UE frequent acquisitions of SIB or MCCH.

Proposal 7: For a quasi-earth-fixed or earth-moving cell, NW can provide ISA for MBS broadcast services that are to be provided by the upcoming satellite(s) and indicate the time interval during which the service is going to broadcast.

- ETWS service area

Observation 3: ETWS primary notification SIB6 without segmentation can still include up to 367 octets for warning area coordinates, which is sufficient to signal a warning area to some accuracy extend. If needed, a more accurate warning area can be further signaled in ETWS secondary notification SIB7 with segmentation.

Proposal 8: The field warningAreaCoordinatesSegment is included in SIB6/SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

-	NTT Docomo supports this

The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

Proposal 9: The size of the included field warningAreaCoordinatesSegment should be ensured to fit into SIB6 message which has up to 2976 bits.

-	NTT Docomo thinks that CT1 is already discussing this

R2-2502946	Discussion on MBS broadcast over NTN	Huawei, HiSilicon, China Southern Power Grid, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 discuss the following enhancements to allow UE to skip unnecessary MCCH acquisition:

Alt.1: The UE decides whether it is within the ISAs belonging to the serving cell, according to the new SIB or the SIB20, upon entering a new cell. If not, the UE does not perform MCCH acquisition. Besides, if there is new SIB update or the SIB20 update showing that the new add-in areas cover the UE’s location, the UE needs to perform MCCH acquisition.

Alt.2: The UE reads the MCCH when entering a new Cell. The UE decides to perform MCCH re-acquisition only when the MCCH change notification has 1 bit used to indicate that the mapping between the MBS session and ISA has changed.

Alt.3: The UE reads the MCCH when entering a new Cell. The UE outside the ISAs of interested MBS session monitor the MCCH change notification every new time interval introduced in SIBx, instead of every MCCH modification period.

Proposal 2:  Both current serving cell and neighbour cells MBS session specific intended service area ID(s) are indicated in the MCCH.

Proposal 3: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

Proposal 4: SIB21 can indicate the subset of ISAs used for each frequency and the UE needs not prioritize (RRC_IDLE) or indicate (RRC_CONNECTED) the frequency of an MBS service of interest if the UE is not in any of the ISAs used in the frequency that provides the service.

R2-2502039	Discussions on supporting broadcast service	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: Behaviours with respect to the initiation of MII procedure upon entering or leaving the area of broadcast service has already been defined.

Proposal 1: RAN2 confirms that, with respect to the MBS service, the term ‘broadcast service area’ refers to the ‘intended service area’.

Proposal 2: UE follows legacy specification to initiate MII procedure when it enters or leaves MBS intended service area.

Proposal 3: UE may skip monitoring MCCH when it is not in any of the ISAs in the new SIB.

Proposal 4: UE monitors changes of MCCH information from PDCCH when it is not in any ISA of interested MBS services but in at least one of the ISAs in the new SIB.

R2-2502700	Considerations on broadcast service for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: It could be up to UE implementation to monitor or skip MCCH (re)acquisition based on the ISA included in the new SIB and its own location information.

Proposal 2: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 3: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

-	Nokia thinks it’s not so simple and might have impacts to other groups. Ericsson shares the same concerns.

-	CATT also has some concerns, also regarding the signalling overhead.

-	Lenovo supports this and thinks we can use the intended service area ID in SIB21. Apple and vivo agree

-	ZTE also supports this.

Continue in offline 304 (Apple)

Proposal 4: For the MBS Interest Indication initiation, we could just follow the legacy mechanism, further enhancement is not needed.

Proposal 5: For SIB6, introduce a new IE(e.g. warningAreaCoordinates) to provide a full warning area coordinates information.

Proposal 6: For SIB7, reuse warningAreaCoordinatesSegment in SIB8 directly.

[AT129bis][304][R19 NR NTN] Service continuity (Apple)

Scope: discuss the proposal to provide intended service area IDs for each MBS broadcast service session in SIB21, or other alternative approaches to support service continuity

Intended outcome: summary of the offline discussion

Deadline for companies' feedback:  Wednesday 2025-04-09 20:00

Deadline for offline discussion summary:  Thursday 2025-04-10 08:00

R2-2503057	Report of [AT129bis][304][R19 NR NTN] Service continuity	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1:

WA: UE may prioritize the frequency for an interested service when UE can receive the service on the frequency and the UE is in intended service area associated with that frequency/service. Otherwise, UE may no longer prioritize the frequency.

FFS whether and how the association between intended service area and frequencies is acquired.

-	Nokia and Ericsson could be ok if we remove frequency

-	ZTE suggests to change to “the service provided in the frequency”

UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency.

- FFS whether and how the association between intended service area and frequencies is acquired.

R2-2501775	Remaining Issues on MBS Broadcast in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2501843	Discussion on the support of broadcast service	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502041	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502044	Discussion on the support of broadcast service in NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502058	Further discussion on support of broadcast service in NR NTN	CATT	discussion	Rel-19

R2-2502064	Further Discussion on Support of MBS Broadcast Service	TCL	discussion

R2-2502247	The signaling design of service area for PWS and MBS	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502353	MBS broadcast service continuity in NR NTN	Lenovo	discussion	Rel-19

R2-2502354	Further considerations on ETWS support in NR NTN	Lenovo	discussion	Rel-19

R2-2502376	Remaining issues on intended service area	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502514	Discussion on broadcast service over NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502526	Consideration on broadcast service ehancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502537	Discussion on the support of broadcast service	Xiaomi	discussion

R2-2502551	Remaining Aspects of MBS in Rel-19 NR NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502651	MBS broadcast service area information	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502677	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502741	Discussion on support for broadcast service in NTN	LG Electronics Inc.	discussion

Agreements:

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

### 8.8.5	Support of regenerative payload

Contributions, if any, should focus on the needed updates for Stage 2 description and on whether any other existing essential features (not considered so far) would be affected - and potentially need any modifications - in a regenerative payload architecture.

R2-2502885	Regenerative payload	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Clarify that the 38.300 figure in 16.14.2.1 is for transparent payload.

Proposal 2	Consider the text proposal in section 4.

Stage 2 rapporteur is invited to submit a 38.330 CR to the next meeting including the RAN3 endorsed changes for regenerative payload so that we can check which additional changes might be needed.

Proposal 3	Specific configurations of common TA and Kmac in regenerative architecture are not captured in Stage 2.

Specific configurations of common TA and Kmac in regenerative architecture are not captured in the specs.

R2-2502494	Satellite switch with re-sync in regenerative payload	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2502630	Regenerative payload for NTN for NR Ph3	TOYOTA Info Technology Center	discussion	Rel-19

Agreements:

1.	Specific configurations of common TA and Kmac in regenerative architecture are not captured in the specs.

### 8.8.6	LTE to NR NTN mobility

Contributions, if any, should focus on any possible missing aspects for the support of idle mode mobility between LTE and NR NTN.

R2-2501776	Remaining Issues on LTE to NR NTN Mobility	vivo	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

Proposal 1: In case the satellite ID is not present in the redirection information for NR NTN and SIB33 is broadcast, the UE considers the cells on the frequency to be terrestrial cells.

Proposal 2: In case the satellite ID is present in the redirection information for NR NTN, but there is no corresponding satellite information for the satellite ID in SIB33, it is up to UE implementation whether to perform NTN neighbour cell measurements on the NTN cell.

Proposal 3: UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15.

-	Xiaomi thinks there is no need to capture this in the specs.

-	ZTE supports this and this is related to the proposal in CATT’s paper

RAN2 understands that UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15 (no need to capture this in the spec)

R2-2502056	Discussion on a remaining issue for LTE_TN_NR_NTN_mob WI	CATT	discussion	Rel-19

Proposal: RAN2 discusses whether/how to support the dedicated priority configuration for E-UTRA TN to NR NTN cell reselection, and down-selects from below options:

- Option #1: Introduce a new capability signaling specific for E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability reported from the UE (based on NW implementation).

- Option #2: Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

- Option #3: No UE capability signaling is needed to indicate support of E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives supported NR NTN band(s) indicated in UE capability signaling. FFS whether any clarification is needed in the Spec.

-	QC supports option 2. Apple also.

Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

R2-2502690	Remaining issues on E-UTRAN to NR NTN mobility	Samsung	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

Proposal 1: The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

Agreed

Proposal 2: RAN2 confirms that the following aspects apply to redirection to NR NTN (without specification impact):

-	AS security must be activated to redirect to NR

-	MPS redirection to NR NTN

Agreements:

1.	RAN2 understands that UE supporting the ntn-RedirectionNR-r19 shall also indicate the support of supportedBandListNR-SA-r15 (no need to capture this in the spec)

2.	Extend the capability signaling for E-UTRA TN to NR NTN redirection to also indicate the support of IRAT E-UTRA TN to NR NTN cell reselection. E-UTRAN that broadcasts NR satellite assistance info in SIB33 may configure dedicated priority for an NR NTN frequency, only if it receives this capability common to E-UTRA TN to NR NTN redirection and cell reselection (based on NW implementation).

3.	The redirection to NR NTN information element is enclosed in an NTN specific CarrierInfoNR, which is introduced in the choice structure as nr-19.

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243278)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Incoming LSs

R2-2501720	Reply LS on PWS support in RRC_CONNECTED for NB-IoT NTN (R1-2501613; contact: InterDigital)	RAN1	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2, RAN

Noted

R2-2501753	Reply LS on Satellite IDs for store-and-forward operation (S2-2502450; contact: CICT Mobile, CATT)	SA2	LS in	Rel-19	5GSAT_Ph3-ARC	To:RAN2	Cc:RAN3

Noted

Running CRs

R2-2502887	Draft Introduction of IoT NTN phase 3	Ericsson	draftCR	Rel-19	36.300	18.4.0	B	IoT_NTN_Ph3-Core

-	Xiaomi and Nokia has some concerns with naming the feature as CB-EDT as also normal EDT is based on RACH. ZTE agrees

-	Ericsson and QC thinks that referring to CB-msg3 is also not good and prefer CB-EDT or something similar

-	CATT thinks that if we carefully defined what CB-EDT is there should be no problem. HW agrees

We tentatively refer to the new procedure as CB-msg3-EDT in the specs (can come back)

To be reviewed in post meeting email discussion

R2-2501969	RRC Running CR for IoT NTN	Huawei, HiSilicon	draftCR	Rel-19	36.331	18.5.0	B	IoT_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2501979	Running CR for TS36.304 for IoT-NTN	Nokia 	draftCR	Rel-19	36.304	18.3.0	B	IoT_NTN_Ph3-Core

To be reviewed in post meeting email discussion

R2-2502768	MAC Running CR for Rel-19 IoT NTN	MediaTek Inc.	draftCR	Rel-19	36.321	18.4.0	B	IoT_NTN_Ph3-Core	R2-2501158

To be reviewed in post meeting email discussion

Agreements;

1.	We tentatively refer to the new procedure as CB-msg3-EDT in the specs (can come back)

[Post129bis][309][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][310][R19 IoT NTN] RRC CR (Huawei)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][311[R19 IoT NTN] MAC CR (Mediatek)

Scope: discuss the running MAC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][312][R19 IoT NTN] 36.304 CR (Nokia)

Scope: discuss the running 36.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][313][R19 IoT NTN] capability CR (Qualcomm)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

### 8.9.2	Support of Store & Forward

Contributions should focus on possible impacts to the radio interface.

R2-2502053	Discussion on RAN2 impacts for the support of S&F operation	CATT	discussion	Rel-19

- On MME-configured satellite list: Cell (re)selection enhancement

Observation 1: SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, there is some room left for the UE to decide which S&F satellite to access and it is specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the MME-configured satellite list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempted to a satellite not in the MME-configured list).

Proposal 1: During cell reselection procedure, UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list.

-	Google supports this. CMCC supports this but thinks this would apply to UEs supporting S&F

-	IDC thinks we don’t have cell specific priorities but only frequency priorities. Also what does it means that the UE prioritizes. IDC also thinks that NB-IoT does not support priority based cell reselection.

-	Nokia does not support this and thinks the observations do not refer to cell reselection

-	QC thinks p1 and p1a are in contradiction (p1 relates to MME configured satellite list)

-	Novamint supports the principle of p1 and p1a

-	Samsung wonders if there will be many satellites the UE can prioritize

-	Xiaomi thinks p1 makes sense if there are only S&F mode cells around.

Come back in the next meeting (the proponents could show the possible spec impact if we decide to go in this direction)

Proposal 1a: A frequency list and/or cell list associated with each satellite ID (indicated by the MME) are provided to the UE in system information.

- On MME-configured satellite list: Relaxation on IDLE mode tasks

Proposal 2: RAN2 discusses whether the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.), if the UE determines that it is out of coverage of all target satellite(s) indicated by MME. How the UE determines out of coverage of all target satellites is up to UE implementation.

Observation 4: To support the power saving operation for S&F operation in Proposal 2, satellite assistance information similar to that used for discontinuous coverage (i.e., footprint info/start serving time and the long-term valid ephemeris format (TLE set format)) needs to be provided by system information.

Proposal 2a: If Proposal 2 is agreed, introduce a separate list in system information to provide the satellite assistance information for S&F operation (i.e., used by UE to predict the coverage time of the satellites indicated by MME).

Proposal 2b: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

- On transition time

Proposal 3: The UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. No other AS impact is expected.

-	Apple supports this but thinks we should also indicate the other direction. QC also agrees on the need for indication in the opposite direction.

-	vivo is ok with this. Nokia also

For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

Proposal 4: Send LS to CT1, informing CT1 of RAN2 agreements related to the "S&F operation mode" to "normal mode" transition time. Take the draft LS in Annex as baseline.

-	HW is fine but thinks we should add additional info on why RAN2 agreed to introduce this

Agreed

Proposal 5: Do not introduce transition time to indicate when the current satellite operation mode will transit from normal operation to S&F operation.

-	Google thinks we should have such an indication redefining the time information we already agreed. Oppo agrees.

-	Lenovo also agrees we need such an indication

-	Samsung thinks that when we move to S&F there will be a system information update

-	Samsung thinks we should not reuse T-service for this. Nokia agrees. ZTE also

-	Honor thinks we need a new indication, different from T-service.

-	Samsung wonders what we use this for.

We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS, and we inform CT1 about this in the LS

- On new release cause

Proposal 6: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the feeder link becomes unavailable.

-	vivo thinks we could ask CT1

-	Xiaomi supports p6

-	Samsung also supports p6

We don’t introduce any new release cause related to S&F operation

Proposal 7: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC_IDLE because the service link becomes unavailable when the cell is operating in S&F mode.

R2-2502116	Store and Forward Cell Access Restrictions	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3: Introduce a flag to indicate whether t-Service represents the remaining time of normal mode before switching to S+F mode, or whether it represents the end of service (per legacy).

R2-2501818	Remaining issues on S&F operation in IoT NTN	ZTE Corporation, Sanechips	discussion	IoT_NTN_Ph3-Core

Proposal 1: Besides the agreed time information on transition from S&F operation mode to Normal mode, a duration of Normal mode could be provided to R19 UE, e.g., via SystemInformationBlockType31(-NB) message.

Proposal 2a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-ServiceStartNeigh in SystemInformationBlockType33(-NB) or t-ServiceStart in SystemInformationBlockType32(-NB) either the time when the neighbor satellite starts to operate in Normal mode or the time when the neighbor satellite starts serving the area.

Proposal 2b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Proposal 2c: It’s suggested to introduce a new time information when a neighbor satellite transits from S&F operation mode to Normal mode for each neighbor satellite in SystemInformationBlockType33(-NB).

Proposal 3a: For quasi-earth fixed cell, it can be up to network’s implementation to set the t-Service in SystemInformationBlockType3(-NB) the time when the serving satellite starts to operate in S&F mode or the time when the serving satellite stops serving the area.

Proposal 3b: The earth moving cell case can be further discussed based on the progress for quasi-earth fixed cell case.

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

[Post129bis][301][R19 IoT NTN] LS to CT1 on S&F mode indications to NAS (CATT)

Scope: Draft an LS to CT1 with meeting agreements on indication to NAS regarding transitions between normal mode and S&F mode operation

Intended outcome: Approved LS

Deadline: 1-week

R2-2503070	LS on S&F mode indications to NAS	CATT	LSout	Rel-19	IoT_NTN_Ph3-Core	To:CT1

Discussed in [Post129bis][301]

R2-2501766	Discussion on Store and Forward operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501777	Further Discussion on S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501844	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502046	RAN2 impacts for SF Operation	Nokia , Nokia Shanghai Bells	discussion

R2-2502068	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502073	Discussion on Store & Forward operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2502101	On the support for the S&F operation	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502245	Further consideration of IoT NTN Store & Forward	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502355	Mode transition time for S&F operation	Lenovo	discussion	Rel-19

R2-2502426	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2502503	Radio Interface Aspect of S&F	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502515	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502613	Further consideration on Store and Forward	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502620	Discussion on Paging and Mode Switching	Toyota ITC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502622	CIoT UP solution for Store & Forward satellite operation	SHARP Corporation	discussion	Rel-19

R2-2502655	Switching of S&F mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502679	Support for store and forward in IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502685	IoT-NTN S&F Mode Change Indication	PANASONIC	discussion

R2-2502688	Further discussion on Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502698	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502769	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2501159

R2-2502805	Discussion on Store and Forward satellite operation	ETRI	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502845	Discussion on Store and Forward satellite operation	ETRI, Korea University	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2502805

R2-2502825	Discussion on time information for S&F	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2500418

R2-2502871	Discussion on Store & Forward satellite operation	TCL	discussion	Rel-19

### 8.9.3	Uplink Capacity Enhancement

Contributions should focus on the possible enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

Including outcome of email discussion [Post129][307].

R2-2502773	Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4	MediaTek Inc.	discussion

- CB-Msg3 configuration for both eMTC and NB-IoT

Proposal 1: RAN2 confirms that the start of DSA transmission window is aligned with the start of time domain resources for (N)PUSCH occasions.

-	ZTE supports this and p2 as well

The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

Proposal 2: RAN2 to discuss whether DSA window length could be defined in unit of PUSCH resources periodicity.

Continue in the Post meeting email discussion/ next meeting

Proposal 3: For DSA transmission window, RAN2 to discuss whether the length and periodicity should always be the same. If yes, only one of the window length and window periodicity is needed (not both).

The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

Proposal 4: Send the LS to RAN1 on CB-Msg3 configurations for confirmation.

- CB-Msg3 configuration for eMTC

Proposal 5: For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

Agreed

Proposal 6: For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed, whether TDD related parameters are needed).

For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

We will not support TDD related parameters.

Proposal 7: For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether we should preclude CE mode B parameters, whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16, and locationCE-ModeB-r16 are needed, whether power parameters p0-UE-PUSCH-r19 and alpha-r19 should be modified, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of or shared frequency-domain resources)

For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

RAN2 assumes power ramping should be supported for CB-msg3-EDT (both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

Proposal 8: For eMTC, introduce PDSCH configuration in shared resources configuration. It includes frequency hopping (as in pur-PDSCH-FreqHopping) and maximum TBS (as in pur-PDSCH-maxTBS). Confirm with RAN1 on the detail parameters.

For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

Proposal 9: For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

Agreed

Proposal 10: Consider the TP in Annex A as a baseline for eMTC shared resources configuration. This could be further discussed in RRC running CR.

Agreed

- CB-Msg3 configuration for NB-IoT

Proposal 11: For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB. FFS whether this IE is needed in SIB22-NB for non-anchor carrier.

-	Xiaomi thinks we should not have a restriction for non-anchor carrier

For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

Proposal 12: For NB-IoT, introduce below physical later parameters in shared resources configuration as below:

•	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

•	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

•	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS parameter for 15kHz SCS is needed. FFS whether subcarriers are provided as a contiguous set.

•	MCS configuration for NPUSCH (as in npusch-MCS-r16). FFS parameter for 15kHz SCS is needed.

•	Power parameters (as in p0-UE-NPUSCH-r16 and alpha-r16). FFS whether update is needed.

•	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

•	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

•	NOTE: confirm with RAN1 is needed

For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

- •	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

- •	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

- •	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

- •	MCS configuration for NPUSCH (as in npusch-MCS-r16).

- •	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

- •	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

- •	NOTE: confirm with RAN1 is needed

Proposal 13: For NB-IoT, RAN2 assume that the periodicity of CB-Msg3 resource may be larger than H-SFN duration.

For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Proposal 14: Consider the TP in Annex B as a baseline for NB-IoT shared resources configuration. This could be further discussed in RRC running CR.

Agreed

- Msg4 monitoring window and RNTI for DSA

Proposal 15: RAN2 to discuss the following options:

•	Option 1: Single Msg4 monitoring window. It starts at the end of Msg3 transmission window plus UE-eNB RTT. FFS processing time is needed or not.

•	Option 2: Multiple Msg4 monitoring windows. Each starts at the end of transmission of each replica plus UE-eNB RTT. FFS processing time is needed or not.

Proposal 16: RAN2 to discuss the following options:

•	Option 1: Single RNTI for MSG4 monitoring

o	Option 1a – The RNTI is derived on the resource of DSA transmission window.

o	Option 1b – The RNTI is preconfigured/predefined.

•	Option 2: Multiple RNTI for MSG4 monitoring. Each RNTI is derived on the resource of each replica.

-	Session chair thinks it’s a bundle: if we go for option 1 for p15 we should go for option 1 for p16 (and viceversa)

-	Xiaomi thinks the false alarm issue is the most important one.

-	Samsung thinks that with option 2 the overall monitoring window is longer and this is the biggest issue

-	QC and Samsung think that option 1 is the one that allows multiplexing multiple UEs in msg4. Ericsson agrees.

-	ZTE thinks that msg4 multiplexing should be an optimization but not the driver for this discussion.

Continue in offline 305

R2-2503058	Report of [AT129bis][305][R19 IoT NTN] CB-msg3-EDT – phase1	Mediatek	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: If typical DSA window length is larger than RTT and the UE consider to start monitor window after 1st replica + RTT, the UL/DL conflict issue should be taken into account.

Observation 2: Single v.s. multiple RNTI seems tradeoff between MSG4 multiplexing and false alarm. The traffic load should be consider for this design.

Proposal 1: Single Msg4 monitoring transmission window and Single RNTI. The UE starts a Msg4 monitoring window in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS processing time). If the monitoring window is overlapping with replica, the UE prioritize the replica transmission.

For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

-	session chair suggests to limit the discussion on FFS whether

A) the Msg4 monitoring always starts at the end of Msg3 transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not) or

B) We allow the network to configure one of the two options:

1)	The Msg4 monitoring starts at the end of Msg3 transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

2) 	The Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time) and is restarted after each replica plus UE-eNB RTT (FFS NW/UE processing time).  If the Msg4 monitoring window is overlapping with replica, FFS if the UE prioritize the replica transmission or monitoring

-	Nokia, ZTE and CMCC think that option 2 should be considered

-	Apple thinks we should not specify two options.

-	Ericsson thinks we should go for the simpler option (A). ESA agrees

-	HW also supports option B

-	NEC thinks A is sufficient. Oppo agrees. Viasat also agrees

The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

- SA vs DSA

R2-2502099	Further consideration on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 7: DSA is an optional sub-feature for CB-Msg3.

Proposal 8: CB-Msg3 resources can be shared between SA-capable UEs and DSA-capable UEs that support CB-Msg3.

Proposal 9: RAN2 to capture the UE behavior for CB-Msg3 SA and CB-Msg3 DSA UEs separately.

R2-2502886	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 2	When DSA is supported, SA is a special setting of the DSA parameters.

R2-2502457	On procedures for contention-based Msg3	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 8: DSA is a mandatory feature of contention-based Msg3/Msg4.

Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

For CB-msg3-EDT, the Transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

- support of CB-Msg3 EDT for NB-IoT with 15kHz SCS and eMTC CE mode B / support of OCC

R2-2501767	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

- Applicability of CB-msg3 EDT

Proposal 3: Send LS to RAN1 asking whether CB-Msg3 EDT for NB-IoT with 15kHz SCS and eMTC CE mode B is supported or not in this release.

- CB-msg3 EDT with OCC

Proposal 4: RAN2 to not consider CB-msg3 EDT with OCC in this release.

-	MTK and Nokia support this, as this would require separate resources and because of the power imbalance issue. Ericsson agrees

-	QC thinks that if we separate resources there should be no problem. QC agrees with QC and we should support OCC.

R2-2502701	Further discussion on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Provide different RSRP ranges for CB-msg3 with and without OCC respectively. And only when corresponding RSRP condition is fulfilled, CB-msg3 combined with OCC could be used.

Proposal 2: Support dedicated signaling to configure the TA validation parameters (e.g. TAT timer at least) for 15kHz SCS NB-IoT and eMTC CE mode B.

R2-2502516	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Msg4 delivery:

Proposal 1: RAN2 do not pursue TA validation solution for NB-IoT with 15kHz SCS and eMTC CE mode B if RAN1 has no further input.

-	HW thinks this would not work without TA validation.

We don’t specify support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

R2-2502656	CB-Msg3 and Msg4 enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	Separate resources are configured for CB-Msg3 transmission using OCC.

If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

Agreements;

1.	Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

2.	We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

3.	For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

6.	If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

18.	For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Agreements (part 2):

1.	For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

2.	The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

3.	The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

4.	FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

R2-2502771	Discussion on CB-Msg3 procedure	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2501164

- TA validation

Proposal 1: RAN discusses the following options regarding the TA validation.

Option 1: NB-IoT with 15kHz SCS and eMTC CE mode B are precluded from the scope of CB-Msg3.

Option 2: All type of NB-IoT and eMTC UE are supported in CB-Msg3

Option 3: RAN2 send LS to RAN1 asking for progress.

- Details of DSA mechanism

Proposal 2: DSA transmission is an optional feature without UE capability signaling for UE supporting CB-Msg3.

Proposal 3: If UE only supports SA, the DSA configuration can be reused.

Proposal 4: The maximum TBS is specific to CE level.

Proposal 5: The DSA transmission window length is specific to CE levels.

- Msg4 monitor window

Proposal 6: The Msg4 response window configuration is specific to CE level.

Proposal 7: Multiple Msg4 monitoring windows is supported. Each window starts at the end of transmission of each replica plus UE-eNB RTT (FFS whether additional processing time is needed).

- CB-Msg3 RNTI

Proposal 8: Send LS to RAN1 to confirm that CB-Msg3 RNTI is used to scramble the contention-based Msg3.

Proposal 9: Multiple Msg4 monitoring windows. Each RNTI is derived on the resource of each replica.

Proposal 10: RAN2 confirm the working assumption that one CB-Msg4 can target multiple UEs simultaneously.

Proposal 11a: The Msg4s responding to the Msg3s from the same time domains, but the different frequency domains can be multiplexed in the same MAC PDU.

Proposal 11b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for CB-Msg3 RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Proposal 12a: For NB-IoT, CB-Msg3 transmission on non-anchor carrier shall be supported.

Proposal 12b: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for CB-MSg3 RNTI calculation.

- Details of Msg4

Proposal 13: A Msg4 for CB-Msg3 with only MAC CE(s) is allowed when RRC message is not mandatory.

Proposal 14: Do not support the use of L1 ACK as the Msg4 for the CB-Msg3.

Proposal 15: RAN2 to discuss whether and how the HARQ ACK/NACK is applied to CB-Msg4.

Proposal 16: For NB-IoT, if the Msg4 multiplex is used, the same subcarrier index used to transmit the Msg3 can be used to transmit the HARQ ACK/NACK of Msg4.

- Fallback to RRC Connected mode

Proposal 17: CB-Msg3 fallback to RRC connected mode is supported.

Proposal 18: A TAC MAC CE can be included in the Msg4 when fallback to RRC Connected mode.

Proposal 19: UE starts the time alignment timer when receiving Msg4 which indicates fallback to RRC Connected mode.

Proposal 20: New C-RNTI should be included in the Msg4 for the CB-Msg3 when fallback to RRC Connected mode.

- Fallback to 4-Step RA

Proposal 21: UE can initiate the legacy 4-step RA when the  CB-Msg3 procedure fails.

Proposal 22: New MAC CE in Msg4 is introduced to indicate the corresponding UEs to stop current CB-Msg3 and initiate legacy 4-step RA.

Proposal 23: A new backoff MAC CE in Msg4 can be introduced when UE fallback to 4-step RA as indicated by the network.

Proposal 24: RAN2 to discuss whether the CE level for 4-step RA starts from the beginning or from the CE level used in CB-Msg3.

- Re-attempt

Proposal 25: Re-attempt in the same CE level due to contention resolution failure is supported.

Proposal 26: Parameter for maximum re-attempt number per CE level is introduced. UE can re-attempt in the same CE level until the max re-attempt number has been reached.

Proposal 27: Backoff MAC CE in Msg4 is supported for re-attempt.

Proposal 28: Power ramping parameters are optionally configured by the network.

R2-2502516	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Msg4 delivery:

Proposal 6: For NB-IoT, time domain factor (SFN at least) and carrier_id of CB-Msg3 EDT resources are considered in the RNTI derivation.

Proposal 7: The frequency domain factor (f_id) of CB-Msg3 EDT resource and OCC index can be  explicitly indicated in MAC subheader for Msg4.

Proposal 8: For eMTC, RAN2 to discuss whether frequency domain info is considered in RNTI calculation (RAN1 confirmation is needed).

Proposal 9: Msg4 should be able to comprise of all or a subset of the following parts:

1) Contention resolution indication on successfully received Msg3 from a certain UE (confirmed last meeting);

2) RRC message part in response to the RRC message contained in received Msg3;

3) MAC subheader with Backoff indicator to other UE(s) whose Msg3 transmission do not succeed.

Proposal 10: Confirm the working assumption that one CB-MSG4 can target multiple UEs simultaneously, assuming one MSG4 RNTI can associate with multiple PUSCH occasions.

Proposal 11: UE stops monitoring Msg4 RNTI after receiving Contention resolution Identity, no matter RRC message is received or not. It is network responsibility to assign a UE specific C-RNTI to UE if network would like to continue DL transmission to the UE.

Proposal 12: RAN2 to discuss whether to support multiplexing of multiple Msg4-RRC messages for more than one UE(s).

[AT129bis][305][R19 IoT NTN] CB-msg3-EDT (Mediatek)

Scope:

Phase 1: discuss single v.s. multiple RNTI/window design

Phase 2: discuss remaining CB-msm33-EDT issues (mainly based on R2-2502771) and LS to RAN1 on CB-msg3-EDT shared resource configuration

Intended outcome: summaries of the offline discussion and Draft LS to RAN1

Deadline for companies' feedback (for phase2):  Thursday 2025-04-10 20:00

Deadline for offline discussion summary (for phase2) and Draft LS:  Friday 2025-04-11 09:00

R2-2503059	Report of [AT129bis][305][R19 IoT NTN] CB-msg3-EDT – phase2	Mediatek	discussion	Rel-19	IoT_NTN_Ph3-Core

Easy - 1

Proposal 1: The DSA configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specified.

The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

Proposal 2: Msg4 response window configuration is CE level specified.

The Msg4 monitoring window configuration (e.g. length) is CE level specific

Proposal 4: RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

Easy - 3

Proposal 8: RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT.

•	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

•	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

Agreed

RAN2 also intends to support CB-msg3-EDT for MT cases

Proposal 10: The C-RNTI MAC CE shall be included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4.

The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

Potential easy - 1

Proposal 3: Introduce one RNTI (e.g. CB-Msg3-EDT RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. FFS whether we send LS to RAN1 for confirmation on RAN1 SPEC impact.

Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

Proposal 5: A CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution. FFS the CB-Msg4 structure.

-	Samsung thinks we need an RRC message

Come back in the next meeting

Proposal 9: The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

Agreed

Potential easy - 2

Proposal 11: Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

Agreed

Proposal 12: Backoff information could be included in CB-Msg4.

Agreed

Proposal 13: UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. FFS on the condition for the UE to initiate the fallback to legacy 4-step RA.

-	Ericsson thinks this should be formulated in a way that failure of the procedure is indicated to upper layers. MTK agrees. ZTE wonders which UL

Continue in the next meeting

May need some discussion (R1 related?)

Proposal 6: L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

Agreed

Proposal 7: HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how

the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

Agreed

R2-2503060	Draft LS on CB-msg3-EDT shared resource configuration	Mediatek	LS out	Rel-19	IoT_NTN_Ph3-Core	To:RAN1

Revised in R2-2503175

R2-2503175	LS on CB-msg3-EDT shared resource configuration	Mediatek	LS out	Rel-19	IoT_NTN_Ph3-Core	To:RAN1

Discussed in [Post129bis][302]

[Post129bis][302][R19 IoT NTN] LS to RAN1 on CB-msg3-EDT (Mediatek)

Scope: Draft an LS to RAN1 with meeting agreements CB-msg3-EDT

Intended outcome: Approved LS

Deadline: 1-week

Agreements (part3):

1.	The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific.

2.	The Msg4 monitoring window configuration (e.g. length) is CE level specific

3.	RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

4.	RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

5.	RAN2 also intends to support CB-msg3-EDT for MT cases

6.	The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI)

7.	Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

8.	The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4.

9.	Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached.

10.	Backoff information could be included in CB-Msg4.

11.	L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported.

12.	HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

R2-2501767	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501778	Discussion on CB-EDT Mechanism	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501794	Further considerations on Locating of CB-Msg3 Replicas for DSA	NTPU	discussion

R2-2501819	Further consideration on UL capacity enhancements in IoT NTN	ZTE Corporation, Sanechips	discussion	IoT_NTN_Ph3-Core

R2-2501845	Discussion on UL capacity enhancement	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501966	Further consideration on UL capacity enhancement	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502054	Further consideration on UL capacity enhancements	CATT	discussion	Rel-19

R2-2502069	Discussion on CB-msg3 EDT and msg4 enhancement	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502099	Further consideration on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502117	CB-EDT	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502356	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2502402	Discussion on UL Capacity Enhancement for IoT-NTN	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502427	Discussion on Uplink Capacity enhancement	Transsion Holdings	discussion	Rel-19

R2-2502428	Remaining issues on CB-Msg3 transmission	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502457	On procedures for contention-based Msg3	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502656	CB-Msg3 and Msg4 enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502701	Further discussion on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502886	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

### 8.9.4	Support of PWS

Contributions should focus on the introduction of support for broadcast of PWS messages for NB-IoT, re-using the LTE mechanisms.

R2-2502680	Enhancements to support PWS in NB-IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

- Enhancements to eDRX

Proposal 1	Investigate ways to improve the reception of PWS messages for NB- NTN UEs in challenging scenarios (e.g., eDRX).

- SIB1-NB acquisition

Proposal 2	A UE interested in receiving PWS may indicate its interest to the network to adjust the assigned power saving configuration.

Proposal 3	RAN2 studies ways for allowing an NB-IoT UE to skip SIB1-NB acquisition to speed up the delivery of the ETWS primary notification.

- PWS notification reception in RRC_CONNECTED

Proposal 4	Support PWS delivery in RRC_CONNECTED for NB-IoT UEs without impact in other WGs.

Proposal 5	RAN2 to discuss how to provide the PWS notification in RRC_CONNECTED and down select among the following possibilities:

a.	Introduce a scheduling gap to obtain the PWS notification.

b.	Introduce time-critical information in dedicated signaling.

c.	Acquire system information before executing the actions to be released.

d.	Skip the 10 seconds delay upon receiving an RRCRelease message.

-	Google supports b

-	QC wonders if any option should be considered

-	Samsung thinks the RAN plenary agreement is not to support PWS in connected mode. Xiaomi agrees. Nokia agrees. Oppo agrees (no need for any change)

-	HW thinks we can just release to idle possibly with a release cause.

-	CMCC supports b

-	Google thinks that sending the UE back to idle is not the good solution.

-	Sasmung thinks it’s possible to skip the 10 seconds delay.

In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

- Reception of PWS segments when switching serving cell

Proposal 6	Provided it is served by the same gNB, the UE should be allowed to assemble warningMessageSegments received from different cells/satellites.

Proposal 7	The network can indicate whether the UE should discard or continue using the previously buffered warningMessageSegments upon cell/satellite switch.

R2-2501983	Remaining issues on PWS support for NB-IoT	Huawei, HiSilicon, Novamint, Sateliot, Thales, Inmarsat, Viasat, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal: To deliver PWS messages to UEs currently in RRC_CONNECTED, the network may release the UEs to RRC_IDLE and instruct the UEs to read the PWS SIB(s) immediately after transitioning to RRC_IDLE.

R2-2502770	Discussion on supporting PWS for NB-IoT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2501161

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

Proposal 2: The network will release the UE in connected mode to acquire PWS.

Proposal 3: New release cause PWS notification is used.

R2-2502458	Acceptable cell camping for NB-IoT	Samsung, Iridium, Vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: A UE shall be able to camp on an NB-IoT NTN cell for the purpose of receiving emergency broadcast, without the cell being a part of UEs PLMNs.

Proposal 2: RAN2 to discuss introducing acceptable cell category for NB-IoT NTN.

Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

R2-2501768	PWS support for NB-IoT over NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501779	Further Discussion on PWS Support for NB-IoT	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2501820	Remaining issues on PWS support in IoT NTN	ZTE Corporation, Sanechips	discussion	IoT_NTN_Ph3-Core

R2-2502055	Remaining issue on support of PWS for NB-IoT NTN UE	CATT	discussion	Rel-19

R2-2502070	Discussion on PWS for NB-IoT	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502102	Remaining issues on support of PWS for NB-IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502103	On the support for PWS in NB-IoT	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502357	Further considerations on PWS broadcast support in IoT NTN	Lenovo	discussion	Rel-19

R2-2502657	Discussion on PWS in NB-IoT NTN	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2502702	Support of PWS messages for NB-IoT	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

## 8.17	IoT-NTN TDD mode

(IoT_NTN_TDD; leading WG: RAN1; REL-19; WID RP-243293)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdoc

Workplan

R2-2501988	Work plan for WID: introduction of IoT-NTN TDD mode	Iridium Satellite LLC	Work Plan	Rel-19	IoT_NTN_TDD

Noted

R2-2501780	Discussion on RAN2 Imapcts of IoT-NTN TDD mode	vivo	discussion	Rel-19	IoT_NTN_TDD-Core

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

R2-2502059	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19

R2-2502071	Discussion on IoT NTN TDD mode	OPPO	discussion	Rel-19	IoT_NTN_TDD

R2-2502100	Discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

R2-2502193	Discussion on support of IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD-Core

R2-2502358	Discussion on TDD support in IoT NTN	Lenovo	discussion	Rel-19

R2-2502459	On RAN2 aspect of IoT NTN TDD	Samsung	discussion	Rel-19	IoT_NTN_TDD

R2-2502517	Discussion on supporting IoT NTN TDD mode	Apple	discussion	Rel-19	IoT_NTN_TDD

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

R2-2502538	Discussion on the IoT NTN TDD mode	Xiaomi	discussion

R2-2502560	On SI scheduling, H-SFN change and postponing impacts in IoT-NTN TDD mode	Nordic Semiconductor ASA	discussion	Rel-19

R2-2502621	Indication of IoT-NTN TDD Mode Support	Toyota ITC	discussion	Rel-19	IoT_NTN_TDD

R2-2502658	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

R2-2502703	Support of IoT-NTN TDD mode	CMCC	discussion	Rel-19	IoT_NTN_TDD

- Paging

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: RAN2 discusses and decides which of the following options is used for Paging occasion determination:

Option 1: New paging DRX cycle with multiple 9 radio frames and radio frame offset for PF determination are used to determine the PF and PO accurately.

Option 2: Legacy NB-IoT PO determination mechanism is used. When the determined paging subframe#n is not a valid downlink subframe, the PO is postponed to the nearest valid downlink subframe#n.

-	Iridium prefers option 2 and thinks it would be easier for chipset implementations. Nordic and QC agree. QC thinks the same applies to a number of other open issues

-	vivo thinks we should align to other procedures discussed in RAN1 where there is the option to drop rather than postpone some occasions. Nokia thinks we can decide in RAN2. HW agrees with Nokia.

Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

-	Cell barring

R2-2501989	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19	IoT_NTN_TDD

Proposal 13: In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

Agreed

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: The IE cellBarred in the SIB1-NB message can be reused to control whether a UE supporting NB-IoT NTN TDD mode is allowed to access an NB-IoT TDD NTN cell.

- Timers value range extension

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 4: RAN 2 confirms that the existing value range of timers with unit of pp are enough for IoT NTN TDD.

-	QC wonders if we need to take into account that invalid subframes need to be considered. Xiaomi thinks the issue of invalid subframes is not specific of IoT NTN but NB-IoT in general and this is not considered a problem in NB-IoT.

Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes)

-	PUR transmission

R2-2502059	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19

Proposal 5: For SPS/PUR, RAN2 discusses whether to capture in the MAC Spec that UE skips an SPS/PUR occasion if it is a non-UL slot.

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 2: When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the latest UL subframe for PUR transmission.

-	HW thinks we should align the behaviour for PUR and SPS. Vivo agrees.

-	Xiaomi also would like to have a unified solution to postpone the transmission

When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

- UL SPS

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal 2a: To reduce the overlap of UL SPS with non-U NB-IoT subframes, new SPS periodicities should be introduced to align with the TDD structure.

-	CMCC supports this

-	Xiaomi thinks this cannot avoid collisions and then we might not need this

Proposal 2b: If UL SPS transmission can’t be finished within a TDD pattern, the remaining repetitions can be postponed to the NB-IoT UL subframes of the following TDD patterns.

R2-2502527	Consideration on IoT-NTN TDD mode	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 8: UL SPS transmission is dropped when the UL SPS overlaps with non-U NB-IoT subframes

When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

- SI reception

R2-2501780	Discussion on RAN2 Imapcts of IoT-NTN TDD mode	vivo	discussion	Rel-19	IoT_NTN_TDD-Core

Proposal 4: Introduce new periodicities based on the time unit of the TDD frame and/or the hyper TDD frame to decide the SI window and SI transmission resource.

R2-2501970	Discussion on RAN2 impacts of IoT-NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Proposal 7: For the SI reception within an SI-window, UE should prioritize the previous SI reception in case there is collision between the previous SI repetitions and the subsequent SI reception.

R2-2501989	Discussion on IoT-NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19	IoT_NTN_TDD

Proposal 5: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Proposal 6: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

Proposal 7: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

R2-2502059	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19

Proposal 2: RAN2 discusses whether to capture in RRC Spec (system information procedure) that if a subframe is within the SI window as determined by the current Spec but is a non-DL subframe, UE skips the reception of the corresponding SI thereon.

R2-2502560	On SI scheduling, H-SFN change and postponing impacts in IoT-NTN TDD mode	Nordic Semiconductor ASA	discussion	Rel-19

Proposal-3: The scheduling procedure of other SI messages remains unchanged.

Proposal-4: Postponing of other SI subframes to the next 90ms TDD frame is supported.

Proposal-5: A network configuration shall ensure SI repetitions within a SI-window never overlap.

Proposal-6: Other SI transmission that overflows a configured SI-window shall be truncated.

- k-mac

R2-2502193	Discussion on support of IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD-Core

Proposal 7	Support k-Mac-r19 with a value range of 1 – 1023 ms

For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

Can further discuss extending this proposal to IoT NTN and NR NTN in the next meetings

R2-2502100	Discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

Proposal 4: RAN2 to confirm that extending the Kmac beyond 512 ms will block the cell for legacy release 17 and 18 NTN UEs.

Proposal 5: Further details about the scenarios requiring an extended Kmac should be provided to justify the Kmac value extension.

Agreements:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

## 8.18	TEI19

Time budget: 1 TU

Tdoc Limitation: 1 tdoc for new proposals and 1 tdoc for old proposals.

1 additional tdoc for primary co-sourcing company on top of the limit is allowed for co-sourced contribution with 4 or more companies.

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#129bis.

LTE TN to NB-IoT NTN mobility

R2-2501781	Introduction of LTE TN to NB-IoT NTN Mobility UE Capability	vivo, Samsung, Google, THALES, MediaTek Inc.	draftCR	Rel-19	36.306	18.4.0	TEI19

R2-2502803	36331CR for the inclusion of NB-IoT satellite information in E-UTRAN	Google, Samsung, vivo, THALES	draftCR	Rel-19	36.331	18.5.0	B	TEI19

R2-2502804	36300CR for the inclusion of NB-IoT satellite information in E-UTRAN	Google, Samsung, vivo, THALES	draftCR	Rel-19	36.300	18.4.0	B	TEI19

R2-2501971	ETWS geo-fencing for eMTC NTN and for TN	Huawei, HiSilicon, China Southern Power Grid, Turkcell	discussion	Rel-19	TEI19

R2-2502689	Redirection to IoT NTN and NR NTN	Samsung	discussion	Rel-19	TEI19

All contributions in this AI will be treated in the next meeting

## 8.19	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

R2-2501718	Reply LS on Ku band numerology (R1-2501609; contact: Eutelsat)	RAN1	LS in	Rel-19	NR_NTN_Ku_bands-Core	To:RAN4	Cc:RAN, RAN2

Noted

# Summary

In-Principle Agreed CRs

R17 IoT NTN (LTE_NBIOT_eMTC_NTN)

R17 NR NTN (NR_NTN_solutions-Core)

R2-2502519	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.331	17.12.0	5303	-	F	NR_NTN_solutions-Core

R2-2502520	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-17	38.306	17.12.0	1255	-	F	NR_NTN_solutions-Core

R2-2502521	Corrections to location-based measurement initiation	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	38.306	18.5.0	1256	-	A	NR_NTN_solutions-Core

R18 IoT NTN (IoT_NTN_enh-Core)

R2-2503061	Correction on SIB33 reception in RRC_CONNECTED	Huawei, HiSilicon	CR	Rel-18	36.331	18.5.0	5105	1	F	IoT_NTN_enh-Core

R2-2503062	Corrections to location-based measurement	ZTE Corporation, Ericsson, CATT, Sanechips	CR	Rel-18	36.300	18.4.0	1417	1	F	IoT_NTN_enh-Core

R18 NR NTN (NR_NTN_enh-Core)

R2-2501782	Correction on NTN in FR2-NTN bands	vivo, ZTE Corporation, Ericsson, Sanechips	CR	Rel-18	38.306	18.5.0	1200	1	F	NR_NTN_enh-Core	R2-2409544

R2-2503066	Clarification on ssb-TimeOffset	Huawei	CR	Rel-18	38.331	18.5.0	XXXX	1	F	NR_NTN_enh-Core

R2-2502669	Correction to Rel-18 NR NTN CHO with only location/time-based trigger	Samsung	CR	Rel-18	38.331	18.5.1	5314	-	F	NR_NTN_enh-Core

R2-2502670	Correction to Rel-18 NR NTN CHO with only location/time-based trigger	Samsung	CR	Rel-18	38.306	18.5.0	1263	-	F	NR_NTN_enh-Core

Approved LSs out

R2-2503068	LS on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

R2-2503069	Reply LS on soft satellite switch with re-sync	Huawei	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN4

[Post129bis] Email discussions

Short

[Post129bis][301][R19 IoT NTN] LS to CT1 on S&F mode indications to NAS (CATT)

Scope: Draft an LS to CT1 with meeting agreements on indication to NAS regarding transitions between normal mode and S&F mode operation

Intended outcome: Approved LS

Deadline: 1-week

[Post129bis][302][R19 IoT NTN] LS to RAN1 on CB-msg3-EDT (Mediatek)

Scope: Draft an LS to RAN1 with meeting agreements CB-msg3-EDT

Intended outcome: Approved LS

Deadline: 1-week

[Post129bis][303][TEI18] LS to RAN4 on NR NB-IoT in-band operation (Ericsson)

Scope: Draft a reply LS to RAN4 on NR NB-IoT in-band operation

Intended outcome: Approved LS

Deadline: 1-week

Long

[Post129bis][304][R19 NR NTN] Stage2 CR (Thales)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][305][R19 NR NTN] RRC CR (Ericsson)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][306][R19 NR NTN] MAC CR (Interdigital)

Scope: discuss the running MAC CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][307][R19 NR NTN] 38.304 CR (ZTE)

Scope: discuss the running 38.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][308][R19 NR NTN] capability CR (Apple)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][309][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: discuss the running Stage 2 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][310][R19 IoT NTN] RRC CR (Huawei)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][311[R19 IoT NTN] MAC CR (Mediatek)

Scope: discuss the running MAC CR

Intended outcome: Endorsed CR and list of open issues

Deadline: long

[Post129bis][312][R19 IoT NTN] 36.304 CR (Nokia)

Scope: discuss the running 36.304 CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][313][R19 IoT NTN] capability CR (Qualcomm)

Scope: discuss the running capability CR

Intended outcome: Endorsed CR

Deadline: long

[Post129bis][314][LTE to NR NTN] RRC CR (CATT)

Scope: discuss the running RRC CR

Intended outcome: Endorsed CR

Deadline: long