3GPP TSG-RAN WG2 Meeting #129bis	R2-2501775

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.8.4

Source:	vivo

Title:	Remaining Issues on MBS Broadcast in NTN

Document for:	Discussion and Decision

- Introduction

In the RAN2#129 meeting, the following agreements were made for MBS broadcast within intended service area [1]:

1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

With all the achieved agreements, the Rel-19 MBS broadcast function in NTN is almost ready. In this contribution, we would like to provide our consideration of some remaining issues.

- Discussion

  - 2.1	Signalling details

In the previous meeting, it was agreed to include the network-indicated area ID list in MBS broadcasat MCCH message. Firstly, we would like to give some more details on the signalling design. The spontaneous thinking is to include the area ID into the MBS broadcast session configuration IE (MBS-SessionInfo-r17). Unfortunately, no extension marker is reserved. So there is no way to include the area ID in MBS-SessionInfo-r17.

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

Instead, we can put the area ID list into MBSBroadcastConfiguration-r17-IEs. Specifically, we can include the area ID list with the same number of entries, and listed in the same order, as in MBS-SessionInfo-r17. The first element in the new area ID list corresponds to the first broadcast session included in the first entry of MBS-SessionInfo-r17. Then the second one corresponds to the second broadcast session, and so on. Meanwhile, as we agree that one broadcast session can be associated with multiple area IDs,  each entry of the area ID list should be able to indicate one or more than area IDs. RAN2 should discuss the maximum number of area IDs that can be associated with one broadcast session.

Last but not least, it is possible that one broadcast service is supposed to be received within the cell while some others are supposed to be received within the associated service intended area. For that service for the whole cell area reception, the network should just make the corresponding entry in the area ID list empty. With that, the UE assumes the broadcast service can be received within the whole cell.

Proposal 1: The service area ID list is included in MBSBroadcastConfiguration-r17-IEs of broadcast MCCH by using the non-critical extension way.

Proposal 2: The service area ID list includes the same number of entries, and listed in the same order, as in mbs-SessionInfoList-r17 (i.e. the first element in service area ID list corresponds to the first broadcast session in mbs-SessionInfoList-r17)

Proposal 3: RAN2 discusses the maximum number of area IDs associated with one broadcast session (e.g. 64).

Proposal 4: If a broadcast session is supposed to be received within the whole cell area, the associated element in the service area ID list should be empty.

An example of MCCH ASN.1-level siganlling modification is given below for reference.

MBSBroadcastConfiguration message

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  MBSBroadcastConfiguration-v19xy-IEs                                                           OPTIONAL

}

MBSBroadcastConfiguration-v19xy-IEs ::= SEQUENCE {

mbs-SessionAreaList-r19      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionAreaIdList-r19                                         OPTIONAL,   -- Need R

nonCriticalExtension             SEQUENCE {}             OPTIONAL

}

MBS-SessionAreaIdList-r19                   SEQUENCE (SIZE (1..maxSession-AreaInfo-r19)) OF ServiceAreaId-r19

ServiceAreaId-r19 ::=  INTEGER (1.. maxServiceArea-r19)

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

  - 2.2	UE behavior on MCCH (re)acquisition

With the network siganlling as mentioned above, UE can interpret the intended service area for the MBS broadcast session via MCCH (re)acquisition. That is when the UE becomes interested in MBS broadcast services, it will apply the MCCH information acquisition procedure. For the Rel-19 NTN case, we think the same behavior can be reused, i.e., UE acquires the MCCH when entering the cell providing SIB20 after becoming interested in MBS broadcast services and re-acquires the MCCH upon the MCCH information has changed.

In the past meetings, there raised a view that we should consider MCCH skipping for the power-saving purpose. For example, after the UE acquires the new SIB, if the UE is not in any of the indicated areas, the UE can skip acquiring the MCCH. It makes no sense to us. On one hand, the indicated area in the new SIB may only used for SIB6, but not broadcast. On the other hand, the network may currently provide a service for the whole cell coverage, which is not linked to any indicated service area. The UE should, as Rel-17 behavior, acquire MCCH and check whether the user is interested in the providing service or not. Blindly skipping the MCCH acquisition may lead to service reception disruption. Take one step back, if power matters, we might leave it to pure UE implementation to skip the MCCH reacquisition. For another example, the UE could use stored service area configuration from the previously received MCCH. And then it may skip re-acquiring the up-to-data MCCH due to PTM configuration changes of the ongoing sessions (i.e. the LSB of MCCH change notification field is set to ‘1’) when it is outside the service area, based on the hypothesis that the service area configuration/association is static in fixed satellite cell. Therefore, we propose,

Proposal 5: If the MCCH-based solution is adopted, RAN2 considers the following options,

Opt 1: Reuse the Rel-17 MCCH information (re)acquisition procedure, i.e., the UE attempts to acquire MCCH upon receiving a change notification for MCCH or becomes interested in receiving MBS broadcast;

Opt 2: It is up to UE implementation to use the service area info acquired in the received MCCH to avoid re-acquiring the MCCH when the UE is outside the service area.

  - 2.3	broadcast service continuity

In the existing MBS broadcast framework, for service continuity, the IDLE/INACTIVE UE may consider the frequency that providing the MBS broadcast service the UE interested in to be the highest priority or consider the frequency that not providing the MBS broadcast service to be the lowest priority. Additionally, the IDLE/INACTIVE UE may request unicast reception (thus triggering RRC setup/resume request procedure) of the broadcast service content before moving to a cell not providing the MBS broadcast service(s) via PTM mode, according to the neighbour cell list in MCCH. Regarding CONNECTED UE, it may send the MBS Interest Indication to the gNB, including the information on MBS frequencies/services UE is receiving or interested to receive and the priority between broadcast and unicast/multicast. With that the gNB may provide the UE with an RRC reconfiguration, select a proper target cell, and deprioritize unicast/multicast scheduling, to allow the UE to receive the MBS services the UE is interested in.

To guarantee service continuity in NTN case with the intended service area, it is worth investigating whether the existing solution is sufficient or not. For the frequency prioritization function of MBS broadcast, the following UE behaviours are captured in TS 38.304 and TS 38.300, respectively,

TS 38.304 sub-clasuse 5.2.4.1 Reselection priorities handling -------------------------<the unrelated part is omitted>----------------------- If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service. NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell. -------------------------<the unrelated part is omitted>----------------------- TS 38.300 sub-clasuse 16.10.6.5.1 Service Continuity in RRC_IDLE or RRC_INACTIVE -------------------------<the unrelated part is omitted>----------------------- In RRC_IDLE and RRC_INACTIVE, the UE applies the normal cell reselection rules with the following modifications: - the UE which is receiving or interested to receive MBS broadcast service(s) via PTM and can only receive these MBS broadcast service(s) via PTM while camping on the frequency providing these MBS broadcast service(s) is allowed to make this frequency highest priority when the conditions described in TS 38.304 [10] are met; - when the MBS broadcast service(s) which the UE is interested in are no longer available (after the end of the session) or the UE is no longer interested in receiving the service(s), the UE no longer prioritises the frequency providing these MBS broadcast service(s).

In the Rel-19 case, it seems a spontaneous logic that the UE may set one frequency associated with broadcast service provision with the highest frequency priority only when it will be in the intended service area after the cell reselection. Otherwise, there is no need to do this as the service, even though the user is interested in that, is not intended to be received. Specifically, if UE is receiving or interested in receiving an MBS broadcast service associated with the intended service area and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session when its location is within the intended service area of the corresponding session after cell reselection. And once the UE goes outside of the intended service area, the UE should no longer prioritize the frequency that providing the MBS broadcast service. In addition, to facilitate the above frequency prioritization behaviour, SIB21 should be allowed to include the service area information so that the UE can obtain the intended service area of the interested broadcast service that is not provided in the current cell.

Proposal 6: SIB21 can include a service area ID list for broadcast service continuity.

Proposal 7: UE may consider the frequency that providing the broadcast service to be the highest priority only when the UE is located within the service area of that interested broadcast service.

Proposal 8: UE no longer prioritizes the frequency that providing the MBS broadcast service when the UE is outside the service area of that interested broadcast service.

For the unicast reception request for the broadcast service content, since it is up to UE implementation to do this without stage-3 spec normative text, we think we can leave it to pure UE implementation in the Rel-19 case. For example, when the UE is moving to a cell not providing the MBS broadcast service and will continue to stay within the intended service area, it may trigger an RRC setup/resume request for the unicast reception of the broadcast service content. No further enhancement is needed for the neighbour cell list solution.

Proposal 9: No enhancement is considered for the MCCH neighbour cell list mechanism in Rel-19.

Last but not least, we would like to look at the MBS Interest Indication (MII) reporting mechanism. In TS 38.331, the following MBS Interest Indication reporting behaviour is specified for MBS broadcast,

5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

We can observe that the RRC_CONNECTED UE may initiate the MII reporting procedure upon entering or leaving the broadcast service area (i.e. cell/tracking area) based on the USD, as highlighted above. We think the same technical logic can be applicable to the Rel-19 case. That is the UE may report MII when entering or leaving the intended service area, as shown in Figure 1.

Specifically, for CONNECTED UE, MII can be used for scheduling prioritization handling between broadcast and unicast/multicast (e.g. if the UE prioritizes broadcast service(s) over unicast/multicast, the network may release some DRBs/multicast MRBs to save more radio bearer capability for broadcast reception). Then the CONNECTED UE should report it to the network when it goes inside the intended service area so that the network can do the proper scheduling prioritization handling. Furthermore, if the UE goes outside the intended service area, the UE should also indicate the MII since the network has to know it does not need to consider scheduling prioritization handling between broadcast and unicast/multicast anymore for this UE.

Figure 1: MII reporting when moving inside/outside the intended service area

Based on the above, we propose,

Proposal 10: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area indicated by AS signalling.

3 Conclusion

In this contribution, we have discussed the remaining issues on MBS broadcast in Rel-19 NTN. And we make the following proposals,

Proposal 1: The service area ID list is included in MBSBroadcastConfiguration-r17-IEs of broadcast MCCH by using the non-critical extension way.

Proposal 2: The service area ID list includes the same number of entries, and listed in the same order, as in mbs-SessionInfoList-r17 (i.e. the first element in service area ID list corresponds to the first broadcast session in mbs-SessionInfoList-r17).

Proposal 3: RAN2 discusses the maximum number of area IDs associated with one broadcast session (e.g. 64).

Proposal 4: If a broadcast session is supposed to be received within the whole cell area, the associated element in the service area ID list should be empty.

Proposal 5: If the MCCH-based solution is adopted, RAN2 considers the following options,

Opt 1: Reuse the Rel-17 MCCH information (re)acquisition procedure, i.e., the UE attempts to acquire MCCH upon receiving a change notification for MCCH or becomes interested in receiving MBS broadcast;

Opt 2: It is up to UE implementation to use the service area info acquired in the received MCCH to avoid re-acquiring the MCCH when the UE is outside the service area.

Proposal 6: SIB21 can include a service area ID list for broadcast service continuity.

Proposal 7: UE may consider the frequency that providing the broadcast service to be the highest priority only when the UE is located within the service area of that interested broadcast service.

Proposal 8: UE no longer prioritizes the frequency that providing the MBS broadcast service when the UE is outside the service area of that interested broadcast service.

Proposal 9: No enhancement is considered for the MCCH neighbour cell list mechanism in Rel-19.

Proposal 10: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area indicated by AS signalling.

4 Reference

[1] R2-2501702, RAN2#129 Meeting Report, MCC.