3GPP TSG-RAN WG2 Meeting #129bis	R2-2501814

Wuhan, China, 7th – 11th, April 2025

Source:	vivo

Title:	AIoT Data Transmission

Agenda Item:	8.2.4

Document for:	Discussion and Decision

# Introduction

According to the chair notes in RAN2#129[1], there are the following agreements regarding AIoT data transmissions and other general aspects:

Agreements on AS ID - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID. - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

Agreements on segmentation - To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient. - Segment retransmission is supported. - For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3 - R2D segmentation is not supported for R19 A-IoT.

Agreements on types of procedure From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

In this contribution, we will further discuss the issues regarding AIoT data transmission and other general aspects, including:

- Validity of AS ID

- D2R Segmentation

- MAC PDU format

# Discussion

# Validity of AS ID

Based on summary Report of [POST129][036][AIoT] AS ID (Xiaomi) as highlighted below box [2], there are still some unresolved issues:

Validity of AS ID Ph2-Proposal 9a (3): For validity of AS ID, exclude option 2 The device releases the AS ID upon timer expiry Ph2-Proposal 9b (3): For validity of AS ID, exclude option 4 The device releases the AS ID after completion of the command procedure Ph2-Proposal 9c-alt1: For validity of AS ID, RAN2 further down-selection among option 6 (11) and Combined option 1+3, taking into account of the pros/cons in the discussion, FFS on whether the AS ID is only valid in the same paging round, paging session, whether “the explicit indication is missing” is a rare case or not. Ph2-Proposal 9c-alt2: For validity of AS ID, RAN2 further down-selection among option 1(10), option 3 (7) and option 6 (11), taking into account of the pros/cons in the discussion, FFS on whether the AS ID is only valid in the same paging round, paging session, whether “the explicit indication is missing” is a rare case or not.

In this section, we further discuss the highlighted FFS issues and provide our views as follows.

Issue 1: FFS on whether the AS ID is only valid in the same paging round or paging session

Firstly, we think the motivation for the device to release the AS ID across different paging rounds within the same service (i.e., paging with the same transaction ID) is unclear. In our understanding, the AS ID, if assigned, should be retained throughout the same service since the precise timing for the reader to receive command(s) from the CN for a particular device is unpredictable and fluctuating, e.g., another command might come in the next paging round. Thus, allowing the AS ID to be consistent across various paging rounds with the same transaction ID can enhance the effectiveness of inventory and command procedures, meaning that the device can continue utilizing the assigned AS ID to monitor its R2D messages without initiating a new RACH procedure. Besides, the argument that a device lacking enough energy to continue monitoring should not hinder the support for maintaining the AS ID through various paging rounds. According to the Report of [POST129][036][AIoT] AS ID (Xiaomi), companies have consensus that “the device releases the AS ID upon power off (no specification impact)”,  implying that such a device without enough energy might release its AS ID upon powering off. For other devices that have enough energy to keep monitoring, efficiency in inventory and command procedures can be realized. Therefore,

- From the device perspective, the assigned AS ID is considered valid during the same paging session; the assigned AS ID becomes invalid only when a paging message with a different transaction ID is received compared to the last received one.

Issue 2: FFS on whether “the explicit indication is missing” is a rare case or not

Basically, we believe the situation where “The explicit indication is missing” primarily occurs in device mobility scenarios, such as when the device moves out of its serving reader’s coverage area. Moreover, device mobility has also been discussed for the real deployment and service requirement. According to 3GPP SA1 specification TS 22.369 [3], device speed is specified as 3km/h – 10km/h, see Table as highlighted below:

- According to SA1 TS 22.369, Device mobility scenario i.e., Device speed is considered for Inventory use case.

Besides, according to RAN1 coverage evaluations captured in TR 38.769 [4], the BLER target for R2D is 1%/10% as highlighted below. And based on all the evaluation results provided by company contributions, most companies give the typical BLER 10% as the design target.

From TR 38.769 ## 7.1 Coverage evaluations For an evaluation scenario: - For each link i, - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements if Budget-Alt2 is used for this link i. - Step 2: Obtain the receiver sensitivity using the method Budget-Alt1 (if a predefined threshold is assumed to derive the receiver sensitivity) or Budget-Alt2 (if no predefined threshold is assumed to derive the receiver sensitivity). See Clause 4.3.1 for the Budget definition. - Step 3: Obtain the coverage performance for link i based on the receiver sensitivity from step 2 and link budget template. - The coverage results for each link are provided. RAN1 has evaluated the coverage for InF-DH NLOS scenarios for D1T1 and InH-Office LOS/InF-DL NLOS scenarios for D2T2, across all devices 1/2a/2b. Coverage evaluation results comparing various R2D Tx power/EIRP value, R2D Rx sensitivity, BLER 1%/10% target, data rate ~1 kbps/~5-7 kbps, D2R coherent receiver/non-coherent receiver, R2D budget-Alt1 and Alt2, CW cancellation, backscatter loss and modulation schemes, D2R with/without channel coding schemes, etc, have been provided. The observations for coverage evaluation results can be found in Clauses 7.1.1.1.2, 7.1.1.2.2, 7.1.1.3.2, 7.1.2.1.2, 7.1.2.2.2, and 7.1.2.3.2. All the evaluation results are contained in the attached spreadsheet file [189].

- According to RAN TR 38.769, the BLER 1%/10% is captured as the R2D coverage design target, wherein the BLER 10% is more typical in company evaluation results.

Based on the above observations, we believe that “the explicit indication is missing” is a valid case, and thus the specification should address such a case. Last but not the least, the potential reception failure of R2D messages on the device side is not only applicable to the explicit indication message for AS ID release (i.e., Option 6), but also apply to other R2D messages like Paging (i.e., Option 1) or R2D message to assign new AS ID (i.e., Option 3).

- For AS ID release solution alternatives, besides the device possibly not receiving the explicit release indication (Option 6), failure may also happen with other R2D messages such as Paging (Option 1) or R2D message for assigning new AS ID (Option 3).

Therefore,

- For every candidate AS ID validity solution option, RAN2 to take into account the scenario of R2D message reception failure, not only for Option 6 i.e., “the explicit indication to release AS ID is missing”.

Issue 3: Further Down-selection for AS ID validity Options

As stated in the Report of [POST129][036][AIoT] AS ID (Xiaomi) [2], there are two candidate alternatives as the solution for validity of AS ID as below:

- Alt1: option 6 and Combined option 1+3

- Alt2: option 1, option 3 and option 6

Moreover, each option mentioned above is described as below:

- Option 1: The device releases the AS ID upon receiving Paging with same/new transaction id, i.e., same/different session/service

- Option 3: The device releases the AS ID upon new random ID is generated or receiving new assigned AS ID from the reader; Note: It will not lead new AS ID assignment option. FFS on whether the AS ID can be assigned at any time.

- Combined option 1+3: The device releases the AS ID upon receiving Paging with same/new transaction id, i.e., same/different session/service, and upon receiving a new assigned AS ID (follow AS ID assignment conclusion).

- Option 6: The device releases the AS ID upon receiving explicit release indication from the Reader

Firstly, regarding the controversial part of “Combined option 1+3” vs. “option 1 or option 3”, we believe that “Combined option 1+3” should be ruled out. The explanation is that, the 1st condition of “upon receiving Paging with same/new transaction id, i.e., same/different session/service” and the 2nd condition of “upon receiving a new assigned AS ID” occur in two different R2D messages. Thus, choosing either Option 1 or Option 3 suffices the purpose to release AS ID. “Combined option 1+3” seems over-optimization. Moreover, we have a slight preference for Option 1 because the motivation for Option 3 why a reader re-assign a new AS ID for the same device is vague. Currently we can assume that the AS ID for an individual device would be assigned just once and utilized until the inventory and command procedure is finished.

Secondly, we observe that the common part of “alt1” and “alt2” is Option 6 and suggest it to be considered as a baseline. Typically, the AS ID is explicitly released by the reader using Option 6, and Option 1 serves as a supplement to Option 6 when the scenario where “the explicit indication is missing” happens. Therefore,

- RAN2 to rule out Option 3 as well as Combined Option 1+3 for validity of AS ID.

- RAN2 to agree Option 1 (new transaction ID) and Option 6 as the overall solution for validity of AS ID.

- If Proposal 3&4 are agreed, then from the device perspective, the AS ID is released when one of the following conditions is met:

- upon receiving the explicit release indication message with the previously assigned AS ID; or,

- upon receiving a paging message with a different transaction ID compared to the last received one.

# D2R Segmentation

# Triggering of D2R segmentation

One basic issue is whether to predefine certain condition for AIoT device to determine to transmit the D2R message in segments or introduce certain allowance parameter in the R2D scheduling message to configure the AIoT device to transmit an D2R message in segments. To reduce overhead of the R2D scheduling message, it is preferred that the following policies can be predefined for the AIoT device:

- If the scheduled D2R resource is enough to carry the whole D2R message, the AIoT device should just transmit the whole D2R message without segmentation;

- Otherwise, i.e., if the scheduled D2R resource is not enough to carry the whole unsegmented D2R message, the AIoT device should segment the D2R message and transmit the D2R message segment by segment.

Therefore,

- The AIoT device performs segmentation when the received TBS excluding the MAC header size is not enough to accommodate the whole D2R message, i.e., no explicit segmentation allowance from the reader is needed.

# 1-bit indication and D2R message size report

In RAN2#129, there were the following agreements regarding the 1-bit indication and the D2R message size:

To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient.

According to the above agreements, there is an open issue on how to report the remaining D2R message size, i.e., whether to simply indicate the AIoT device has more data to be transmitted using single-bit indication or to indicate accurate remaining D2R message size using multiple bits. If only 1-bit indication is used, the reader cannot determine the accurate number of remaining D2R message bits. In this situation, the reader may over-allocate resources for the D2R end segment, which requires additional padding bits in the D2R MAC PDU. The potential number of padding bits could be much larger than the D2R message size report size (e.g.,1 byte) considering the maximum TBS can be up to ~1000 bits and the minimum size of the end segment of the D2R message can be 8 bit. In order to avoid large number of padding bits, it is preferred that the AIoT device report the accurate remaining D2R message size.

Then one further issue is whether a 1-bit indication is still needed if a remaining message size report is supported. One feasible design is that the 1-bit indication can be used as an end/non-end segment indication of the D2R message. The 1-bit indicator indicates the end segment in the MAC PDU comprising an unsegmented D2R message or the last segment of a segmented D2R message. When the reader receives the first D2R MAC PDU with an end segment indication without an associated remaining D2R message size report, the reader can determine that the received D2R MAC PDU comprises an unsegmented D2R message. In such a way, the overhead of the remaining D2R message size report (e.g., 8-bit) for unsegmented D2R message can be avoided. If the device determines that the D2R message should be segmented, the AIoT device includes the remaining D2R message size report and non-end segment indication in the D2R MAC PDU containing the first (non-end) segment. Upon reception of this D2R MAC PDU containing the non-end segment indication, the reader can know the D2R message is segmented and can further derive the remaining message size report accordingly. Further, there is no need for subsequent remaining D2R message size reporting. Therefore,

- Support 1-bit end/non-end segment indication.

- Support remaining D2R message size report from device to reader.

- The remaining D2R message size in relation to a D2R message is only reported in the MAC PDU comprising the first non-end segment of the D2R message.

Considering the maximum upper layer D2R message size is ~1000 bits, the remaining D2R message size should be below ~1000 bits. To reserve some margin for possible D2R message size variance, the maximum remaining message size range of 0~2048 (i.e.,2^11) bits can be defined. As the D2R message is an upper layer message which is byte aligned, the remaining D2R message size could also be reported in bytes.

- The remaining D2R message size report comprises an 8-bit field to indicate the number of remaining bytes of the D2R message.

Based on the preceding discussions regarding the D2R message transmission, the following content of a D2R MAC PDU for data transmission can be derived:

- The D2R MAC PDU for data transmission contains the following fields:

- Single-bit end/non-end segment indication;

- Remaining msg size report, if any;

- Length indication of an D2R MAC SDU/MAC SDU segment;

- An D2R MAC SDU or MAC SDU segment.

# Segment retransmission

In RAN2#129, there were the following agreements regarding segment retransmission of D2R message:

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

According to the above agreements, an offset indication will be included in the scheduling message for segment (re)transmission of D2R message, while the detail design of the scheduling message content/format for scheduling segment (re)transmission of a D2R message needs to be further discussed, for instance:

- Issue 1: Whether the scheduling message for every segment (re)transmission always carries the corresponding upper layer service request (i.e., MAC SDU);

- Issue 2: Whether the scheduling message for every segment (re)transmission always carry an offset indication.

For Issue 1, there are the following two alternatives:

- Alt 1: the upper layer service request (i.e., MAC SDU) triggering the D2R message is carried in the scheduling message for (re)transmission of each segment of the D2R message. Upon reception of the scheduling message for (re)transmission and offset indication (the offset indication may be included in every scheduling message or only in the scheduling message for subsequent segments after the first segment), the AIoT device simply generates this D2R message segment. In this way the AIoT device does not need to store the start/end location of the generated D2R message for subsequent segment. The cost is that the reader needs to repeatedly transmit the same upper layer service request to AIoT device in each scheduling message for the segments of the D2R message.

- Alt 2: the upper layer service request (i.e., MAC SDU) triggering the D2R message is only carried in the scheduling message for transmission of the first segment of the D2R message. Upon reception of the upper layer service request, the AIoT device generates the first segment and the subsequent segments. This means that the AIoT device does not store the generated D2R message for subsequent segment generation. Compared to Al1 1, the overhead to transmit the upper layer service request to AIoT device after the successful transmission of the first segment can be avoided. The cost is that the AIoT device needs to store the generated D2R message related information (e.g., start/end location of the D2R message), for subsequent segment generation after the first segment.

- RAN2 consider the following options of scheduling message design in relation to D2R message segment (re)transmission:

- Option.1: each scheduling msg for D2R segment carries the upper service request (i.e., MAC SDU).

- Option.2: only the scheduling msg for the first D2R segment carries the upper service request (i.e., MAC SDU).

For Issue 2, there are also two alternatives:

- Alt a: the scheduling message of D2R message always carry an offset indication for D2R message.

For this alternative, there is offset indication in every scheduling message for D2R message segments, including for non-end segments and for end segment. If the segmentation is needed for the D2R message, the AIoT device generates the segment using the offset indication. The reader indicates offset “0” for the first segment of the D2R message.

- Alt b: the scheduling message for the first segment of D2R message does not carry the offset indication while each scheduling message for subsequent segment (re)transmission carries the corresponding offset indication.

For this alternative, when the AIoT device receives a scheduling message without offset indication but with upper layer service request (i.e., MAC SDU), the AIoT device generates the D2R message and generates the first segment from the start location of the generated D2R message. In each scheduling message for subsequent segments of the D2R message, there is no upper layer service request but an offset indication. Upon reception of the scheduling message without upper layer service request but an offset indication, the AIoT device can determine to generate the segment based on the recently generated D2R message and the received offset indication. For this alternative, the offset indication in the scheduling message for the first segment of the D2R message and the upper layer service request (i.e., MAC SDU) in the scheduling messages for transmission of subsequent segments can be avoided. The cost is that the AIoT device should store the D2R message related information for subsequent segment generation.

- RAN2 down-select from the following two alternatives for the offset indication:

- Alt.a: the scheduling message always carries offset indication for D2R message.

- Alt.b: the scheduling message for the first segment of D2R message does not carry the offset indication but each scheduling message for subsequent segment transmission/retransmission carries an offset indication.

As the maximum message size for AIoT service is ~1000bits the offset indication value range can be 0 to ~1000 bits. Considering that the D2R message is byte aligned, the offset indication can also be in bytes instead of bits. Hence, the offset indication in the scheduling message can be defined as an 8-bit field in granularity of bytes.

- The offset indication for scheduling segment transmission/retransmission of a D2R message is 8-bit length in byte.

One further issue is to investigate the feasibility on whether the segment retransmission procedure can be reused for retransmission of an unsegmented D2R message. D2R failure of an unsegmented D2R message can be resolved follows:

- Case a: the reader transmits the upper layer service request to the AIoT device again to trigger the AIoT device to transmit the D2R message.

- Case b: the reader can schedule the retransmission of the D2R message, e.g., by reusing the segment retransmission procedure for unsegmented D2R message transmission.

As the reader cannot know whether the device received the R2D message or not, if the reader schedules the retransmission of the unsegmented D2R message by reusing the segment retransmission procedure while the AIoT device did not receive the corresponding upper layer service request, there can be mismatch between the device and reader. Thus, the segment retransmission procedure with offset indication should not be reused when the device fails to receive the first R2D message, as the reader has no mean to know whether the D2R message is segmented or not. For safety, the reasonable behaviour for reader is to transmit the service request again to trigger the AIoT device to transmit the D2R message.

- The segmentation retransmission procedure is not reused for retransmission of an unsegmented D2R message.

One more issue is whether to support segmentation for Message 1 of CFRA and Message 3 of CBRA. There are two following aspects:

- In one aspect, Message 1 of CFRA and Message 3 of CBRA are used to carry the upper layer device ID of AIoT device. According to Annex A of clause “8.2.1 Identifier and Identification Management” of the TR [7] in the following, the typical size of EPC (i.e., device ID) length is 96 bits. That is, device ID length is very small compared to the maximum message size (~1000 bits) and the benefit to segment a device ID is rather small.

From TR 23.700-13 As shown in Table A-3, the key idea is that a binary representation of an EPC starts with a header value that indicates the EPC scheme followed by the bitwise representation of a particular encoding scheme. For instance, SSCC-96 defines how the GS1 Company Prefix and the Serial Reference of the Serial Shipping Container Code are represented as a string of bits. The typical size of EPCs, e.g. for the Serialised Global Trade Item Number using the SGTIN-96 coding scheme is 96 bits (including the Header value). Table A-3: The EPC binary representation consists of the header value that identifies the EPC scheme and coding scheme specific bits, e.g. SSCC-96 for the Serial Shipping Container Code as defined in GS1 TDS Release 2.1 [10]. (Note: This table shows a subset only) EPC scheme; Header value; Coding scheme specific bits. Serial Shipping Container Code (SSCC); 0011 0001; SSCC-96 as defined in clause 14.6.2.1 of GS1 TDS Release 2.1 [10]. Serialised Global Trade Item Number (SGTIN); 0011 0000; SGTIN-96 as defined in clause 14.6.1.1 of GS1 TDS Release 2.1 [10].

- In another aspect, if segmentation of Message 1 of CFRA and Message 3 of CBRA is supported, one specific Msg2 format for CFRA and another specific Msg2 format for CBRA should be defined for scheduling segment transmission and retransmission, which means extra standardization effort and device complexity.

- Segmentation for Msg1 of CFRA or Msg3 of CBRA is not supported.

Based on the previous discussion, including the scheduling message for scheduling (re)transmission of D2R segments and that of unsegmented D2R message, the R2D MAC PDU content can be derived as the follows:

- The R2D MAC PDU for data transmission contains:

- Message type;

- Resource allocation parameters for the corresponding D2R message transmission;

- Offset indication for D2R message segment, if any;

- MAC SDU of service request from upper layer, if any.

# MAC PDU format

# General MAC PDU structure

The AIoT MAC PDU primarily consists of MAC control information and the payload. Firstly, as RAN2#129 has agreed that “Aim to design simple MAC PDU format design”, the AIoT MAC PDU format could just include MAC control information and a single SDU. The SDU is used to contain the payload. The MAC control information comprises the control/scheduling information, which can either be directly included in the AIoT MAC PDU or added to the MAC header, thus eliminating the need for the MAC CE concept. Secondly, we believe it is unnecessary to support multiple subPDUs. As AIoT devices lack support for multiple LCID, it is possible to bypass SDU concatenation; otherwise, the subPDU will incur extra overhead of subHeader and subPDU multiplexing.

- AIoT MAC Header comprises MAC control information. AIoT MAC PDU does not support MAC CE or SDU concatenation.

# R2D Msg Structure

Regarding the R2D messages, we discuss whether new R2D messages and padding are required. Based from insights provided in contributions [5] and [6], the following is a summary of AIoT R2D messages:

Table 2.3-1: overview of AIoT R2D messages

| # | Msg | Direction | MAC Header (Control Info) | MAC SDU |
| --- | --- | --- | --- | --- |
| 1 | Paging | R2D | Message type; Transaction ID; RA resource allocation information, e.g., Q-value. | Paging Identifier info |
| 2 | R2D Trigger | R2D | Message type; Transaction ID. | n/a |
| 3 | CBRA Msg2 | R2D | Message type; RN_16: 16bits; AS_ID; MSG3 resource allocation. | n/a |
| 4 | CFRA Msg2 | R2D | Message type; AS ID. | Upper layer data |
| 5 | R2D data | R2D | Message type; AS ID; Following D2R resource allocation; R2D Control info: payload size for PDRCH, Segmentation offset indication etc. | Upper layer data |
| 6 | NACK | R2D | Message type; AS ID: optional. | n/a |

Based on RAN2#129 agreement “At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s).”, R2D MAC PDU  includes the MAC Header along with the MAC SDU. The R2D MAC header includes MAC control information. An illustration of an AIoT R2D MAC PDU is shown in the figure below:

Figure 2.3-1: Example of an AIoT R2D MAC PDU

As for R2D trigger message, referring contribution [5], introducing a new R2D trigger message other than the paging message, e.g., QueryRep-like, is necessary.

- Introduce R2D trigger message other than the paging message, e.g., QueryRep-like message.

As discussed in contribution [6], we suggest to introduce MAC NACK msg for D2R transmission failure indication.

- Introduce a MAC NACK for D2R transmission failure indication.

Since the R2D message comprises message type, length for SDU and variable parts. the device can decode the R2D messages based on the message type, the length indication or post-amble. Then, the padding is not required for R2D MAC PDU.

- R2D MAC PDU does not comprise padding.

# D2R Msg structure

Based on our contribution [5] [6], below is the summary of AIoT D2R messages:

Table 2.3-2: overview of AIoT D2R messages

| # | Msg | MAC Header (Control Info) | MAC SDU, Padding(opt) |
| --- | --- | --- | --- |
| 1 | CBRA Msg1 | RN16 | n/a |
| 2 | CBRA Msg3 | Length of SDU | Upper layer payload, e.g. device ID; Padding(opt). |
| 3 | CFRA Msg1 | Length of SDU | Upper layer payload, e.g. device ID; Padding(opt). |
| 4 | D2R data | Length of SDU; Segmentation Indication; Remaining message size. | Upper layer data; Padding(opt). |

We will discuss whether for D2R we need message type field, any length and need for padding.

Msg Type: the D2R message type can be determined according to R2D context. Although adding msg type can make the AIoT procedure steps clearer, it anyway results in more signaling overhead. Hence, we suggest not need Msg Type for D2R. the D2R MAC Header may comprise MAC control information, e.g., length for SDU and variable part(s).

- The MAC header for D2R message comprises length for “SDU and variable part(s)”. Message type field is not required for D2R.

Padding: As per RAN1 agreement “The payload size (i.e., TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.”, the D2R Padding is required if the total size of MAC Header and MAC SDU is less than the transport block size indicated in the corresponding R2D control information. furthermore, Padding is placed at the end of MAC PDU without any subheader or length info.

One example of an AIoT D2R MAC PDU is shown as below:

Figure 2.3-2: Example of an AIoT D2R MAC PDU

- Padding is required for D2R MAC PDU if the total size of MAC Header and MAC SDU is less than the transport block size indicated in the corresponding R2D control information. Padding is placed at the end of MAC PDU without subheader or length info.

# Byte alignment

AIoT protocol stack architecture is a hierarchical structure, each layer, e.g., AIoT upper layer, AIoT MAC layer to be byte aligned, which is clear and convenient for maintain and implementation.

- The AIoT MAC PDU, MAC Header and MAC SDU should be byte aligned (i.e., multiple of 8 bits) in length.

# Msg Type coding

We should design the Msg Type coding short and future compatible. Based on the summary of R2D and D2R messages, 4-bit length of message type can provide a suitable space for required message types and the potential capability for future requirement.

- RAN2 supports 4-bit length of message type in R2D.

# Multiplexing

RAN2#129 agreed to “Support multiplexing of information for multiple devices in R2D message for msg2.” Other multicast messages need further study. In our understanding, whether supporting multiplexing for other R2D messages except msg2 relies on RAN1 discussion. RAN1 is discussing resource allocation and/or TDMA/FDMA for Msg1/Msg3 etc. According to RAN1 discussion, only need to consider multiplexing for Msg2.

- RAN2 assumes to only support multiplexing for Msg2.

# Conclusion

In this contribution, remaining issues on AIoT data transmission and other general aspects are discussed, and we have the following observations:

- According to SA1 TS 22.369, Device mobility scenario i.e., Device speed is considered for Inventory use case.

- According to RAN TR 38.769, the BLER 1%/10% is captured as the R2D coverage design target, wherein the BLER 10% is more typical in company evaluation results.

- For AS ID release solution alternatives, besides the device possibly not receiving the explicit release indication (Option 6), failure may also happen with other R2D messages such as Paging (Option 1) or R2D message for assigning new AS ID (Option 3).

Based the above discussions and observations, we have the following proposals:

AS ID

- From the device perspective, the assigned AS ID is considered valid during the same paging session; the assigned AS ID becomes invalid only when a paging message with a different transaction ID is received compared to the last received one.

- For every candidate AS ID validity solution option, RAN2 to take into account the scenario of R2D message reception failure, not only for Option 6 i.e., “the explicit indication to release AS ID is missing”.

- RAN2 to rule out Option 3 as well as Combined Option 1+3 for validity of AS ID.

- RAN2 to agree Option 1 (new transaction ID) and Option 6 as the overall solution for validity of AS ID.

- If Proposal 3&4 are agreed, then from the device perspective, the AS ID is released when one of the following conditions is met:

- upon receiving the explicit release indication message with the previously assigned AS ID; or,

- upon receiving a paging message with a different transaction ID compared to the last received one.

D2R Segmentation

- The AIoT device performs segmentation when the received TBS excluding the MAC header size is not enough to accommodate the whole D2R message, i.e., no explicit segmentation allowance from the reader is needed.

- Support 1-bit end/non-end segment indication.

- Support remaining D2R message size report from device to reader.

- The remaining D2R message size in relation to a D2R message is only reported in the MAC PDU comprising the first non-end segment of the D2R message.

- The remaining D2R message size report comprises an 8-bit field to indicate the number of remaining bytes of the D2R message.

- The D2R MAC PDU for data transmission contains the following fields:

- Single-bit end/non-end segment indication;

- Remaining msg size report, if any;

- Length indication of an D2R MAC SDU/MAC SDU segment;

- An D2R MAC SDU or MAC SDU segment.

- RAN2 consider the following options of scheduling message design in relation to D2R message segment (re)transmission:

- Option.1: each scheduling msg for D2R segment carries the upper service request (i.e., MAC SDU).

- Option.2: only the scheduling msg for the first D2R segment carries the upper service request (i.e., MAC SDU).

- RAN2 down-select from the following two alternatives for the offset indication:

- Alt.a: the scheduling message always carries offset indication for D2R message.

- Alt.b: the scheduling message for the first segment of D2R message does not carry the offset indication but each scheduling message for subsequent segment transmission/retransmission carries an offset indication.

- The offset indication for scheduling segment transmission/retransmission of a D2R message is 8-bit length in byte.

- The segmentation retransmission procedure is not reused for retransmission of an unsegmented D2R message.

- Segmentation for Msg1 of CFRA or Msg3 of CBRA is not supported.

- The R2D MAC PDU for data transmission contains:

- Message type;

- Resource allocation parameters for the corresponding D2R message transmission;

- Offset indication for D2R message segment, if any;

- MAC SDU of service request from upper layer, if any.

MAC PDU format

- AIoT MAC Header comprises MAC control information. AIoT MAC PDU does not support MAC CE or SDU concatenation.

- Introduce R2D trigger message other than the paging message, e.g., QueryRep-like message.

- Introduce a MAC NACK for D2R transmission failure indication.

- R2D MAC PDU does not comprise padding.

- The MAC header for D2R message comprises length for “SDU and variable part(s)”. Message type field is not required for D2R.

- Padding is required for D2R MAC PDU if the total size of MAC Header and MAC SDU is less than the transport block size indicated in the corresponding R2D control information. Padding is placed at the end of MAC PDU without subheader or length info.

- The AIoT MAC PDU, MAC Header and MAC SDU should be byte aligned (i.e., multiple of 8 bits) in length.

- RAN2 supports 4-bit length of message type in R2D.

- RAN2 assumes to only support multiplexing for Msg2.

- Reference

- Chair notes, RAN2#129.

- R2-2501965 Report of [POST129][036][AIoT] AS ID (Xiaomi).

- 3GPP TS 22.369 V19.3.0 (2024-09) Service requirements for ambient power-enabled IoT; Stage 1.

- 3GPP TR 38.769 V19.0.0 (2024-12) Study on solutions for ambient IoT (Internet of Things) in NR.

- R2-2501812	Discussion on AIoT Paging.

- R2-2501813	Random Access Procedure for AIoT Device.

- 3GPP TR 23.700-13 V19.0.0 (2025-03) Study on Architecture support of Ambient power-enabled Internet of Things.