3GPP TSG-RAN WG2 Meeting #129bis	R2-2501816

Wuhan, China, 7th – 11st Apr, 2025

Agenda Item:	8.7.4.2

Source: 	OPPO

Title:  	Discussion on DSR enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on DSR enhancements for scheduling.

# Discussion on DSR enhancements

In RAN2#129 meetings, the following agreements were made. Here we further discuss the leftover issues.

Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

Co-existence with legacy DSR MAC CE

One leftover issue is the co-existence with legacy DSR MAC CE. If networks configure them at the same time, considering the new DSR MAC CE is also compatible to legacy DSR MAC CE, it is straightforward that in case there is at least one LCG configured with new DSR, UE always uses a new DSR MAC CE for all LCGs.

- UE always uses the new DSR MAC CE for all LCGs in case there is at least one LCG configured with new DSR.

General data volume calculation mechanism

Another important topic discussed in the last meeting is the data volume calculation mechanism for Rel-19 DSR. In our understanding, taking the data volume calculation in PDCP as example, the data volume calculation mechanism could be specified based on the following principles:

1/ define “n-portion Delay-reporting PDCP SDU”, with/without PDU set discard, based on multiple reporting thresholds

2/ rely on “n-portion Delay-reporting PDCP SDU” to specify the n-portion data volume calculation in PDCP, including

- Calculate data volume for each portion of a LCH in an ascend order.

- The n-portion data volume calculation includes the data volume of n-portion Delay-reporting PDCP SDUs, and up to UE capability, if any PDCP SDU is ahead of any one of n-portion Delay-reporting PDCP SDUs, its data volume is considered in the n-portion data volume calculation.

- A PDCP SDU that has been included in the data volume calculation for one portion should not be counted again in the data volume calculation for another portion, regardless of its remaining time, in order to avoid redundant calculation of PDCP SDUs that have been calculated in the previous portions.

- The transmitting PDCP entity provides a per-portion delay-reporting indication for the PDCP Data PDU to lower layers.

Here are two examples for data volume calculation including both delay-reporting data and non-delay-reporting data with or without PDU set discard.

The relevant issues have already been partially discussed in post-meeting email discussion on running CRs. Based on the current running CRs and the post-meeting email discussion, two open issues on delay-reporting indication are proposed to discuss in this meeting, which includes

1/ Whether to specify delay-reporting indication in the PDCP specification or leave it up to UE implementation,

2/ Whether to specify update of delay-reporting indication in the PDCP specification or leave it up to UE implementation.

Here we further discuss on these two open issues.

Although this kind of cross-layer indication is between PDCP and RLC within UE itself and more like UE implementation, note that the current PDCP specification still captures the delay-critical indication for the data volume calculation for Rel-18 DSR roughly.

For the data volume calculation for Rel-19 DSR, similar way is suggested, i.e.,

1/ Based on the current per-portion data volume calculation mechanism for Rel-19 DSR, as what we have already done in current PDCP running CR, PDCP specification should also capture the per-portion delay-reporting indication roughly, i.e., the transmitting PDCP entity provides a per-portion delay-reporting indication for the PDCP Data PDU to lower layers.

- RAN2 confirms per-portion delay-reporting indication is specified in PDCP specification.

2/ For the update of the per-portion delay-reporting indication, it is captured in the current PDCP running CR as shown below.

A delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold as its remaining time decreases. The transmitting PDCP entity provides a delay-reporting indication for the PDCP Data PDU to lower layers each time the delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold.

As highlighted above, it requires that UE has to perform cross-layer interaction every time in case of the change of per-portion delay-reporting PDCP SDU, which may add unnecessary complexity and restriction for UE implementation. We believe that it would be better to mention in PDCP specification roughly and leave more to UE implementation.

What we suggest capturing in PDCP specification is that in case that a delay-reporting PDCP SDU changes its associated portion, the transmitting PDCP entity may update the per-portion delay-reporting indication to lower layers, but the details on how to perform it are left to UE implementation.

- The transmitting PDCP entity may update the per-portion delay-reporting indication to lower layers but leave the details to UE implementation.

Another FFS issue mentioned in the EN of the RRC running CR is to discuss whether the entries within dsr-ReportingThresList need to be configured in ascending order, as shown below.

dsr-ReportingThresList List of remaining time thresholds for reporting delay status information (DSR reporting threshold) in the Enhanced DSR, as specified in TS 38.321 [3]. Value for the IE DSR-ReportingThreshold in number of milliseconds. Editor's NOTE: exact name of the DSR MAC CE introduced in R19 to be further discussed and aligned with the MAC spec. Edirot’s NOTE: Discuss whether the entries within the list need to be configured in order (ascending order or decreasing order).

Note that per-portion data volume calculation mechanism in the current PDCP/RLC running CR is fully based on ascending-ordered dsr-ReportingThresList. If dsr-ReportingThresList is not configured in ascending order, much effort is needed to change the current PDCP/RLC running CR. Alternatively, UE has to sort the entries within dsr-ReportingThresList before using in PDCP/RLC, which adds unnecessary complexity for UE implementation. Therefore, we believe the simplest way is that the entries within dsr-ReportingThresList needs to be configured in ascending order.

- RAN2 confirms that the entries within dsr-ReportingThresList are configured in ascending order.

Control PDUs and retransmitted data

Another leftover issue on data volume calculation mechanism is how to deal with some Control PDUs and retransmitted data, including PDCP control PDU, PDCP SDUs to be retransmitted, PDCP Data PDUs to be retransmitted, RLC data PDUs that are pending for retransmission and RLC STATUS PDU.

In our understanding, there are two options put on the table, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the data volume calculation of either

- Option-1: the first reported portion in DSR, or

- Option-2: the first configured portion in DSR.

We need to analyze whether additional effort is needed or not for both Option-1 and Option-2.

With the current DSR mechanism, the DSR report in MAC layer is per LCG which includes multiple LCHs associated to different PDCP/RLC entities, and data volume calculation in PDCP/RLC layer is per PDCP/RLC entity.

If Option-1, i.e., first reported portion, is adopted, additional effort is needed for cross-layer/entity interaction. For example,

1/ if LCHs within one LCG have delay-reporting data but the first reported portions of these LCHs are different, it is problematic on which portion to capture Contol PDU and retransmitted data. At least there are two different interpretations, i.e., either the first reported portion of each LCH or the first reported portion of whole LCG, where the latter case introduces the dependency between different PDCP/RLC entities during data volume calculation.

2/ for the LCH within one LCG but does not have delay-reporting data, i.e., only contains Contol PDU and retransmitted data, it is also problematic on which portion to capture Contol PDU and retransmitted data. The potential approaches include either it is captured in the first reported portion of the whole LCG which also requires the interaction between different PDCP/RLC entities during data volume calculation, or it is not calculated during data volume calculation which leads to missing data volume.

If Option-2, i.e., first configured portion, is adopted, the only small issue is that in case of no delay-reported data associated to the first configured portion, the first configured portion still needs to be present in DSR MAC CE to capture Contol PDU and retransmitted data if any, which leads to 2-octet signalling overhead per LCG for DSR MAC CE, and needs to consider how to set its remaining time without the delay-reporting data.

Another case needs to be analyzed is that for DC one DRB is associated to at least two RLC bearers and LCHs. For the associated LCHs due to DC, they share one PDCP entity, but it still has chance that one of the LCHs only contain the retransmitted data without any delay-reporting data due to PDCP duplication/split. The problem for Option-1, i.e., first reported portion, is similar to the case of LCHs within one LCG.

Therefore, we propose to go for Option-2, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the data volume calculation of the first configured portion of the associated LCG.

- For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be considered into the first configured portion of the associated LCG.

UE capability

Another issue needed to be started to discuss is the capability for DSR enhancement. The DSR enhancement introduced in Rel-19 XR should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

- Introduce a new capability on whether the UE supports DSR enhancement for Rel-19 XR, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- UE always uses the new DSR MAC CE for all LCGs in case there is at least one LCG configured with new DSR.

- RAN2 confirms per-portion delay-reporting indication is specified in PDCP specification.

- The transmitting PDCP entity may update the per-portion delay-reporting indication to lower layers but leave the details to UE implementation.

- RAN2 confirms that the entries within dsr-ReportingThresList are configured in ascending order.

- For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be considered into the first configured portion of the associated LCG.

- Introduce a new capability on whether the UE supports DSR enhancement for Rel-19 XR, which is an optional capability with signalling.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia