3GPP TSG-RAN WG2 Meeting #129bis	R2-2501817

Wuhan, China, 7th – 11st Apr, 2025

Agenda Item:	8.5.4

Source: 	OPPO

Title:  	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

# Introduction

In Rel-19 NES WID [1], the objective on adaptation of common signal/channel transmissions is captured as follows.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution would like to provide our considerations on this objective of Rel-19 NES.

# Discussion

# Time-domain RACH Adaptation

Most work on the RACH enhancement is within R1 scope. From R2 perspective, what we need to discuss includes whether the RACH adaptation mechanism applies to different types of RACH resources (e.g., CBRA/CFRA, 2/4-step RA type, feature/feature combination-specific RACH resources).

Here we further discuss the leftover issue based on the following agreements made in the last meeting.

RAN2 starts CBRA for RACH adaptation. RAN2 starts 4-step RACH adaptation. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. From R2 perspective, RACH adaptation is not modelled as RA feature(s). From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

For CBRA/CFRA, R2 has already agreed to support CBRA, but it still FFS on CFRA from R2 perspective.

Note that based on the newest R1 agreement made in R1#120 meeting as shown below, R1 has already worked on supporting RACH adaptation for CFRA triggered by (normal) PDCCH order, e.g., PDCCH order enhancement to indicate whether the additional PRACH resource(s) is available for the triggered RACH.

Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication

- R1 has already worked on supporting RACH adaptation for CFRA initiated by (normal) PDCCH order.

In the current RACH framework, CFRA could be initiated by a PDCCH order, by the MAC entity itself, or by RRC signalling. For different CFRA cases, their CFRA resources may be configured differently, i.e.,

- CFRA for additional PCI initiated by PDCCH order: the CFRA resources are configured by rach-configGeneric from additionalRACH-perPCI-ToAddModList-r18, which is mandatory present.

- CFRA for LTM early sync initiated by PDCCH order: the CFRA resources are configured by rach-configGeneric from EarlyUL-SyncConfig of each LTM candidate cell, which is mandatory present.

- CFRA initiated by normal PDCCH order: as observed above, up to the 1-bit indication in PDCCH order, the CFRA resources are configured by rach-configGeneric from either rach-configCommon or the new IE for Rel-19 RACH adaptation.

- CFRA for LTM cell switch initiated by MAC entity itself (via an LTM Cell Switch Command MAC CE): the CFRA resources are configured by rach-configGeneric from the rach-ConfigDedicated (if provided, otherwise from rach-configCommon) of each LTM candidate cell.

- CFRA for HO initiated by RRC signalling (via RRCReconfiguration with ReconfigurationWithSync): the CFRA resources are configured by rach-configGeneric from the rach-ConfigDedicated (if provided, otherwise from rach-configCommon) of target cell.

- CFRA for BFR initiated by MAC entity itself: the CFRA resources are configured by rach-configGeneric from BeamFailureRecoveryConfig.

Considering R1 has already supported RACH adaptation for 3), it is straightforward to confirm it and further discuss the other CFRA cases.

In our understanding, for 1), 2) and 6), no change for RACH adaptation is needed, since one can simply configure the rach-configGeneric (which is separately configured) same as the one for additional RACH resources.

For 4), and 5), the rach-configGeneric comes from the target/candidate cell. If rach-configGeneric is not present in rach-ConfigDedicated for a target/candidate cell, rach-configGeneric from rach-configCommon would be used. In this case, companies think that there is a need for an additional indication to indicate between legacy RACH resources and additional RACH resources, which has additional specification impacts. We understand for 4) and 5), the same approach mentioned for 1), 2), and 6) is also applicable. If network would like to configure additional RACH resources for CFRA, it can simply configure the rach-configGeneric in rach-ConfigDedicated same as the one for additional RACH resources, which is fully up to network implementation without any specification impact. And we believe that the signalling overhead is not a big issue.

- If network would like to configure additional RACH resources for CFRA, network may configure the rach-configGeneric (which is separately configured) same as the one for additional RACH resources, for other CFRA cases including 
1) CFRA for additional PCI initiated by PDCCH order, 
2) CFRA for LTM early sync initiated by PDCCH order, 
3) CFRA for BFR, 
4) CFRA for HO initiated by RRC signalling, and 
5) CFRA initiated by LTM Cell Switch Command MAC CE.

- R2 confirms time-domain RACH adaptation is supported for CFRA initiated by normal PDCCH order, but not for other CFRA cases. Send LS to R1 if any concern.

For 4/2-step RACH, 4-step RA type is supported as baseline. It is still open on the support of 2-step RA type from R2 perspective. In our understanding, 2-step RA type may need additional R1 effort which at least includes:

- impact on DCI design in R1, e.g., indicating 2-step and/or 4-step RA type in DCI.

- impact on the associated MsgA PUSCH resource in case of MsgA PRACH resource adaptation, i.e., R1 may need to check whether these time-domain parameters are still feasible, e.g., msgA-PUSCH-TimeDomainOffset, startSymbolAndLengthMsgA-PO, msgA-PUSCH-timeDomainAllocation, nrofMsgA-PO-perSlot.

Considering the main impact of 2-step RA type needs to be evaluated in R1, it is good to send LS to R1 to further check the support of 2-stpe RA.

- Up to R1 to evaluate the impact on supporting of time-domain RACH adaptation for 2-step RA type. Send LS to R1 to check whether to apply to 2-step RA or not.

For feature (combination) specific RACH partitioning, the legacy RACH resource can be applied to all existing features, i.e. RedCap, SDT, Slicing, and CE, and their feature combinations, but for additional RACH resource introduced by Rel-19 RACH adaptation mechanism, CE and its related feature combinations are excluded, considering it is not reasonable to configure both NES and CE features for UE’s SCell.

- CE and its related feature combinations are not supported for additional RACH resource introduced by RACH adaptation.

The issue lies in the disparity between the legacy RACH resources and the new additional RACH resources introduced in R19. If network needs to allocate RACH resources to the feature of CE (or CE-related feature combinations) using the legacy RACH resources, it will face a significant constraint with the new additional RACH resources considering it cannot be configured to support CE, if both types of RACH resources tend to share the same feature or feature combination-based RACH partition scheme. It is unclear that how to enable feature(-combination) based RACH resource partitioning for the two types of RACH resources, i.e., legacy RACH resources and new additional RACH resources.

Two approaches could be considered for two types of RACH resources, as following:

- utilizing two distinct feature (combination)-based RACH partitioning schemes including/not including the support of CE-related feature (combination), or

- utilizing a unified feature (combination)-based RACH partitioning scheme for them, and additional indication to address the specific exclusion of CE-related features in the new additional RACH resources.

- R2 discusses the approaches to enable feature(-combination) based RACH resource partitioning for legacy RACH resources and new additional RACH resources, including 
1) two distinct feature (combination)-based RACH partitioning schemes,
2) a unified feature (combination)-based RACH partitioning scheme.

Another issue is how to calculate RA-RNTI, in case of additional PRACH resources introduced due to RACH adaptation. Although RA-RNTI calculation is specified in 38321, the main solution is within R1 scope, e.g., separate search space for additional PRACH resources can be easily used to solve this issue (as R2 discussed for the RACH separation issue since R15), or RA-RNTI formula enhancement (which invites more solutions since it has to be backwards compatible with legacy UEs). We propose to rely on R1 to further discuss on this issue.

- Up to R1 to discuss the potential RAR differentiation issue due to RACH adaptation.

# Paging Adaptation

One leftover issue mentioned in the EN of the RRC running CR is whether to introduce a separate pei-ConfigBWP for paging adaptation or not, as shown below.

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations. FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

Note that pei-ConfigBWP includes the PEI-dedicated search space configuration (i.e., pei-SearchSpace), and PEI-O location (i.e., firstPDCCH-MonitoringOccasionOfPEI-O). We understand that the intention of the current agreement of introducing a separate PEI configuration has already covered separate PEI-O location, therefore the main issue is whether the pei-SearchSpace needs to be separate or not. From R2 perspective, no motivation to introduce separate pei-SearchSpace, so a separate PEI-O location is sufficient instead of a separate pei-ConfigBWP.

- No motivation to introduce separate pei-SearchSpace from R2 perspective.

- R2 clarifies that separate PEI-O location for Rel-19 NES UEs is sufficient and no need to introduce a separate PEI search space.

Another leftover issue mentioned in the EN of the 38304 running CR is the UE behaviour when Rel-19 PEI configuration is provided, as shown below.

For a UE supporting paging adaptation and PEI, if pagingAdaptationPEI-Config is signaled in system information, its behavior is FFS. Editor’s note: FFS the UE behavior if pagingAdaptationPEI-Config is configured.

Companies think that there are two potential behaviours for Rel-19 NES UEs, i.e., either Option-1) Rel-19 NES UEs only monitor Rel-19 PEI, or Option-2) Rel-19 NES UEs monitor legacy PEI for legacy POs and Rel-19 PEI for Rel-19 POs. Regarding this issue, we share the same view as CR Rapporteur, i.e., Option-2) is conflicted with the R2 agreements made in the R2#127bis meeting, as shown below.

R2#127bis agreements on paging adaptation - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Thus, we propose to further clarify on this.

- Considering R2 has already agreed that “Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration” in the R2#127bis meeting, Option-2), i.e., Rel-19 NES UEs monitor legacy PEI for legacy POs and Rel-19 PEI for Rel-19 POs, is conflicted with the current R2 agreement.

- R2 clarifies that Rel-19 NES UEs only monitor Rel-19 PEI for Rel-19 POs.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Time-domain RACH Adaptation

- R1 has already worked on supporting RACH adaptation for CFRA initiated by (normal) PDCCH order.

- If network would like to configure additional RACH resources for CFRA, network may configure the rach-configGeneric (which is separately configured) same as the one for additional RACH resources, for other CFRA cases including 
1) CFRA for additional PCI initiated by PDCCH order, 
2) CFRA for LTM early sync initiated by PDCCH order, 
3) CFRA for BFR, 
4) CFRA for HO initiated by RRC signalling, and 
5) CFRA initiated by LTM Cell Switch Command MAC CE.

- R2 confirms time-domain RACH adaptation is supported for CFRA initiated by normal PDCCH order, but not for other CFRA cases. Send LS to R1 if any concern.

- Up to R1 to evaluate the impact on supporting of time-domain RACH adaptation for 2-step RA type. Send LS to R1 to check whether to apply to 2-step RA or not.

- CE and its related feature combinations are not supported for additional RACH resource introduced by RACH adaptation.

- R2 discusses the approaches to enable feature(-combination) based RACH resource partitioning for legacy RACH resources and new additional RACH resources, including 
1) two distinct feature (combination)-based RACH partitioning schemes,
2) a unified feature (combination)-based RACH partitioning scheme.

- Up to R1 to discuss the potential RAR differentiation issue due to RACH adaptation.

Paging Adaptation

- No motivation to introduce separate pei-SearchSpace from R2 perspective.

- R2 clarifies that separate PEI-O location for Rel-19 NES UEs is sufficient and no need to introduce a separate PEI search space.

- Considering R2 has already agreed that “Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration” in the R2#127bis meeting, Option-2), i.e., Rel-19 NES UEs monitor legacy PEI for legacy POs and Rel-19 PEI for Rel-19 POs, is conflicted with the current R2 agreement.

- R2 clarifies that Rel-19 NES UEs only monitor Rel-19 PEI for Rel-19 POs.

# 4. Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson