3GPP RAN WG2 Meeting #129bis	R2-2501827

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.1.2.3

Source:	HONOR

Title:	Discussion on LCM for Positioning Use Case

Document for:	Discussion and Decision

- Introduction

In Rel-19 AI/ML for NR air interface WID, the following objectives were achieved:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In RAN2#129 meeting, the following agreements were made:

1: Introduce AI/ML positioning Case 1 as a new positioning method. 2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR. 3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation, 4: The content of error cause is discussed while drafting stage3 CRs. 5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. 6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

In this contribution, we discuss the remaining issues on AI/ML positioning.

- Discussion

2.1 LCM procedure

In RAN2#127bis, the following procedures for LCM for UE sided model for AI positioning case 1 were achieved as the baseline:

- Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. - Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. - Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). - Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. - Step 5: The LMF requests the inferred location information using the LPP request location information message. - Step 6: UE reports the inferred location using LPP provide location information message.

In step 3, although the assistance data is sent from LMF to UE, due to that the LMF does not know the applicable functionality for using AI/ML model, the provided assistance data may not be appropriate for inference.

In order to avoid this problem, the inference configuration can be provided to UE after the UE reports the applicable functionality to the LMF via step 4. That is to say, the applicable functionality related to the inference configuration can be send from LMF to UE via step 5.

Proposal 1: The inference configuration should be configured via step 5.

In the last meeting, an FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step 4 is sent reactively or proactively) was left. From the above analysis, we know that the AI/ML based method for positioning can be executed based on the inference configuration of one applicable functionality. In step 3, the LMF can provide the network side additional condition to UE simultaneously. The UE determines the applicable functionality based on the additional condition and reports the applicable functionality to the LMF via step 4. The provided additional condition can be viewed as an implicit request indication.

Proposal 2: If the additional condition is provided in step 3, the UE reports the applicable functionality to the network via step 4.

2.2 Switching between non-AI/ML and AI/ML methods

In RAN2#128, it was agreed, if the AI/ML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AI/ML based positioning, and replies with LPP ProvideLocationInformation message with an error cause. If this happens, the UE needs to perform fallback operation. The issues how to perform fallback is identified, and the following two options can be taken into account.

Option 1: Request fallback after sending ProvideLocationInformation message with error cause.

When the AI/ML based positioning method becomes non-applicable, the UE will send ProvideLocationInformation message with error cause. Then, the UE requests fallback operation. After receiving response from LMF, the UE will perform fallback to non-AI/ML methods.

Option 2: Perform fallback directly with preconfigured fallback in step 5.

In step 5, the LMF provides an indication of performing fallback operation, if the AI/ML based positioning method becomes non-applicable. Optionally, the LMF provides the non-AI/ML methods related assistance information simultaneously.

After sending ProvideLocationInformation message with error cause, if the UE has already obtained the non-AI/ML methods related assistance information, the UE can perform fallback non-AI/ML methods directly.

Comparing with the above two options, option 1 results in additional signaling overhead. Option 2 can be performed based on the presence of non-AI/ML methods related assistance information.

Proposal 3: It is suggested to preconfigure fallback via step 5, and the UE performs fallback directly if needed.

If the applicability conditions are not satisfied at the time the UE reports the applicable functionality, the LMF may request non-AI/ML method to perform positioning. However, if the applicability conditions change, the network can reconfigure the updated applicability conditions via step 5 simultaneously. Since there is a change of applicable conditions, the applicability conditions may become satisfied. The UE will report the applicable functionality to the LMF by the LPP. If the LMF requests the inferred location information with AI/ML, the UE will switch from non-AI/ML to AI/ML methods to perform positioning.

Proposal 4: If the LMF requests the inferred location information with AI/ML instead of the previous request non-AI/ML method to perform positioning, the UE switches from non-AI/ML to AI/ML method to perform positioning.

2.3 Performance Monitoring

In RAN1#116-bis, the following agreements on performance monitoring were achieved:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

From RAN1 discussion, both UE and LMF are capable of generating monitoring metric, and it is up to implementation to determine one exact method to perform the monitoring metric calculation. In order to reduce the signalling overhead associated with transmitting of monitoring metric, it would be appropriate for the same node to be responsible for monitoring metric calculation and performance monitoring.

Proposal 5a：For AI/ML positioning Case 1, if the monitoring metric is generated by UE，UE is also responsible for making decision based on performance monitoring.

Proposal 5b：For AI/ML positioning Case 1, if the monitoring metric is generated by LMF，LMF is also responsible for making decision based on performance monitoring.

- Conclusions

In this contribution, we discussed the issues related to LCM for Positioning Use Case. Based on the discussion, the following proposals are concluded:

Proposal 1: The inference configuration should be configured via step 5.

Proposal 2: If the additional condition is provided in step 3, the UE reports the applicable functionality to the network via step 4.

Proposal 3: It is suggested to preconfigure fallback via step 5, and the UE performs fallback directly if needed.

Proposal 4: If the LMF requests the inferred location information with AI/ML instead of the previous request non-AI/ML method to perform positioning, the UE switches from non-AI/ML to AI/ML method to perform positioning.

Proposal 5a：For AI/ML positioning Case 1, if the monitoring metric is generated by UE，UE is also responsible for making decision based on performance monitoring.

Proposal 5b：For AI/ML positioning Case 1, if the monitoring metric is generated by LMF，LMF is also responsible for making decision based on performance monitoring.

- References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface