3GPP TSG-RAN WG2 Meeting #129bis	R2-2501840

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.7.5

Source:	HONOR

Title:	Discussion on RLC enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#129, the following agreements were agreed for enhancing RLC retransmissions [1]:

Agreements on RLC enhancements Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly. - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

In this contribution, we discuss the potential solutions related to RLC enhancements.

2. Discussion

2.1 Avoiding unnecessary RLC retransmissions

In the last meeting, it was agreed that the new RLC timer at the RX side is per RLC entity. It is similar to t-Reassembly, i.e. only one new timer t-RxDiscard is running at a given time.

In the RLC running CR, it is FFS whether the receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires. The current SR triggering and sending mechanism ensures that the SR carries the appropriate receiving status of SDUs and is sent in a timely manner without being too frequent. The t-RxDiscard does not need to trigger SR more frequently. Therefore, the current mechanism of triggering SR by t-Reassembly expiration works well. There is no need to trigger SR when the t-RxDiscard expires every time.

Observation 1: The t-Reassembly triggered SR works well, and there is no need for the expiry of the t-RxDiscard to trigger SR every time.

However, there is a scenario that the receiving status of SDUs associated with t-Reassembly are positively acknowledged due to the t-RxDiscard expires.

As shown in Fig.1a, the expiry of t-RxDiscard determines the SDUs whose SN are smaller than x+3 as outdated SDU. And from SDU(x+3) to SDU(x+5), these SDU are delivered to upper layer. All SDUs associated with t-Reassembly can be positively acknowledged. In this case, the SR can be triggered by the expiration of the t-RxDiscard.

In Fig.1b, the expiry of t-RxDiscard determines the SDUs whose SN are smaller than x+3 as an outdated SDU. And the reception status of SDU(x+4) is not determined. In this case, it is no need to trigger SR when the t-RxDiscard expires. The SR can be triggered by the expiration of t-Reassembly.

Fig.1 The expiry of t-RxDiscard can trigger SR in some cases.

Therefore, SR can be triggered when the t-RxDiscard expires and all SDUs maintained by t-Reassembly are outdated or ACK.

Proposal 1: When the t-RxDiscard expires and all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, the expiration of t-RxDiscard can trigger an SR.

If it is agreed that the t-RxDiscard can trigger SR in some cases, the RX_Highest_Status which has not been updated by t-Reassembly expiration needs to be updated before triggering SR. This variable affects the information carried in SR.

Proposal 2: If it is agreed that the t-RxDiscard expiration can trigger an SR in some cases, RX_Highest_Status should be updated before triggering SR.

The SR triggered by the expiry of t-RxDiscard indicates that all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, t-Reassembly is no longer needed to continue running. Therefore, in the case that all SDUs maintained by t-Reassembly are outdated or delivered to upper layer due to the t-RxDiscard expires, t-Reassembly should be stopped or restarted.

Proposal 3: In the case that all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, t-Reassembly should be stopped or restarted.

2.2 Timely RLC retransmission

In the last meeting, it was agreed autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold, which is separated for autonomous reTx and for polling. It is FFS if the remaining time is determined based on the discardTimer on PDCP or the new timer on RLC.

2.2.1 Based on the discardTimer on PDCP or the new timer on RLC

Based on the discardTimer on PDCP

Assume that the remaining time of an RLC SDU is determined based on the discardTimer on PDCP. When the remaining time of a PDCP SDU falls below a corresponding threshold, the autonomous retransmission and/or polling should be indicated to lower layers, if the corresponding PDCP Data PDU has already been submitted to lower layers.

When indicated from upper layer (e.g. PDCP) to retransmit a particular RLC SDU autonomously, the transmitting side of an AM RLC entity shall consider the indicated RLC SDU or the RLC SDU segment which has not been autonomously retransmitted for retransmission.

In the same way, when indicated from upper layer (e.g. PDCP) to poll about a particular RLC SDU, the transmitting side of an AM RLC entity shall consider include a poll in the AMD PDU.

Observation 2: The autonomous retransmission and/or polling indication from PDCP triggers AM RLC entity to consider the indicated RLC SDU for autonomous retransmission and/or including a poll.

Based on the new timer on RLC

The new timer introduced in the TX RLC entity can be per RLC or per SDU.

If the new timer is per RLC, the remaining time of each SDU cannot be accurately determined. When the per RLC timer meets the triggering conditions of POLL or AT, the SDU that performs poll and AT can be the one associated with the timer, or the SDU received by RLC during the timer running. In either case, timely retransmission of SDU cannot be achieved.

If it is a per SDU timer, the timer associated with the SDU is started when the SDU is received from the upper layer. When the remaining time till the timer expiry is less than EnhancedPollingThreshold, the poll is included in an AMD PDU. When the remaining time till the timer expiry is less than AutonomousReTxThreshold, RLC considers retransmitting the SDU associated with the timer.

Proposal 4: If the remaining time is determined based on a new timer at RLC, the new timer at the TX is per SDU.

2.2.2 The procedure of autonomous retransmission and polling

Autonomous retransmission

In the last meeting, it was agreed that only a single autonomous retransmission will be triggered per RLC SDU. Among the SDUs that have not been autonomously retransmitted, priority is given to autonomously retransmitting the SDU whose reception status has never been received, i.e. the TX side has never received a status report containing the SN that SDU. Autonomous retransmission gives the retransmission opportunity for an SDU which without any reception status and is about to become outdated.

Proposal 5: The TX side can perform autonomous retransmission of an SDU, if the SDU has not been autonomously retransmitted and a status report containing the SN of that SDU has never been received.

Polling

In the RLC running CR, the modifications for the polling are as following:

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte; or - if the remaining time of an RLC SDU or an RLC SDU segment falls below the [specified threshold for polling enhancement] [as indicated from PDCP]: - include a poll in the AMD PDU as described below;

Since the pollbyte and pollPDU are applied for the new transmission, upon the notification of a transmission opportunity by lower layer, it is restricted the transmission needs to contain the AMD PDU which contains a not previously transmitted SDU/SDU segment. Therefore, it excludes the transmission with only retransmission SDU(s).

As we all know, the transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

Fig.2 Only retransmission PDUs in a transmission opportunity.

In Fig.2, upon notification of a transmission opportunity by lower layer, the remaining time of SDU3 and SDU5 fall below the specified threshold, there is no AMD PDU that contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment. Therefore, it is no opportunity to include a poll in an AMD PDU in such case. It is beneficial to include a poll in an AMD PDU for only retransmission case for requesting SR.

Proposal 6: The transmission with only retransmission SDU(s) should include a poll in AMD PDU(s) when there is an SDU whose remaining time below the specified threshold.

Regardless of whether pollPDU or pollByte, the poll is contained in an AMD PDU that reaches the trigger conditions, and then TX resets the trigger condition.

According to this logic, the poll triggered by the remaining time should be included in the particular PDU whose remaining time is less than a threshold. Because the particular PDU may not have the opportunity to be retransmitted, the poll may also be included in one or more PDUs with a transmission opportunity.

Proposal 7: The poll triggered by the remaining time may be included in the PDU whose remaining time is less than a threshold, or the PDUs in a transmission opportunity.

On the other hand, if the poll triggered by the remaining time is included in an AMD PDU, whether or not to reset the trigger condition should be considered. This determines how many times polling can be triggered by an SDU with a remaining time less than a specified threshold.

Proposal 8: RAN2 shall consider how many times polling can be triggered if the remaining time of SDU fall below the specified threshold.

3. Conclusions

- Avoiding unnecessary RLC retransmissions

Observation 1: The t-Reassembly triggered SR works well, and there is no need for the expiry of the new timer to trigger SR every time.

Proposal 1: When the new timer expires and all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, the expiration of new timer can trigger an SR.

Proposal 2: If it is agreed that the new timer expiration can trigger an SR in some cases, RX_Highest_Status should be updated before triggering SR.

Proposal 3: In the case that all SDUs maintained by t-Reassembly are outdated or delivered to upper layer, t-Reassembly should be stopped or restarted.

- Timely RLC retransmission

Observation 2: The autonomous retransmission and/or polling indication from PDCP triggers AM RLC entity to consider the indicated RLC SDU for autonomous retransmission and/or including a poll.

Proposal 4: If the remaining time is determined based on a new timer at RLC, the new timer at the TX is per SDU.

Proposal 5: The TX side can perform autonomous retransmission of an SDU, if the SDU has not been autonomously retransmitted and a status report containing the SN of that SDU has never been received.

Proposal 6: The transmission with only retransmission SDU(s) should include a poll in AMD PDU(s) when there is an SDU whose remaining time below the specified threshold.

Proposal 7: The poll triggered by the remaining time may be included in the PDU whose remaining time is less than a threshold, or the PDUs in a transmission opportunity.

Proposal 8: RAN2 shall consider how many times polling can be triggered if the remaining time of SDU fall below the specified threshold.

4. References

[1]	RAN2 #129 Chairman Notes