3GPP RAN WG2 Meeting #129bis	R2-2501845

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.9.3

Source:	HONOR

Title:	Discussion on UL capacity enhancement

Document for:	Discussion and Decision

1. Introduction

In Rel-19 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID, the following objectives were achieved:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#129 meeting, the following agreements were made:

1. It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used 2. RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC 3. RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS 4. For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. 5. The UE shall at most have one ongoing CB EDT procedure at any time. 6. The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. 7. The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. 8. CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. 7. As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. 8. RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: 1. One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) 2. For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19

In this contribution, we discuss the remaining issues on CB-Msg3 mechanism.

2. Discussion

The CB-MSG3 will be triggered if the configured condition of TBS is met. Based on the mechanism of CE, when the UE determines one CE level to trigger CB-MSG3, if the number of CB-MSG3 transmissions reaches maximum number of transmissions of this CE level, it will transfer to the next level to continue the transmission. However, if TBS is configured per CE level, the condition of TBS for the next level may not be met. If so, the mechanism of CB-MSG3 will no longer be able to continue executing, and the UE will not be able to obtain gain from it. In view of this, a common maximum TBS is appropriate.

Proposal 1：A common maximum TBS should be configured for different CE levels.

When DSA is applied for CB-MSG3 transmission, one issue to be discussed is whether the UE needs to monitor one or multiple PDCCH monitoring windows. Two options are shown as follows:

Option 1: multiple/parallel PDCCH monitoring windows

For multiple PDCCH monitoring windows, one PDCCH monitoring window will be started/restarted after sending every replica. In detail, one PDCCH monitoring window will be restarted after sending the next replica, and the previous window will be stopped, i.e. there is only one window running every time.

Alternatively, parallel PDCCH monitoring window can be started after every replica for the reception of the corresponding response while the previous window is still running. That is, the UE might manage up to 4 windows at the same time, which will be the UE complexity. Therefore, we should carefully consider parallel PDCCH monitoring window.

Option 2: one PDCCH monitoring window

We have noticed an issue that, if the one PDCCH monitoring window is started after the last replica, the UE might not receive any response in the duration of the window, since the NW has already provided the response after receiving the previous replica and mistakenly thinking this is the last replica. To solve this issue, there are two solutions as follows:

One solution is to start PDCCH monitoring window after the end of DSA resource window. However, if the UE selects the earlier occasion DSA resource within this window time range, the monitoring window has to wait for the end of DSA time window, which might have a slight impact on UL capacity.

Another solution is to start PDCCH monitoring window, after completing the sending of the first replica. Compared with multiple PDCCH monitoring windows, if the interval between the selected two replica sending occasions is greater than the length of the PDCCH monitoring window, unnecessary PDCCH monitoring would be introduced.

Based on the analysis on the two options, we have come to the following proposal:

Proposal 2: It is preferred to have multiple PDCCH monitoring windows and the window will be restarted after sending every replica.

In two-step RACH, each MSGA payload resource is associated with the resource of transmitting preamble. Similar to two-step RACH, the window for DSA CB-Msg3 occasion selection can be configured corresponding to each CB-MSG3 resource. Based on the relationship between DSA resource and CB-MSG3 resource, the RNTI for DSA replicas can be calculated based on the associated CB-MSG3 resource.

Proposal 3: The DSA CB-Msg3 time window can be mapped to the CB-MSG3 resource.

In the previous meeting, one FFS has been left as that one common RNTI or multiple RNTIs shall be used for monitoring respond message. If multiple RNTIs are used, after the NW sends the response towards UE in a short time duration (i.e., UE-eNB RTT), it may receive a new replica from the same UE. Since the NW doesn't know if this replica comes from the same UE as the previous one, the NW has to respond to this replica, which may result in unnecessary signaling overhead. Moreover, due to that the respond message is used for contention resolution, if the winner of contention resolution is same as that of the previous replica, it will have a negative impact on UL capability enhancements. Therefore, to solve this issue, one RNTI should be used. If proposal 3 is agreed, the common RNTI can be calculated based on the transmission resource of CN-MSG3.

Proposal 4a: It is suggested to use one common RNTI.

Proposal 4b: The common RNTI can be calculated based on the transmission resource of CN-MSG3.

In two-step RACH, the MAC PDU (MSGB) consists of one or more MAC subPDUs and optionally padding. Each MAC subPDU consists one of the following [38.321]:

-	a MAC subheader with Backoff Indicator only;

-	a MAC subheader and fallbackRAR;

-	a MAC subheader and successRAR;

-	a MAC subheader and MAC SDU for CCCH, DCCH or DTCH;

-	a MAC subheader and padding.

Since the cell specific PUSCH resources are configured for Msg3 transmission, after receiving Msg3, a contention resolution mechanism should be applied. The UE contention resolution identity included in the successRAR is used to determine whether the contention is successful. If the UE contention resolution identity matches the one used in CCCH SDU, that would mean success from the contention.

For CB-MSG3, the MSG4 consists of the similar MAC subPDUs as that of MSGB. That is, it is feasible to transmit the contention resolution indication together with RRC message. However, whether to transmit RRC message together with contention resolution indication depends on the design of signaling procedure. The Backoff Indicator, fallbackRAR and successRAR will be considered essential functions, which should be included in Msg4.

Proposal 5: The Msg4 contains at least one or more MAC subPDUs: Backoff Indicator, fallbackRAR, successRAR and optionally padding.

For UE who won the competition, the UE can further determine its own behavior based on the type of RRC message. If the RRC message is RRCEarlyDataComplete, the UE will consider this service transfer to be complete. If the RRC message is RRCConnectionSetup, the UE will enter connected mode and will continue service transmission. And the C-RNTI can be allocated by RRCConnectionSetup.

Alternatively, the contention resolution indication can be transmitted separately and followed by subsequent RRC message, if needed.

In principle, either the transmission of the contention resolution indication together with RRC message or transmission of the contention resolution indication followed by subsequent RRC message could be executed. However, if there is downlink data needed to be transmitted, the downlink data should be transmitted by the subsequent RRC message. If not, it will increase the complexity of transmitting the downlink data via MSG4, e.g. the impact of downlink data volume.

Proposal 6: For MSG4 transmission, it is suggested to transmit the contention resolution indication separately and followed by subsequent RRC message, if needed.

Whether to detect and receive the subsequent RRC message based on the indication carried by successRAR. For instance, if the C-RNTI is included in successRAR, there is the subsequent RRC message

Proposal 7: For success RAR, if the field C-RNTI is present, it indicates that there is subsequent RRC message and/or DL data.

In 2-step RACH, the A MAC subheader for fallbackRAR consists of three header fields E/T1/RAPID. The field RAPID is used to indicate that UEs that use the preamble with this RAPID need to perform a fallback operation. However, in CB-MSG3, it has no preamble transmission. Therefore, we need to consider other identifiers to indicate the UEs that need to perform a fallback operation

Proposal 8: For fallback RAR, the issue of distinguishing which UE(s) to perform fallback operations on should be taken into consideration.

3. Conclusions

In this contribution, we discussed the issues related to CB-Msg3 mechanism. Based on the discussion, the following proposals are concluded:

Proposal 1：A common maximum TBS should be configured for different CE levels.

Proposal 2: It is preferred to have multiple PDCCH monitoring windows and the window will be restarted after sending every replica.

Proposal 3: The DSA CB-Msg3 time window can be mapped to the CB-MSG3 resource.

Proposal 4a: It is suggested to use one common RNTI.

Proposal 4b: The common RNTI can be calculated based on the transmission resource of CN-MSG3.

Proposal 5: The Msg4 contains at least one or more MAC subPDUs: Backoff Indicator, fallbackRAR, successRAR and optionally padding.

Proposal 6: For MSG4 transmission, it is suggested to transmit the contention resolution indication separately and followed by subsequent RRC message, if needed.

Proposal 7: For success RAR, if the field C-RNTI is present, it indicates that there is subsequent RRC message and/or DL data.

Proposal 8: For fallback RAR, the issue of distinguishing which UE(s) to perform fallback operations on should be taken into consideration.

4. References

[1]