3GPP TSG-RAN WG2 Meeting #129bis                                                   R2-2501872

Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Consideration on XR-specific RLC Enhancement

Agenda Item:	8.7.5

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the issues on timely RLC retransmissions including enhanced polling and autonomous retransmission, and avoiding unnecessary retransmission for XR, our views on these issues are given in the following part.

Discussion

Timely RLC retransmissions for XR

In RAN2#129 meeting, the agreement on timely retransmissions on RLC layer is as follows [1].

Autonomous retransmissions and polling enhancements Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC Only a single autonomous retransmission will be triggered per RLC SDU. There is no dynamic activation/deactivation of the autonomous retransmission mechanism. We have separate thresholds for autonomous reTx and for polling.

The triggering condition

Based on above agreement, autonomous retransmission and/or polling will be triggered when the remaining time of an RLC SDU is below a specified threshold, but it is still FFS the discardTimer on remaining time is at PDCP layer or new timer at RLC layer. Two options will be used for down-selection for autonomous retransmission and/or polling triggering.

- Option 1: When the remaining time of a SDU based on PDCP discard timer is lower than a threshold.

- Option 2: When a new RLC retransmission timer is expired, this new timer may start upon submission of RLC PDU to MAC.

For above two options, option 1 will reuse the delay-critical data mechanism in PDCP layer but with a new separate remaining time threshold, which could reflect the real remaining time and retransmission requirement from the view of delay budget for each RLC SDU. The option 2 will give the sufficient time to ARQ/HARQ feedback and avoid the repeated retransmission of same RLC SDU in lower layer. However, usually, the autonomous retransmission and enhanced polling are always to be triggered when the remaining time is near to a small value, which are considered as a supplementary method to normal behavior, hence a suitable configured remaining time threshold in option 1 will give low layers sufficient time to perform the legacy (re)transmission options. Besides, PDCP layer already gives types of indications to RLC layer, such as indication for DSR, it is preferable to use option 1 for autonomous retransmission and enhanced polling.

Proposal 1: Autonomous retransmission and/or polling will be triggered when the remaining time of a RLC SDU based on PDCP discard timer is lower than a threshold.

The priority of autonomous retransmission

In current running CR at RAN2#129 Post Email [512], there is a FFS issue as below:

5.2.3.1 Transmit operations 5.2.3.1.1 General The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments. Editor’s Note: FFS the transmitting priority between AMD PDUs for ARQ and AMD PDUs for autonomous retransmission.

Similar to the legacy procedure, the autonomous retransmission will have higher priority than transmission of not previously transmitted RLC SDUs or RLC SDU segments. For the autonomous retransmission and legacy retransmission, there is no significant benefit is seen when the priority among them is considered. It is suggested they will be transmitted in the order of SN of RLC SDU or segment.

Proposal 2: For the autonomous retransmission and legacy retransmission, they will be transmitted in the order SN of RLC SDU or segment, no further spec impact will be introduced.

One or multiple enhanced polling:

38.322 Running CR for RAN2#129 Post Email discussion Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple

For above FFS issue on polling, more remaining time is needed for receiving status reporting, so there is not so much time remained for multiple pollings when the remaining time is limited. Since there is only one time of autonomous retransmission, so, one enhanced polling is preferable, which is simple and sufficient in the congestion scenario.

Proposal 3:  Only a single enhanced polling will be triggered per RLC SDU.

The impact on RLF

In legacy RLC layer procedure, the following procedure is applied for RLF.  It can be seen that the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

5.3.2 Retransmission The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

Observation 1: In legacy procedure, the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

When the autonomous retransmission is introduced, the number of autonomous retransmission(s) could not be used to deduce that channel quality is bad enough to indicate the RLF to high layer, it is not necessary to increase the RETX_COUNT when autonomous retransmission is performed, there is no impact to spec.

Proposal 4: It is unnecessary to increase the RETX_COUNT when autonomous retransmission is performed, and there is no specification impact foreseen.

Avoid unnecessary retransmission

In RAN2#129 meeting, the agreements on avoiding unnecessary retransmission on RLC layer are listed as below.

Agreements on unnecessary RLC retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

Besides, one FFS issue in RAN2#129 PostEmail [512] is as follow:

[FFS - Detection of obsolescence of an AMD PDU: - The receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires.] NOTE 2: The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated. NOTE X: The expiry of t-RxDiscard triggers both RX_Next to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next is updated.

Based on above discussion in running CR, one issue is whether status report will be triggered after the t-RxDiscard is expiry. Considering the common understanding that Tx will advance its window based on status report from Rx side in avoiding unnecessary retransmission mechanism, once t-RxDiscard expires and Rx side updates the associated status variable, it is better to transmit the STATUS report with fake ACK for outdated data as soon as possible to make TX side move Tx window forward and stop retransmission of outdated RLC PDU or segment, which could avoid the window stalling and unnecessary retransmission of outdate RLC PDU or segment.

Proposal 5: The status report will be triggered by Rx side when t-RxDiscard expires.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Autonomous retransmission and/or polling will be triggered when the remaining time of a RLC SDU based on PDCP discard timer is lower than a threshold.

Proposal 2: For the autonomous retransmission and legacy retransmission, they will be transmitted in the order SN of RLC SDU or segment, no further spec impact will be introduced.

Proposal 3:  Only a single enhanced polling will be triggered per RLC SDU.

Observation 1: In legacy procedure, the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

Proposal 4: It is unnecessary to increase the RETX_COUNT when autonomous retransmission is performed, and there is no specification impact foreseen.

Proposal 5: The status report will be triggered by Rx side when t-RxDiscard expires.

Reference

- RAN2#129 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei.