3GPP TSG-RAN WG2 Meeting #129-bis	R2-2501878

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.11.2

Source:	Huawei, HiSilicon

Title:	Impacts on the random access by the evolution of duplex operation

Document for: Discussion and Decision

# 1	Introduction

In the RAN2#129 meeting, RAN2 achieved the following agreements about the impacts on random access operation by the evolution of duplex operation:

RACH configuration - When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. - When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. RACH procedure, RO selection criteria - For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. - If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. RACH procedure, RO selection before or after selecting a set of RA resources - FFS whether RO type selection is performed before or after the RA type selection. RACH procedure, fallback behaviour - FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In this contribution, we will further analyse the remaining RAN2 impacts to support random access in SBFD symbols.

# 2	Discussion

## 2.1 RACH events

In the RAN2#127 meeting, there was one working assumption on the appliable RACH events for SBFD resource. Considering that random access in SBFD symbols aims to reduce the random access latency, to reduce the PRACH collision probability and to improve the coverage of PRACH and Msg3 transmission, we do not see strong reason to drop any of the RACH events and all of them should benefit from the advantages of RACH operation in SBFD resource.

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

## 2.2 RACH resource selection

In the last meeting, RAN2 discussed the RACH resource selection issue, and achieved the following agreements:

- For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. - If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

The only remaining issue is how to use the SSB RSRP threshold when determine the RACH resource type. Considering that the SBFD RACH resource can be used for the following purposes: 1) reduce the latency for UE RACH procedure; 2) increase RACH capacity;; 3) improve the cell coverage; 4) improve uplink reception performance for especially border UEs. Reducing the CLI can be considered as one benefit for not using SBFD resource. Among these benefits, it is not feasible for UE to achieve them all at the same time. For example, using SBFD recourse can be beneficial for latency reduction for UEs at cell-center, it can cause higher CLI by cell-edge UEs using SBFD resource. If cell border UEs with lower RSRP measurement than a RSRP threshold are filtered to use conventional RO instead of SBFD resource, they cannot achieve coverage enhancement by using long PRACH sequences on the SBFD symbols.

As can be seen from above, the decision for the different UEs on using and not using SBFD resource needs to be done and it can be done by the gNB, as the gNB would have better knowledge than the UEs about the related information such as cell size, UE distribution, interference between cells etc. According to those information, the gNB can decide and signal to the UEs on how to compare the SSB RSRP against a threshold, i.e. the gNB indicates to use the SBFD RACH resource when the RSRP is higher than the SSB RSRP threshold or is lower than the SSB RSRP threshold. This decision making can be left to gNB implementation.

Proposal 2: Support the network indicates to use the SBFD RACH resource when the RSRP is higher than the SSB RSRP threshold or is lower than the SSB RSRP threshold. The decision making is left to gNB implementation.

## 2.3 Resource fall-back behaviour

Another aspect to discuss on resource selection is the UE fall-back behaviour. For example, after multiple times of RACH failures with resource in SBFD symbols, UE shall be allowed to switch from the PRACH resources in SBFD symbols to the PRACH resources in non-SBFD symbols at least. This direction of fallback behaviour has already be agreed, the opposite direction fallback, from using non-SBFD symbols to using SBFD symbols should also be supported. This is based on the consideration that using addition ROs can achieve coverage enhancement for cell-border UEs, as well as fallback to SBFD symbols if the previous RACH failures are caused by low capacity of non-SBFD resource.

Proposal 3: Support to switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols.

Regarding whether to reset the PRACH power ramping counter when switching from legacy-ROs to additional-ROs or vice versa, we think there is no need to reset, considering that the network would have already considered there may be RO type switching in one RACH procedure and have configured the suitable parameters of power ramping.

Proposal 4: No need to reset PRACH power ramping counter during RACH resource fall-back from RAN2 perspective.

## 2.4 Msg3 early indication

In the previous RAN2 meeting, Msg1 resource-based early identification was agreed. Whether Msg3 based EI is needed is to be further discussed. We think it is still useful for network to know the UE capability earlier, when Msg1 based EI is not used, so that network can schedule subsequent transmissions/receptions in SBFD symbols. For Msg3 early indication solution, the reserved LCID can be allocated to the SBFD aware UEs to indicate the supporting of SBFD, which will have no impact to legacy UE. As there are plenty of reserved LCID, the concern on the limited LCID would not be critical.

Proposal 5: Msg3 based early identification solution is supported, e.g., using reserved LCID to indicate supporting SBFD operation.

## 2.5 RACH-RNTI calculation

In the previous RAN2 meetings, some contributions mentioned the RA-RNTI calculation issue caused by possible overlapping SBFD RACH resource configuration.

In the RAN1#119 meeting, there was agreement about RO determination for the case that additional ROs overlap with legacy ROs:

Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

In the RAN1#120 meeting, RAN1 discussed this issue and agreed to not do any optimize:

Conclusion For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1: - Alt-1: do not optimize for this case

Based on the above RAN1 conclusion, we understand RAN2 does not need to enhance the calculation of RACH-RNTI, considering that the contention resolution mechanism of RACH procedure can deal with this issue anyway, if two same RA-RNTIs are used at the same time. In addition, from the perspective of the RAN2, if it is unclear whether the overlapping ROs is invalid for a UE, i.e. different UEs have different ideas about this situation, RAN2 cannot solve the RA-RNTI conflict issue without the involvement of RAN1.

Proposal 6: RAN2 do not optimize on the possible RA-RNTI conflict issue.

## 2.6 Order of RO selection and RA selection

In the last meeting, RAN2 discussed the order of RACH procedure as below:

RACH procedure, RO selection before or after selecting a set of RA resources - FFS whether RO type selection is performed before or after the RA type selection.

In the RAN1#120 meeting, RAN1 discussed this issue of whether to support 2 step RACH procedure using SBFD resource and achieved the following agreement:

Conclusion There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

Considering that RAN1 has agreed that SBFD for 2-step RACH is not supported, to avoid the impact on the current RA type selection procedure, we think the SBFD aware UE should first perform RA type selection first. If the UE first choose 4 step RACH procedure, the UE can further determine to use SBFD RACH resource or legacy RACH resource; if the UE choose 2-step RACH procedure first, the UE can only use legacy RACH resource. That is, there would be no point for UE to determine to use SBFD resource then to choose 2-step RA type.

Proposal 7: Support to select RO type after the RA type selection.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

RACH events

Proposal 1: Confirm the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events".

RACH resource selection

Proposal 2: Support the network indicates to use the SBFD RACH resource when the RSRP is higher than the SSB RSRP threshold or is lower than the SSB RSRP threshold. The decision making is left to gNB implementation.

Resource fall-back behaviour

Proposal 3: Support to switch from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols.

Proposal 4: No need to reset PRACH power ramping counter during RACH resource fall-back from RAN2 perspective.

Msg3 early indication

Proposal 5: Msg3 based early identification solution is supported, e.g., using reserved LCID to indicate supporting SBFD operation.

RACH-RNTI calculation

Proposal 6: RAN2 do not optimize on the possible RA-RNTI conflict issue.

Order of RO selection and RA selection

Proposal 7: Support to select RO type after the RA type selection.

# 4	Reference

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)